Source: 	Nokia Corporation

Title: 	Additional use case on multiple rooms for VR conferencing

Document for	Agreement

Agenda item: 	11.8

# Introduction

The ITT4RT Permanent Document [1] currently lists one core use case with two scenarios. In both scenarios, there is only a single source of 360-degree video. There has been some discussion on multiple sources of 360-degree video. This document combines some of these discussions to contribute a third scenario with multiple rooms and overlays. The scenario maintains unidirectional 360-degree video. The purpose of this document is to formally recognize these suggestions as core part of the ITT4RT use case.

# Use Case Description

In the proposed third scenario, multiple conference rooms are sending 360-degree video to an MRF/MCU. The rooms may choose to receive 2D video streams from other participants including one of the other rooms, which is displayed on the screen in the room. A pictorial representation is shown in Figure 1.

X

Figure 1: Multiple rooms with 360-degree video

Furthermore,

- The remote (single) users can choose to view any one or none (e.g., when only viewing the screenshare) of the available 360-degree videos from the multiple rooms. Switching from one room to another may be triggered manually, or using other mechanisms, such as, viewing direction or dominant speaker. However, for the case of the dominant speaker, it is important to consider effects of motion sickness while switching from one video to another as the user may not be sufficiently prepared for the switch.

- The MRF/MCU may signal to pause the receiving 360-degree video from any of the rooms that do not currently have any active viewers. The streams can be resumed if and when there are viewers [2].

- The presentation/screenshare stream is distributed to the single users and the rooms as a separate stream and can be identified using an explicit “a=content:slides” SDP field [3]. The single users may view the stream as an overlay on top of the 360-degree video.

- If the area of interest in the 360-degree video is limited (the preferred FoV is defined and negotiated between the participants) or the motion outside the FoV is limited, the background area (outside the viewport) can be transmitted as a still image to maintain continuity in the 360-degree view. The still-image may be updated at regular intervals or as needed.

- A still-image background area can also be used for placing overlays and avoiding unnecessary bandwidth utilization. In the scenario in the figure, this background area would be the wall with the screen and the presentation overlay may be placed on top of the screen for a better visual experience.

- Overlays may also be used for other 2D streams. These include videos from the single users and 2D video streams from the other rooms. Spatial audio should be associated with overlays when placed in the 360-degree view, i.e., the sound should appear to originate from the placement of the overlay, not necessarily associated with the elements within the video.

- Network-base Media Processing (NBMP) may be used for creating 360-degree views combining multiple input sources, e.g., 360-degree video with limited FoV, still image for background, overlays of screenshare and other 2D videos, etc., when the device capabilities are limited.

# Requirements

The above use case has the following additional requirements:

- The capability to place overlays in the 360-degree video either within the device or pre-rendered through a network element.

- Signalling to establish the format of the background area, especially if a limited FoV is used for immersive video content. For example, a still background image.

# Proposal

We propose that this scenario and the related requirements in sections 2 and 3 are included in the ITT4RT Permanent Document [1].

# References

[1] ITT4RT Permanent Document – Requirements, Working Assumptions and Potential Solutions, 3GPP TSG SA WG4 #104 meeting, Cork, Ireland, Tdoc S4-190779.

[2]	IETF RFC 7728 (2016): "RTP Stream Pause and Resume".

[3] 3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

Contact: Igor Curcio, Saba Ahsan, Nokia Technologies, Finland. Emails: igor.curcio, saba.ahsan@nokia.com.