Source:	Nokia Corporation

Title:	On IVAS audio formats and complexity design constraints

Document for:	Discussion and Agreement

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

Draft IVAS design constraints [1] do not set any specific design constraints on the complexity of the codec solution. It has been proposed to set the complexity constraints based on the complete IVAS solution that includes the IVAS renderer [2]. The source has furthermore proposed to set the complexity constraint such that the wide range of device types and capabilities are taken into account, e.g., by considering complexity constraints for individual audio input formats [3].

The IVAS codec is expected to significantly expand the opportunities for immersive services and advanced use cases over EVS. This is to be achieved, e.g., by support of a multitude of audio input formats suitable for different use cases and also some combinations of them. In general, it can be understood that the encoding complexity increases with additional input channels.

We discuss in this input document aspects to be considered in setting the complexity constraints for IVAS relating to input formats and number of supported channels. As with the previous contribution [3], it is not presently proposed any exact values for complexity limits. Rather this proposal focuses on the principle. Agreement on the principle should help to develop realistic and relevant complexity constraints that enable desired use cases and allow for wide deployment in the mobile space and other related products and applications.

# 2. Input formats and rendered output formats

In draft IVAS-4 [1], the support for the following input types is currently specified:

-	Channel-based audio, including mono (1.0), stereo (2.0), surround (5.1 and 7.1)

-	Binaural audio

-	Scene-based audio, first-order (FOA) ambisonics

-	Object-based audio (definition TBD)

Furthermore, the following input format – output format mapping (“rendered output formats”) has been agreed:

| Encoder Input Format | Rendered Output Format |
| --- | --- |
| Multi-channel 7.1 | Multi-channel 7.1, Binaural audio, Stereo, Mono. |
| Multi-channel 5.1 | Multi-channel 5.1, Binaural audio, Stereo, Mono. |
| Binaural audio | Binaural Audio |
| Stereo | Stereo, Mono |
| Mono | Mono |

Currently, the highest agreed number of channels both at the codec input and output can thus be considered as eight channels. This corresponds to channel-based input and multi-channel output in a 7.1 channel configuration.

# 3. Number of channels and solution complexity

Number of channels supported at the codec input sets some baseline level for the IVAS solution complexity. In general, it can be understood that the amount of processing including allocation and handling of buffers increases with each additional input channel. The same is true for the codec output. The outputs of interest may also to some degree depend on the input formats (as already seen in the input – output mapping agreed for the rendered output formats). In addition, via provision of external rendering interface, the IVAS codec enables more complex renderings where suitable. We note that the requirements for the external rendering interface remain TBD.

It is also noted that there may be other practical reasons to limit the number of concurrent audio input channels. For example, in [3] it is specifically discussed to have a limit of eight channels due to operating system limits on certain mobile phone platforms.

Our view is that this connection between number of channels and the overall solution complexity needs to be considered should there otherwise be clear justification for increasing the number of channels supported. In particular, this relates to the codec input. At least for some input signal types, any higher rendering complexity (e.g., number of output channels for a more complex multi-channel system) might be sufficiently covered by external rendering.

We note that there have previously been even numerical proposals [5] to tie the number of input channels with the complexity design constraint.

# 4. Combinations of input audio formats

The IVAS codec is expected to provide support for combinations of input audio formats. This definition is presently TBD and captured in IVAS-4 [1] as follows:

[In addition, the IVAS codec shall support combinations of the above, totalling to no more than [TBD] audio streams.

Note: It will be necessary to specify how capture/presentations could be achieved in mobile communications.]

This definition also indicates some upper limit of audio streams, “totalling to no more than [TBD] audio streams”. We think this upper limit also needs to be considered relative to the complexity constraint.

It is still to be defined what combinations of input audio formats will need to be supported in the IVAS solution. At least it can be expected that object-based audio is to be supported in combination with some other input audio formats. For IVAS, we believe the focus should be on conversational use cases. Based on good use case justification, some other combinations can also be considered. However, we note that the upper limit of audio streams may preclude some combinations.

In general, it seems useful based on current understanding to allow object-based audio in combination with any other input format (agreed or still under consideration) provided that the total channel limit is not exceeded. Specific example use cases for intended capture and representation may however prove useful.

# 5. Complexity design constraint

In [3], the source proposed to specify the complexity constraint not only for the worst-case complexity of the solution. In addition, it was proposed to take into account the envisioned IVAS device types and their capability levels. One example of this approach is to specify a (weighted) average worst-case complexity figure across the high-level audio input format categories in addition to the maximum worst-case complexity.

## 5.1 Discussion

The motivation for the proposed principle is to avoid an IVAS solution with unnecessarily high complexity at operation points that can be considered less computationally demanding than the expected worst-case operation. For example, an IVAS device with only stereo microphone inputs is likely to mostly operate in a stereo encoding mode. If this simpler operation is wasteful, e.g., in terms of processor cycles, an unnecessary hit on the device battery life is experienced.

Currently, there is agreed support for eight input channels (7.1 channel-based) and IVAS rendering into eight channels (7.1 loudspeaker configuration). This already presents a significantly higher potential complexity than at least some of other input formats such as stereo, binaural, FOA, MASA, etc. If there is good justification and the complexity is not expected to become too high, it may be agreed to increase the number of supported channels beyond eight. For example, for a simple stereo device the implications on battery life could then be highly objectionable if there is only a single constraint on computational complexity.

Thus, it seems important to have more than a single value for the complexity constraint.

The initial proposal by the source could be simplified by considering equal weights for the input categories. In addition, it would still need to be decided what are the high-level categories to consider for the average. We see two alternatives. The first is to list the key input categories separately (e.g., separate stereo from other channel-based audio). The second alternative is to consider the lower and higher end of operation for each input type, e.g., such that for channel-based audio it would be considered complexity for stereo (2.0) and the highest end of operation (currently, 7.1). A combination of these two values would be used for the averaging.

In our current view, combinations of input formats can be considered for the worst-case complexity only at least until there is more clarity on what these combinations will be.

## 5.2 Proposal

According to the proposal, the IVAS solution is required to meet two Figures of Complexity (FoC) for computational complexity. These are:

- FoCTOT: Total worst-case complexity of the solution (highest end-to-end complexity across any input-output combination using the IVAS renderer).

- FoCAVE: Average input-output complexity of the solution (using the IVAS renderer).

Based on currently agreed input audio formats and, for the sake of example, by extending the agreed input format – output format mapping slightly, the average input-output complexity could thus be defined, e.g., as:

FoCAVE=15C2.0→2.0+Cbin→bin+maxC7.1→bin,C7.1→7.1+maxCFOA→bin,CFOA→7.1+maxCOBA→bin,COBA→7.1⁡ ,

where by Cxy we denote complexity of input x with rendering y. Thus, in this example it is for stereo input (2.0) considered stereo (2.0) rendering and for binaural input binaural rendering only. For other agreed input format types, it is considered the maximum complexity of binaural or 7.1 multi-channel rendering.

Mono input could be included, especially if some up-mixing for rendering is considered.

The equation above is one example of the principle of covering a multitude of operation points using a limited set of constraints and not part of the proposal as such. As previously discussed in the SA4#104 Cork meeting, this type of approach does not fully guarantee that a low complexity for a specific operation point is achieved. However, it does penalize a solution that is unnecessarily wasteful in terms of computational complexity at any key operating point, and therefore guides the codec proponents to optimize complexity throughout their respective solutions. We believe this goal is important.

Additionally, it could still be considered the solution complexity for non-rendered outputs. While an external rendering may be mostly expected in case of relaxed complexity constraints, it should be understood what the benefits are in potentially setting a lower complexity constraint value for decoder output(s) without this internal rendering step. For example, it could be foreseen headphone devices where the final rendering is carried out locally in order to minimize the effects of head-tracking related latencies. Particularly freely rotatable representations such as ambisonics, MASA, objects, and their combinations could thus be rendered for binaural presentation based on a corresponding decoder output. It could be considered this aspect also in the definition of the average input-output complexity.

# 6. Summary and Proposal

In the present document, we discussed some aspects relating to the complexity constraint for the IVAS codec. In general, the principle of considering the end-to-end complexity including encoder and decoder/renderer could be applied. Furthermore, in defining any relevant computational complexity constraint for the IVAS codec, it should be taken into account the extensive scope of the intended use cases as well as the multitude of device types and their capability levels. Thus, while it is the understanding of the source that there is no profiling of the codec, we foresee that certain devices will support only a subset of the IVAS input audio formats in their operation. It is further noted that the practical worst-case complexity is connected to the audio input and output formats (e.g., number of channels to be supported).

The source proposes to specify complexity constraint not only for the worst-case complexity of the solution. In addition, it is proposed to apply the principle of specifying complexity constraint that takes into account the envisioned device types and capability levels. A specific proposal to apply this two-fold principle is provided in clause 5.2. We respectfully request an agreement on said principle.

# References

[1]	S4-190839: IVAS Design Constraints (IVAS-4), v0.1.0.

[2]	AHEVS-472: “On delay and complexity design constraints for IVAS”, source: Orange.

[3]	S4-190694: “On IVAS complexity constraint”, source: Nokia Corporation

[4]	S4-180466: “On capture formats for IVAS”, source: Qualcomm Incorporated

[5]	S4-170939: “Considerations on Design Constraints and Performance Requirements for IVAS Codec”, source: Dolby Laboratories, Inc.