Source:		Qualcomm Incorporated

Title:	Status of the HOA Transport Format (HTF)

Agenda Item:	7.5

Document for:	Discussion and Agreement

# 1. Introduction

At the SA4#98 meeting the source proposed a subset of the HOA Transport Format (HTF) as specified in [3] as a capturing format for IVAS [1]. Consecutive input contributions provided additional information and example software to the group. This contribution provides a status update on HTF and requests the inclusion of HTF into IVAS-4 [8].

# 2. Points of Discussion

The SA4 contributions [4] and [5] provided valuable feedback on the HTF proposal. The following section summarizes how the source has addressed the various questions.

# Verification of potential quality impacts

# It would be important to verify the quality impacts of downmixing HOA to up to 8 channels. With the HTF format, intrinsic quality may be degraded for various types of audio scenes and this impact has not been validated in 3GPP.

# To validate the quality impact of various types of audio scenes the source conducted a formal listening test for SA4#100. This test demonstrates the quality impact of a HOA conversion to HTF with different number of transport channels. HTF with six, eight, and ten transport channels were evaluated in a MUSHRA test. The listening test results presented indicate that HTF with 8 or more transport channels consistently provides excellent service quality across various types of audio scenes. The quality with 6 transport channels was in the Good to Excellent MUSHRA range. For more details on the subjective listening test see [2].

# Availability of reference processing stage to convert from HOA to HTF

# For the sake of fairness with all IVAS proponents, it would be required to provide a complete implementation of HTF preprocessing and synthesis stages, with sufficient time to evaluate this technology. The source notes that for the MASA proposal an example implementation has been made available.

At the time of the proposal in April 2018 [1] the ETSI specification has just been finalized and it is reasonable that some SA4 experts had reservations to accept this specification without sufficient time to get familiarized with HTF.

As of now, the HTF specification has been freely available [3] since more than a year without further modifications. HTF is mature and stable and there has now been plenty of time to study the specification.

# Also, the source has developed HTF example software for Matlab/Octave [7] which is freely available upon request.

# With respect to the scene-based and also often channel-based content, it is understood that pre-processing (or pre-mixing) is required to generate the desired audio content. These pre-processing techniques are often device-specific and/or artistically motivated. For example:

- No practical microphone array can natively record in Ambisonics. The Ambisonics signals needs to be generated using a device-specific A-to-B format processing.

- A stereo signal can be captured in many different ways. With the M/S recording technique a specific pre-processing converts M/S into stereophonic.

- Surround and immersive audio capture using microphone trees requires a sensible mixing of the microphone feeds to the target loudspeaker feeds (see e.g., Hamasaki square, Double A-B, Fukada Tree, …).

# Because the codec performance is independent from the pre-processing performance, specifics of the pre-processing can be opaque to the audio codec. Compared to the examples above, the device-specific HTF pre-processor is also located prior the audio codec. Because the codec performance is independent from the pre-processing performance, details of the HTF pre-processing can be opaque to the audio codec. Based on this concept, a methodology for evaluating the performance of IVAS codec candidates for HTF-based input content is proposed in [6]. This evaluation methodology uses a common HOA renderer and decouples the HTF preprocessing from the IVAS codec candidate evaluation. Consequently, the IVAS candidate evaluation process is fair for all proponents.

# Relationship to scene-based audio

# IVAS-4 already includes "scene-based audio, first-order (FOA) and up to [N]-order ambisonics". It would be important to clarify how two very similar input formats would interact.

# The HOA Transport Format provides a compact representation of scene-based audio and is therefore closely related. The HTF proposal supports ACN component ordering and SN3D normalization scheme as already specified for scene-based audio in IVAS-4 [8].

# HOA content can be truncated (e.g., to FOA) to reduce the number of components. To achieve a truncation of the resulting HOA order in the HTF domain, it is possible to zero the upper parts of the metadata vectors.

# Mixed content such as HOA plus some objects can be of interest for certain use cases, consequently HTF plus objects is similarly plausible and could be supported too. In contrast, a simultaneous input of HOA and HTF seems to be less relevant. However, a codec solution could simulate the interaction of HOA and HTF input sound fields, e.g., by superposition of HOA and HTF (after generating HOA from HTF).

# Use case

# The HTF format was proposed based on a potential limitation of the number of inputs formats. Currently in IVAS-4 the HOA order N is not yet decided, however by the time IVAS gets standardized and deployed, the justification for HTF may not be clear. The HOA orders required for optimal quality of experience in mobile applications should be studied in SA4 to draw conclusions on HTF.

# Current audio channel limitations by certain popular mobile operating systems are a given fact. The source is not aware that this channel limitation will be relaxed in future OS releases. Based on the current state, even the capture of second-order Ambisonics is impossible.

# Further, as stated in the initial proposal [1], the motivation for supporting HTF in IVAS is to allow for an overall lean system design for capturing and encoding of ambisonic-based audio content with the goal to optimize for complexity, memory, latency as well as service quality.

# The question of the maximum HOA order should be discussed independently from the HTF proposal because HTF can support any practically high HOA order. TR26.918 already provides subjective test results for HOA orders.

# 3. Summary and Proposal

# With several different input contributions, the source addressed various feedback on the application of HTF as an efficient capturing format for IVAS.

Consequently, the source reiterates its proposal for considering HTF type 3 [3] in IVAS-4 [8] as a spatial audio format that comprises audio channels plus spatial metadata:

| Encoder Input Formats | The encoder shall support the following input formats: - Channel-based audio, including mono (1.0), stereo (2.0), surround (5.1 and 7.1), [surround + height (5.1+4 and 7.1+4), TBD] - Binaural audio - Scene-based audio, first-order (FOA) and up to [N]-order ambisonics. Note: ACN component ordering and SN3D normalization. - [Spatial audio, [N] channels and spatial metadata defined by [HTF (ETSI TS 103 589, type 3) and TBD].] [Editor’s Note FFS: Spatial metadata definition for the spatial audio format will require further input.] - Object-based audio, with support for at least [TBD] individual [mono] object streams. Each audio object shall be defined by [TBD metadata parameters]. [In addition, the IVAS codec shall support combinations of the above, totalling to no more than [TBD] audio streams. Note: It will be necessary to specify how capture/presentations could be achieved in mobile communications.] |
| --- | --- |

# References

- S4-180466, On capture formats for IVAS, SA4#98 Kista, 2018.

- S4-181046, Subjective Test Result of HOA Transport Format, SA4#100 Kochi, 2018.

- ETSI TS 103 589 V1.1.1, Higher Order Ambisonics (HOA) Transport Format, 2018. https://portal.etsi.org/webapp/workProgram/Report_WorkItem.asp?wki_id=53690

- S4-181375, On the HTF format, SA4#101 Busan, 2018

- S4-181126, Comments on the proposed HTF format, SA4#100 Kochi, 2018.

- S4-190370, Discussion on Input Format Testing, SA4#103, Newport Beach, 2019.

- S4-190547, Availability of HTF Example Software, SA4#103, Newport Beach, 2019.

- S4-190839, IVAS Design Constraints (IVAS-4) v0.1.0, SA4#104