3GPP TSG-SA4 Meeting # 105	S4-191053

# Ljubljana, Slovenia, 12 – 16 August 2019									Revision of S4-190974

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.131 | CR | 0079 | rev | 1 | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of WLAN delay requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EXT_UED |  |  |  |  |  | Date: |  |  | 12/08/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Delay-related table captions only refer to LTE which could be misleading. |  |  |  |  |  |  |  |  |
| Summary of change: |  | WLAN is added to delay-related table captions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous requirements with a mismatch between table captions and the text in the main body of the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.12.1, 5.12.2.1, 6.11.1, 6.11.2.1, 7.11.1, 7.11.2.1, 8.11.1, 8.11.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | There is no mirror CR in Rel-16. A corresponding change is implemented in S4-190836, which is a Rel-16 CR to 26.131 (with a different work item code and a wider scope/amount of changes) that was agreed at SA#104. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.12.1	Handset UE

It is in general desirable to minimize UE delays to ensure low enough end-to-end delays and hence a good conversational experience, guidance is found in ITU-T Recommendation G.114.

For UMTS circuit-switched AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 220ms and should be ≤ 185ms.

NOTE: A delay ≤ 185 ms might not be achievable in some cases due to UE implementation trade-offs between delay and other parameters such as speech quality enhancement, performance of noise reduction or UE power consumption optimization, and UE implementation issues such as rebuffering between components.

For MTSI-based speech-only with LTE or WLAN access in error and jitter free conditions and AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) should be ≤ 150ms. If this performance objective cannot be met, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 190ms.

For MTSI-based speech-only with LTE or WLAN access in conditions with simulated packet arrival time variations and packet loss and AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall be less than or equal to the delay requirements in Table 8bis, while meeting the speech quality targets defined.

NOTE: The UE delay requirements for MTSI-based speech-only with LTE or WLAN access are derived from:

-	A speech frame buffering and codec look-ahead of 25ms.

-	An air interface transmission time of 1ms on receive and 1ms on the send direction.

-	A budget allowance for a jitter buffer depth of 40ms for error and jitter free conditions and test conditions 0 and 1 of Table 8bis, and 80ms for test condition 2 of Table 8bis.

-	A budget allowance for vendor specific implementation of 83ms corresponding to the performance objective and 123ms corresponding to the required maximum UE send and receive delay.

Table 8bis: UE delay and speech quality requirements for LTE and WLAN access

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 150ms | TS + TR ≤ 190ms | No requirement, reference score MOS-LQOREF |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ 150ms | TS + TR ≤ 190ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.3 |
| 2 | dly_profile_40msDRX_10pct_BLER_e2e | TS + TR ≤ 190ms | TS + TR ≤ 230ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.3 |
|  | NOTE 1: The delay profiles for test condition 1 and 2 are theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and target BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. Delay profiles are injected at the IP layer of the test system. Delay profiles are attached electronically to document 3GPP TS 26.132 [1]. The delay profiles in test condition 1 and 2 are static delay variation conditions and do not expose the UE to packet delay variations in the full range of the packet delay budget as defined for QCI1 in 3GPP TS 23.203 [18]. A third test condition that exposes the UE to non-stationary packet delay variations experienced in live operation and packet delay variations in the full range of the packet delay budget for QCI1, and accompanied delay and speech quality requirements, is for further study. |  |  |  |
|  | NOTE 2: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132.

#### 5.12.2.1	Wired headset

It is in general desirable to minimize UE delays to ensure low enough end-to-end delays and hence a good conversational experience, guidance is found in ITU-T Recommendation G.114.

For UMTS circuit-switched AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 220ms and should be ≤ 185ms.

NOTE: A delay ≤ 185 ms might not be achievable in some cases due to UE implementation trade-offs between delay and other parameters such as speech quality enhancement, performance of noise reduction or UE power consumption optimization, and UE implementation issues such as rebuffering between components.

For MTSI-based speech-only with LTE or WLAN access in error and jitter free conditions and AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) should be ≤ 150ms. If this performance objective cannot be met, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 190ms.

For MTSI-based speech-only with LTE or WLAN access in conditions with simulated packet arrival time variations and packet loss and AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall be less than or equal to the delay requirements in Table 8ter, while meeting the speech quality targets defined.

NOTE: The UE delay requirements for MTSI-based speech-only with LTE or WLAN access are derived from:

-	A speech frame buffering and codec look-ahead of 25ms.

-	An air interface transmission time of 1ms on receive and 1ms on the send direction.

-	A budget allowance for a jitter buffer depth of 40ms for error and jitter free conditions and test conditions 0 and 1 of Table 8ter, and 80ms for test condition 2 of Table 8ter.

-	A budget allowance for vendor specific implementation of 83ms corresponding to the performance objective and 123ms corresponding to the required maximum UE send and receive delay.

Table 8ter: UE delay and speech quality requirements for LTE and WLAN access

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 150ms | TS + TR ≤ 190ms | No requirement, reference score MOS-LQOREF |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ 150ms | TS + TR ≤ 190ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.3 |
| 2 | dly_profile_40msDRX_10pct_BLER_e2e | TS + TR ≤ 190ms | TS + TR ≤ 230ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.3 |
|  | NOTE 1: The delay profiles for test condition 1 and 2 are theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and target BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. Delay profiles are injected at the IP layer of the test system. Delay profiles are attached electronically to document 3GPP TS 26.132 [1]. The delay profiles in test condition 1 and 2 are static delay variation conditions and do not expose the UE to packet delay variations in the full range of the packet delay budget as defined for QCI1 in 3GPP TS 23.203 [18]. A third test condition that exposes the UE to non-stationary packet delay variations experienced in live operation and packet delay variations in the full range of the packet delay budget for QCI1, and accompanied delay and speech quality requirements, is for further study. |  |  |  |
|  | NOTE 2: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132.

### 6.11.1	Handset UE

It is in general desirable to minimize UE delays to ensure low enough end-to-end delays and hence a good conversational experience, guidance is found in ITU-T Recommendation G.114.

For UMTS circuit-switched AMR-WB speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 220ms and should be ≤ 185ms.

NOTE: A delay ≤ 185 ms might not be achievable in some cases due to UE implementation trade-offs between delay and other parameters such as speech quality enhancement, performance of noise reduction or UE power consumption optimization, and UE implementation issues such as rebuffering between components.

For MTSI-based speech-only with LTE or WLAN access in error and jitter free conditions and AMR-WB speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) should be ≤ 150ms. If this performance objective cannot be met, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 190ms.

For MTSI-based speech-only with LTE or WLAN access in conditions with simulated packet arrival time variations and packet loss and AMR-WB speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall be less than or equal to the delay requirements in Table 16a1, while meeting the speech quality targets defined.

NOTE: The UE delay requirements for MTSI-based speech-only with LTE or WLAN access are derived from:

-	A speech frame buffering and codec look-ahead of 25ms.

-	An air interface transmission time of 1ms on receive and 1ms on the send direction.

-	A budget allowance for a jitter buffer depth of 40ms for error and jitter free conditions and test conditions 0 and 1 of Table 16a1, and 80ms for test condition 2 of Table 16a1.

-	A budget allowance for vendor specific implementation of 83ms corresponding to the performance objective and 123ms corresponding to the required maximum UE send and receive delay.

Table 16a1: UE delay and speech quality requirements for LTE and WLAN access

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 150ms | TS + TR ≤ 190ms | No requirement, reference score MOS-LQOREF |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ 150ms | TS + TR ≤ 190ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.3 |
| 2 | dly_profile_40msDRX_10pct_BLER_e2e | TS + TR ≤ 190ms | TS + TR ≤ 230ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.3 |
|  | NOTE 1: The delay profiles for test condition 1 and 2 are theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and target BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. Delay profiles are injected at the IP layer of the test system. Delay profiles are attached electronically to document 3GPP TS 26.132 [1]. The delay profiles in test condition 1 and 2 are static delay variation conditions and do not expose the UE to packet delay variations in the full range of the packet delay budget as defined for QCI1 in 3GPP TS 23.203 [18]. A third test condition that exposes the UE to non-stationary packet delay variations experienced in live operation and packet delay variations in the full range of the packet delay budget for QCI1, and accompanied delay and speech quality requirements, is for further study. |  |  |  |
|  | NOTE 2: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132.

#### 6.11.2.1	Wired headset

It is in general desirable to minimize UE delays to ensure low enough end-to-end delays and hence a good conversational experience, guidance is found in ITU-T Recommendation G.114.

For UMTS circuit-switched AMR-WB speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 220ms and should be ≤ 185ms.

NOTE: A delay ≤ 185 ms might not be achievable in some cases due to UE implementation trade-offs between delay and other parameters such as speech quality enhancement, performance of noise reduction or UE power consumption optimization, and UE implementation issues such as rebuffering between components.

For MTSI-based speech-only with LTE or WLAN access in error and jitter free conditions and AMR-WB speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) should be ≤ 150ms. If this performance objective cannot be met, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 190ms.

For MTSI-based speech-only with LTE or WLAN access in conditions with simulated packet arrival time variations and packet loss and AMR-WB speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall be less than or equal to the delay requirements in Table 16a2, while meeting the speech quality targets defined.

NOTE: The UE delay requirements for MTSI-based speech-only with LTE or WLAN access are derived from:

-	A speech frame buffering and codec look-ahead of 25ms.

-	An air interface transmission time of 1ms on receive and 1ms on the send direction.

-	A budget allowance for a jitter buffer depth of 40ms for error and jitter free conditions and test conditions 0 and 1 of Table 16a2, and 80ms for test condition 2 of Table 16a2.

-	A budget allowance for vendor specific implementation of 83ms corresponding to the performance objective and 123ms corresponding to the required maximum UE send and receive delay.

Table 16a2: UE delay and speech quality requirements for LTE and WLAN access

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 150ms | TS + TR ≤ 190ms | No requirement, reference score MOS-LQOREF |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ 150ms | TS + TR ≤ 190ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.3 |
| 2 | dly_profile_40msDRX_10pct_BLER_e2e | TS + TR ≤ 190ms | TS + TR ≤ 230ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.3 |
|  | NOTE 1: The delay profiles for test condition 1 and 2 are theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and target BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. Delay profiles are injected at the IP layer of the test system. Delay profiles are attached electronically to document 3GPP TS 26.132 [1]. The delay profiles in test condition 1 and 2 are static delay variation conditions and do not expose the UE to packet delay variations in the full range of the packet delay budget as defined for QCI1 in 3GPP TS 23.203 [18]. A third test condition that exposes the UE to non-stationary packet delay variations experienced in live operation and packet delay variations in the full range of the packet delay budget for QCI1, and accompanied delay and speech quality requirements, is for further study. |  |  |  |
|  | NOTE 2: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132.

### 7.11.1	Handset UE

It is in general desirable to minimize UE delays to ensure low enough end-to-end delays and hence a good conversational experience, guidance is found in ITU-T Recommendation G.114.

For MTSI-based speech-only with LTE or WLAN access in error and jitter free conditions and EVS speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) should be ≤ 157ms. If this performance objective cannot be met, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 197ms.

For MTSI-based speech-only with LTE or WLAN access in conditions with simulated packet arrival time variations and packet loss and EVS speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall be less than or equal to the delay requirements in Table 32a1, while meeting the speech quality targets defined.

NOTE: The UE delay requirements for MTSI-based speech-only with LTE or WLAN access are derived from:

-	A codec algorithmic delay of 32ms including speech frame buffering and encoder lookahead.

-	An air interface transmission time of 1ms on receive and 1ms on the send direction.

-	A budget allowance for a jitter buffer depth of 40ms for error and jitter free conditions and test conditions 0 and 1 of Table 32a1, and 80ms for test condition 2 and 3 of Table 32a1. This budget is also applied in Table 32b1 (when testing in EVS 13.2 kbit/s channel-aware mode).

-	A budget allowance for vendor specific implementation of 83ms corresponding to the performance objective and 123ms corresponding to the required maximum UE send and receive delay.

Table 32a1: UE delay and speech quality requirements for LTE and WLAN access

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 157ms | TS + TR ≤ 197ms | No requirement, reference score MOS-LQOREF |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ 157ms | TS + TR ≤ 197ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.4 |
| 2 | dly_profile_40msDRX_10pct_BLER_e2e | TS + TR ≤ 197ms | TS + TR ≤ 237ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.4 |
| 3 | dly_profile_40msDRX_22pct_BLER_e2e | TS + TR ≤ 197ms | TS + TR ≤ 237ms | MOS-LQOTEST ≥ MOS-LQOREF – 1.0 |
|  | NOTE 1: The delay profiles for test condition 1 and 2 are theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and initial BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. The delay profile for test condition 3 is theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and initial BLER in sending and receiving directions of 22%, with +/- 6ms of EPC jitter. Delay profiles are injected at the IP layer of the test system. Delay profiles are attached electronically to document 3GPP TS 26.132 [1]. |  |  |  |
|  | NOTE 2: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132. When tested in EVS 13.2kbit/s channel aware mode, the UE shall meet the additional requirements in Table 32b1:

Table 32b1: UE delay and speech quality requirements for LTE and WLAN access when tested in EVS 13.2kbit/s channel aware mode

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 157ms | TS + TR ≤ 197ms | No requirement, reference score MOS-LQOREF |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ [157]ms | TS + TR ≤ 197ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.3 |
| 2 | dly_profile_40msDRX_10pct_BLER_e2e | TS + TR ≤ [197]ms | TS + TR ≤ 237ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.3 |
| 3 | dly_profile_40msDRX_22pct_BLER_e2e | TS + TR ≤ [197]ms | TS + TR ≤ 237ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.5 |
|  | NOTE 1: The delay profiles for test condition 1 and 2 are theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and initial BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. The delay profiles for test condition 3 is theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and initial BLER in sending and receiving directions of 22%, with +/- 6ms of EPC jitter. Delay profiles are injected at the IP layer of the test system. Delay profiles are attached electronically to document 3GPP TS 26.132 [1]. |  |  |  |
|  | NOTE 2: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132.

#### 7.11.2.1	Wired headset

It is in general desirable to minimize UE delays to ensure low enough end-to-end delays and hence a good conversational experience, guidance is found in ITU-T Recommendation G.114.

For MTSI-based speech-only with LTE or WLAN access in error and jitter free conditions and EVS speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) should be ≤ 157ms. If this performance objective cannot be met, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 197ms.

For MTSI-based speech-only with LTE or WLAN access in conditions with simulated packet arrival time variations and packet loss and EVS speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall be less than or equal to the delay requirements in Table 32a2, while meeting the speech quality targets defined.

NOTE: The UE delay requirements for MTSI-based speech-only with LTE or WLAN access are derived from:

-	A codec algorithmic delay of 32ms including speech frame buffering and encoder lookahead.

-	An air interface transmission time of 1ms on receive and 1ms on the send direction.

-	A budget allowance for a jitter buffer depth of 40ms for error and jitter free conditions and test conditions 0 and 1 of Table 32a2, and 80ms for test condition 2 and 3 of Table 32a2. This budget is also applied in Table 32b2 (when testing in EVS 13.2 kbit/s channel-aware mode).

-	A budget allowance for vendor specific implementation of 83ms corresponding to the performance objective and 123ms corresponding to the required maximum UE send and receive delay.

Table 32a2: UE delay and speech quality requirements for LTE and WLAN access

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 157ms | TS + TR ≤ 197ms | No requirement, reference score MOS-LQOREF |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ 157ms | TS + TR ≤ 197ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.4 |
| 2 | dly_profile_40msDRX_10pct_BLER_e2e | TS + TR ≤ 197ms | TS + TR ≤ 237ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.4 |
| 3 | dly_profile_40msDRX_22pct_BLER_e2e | TS + TR ≤ 197ms | TS + TR ≤ 237ms | MOS-LQOTEST ≥ MOS-LQOREF – 1.0 |
|  | NOTE 1: The delay profiles for test condition 1 and 2 are theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and initial BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. The delay profile for test condition 3 is theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and initial BLER in sending and receiving directions of 22%, with +/- 6ms of EPC jitter. Delay profiles are injected at the IP layer of the test system. Delay profiles are attached electronically to document 3GPP TS 26.132 [1]. |  |  |  |
|  | NOTE 2: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132. When tested in EVS 13.2kbit/s channel aware mode, the UE shall meet the additional requirements in Table 32b2.

Table 32b2: UE delay and speech quality requirements for LTE and WLAN access when tested in EVS 13.2kbit/s channel aware mode

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 157ms | TS + TR ≤ 197ms | No requirement, reference score MOS-LQOREF |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ 157ms | TS + TR ≤ 197ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.3 |
| 2 | dly_profile_40msDRX_10pct_BLER_e2e | TS + TR ≤ 197ms | TS + TR ≤ 237ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.3 |
| 3 | dly_profile_40msDRX_22pct_BLER_e2e | TS + TR ≤ 197ms | TS + TR ≤ 237ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.5 |
|  | NOTE 1: The delay profiles for test condition 1 and 2 are theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and initial BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. The delay profiles for test condition 3 is theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and initial BLER in sending and receiving directions of 22%, with +/- 6ms of EPC jitter. Delay profiles are injected at the IP layer of the test system. Delay profiles are attached electronically to document 3GPP TS 26.132 [1]. |  |  |  |
|  | NOTE 2: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132.

### 8.11.1	Handset UE

It is in general desirable to minimize UE delays to ensure low enough end-to-end delays and hence a good conversational experience, guidance is found in ITU-T Recommendation G.114.

For MTSI-based speech-only with LTE or WLAN access in error and jitter free conditions and EVS speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) should be ≤ 157ms. If this performance objective cannot be met, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 197ms.

For MTSI-based speech-only with LTE or WLAN access in conditions with simulated packet arrival time variations and packet loss and EVS speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall be less than or equal to the delay requirements in Table 33a1, while meeting the speech quality targets defined.

NOTE: The UE delay requirements for MTSI-based speech-only with LTE or WLAN access are derived from:

-	A codec algorithmic delay of 32ms including speech frame buffering and encoder lookahead.

-	An air interface transmission time of 1ms on receive and 1ms on the send direction.

-	A budget allowance for a jitter buffer depth of 40ms for error and jitter free conditions and test conditions 0 and 1 of Table 33a1, and 80ms for test condition 2 of Table 33a1.

-	A budget allowance for vendor specific implementation of 83ms corresponding to the performance objective and 123ms corresponding to the required maximum UE send and receive delay.

Table 33a1: UE delay and speech quality requirements for LTE and WLAN access

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 157ms | TS + TR ≤ 197ms | ffs |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ 157ms | TS + TR ≤ 197ms | ffs |
| 2 | dly_profile_40msDRX_10pct_BLER_e2e | TS + TR ≤ 197ms | TS + TR ≤ 237ms | ffs |
|  | NOTE 1: The delay profiles for test condition 1 and 2 are theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and target BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. Delay profiles are injected at the IP layer of the test system. Delay profiles are attached electronically to document 3GPP TS 26.132 [1]. The delay profiles in test condition 1 and 2 are static delay variation conditions and do not expose the UE to packet delay variations in the full range of the packet delay budget as defined for QCI1 in 3GPP TS 23.203 [18]. A third test condition that exposes the UE to non-stationary packet delay variations experienced in live operation and packet delay variations in the full range of the packet delay budget for QCI1, and accompanied delay and speech quality requirements, is for further study. |  |  |  |
|  | NOTE 2: P.863 is limited to 14 kHz bandwidth; therefore the speech quality requirements are ffs. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132.

#### 8.11.2.1	Wired headset

It is in general desirable to minimize UE delays to ensure low enough end-to-end delays and hence a good conversational experience, guidance is found in ITU-T Recommendation G.114.

For MTSI-based speech-only with LTE or WLAN access in error and jitter free conditions and EVS speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) should be ≤ 157ms. If this performance objective cannot be met, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 197ms.

For MTSI-based speech-only with LTE or WLAN access in conditions with simulated packet arrival time variations and packet loss and EVS speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall be less than or equal to the delay requirements in Table 33a2, while meeting the speech quality targets defined.

NOTE: The UE delay requirements for MTSI-based speech-only with LTE or WLAN access are derived from:

-	A codec algorithmic delay of 32ms including speech frame buffering and encoder lookahead.

-	An air interface transmission time of 1ms on receive and 1ms on the send direction.

-	A budget allowance for a jitter buffer depth of 40ms for error and jitter free conditions and test conditions 0 and 1 of Table 33a2, and 80ms for test condition 2 of Table 33a2.

-	A budget allowance for vendor specific implementation of 83ms corresponding to the performance objective and 123ms corresponding to the required maximum UE send and receive delay.

Table 33a2: UE delay and speech quality requirements for LTE and WLAN access

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 157ms | TS + TR ≤ 197ms | ffs |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ 157ms | TS + TR ≤ 197ms | ffs |
| 2 | dly_profile_40msDRX_10pct_BLER_e2e | TS + TR ≤ 197ms | TS + TR ≤ 237ms | ffs |
|  | NOTE 1: The delay profiles for test condition 1 and 2 are theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and target BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. Delay profiles are injected at the IP layer of the test system. Delay profiles are attached electronically to document 3GPP TS 26.132 [1]. The delay profiles in test condition 1 and 2 are static delay variation conditions and do not expose the UE to packet delay variations in the full range of the packet delay budget as defined for QCI1 in 3GPP TS 23.203 [18]. A third test condition that exposes the UE to non-stationary packet delay variations experienced in live operation and packet delay variations in the full range of the packet delay budget for QCI1, and accompanied delay and speech quality requirements, is for further study. |  |  |  |
|  | NOTE 2: P.863 is limited to 14 kHz bandwidth; therefore the speech quality requirements are ffs. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132.