3GPP TSG-SA4 Meeting #107 	S4-200084

# Wroclaw, Poland, Jan 20 - 24, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.139 | CR | xxx | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding RTCP Tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | RTCPVer |  |  |  |  |  | Date: |  |  | 2020-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some RTCP tests have headings but are partially or entirely lacking descriptive text. Some RTCP tests are insufficiently described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding and complementing RTCP test descriptions. Editorial corrections and clarifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some RTCP tests will not be fully or correctly described. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-200084 Initial draft |  |  |  |  |  |  |  |  |

First Change

### 6.2.1	General

This clause describes tests that are applicable to all types of RTCP and SRTCP packets. If not explicitly stated otherwise, any reference to RTCP is equally applicable to SRTCP. Tests that are only applicable to SRTCP and not to RTCP are specified in clause 6.4.

If not explicitly stated otherwise, all tests in this clause assume that the system under test and the test setup are set to use:

1)	AVP profile [4] for RTCP tests.

2)	SAVP profile [5] for SRTCP tests.

3)	Non-zero RTP session bandwidth.

4)	Non-zero RTCP bandwidth.

5)	Lossless RTP and RTCP packet transmission

6) If the default, deterministic RTCP report interval of 5 seconds is used:

a)	A minimum RTCP report interval no less than 2 seconds.

b)	A maximum RTCP report interval no longer than 7 seconds.

c)	An average RTCP report interval in the range 4.5 – 5.5 seconds.

NOTE 1:	The minimum, maximum, and average RTCP report intervals are not static parameters for an RTP/RTCP implementation but, as described in RFC 3550 [2], depend on conditions that vary with e.g. RTP session bandwidth, RTCP bandwidth, average size of RTCP packets, number of RTP session participants, and use of optional RTP/RTCP features such as RTCP feedback [7] and/or non-compound RTCP packets [8].

NOTE 2:	The method to set RTCP report interval in the system under test can vary. The RTCP report interval can in some cases be set directly, if the system under test has appropriate means to do so. The RTCP report interval can alternatively be set indirectly by e.g. setting an appropriate RTCP bandwidth in combination with pre-knowledge about the expected average RTCP packet size. If it is not possible to set RTCP bandwidth directly, it could be possible to set an appropriate RTP session bandwidth and rely on RTCP bandwidth becoming the default 5% of that RTP session bandwidth.

### 6.2.2	Basic RTCP Tests

#### 6.2.2.1	Initial Sending No Data RTCP Test

Purpose:	Tests that RTCP packets are generated even if no RTP packets are sent or received at the start of the session.

Status:	Conditionally Mandatory (Precondition 1)

Preconditions:	1) The system under test can set explicit RTCP bandwidth.
2) The system under test is set to not send any RTP packets from the start of the test session, e.g. pre-configured or set as receive-only through test instrument signalling.

Test procedure:	Observe system under test output.

Stop condition:	RTCP packets are sent from system under test, or time has passed corresponding to at least three maximum RTCP intervals (see clause 6.2.1, ca 21 seconds).

Pass criteria:	1) No RTP packets are sent.
2) RTCP packets are sent containing at least a Receiver Report.

Comments:	The test result is agnostic to RTP packets being received by the system under test or not. If the system under test is not receiving any RTP packets, the Receiver Report will be empty (no report blocks).

#### 6.2.2.2	Initial Receiving No Data RTCP Test

Purpose:	Tests that RTCP Sender Reports or Receiver Reports without any Receiver Report blocks are generated even if no RTP packets are received at the start of the session.

Status:	Mandatory

Preconditions:	The system under test is set to not receive any RTP packets from the start of the test session, e.g. pre-configured set as send-only through test instrument signalling.

Test procedure:	Observe system under test output.

Stop condition:	RTCP packets are sent from system under test, or time has passed corresponding to at least three maximum RTCP intervals (see clause 6.2.1, ca 21 seconds).

Pass criteria:	1) No RTP packets are received.
2) RTCP packets are sent containing at least a Sender Report or Receiver Report, with an empty Receiver Report block, indicated by RC=0.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.

#### 6.2.2.3	Sending Data RTCP Test

Purpose:	Tests that RTCP Sender Reports with nonzero content are sent when RTP packets are sent.

Status:	Mandatory

Preconditions:	The system under test is set to send RTP packets during the test session, e.g. pre-configured or set as send-receive or send-only through test instrument signalling.

Test procedure:	Observe system under test output.

Stop condition:	1) One or more RTP packets are sent.
2) An RTCP packet is sent.

Pass criteria:	1) RTP packets are sent.
2) At least one RTCP packet is sent containing at least a Sender Report.
3) The Sender Report contains nonzero Sender Info fields (NTP timestamp, RTP timestamp, sender’s packet count, and sender’s octet count).

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.

#### 6.2.2.4	Mid-Session Sending No Data RTCP Test

Purpose:	Test if the content of Sender Reports sent after ceasing to send RTP packets correctly reflect that RTP is no longer sent, and that RTCP transmission is changed to instead issue Receiver Reports at an appropriate time after RTP packet transmission was stopped.

Status:	Mandatory

Preconditions:	1) The system under test has passed tests 6.2.2.1 and 6.2.2.3.
2) The system under test is possible to control dynamically (e.g. directly through proprietary interfaces or via test instrument signalling) during an ongoing RTP session to turn sending of RTP packets on and off, while keeping the RTP session.

Test procedure:	1) Set the system under test to send RTP packets.
2) Observe the system under test output until at least one RTCP Sender Report is sent.
3) Set the system under test to not send RTP packets.
4) Observe the system under test output.

Stop condition:	At least three RTCP packets are sent after step 3 in the test procedure.

Pass criteria:	The following is fulfilled after step 3 in the test procedure:
1) No RTP packets are sent.
2) The first two RTCP packets sent contain Sender Reports with identical sender information in sender's packet count and sender's octet count.
3) The third RTCP packet sent contains a Receiver Report.

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.
IETF RFC 3550 section 6.4 specifies to send a Sender Report if RTP was sent during this RTCP reporting interval or the previous one, otherwise a Receiver Report is sent. The two Sender Reports contain identical sender information in sender's packet count and sender's octet count since no packets were sent during the latter reporting period.

#### 6.2.2.5	Mid-Session Receiving No Data RTCP Test

Purpose:	Test if RTCP content correctly indicates that no RTP packets were received during the reporting period, even if RTP packets were received previously in the RTP session.

Status:	Mandatory

Preconditions:	1) The system under test has passed tests 6.2.2.2 and 6.2.2.3.
2) The data injection to the system under test is possible to control dynamically (e.g. directly through proprietary interfaces or via test instrument signalling) during an ongoing RTP session to turn sending of RTP packets on and off, while keeping the RTP session.

Test procedure:	1) Set the data injection to send RTP packets to the system under test.
2) Observe the system under test output until at least one RTCP packet is sent.
3) Set the data injection to not send RTP packets to the system under test.
4) Observe the system under test output.

Stop condition:	At least two RTCP packets are sent after step 3 in the test procedure.

Pass criteria:	The second RTCP packet sent contains a Sender Report or Receiver Report with an empty Receiver Report block, indicated by RC=0.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.
If the data injection does not perform step 3 fast enough after step 2, the system under test may receive some RTP packets after step 2. Those RTP packets will then be reported on in the next RTCP packet sent, which is the reason to use the second RTCP packet sent after step 3 as input to the pass criteria.

#### 6.2.2.6	Compound RTCP Packet Format Test

[Editor's note: Should UDP header Length field and IP header Total Length field also be tested for consistency and included in the pass criteria?]

Purpose:	Test if the content and formatting of a compound RTCP packet fulfils basic criteria.

Status:	Mandatory

Preconditions:	1) The system under test has passed tests 6.2.2.1 – 6.2.2.5.
2) Reduced-size RTCP [8] is not negotiated to be used in the session (see the clause 5.3.11 of [8] for reduced-size tests), e.g. indicated by "a=rtcp-rsize" not being present in the SDP answer for the tested RTP media.

Test procedure:	Observe the system under test output from tests 6.2.2.1 – 6.2.2.5.

Stop condition:	Not applicable (implicit through tests in 6.2.2.1 – 6.2.2.5).

Pass criteria:	All RTCP packets sent fulfils:
1) A Sender Report or Receiver Report (equally acceptable for the outcome of this test) is included as the first part of the compound RTCP packet.
2) An SDES packet with a CNAME SDES item is included in the compound RTCP packet (except when a compound RTCP packet is split for partial encryption, if used; see SRTCP tests in clause 5.5).
3) Each RTCP packet in the compound RTCP packet starts on a 32-bit boundary.
4) Each RTCP packet in the compound RTCP packet has a length field with a value that is the number of 32-bit words minus one occupied by that RTCP packet, including the RTCP packet header and any padding.

#### 6.2.2.7	RTCP Report Count Test

Purpose:	Test if RTCP packets contain a correct and consistent report count and length fields.

Status:	Mandatory

Preconditions:	The system under test has passed tests 6.2.2.1 – 6.2.2.5.

Test procedure:	Observe the system under test output from tests 6.2.2.1 – 6.2.2.5.

Stop condition:	Not applicable (implicit through tests in 6.2.2.1 – 6.2.2.5).

Pass criteria:	All RTCP packets sent will fulfil:
1) Regardless if the RTCP packet is a Sender Report or Receiver Report, the report count (RC) field is consistent with the number of report blocks observed to be included in the packet.
2) The RTCP packet length field has a value that is the number of 32-bit words minus one occupied by that RTCP packet, including the RTCP packet header and any padding.

Comments:	This test does not cover a higher report count than 1, which can be required when more than a single RTP stream is sent in an RTP session, e.g. in multi-stream scenarios as described by TS 26.114 [9] Annex S. Tests covering a higher report count than 1 may be added in future revisions of the present document.

### 6.2.3	RTCP Bandwidth and Transmission Timing Tests

#### 6.2.3.1	RTCP Disable Test

Purpose:	Test if setting RTCP bandwidth to zero in SDP disables RTCP transmission.

Status:	Mandatory

Preconditions:	RTCP transmission is disabled for the RTP session, e.g. negotiated in SDP by both b=RS:0 and b=RR:0 being present in the SDP answer (zero RTCP bandwidth).

Test procedure:	Observe the system under test output.

Stop condition:	Time has passed corresponding to at least three maximum RTCP intervals (see clause 6.2.1, ca 21 seconds).

Pass criteria:	No RTCP packets are sent.

Comments:	The test result is agnostic to RTP packets being sent or received by the system under test.

#### [6.2.3.2	RTCP Basic Interval Test

Purpose:	Test if the non-reduced, minimum time interval with which RTCP packets are sent complies with IETF RFC 3550 [2], including prescribed randomization of the time interval.

Status:	Mandatory

Preconditions:	1) The system under test is set to not send any RTP packets, e.g. pre-configured or set as receive-only through test instrument signalling.
2) The system under test is set to use default RTCP timing (based on 5% RTCP bandwidth), e.g. pre-configured or through neither "b=RS" nor "b=RR" being included in the SDP answer through test instrument signalling.
3) The system under test is set to not use the reduced minimum RTCP interval for high bandwidth sessions (see test 6.2.3.5).
4) The system under test is set to use an RTP session bandwidth of at least 1000 kbit/s to ensure that the minimum, non-reduced RTCP interval is used, e.g. pre-configured or negotiated in SDP through "b=AS:1000" (or greater) being included in the SDP answer. If that high RTP session bandwidth cannot be set, the maximum possible RTP session bandwidth should instead be set as long as it is greater than 72 kbit/s (see also test 6.2.3.4).
5) The data injection to the system under test is set to not send any RTP packets, such that the system under test does not receive any RTP packets (that is, no RTP packets will be sent to or from the system under test).

Test procedure:	1) Observe the system under test output.
2) Note the reception time (ti) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets, n, sent from the system under test.
4) For each pair of subsequent RTCP packets, calculate the difference in reception time (called the interval), Ii=ti-ti-1, iϵ2..n.
5) Calculate the average RTCP interval I=1n-1i=2nIi.

Stop condition:	At least 10 minutes, or 100 RTCP packets are received, whichever is less.

Pass criteria:	1) 2 seconds≤miniIi≤I. 
2) I≤maxiIi≤7 seconds. 
3) 4.5 seconds≤I≤5.5 seconds.

Comments:	This test allows an exception to IETF RFC 3550 [2] and the RTP testing strategies in [3] stating that an implementation fails the test if the RTCP packets are sent with a constant interval (not randomized).

#### 6.2.3.3	RTCP Multiparty and Multistream Interval Test

Purpose:	Test if the non-reduced, minimum time interval with which RTCP packets are sent complies with IETF RFC 3550 [2], including prescribed randomization of the time interval.

Status:	Conditionally Mandatory (precondition 1)

Preconditions:	1) The system under test is targeted for RTP multi-party and / or RTP multi-stream scenarios (see test 6.2.3.2 for other scenarios).
2) The system under test is set to not send any RTP packets, e.g. pre-configured or set as receive-only through test instrument signalling.
3) The system under test is set to use default RTCP timing (based on 5% RTCP bandwidth), e.g. pre-configured or through neither "b=RS" nor "b=RR" being included in the SDP answer through test instrument signalling.
4) The system under test is set to not use the reduced minimum RTCP interval for high bandwidth sessions (see test 6.2.3.5).
5) The system under test is set to use an RTP session bandwidth of at least 1000 kbit/s to ensure that the minimum, non-reduced RTCP interval is used, e.g. pre-configured or negotiated in SDP through "b=AS:1000" (or greater) being included in the SDP answer. If that high RTP session bandwidth cannot be set, the maximum possible RTP session bandwidth should instead be set as long as it is greater than 72 kbit/s (see also test 6.2.3.4).
6) The data injection to the system under test is set to not send any RTP packets, such that the system under test does not receive any RTP packets (that is, no RTP packets will be sent to or from the system under test).

Test procedure:	1) Observe the system under test output.
2) Note the reception time (ti) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets, n, sent from the system under test.
4) For each pair of subsequent RTCP packets, calculate the difference in reception time (called the interval), Ii=ti-ti-1, iϵ2..n.
5) Calculate the average RTCP interval I=1n-1i=2nIi.

Stop condition:	At least 10 minutes, or 100 RTCP packets are received, whichever is less.

Pass criteria:	1) 2 seconds≤miniIi≤2.5 seconds.
2) 5.5 seconds≤maxiIi≤7 seconds.
3) 4.5 seconds≤I≤5.5 seconds.

Comments:	The RTP testing strategies in [3] recommends the test to run for at least 20 minutes and highlights that an implementation fails the test if the RTCP packets are sent with a constant interval (not randomized).

#### 6.2.3.4	RTCP Low Bandwidth Test

Purpose:	Test if the non-reduced time interval with which RTCP packets are sent complies with used RTP and RTCP bandwidth, and the recommended minimum time interval as specified by IETF RFC 3550 [2].

Status:	Mandatory

Preconditions:	1) The system under test is set to use default RTCP timing (based on 5% RTCP bandwidth), e.g. pre-configured through neither "b=RS" nor "b=RR" being included in the SDP answer through test instrument signalling.
2) The system under test is set to send and receive RTP packets during the test session, e.g. pre-configured or set as send-receive in SDP answer through test instrument signalling.
3) The RTP session bandwidth is set lower than 72 kbit/s, e.g. through b=AS included in SDP answer.

Test procedure:	1) Set the data injection to send a single RTP stream (one SSRC) to the system under test.
2) Observe the system under test output.
3) Note the reception time (ti) and size (Si) in bits of each RTCP packet sent from the system under test, including UDP and IP headers.
4) Count the number of RTCP packets, n, sent from the system under test.
5) For each pair of subsequent RTCP packets, calculate the difference in reception time,
Ii=ti-ti-1, iϵ2..n.
6) Calculate the average RTCP interval I=1n-1i=2nIi.
7) Calculate the average RTCP bandwidth B=i=1n-1Sii=2nIi.
8) Calculate the average RTCP packet size S=1n-1i=2nSi.
9) Calculate the deterministic RTCP interval, T=max2SBTmin, where Tmin=5 seconds.

Stop condition:	At least 10 minutes, or 100 RTCP packets are received, whichever is less.

Pass criteria:	1) 0.4 * T ≤ mini Ii.
2) maxi Ii ≤ 1.25 * T.
3) 0.95 * T ≤ I ≤ 1.05 * T

Comments:	The values 0.4 and 1.25 above are chosen based on the 0.5T..1.5T uniform distribution randomization of the RTCP interval, divided by the e-1.5 (=1.21828…) reconsideration factor, both taken from section 6.2.1 of IETF RFC 3550.

#### 6.2.3.5	RTCP High Bandwidth Test

Purpose:	Test if the reduced time interval with which RTCP packets are sent for high-bandwidth RTP streams complies with used RTP and RTCP bandwidth, and the recommended minimum time interval for high-bandwidth RTP as specified by IETF RFC 3550 [2].

Status:	Conditionally Mandatory (Precondition 1)

Preconditions:	1) The system under test can use higher RTP bandwidth than 72 kbit/s.
2) The system under test is set to use default RTCP timing (based on 5% RTCP bandwidth), e.g. pre-configured or through neither "b=RS" nor "b=RR" being included in the SDP answer through test instrument signalling.
3) The system under test is set to send and receive RTP packets during the test session, e.g. pre-configured or set as send-receive in SDP answer through test instrument signalling.
4) The RTP session bandwidth is set higher than 72 kbit/s, e.g. through b=AS included in SDP answer.

Test procedure:	1) Set the data injection to send a single RTP stream (one SSRC) to the system under test.
2) Observe the system under test output.
3) Note the reception time (ti) and size (Si) in bits of each RTCP packet sent from the system under test, including UDP and IP headers.
4) Count the number of RTCP packets, n, sent from the system under test.
5) For each pair of subsequent RTCP packets, calculate the difference in reception time,
Ii=ti-ti-1, iϵ2..n.
6) Calculate the average RTCP interval I=1n-1i=2nIi.
7) Calculate the reduced deterministic RTCP interval, T=360BRTP, where BRTP = RTP session maximum bandwidth in kbit/s, e.g. calculated as 2 times the bandwidth from b=AS in SDP answer for point-to-point sessions (see also comment below).

Stop condition:	At least 10 minutes, or 100 RTCP packets are received, whichever is less.

Pass criteria:	1) 0.4T≤miniIi
2) maxiIi≤1.25T
3) 0.95T≤I≤1.05T

Comments:	The values 0.4 and 1.25 above are chosen based on the 0.5T..1.5T uniform distribution randomization of the RTCP interval, divided by the e-1.5 (=1.21828…) reconsideration factor, both taken from section 6.2.1 of IETF RFC 3550.
The calculation of the BRTP factor in the denominator of step 7 is seemingly not in accordance with the definition of IETF RFC 4566 section 5.8 that says b=AS is the RTP session bandwidth (all RTP streams that are sent and received in the RTP session), but 3GPP specifications commonly interpret b=AS as receive bandwidth seen from the part sending the SDP, which is thus only half of the actual RTP session bandwidth in a bandwidth-symmetric point-to-point session.

#### 6.2.3.6	RTCP Explicit Bandwidth Test

Purpose:	Test if the RTCP bandwidth is kept within set maximum limit, e.g. by "b=RS" and "b=RR" in SDP, and the information in those is correctly interpreted with respect to system under test being sender in a point-to-point RTP session.

Status:	Conditionally Mandatory (Precondition 1)

Preconditions:	1) The system under test can set explicit RTCP bandwidth

2) The system under test is set to send and receive RTP packets during the test session, e.g. pre-configured or set as send-receive in SDP answer through test instrument signalling.
3) The RTP session bandwidth is set lower than 72 kbit/s, e.g. through b=AS included in SDP answer such that the reduced RTCP interval calculation in test 6.2.3.5 does not apply.

Test procedure:	1) Negotiate an explicit RTCP bandwidth in SDP, setting "b=RS:625" and "b=RR:1875" in SDP answer.
2) Observe the system under test output.
3) Note the reception time (ti) and size (Si) in bits of each RTCP packet sent from the system under test, including UDP and IP headers.
4) Count the number of RTCP packets, n, sent from the system under test.
5) For each pair of subsequent RTCP packets, calculate the difference in reception time,
Ii=ti-ti-1, iϵ2..n.
6) Calculate the average RTCP bandwidth B=i=1n-1Sii=2nIi.
7) Note the test result from step 6.
8) Negotiate an explicit RTCP bandwidth in SDP, setting "b=RS:0" and "b=RR:2500" in SDP answer.
9) Re-run steps 2-6 and note the test result from step 6.

Stop condition:	At least 10 minutes, or 100 RTCP packets are received, whichever is less, in each of steps 7 and 9.

Pass criteria:	1) B≤1250 bit/s, from step 7 of this test.
2) B≤1250 bit/s, from step 9 of this test.

Comments:	Pass criteria 1 and 2 are identical because the proportion of senders in this point-to-point RTP session (100%) is larger than RS/(RS+RR) and all senders get their proportion of the (RS+RR) sum (see section 2 of IETF RFC 3556 [6]).

#### 6.2.3.7	RTCP Timeout Test

Purpose:	Test if an RTP stream that is stopped at the sender without any explicit RTCP BYE times out correctly and is reflected as such in the subsequent receiver reporting.

Status:	Conditionally Mandatory (Precondition 1)

Preconditions:	1) The system under test is capable of using multiple simultaneous RTP streams in a single RTP session.
2) The system under test has passed tests 6.2.2.1, 6.2.2.3 and 6.2.2.5.
3) The data injection to the system under test is possible to control dynamically during an ongoing RTP session to turn sending of RTP packets on and off, while keeping the RTP session.

Test procedure:	1) Set the data injection to send RTP packets to the system under test.
2) Observe the system under test output until at least one RTCP packet is sent.
3) Set the data injection to not send RTP packets to the system under test, not sending any RTCP BYE for the stopped RTP stream SSRC.
4) Observe the system under test output.

Stop condition:	Six (6) RTCP packets are sent after step 3 in the test procedure.

Pass criteria:	The 6th RTCP packet sent contains a Sender Report or Receiver Report that does not contain any report block for the stopped SSRC from data injection.

Comments:	An SSRC for which no packets have been received during (per default) 5 or more RTCP intervals will be timed out, as described by section 6.3.5 of IETF RFC 3550. The current test procedure and pass criteria do not test if RTCP bandwidth allocated to the SSRC that was timed out is correctly re-allocated to the remaining SSRCs, which may be added in a subsequent version of this specification.

#### 6.2.3.8	RTCP Step Join Backoff Receiver Test

Purpose:	Test if the time between first and second RTCP packet sent from system under test is appropriate (using sufficiently low but not too low RTCP bandwidth) when joining as RTP receiver (non-sender) in an RTP session that is discovered to already contain many participants.

FFS.

#### 6.2.3.9	RTCP Step Join Backoff Sender Test

Purpose:	Test if the time between first and second RTCP packet sent from system under test is appropriate (using sufficiently low but not too low RTCP bandwidth) when joining as RTP sender in an RTP session that is discovered to already contain many participants.

FFS.

### 6.2.4	Sender Report Sender Info Tests

#### 6.2.4.1	Sender SSRC Test

Purpose:	Test if all SSRC values included in a sent RTCP sender information corresponds to the SSRC of a sent RTP stream, and that there are corresponding RTCP sender information for all sent RTP streams (one or more).

Status:	Mandatory

Preconditions:	1) The system under test has passed test 6.2.2.3.
2) The system under test is set to send RTP packets during the test session, e.g. pre-configured or set as send-receive or send-only through test instrument signalling.
3) The number of simultaneously used SSRC identifiers to be sent by the system under test in the RTP session is known a-priori (only one in the single-stream case).

Test procedure:	Observe system under test output.

Stop condition:	1) RTP packets are sent.
2) At least three RTCP packets are sent per used SSRC identifier (at least the first RTCP packet may be sent before first RTP packet for each SSRC).

Pass criteria:	1) The "SSRC of sender" header field in all sent RTCP packets contains a value that is identical to the "synchronization source (SSRC) identifier" header field in at least one of the sent RTP packets.
2) The "synchronization source (SSRC) identifier" header field in all sent RTP packets contains a value that is identical to the "SSRC of sender" header field in at least one of the sent RTCP packets.

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.

#### 6.2.4.2	NTP Test

Purpose:	Test if the NTP timestamp field in successive RTCP packets progresses with a consistent clock rate, and that the used NTP time format is correct with seconds in the high 32 bits.

Status:	Mandatory

Preconditions:	1) The system under test has passed test 6.2.2.3.
2) The system under test is set to send RTP packets during the test session, e.g. pre-configured or set as send-receive or send-only through test instrument signalling.
3) The "NTP timestamp" field in the RTCP packet Sender Information is monotonously increasing.
4) The test network and test equipment contributes with an amount of time jitter that is insignificant to the test result.

Test procedure:	1) Observe the system under test output.
2) Note the reception time (ti) and NTP timestamp field value (wi) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets, n, sent from the system under test.
4) For each pair of subsequent RTCP packets, calculate the difference in reception time,
Ii=ti-ti-1, iϵ2..n, and difference in NTP time, Ni=wi-wi-1, iϵ2..n.
5) Calculate the NTP timestamp clock rate estimates Ci=NiIi, i∈2..n.
6) Calculate the NTP timestamp correlation quotients Qi=CiCi-1, i∈3..n.

Stop condition:	1) RTP packets are sent.
2) At least three RTCP packets are sent (at least the first RTCP packet may be sent before first RTP packet).

Pass criteria:	1) 0.999≤Qi≤1.001, for all i∈3..n.
2) 0.999≤Ci232≤1.001, for all i∈3..n.

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.
NTP time calculations require 64-bit integer arithmetic capability.
The pass criterion corresponds to a minimum clock accuracy of 0.1% but a much higher accuracy should in general be expected, however in some cases the test setup can introduce packet delay jitter that makes accurate measurement impossible and the above values are chosen as a trade-off.

#### 6.2.4.3	Wrapped NTP Test

Purpose:	Test if the NTP timestamp field in successive RTCP packets progresses with a consistent clock rate, even when the NTP timestamp field wraps around.

Status:	Conditionally Mandatory (precondition 3)

Preconditions:	1) The system under test has passed test 6.2.4.2.
2) The system under test is set to send RTP packets during the test session, e.g. pre-configured or set as send-receive or send-only through test instrument signalling.
3) The "NTP timestamp" field in the first RTCP packet Sender Information is set to a value close to 264-1 (when interpreted as an unsigned 64-bit integer), such that the value will wrap during the test.

Test procedure:	1) Observe the system under test output.
2) Note the reception time (ti) and NTP timestamp field value (wi) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets, n, sent from the system under test.
4) For each pair of subsequent RTCP packets, calculate the difference in reception time,
Ii=ti-ti-1, iϵ2..n, and difference in NTP time, Ni=wi-wi-1, iϵ2..n.
5) For any wi and wi-1 with wi<wi-1 when interpreting wi and wi-1 as unsigned 64-bit values, calculate Ni=264-wi-1+wi=264+wi-wi-1 % 264, replacing the Ni from step 4 above.
6) Calculate the NTP timestamp clock rate estimates Ci=NiIi, i∈2..n.
7) Calculate the NTP timestamp correlation quotients Qi=CiCi-1, i∈3..n.

Stop condition:	1) RTP packets are sent.
2) At least three RTCP packets are sent (at least the first RTCP packet may be sent before first RTP packet).
3) The "NTP timestamp" field in the last RTCP packet Sender Information is less than the "NTP timestamp" field in the first RTCP packet Sender Information, when interpreting those fields as unsigned 64-bit integers.

Pass criteria:	0.999≤Qi≤1.001, for all i∈3..n.

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.
NTP time calculations require 64-bit integer arithmetic capability.
The pass criterion corresponds to a minimum timestamp clock accuracy of 0.1% but a much higher accuracy should in general be expected, however in some cases the test setup can introduce packet delay jitter that makes accurate measurement impossible and the above values are chosen as a trade-off.

#### 6.2.4.4	Time Stamp Test

[Editor’s note: Having stop condition allow receiving just three RTCP packets is truly a minimum and a bit arbitrary; would it be desirable to require a larger sample size?]

Purpose:	Test if the RTP timestamp field in successive RTCP packets progresses with a clock rate that is a reasonably constant factor of NTP timestamp clock rate, and that the RTCP timestamp clock rate is reasonably close to the set rate, e.g. from "a=rtpmap" line in SDP.

Status:	Mandatory

Preconditions:	1) The system under test has passed tests 6.2.2.3, 6.2.4.1 and 6.2.4.2.
2) The system under test is set to send RTP packets for a single RTP stream (one SSRC) during the test session, e.g. pre-configured or set as send-receive or send-only through test instrument signalling.
3) The "RTP timestamp" field in the RTCP packet Sender Information is monotonously increasing.

Test procedure:	1) Observe the system under test output.
2) Note the tiNTP timestamp field value (wi) and RTP timestamp field value (ri) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets, n, sent from the system under test.
4) For each pair of subsequent RTCP packets, calculate the 
Ii=ti-ti-1, iϵ2..ndifference in NTP time, Ni=wi-wi-1, iϵ2..n and difference in RTP timestamp, Ti=ri-ri-1, iϵ2..n.
5) Calculate the NTP timestamp difference in seconds Ci=Ni232, i∈2..n.
6) Calculate the RTP timestamp rate estimatesRi=TiIi, i∈2..n Ri=TiCi, i∈2..n.
Ci=NiIi, i∈2..n
7) Calculate the average RTP timestamp rate estimate R=1n-1i=2nRi.
Pi=RiCi, i∈2..n
 Qi=PiPi-1, i∈3..n
8) Retrieve RTP Time Stamp rate information, S, e.g. from "<clock rate>" on the SDP "a=rtpmap:<payload type> <encoding name>/<clock rate>" line, applicable for the RTP stream that the RTCP packets are associated with.

Stop condition:	1) RTP packets are sent.
2) At least three RTCP packets are sent (at least the first RTCP packet may be sent before first RTP packet).

Pass criteria:	0.999≤RiS≤1.001, for all i∈2..n0.999≤RS≤1.001.
0.999≤Qi≤1.001, for all i∈3..n

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.
NTP time calculations require 64-bit integer arithmetic capability.
The pass criteria correspond to a minimum timestamp clock accuracy of 0.1% but a much higher accuracy should in general be expected, however in some cases the test setup can introduce packet delay jitter that makes accurate measurement impossible and the above values are chosen as a trade-off.

#### 6.2.4.5	Wrapped Time Stamp Test

[Editor’s note: Same comment as for 6.2.4.4.]

Purpose:	Test if the RTP timestamp field in successive RTCP packets progresses with a clock rate that is a reasonably constant factor of NTP timestamp clock rate, even when the RTP timestamp field wraps around, and that the RTCP timestamp clock rate is reasonably close to the set rate, e.g. from "a=rtpmap" line in SDP.

Status:	Conditionally Mandatory (precondition 3)

Preconditions:	1) The system under test has passed test 6.2.4.4.
2) The system under test is set to send RTP packets for a single RTP stream (one SSRC) during the test session, e.g. pre-configured or set as send-receive or send-only through test instrument signalling.
3) The "RTP timestamp" field in the first RTCP packet Sender Information is set to a value close to 232-1 (when interpreted as an unsigned 32-bit integer), such that the value will wrap during the test.

Test procedure:	1) Observe the system under test output.
2) Note the tiNTP timestamp field value (wi) and RTP timestamp field value (ri) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets, n, sent from the system under test.
4) For each pair of subsequent RTCP packets, calculate the 
Ii=ti-ti-1, iϵ2..ndifference in NTP time, Ni=wi-wi-1, iϵ2..n, 
5) Calculate the NTP timestamp difference in seconds Ci=Ni232, i∈2..n.
6) For each pair of subsequent RTCP packets with ri≥ri-1 when interpreting ri and ri-1 as unsigned 32-bit values, calculate the difference in RTP time, Ti=ri-ri-1, iϵ2..n.
7) For each pair of subsequent RTCP packets with ri<ri-1 when interpreting ri and ri-1 as unsigned 32-bit values, calculate Ti=232-ri-1+ri=232+ri-ri-1 modulo 232.
8) Calculate the RTP timestamp rate estimatesRi=TiIi, i∈2..n Ri=TiCi, i∈2..n.
Ci=NiIi, i∈2..n
9) Calculate the average RTP timestamp rate estimate R=1n-1i=2nRi.
Pi=RiCi, i∈2..n
Qi=PiPi-1, i∈3..n
10) Retrieve the intended RTP Time Stamp rate information, S, e.g. from "…/S" on the SDP "a=rtpmap" line, applicable for the RTP stream that the RTCP packets are associated with.

Stop condition:	1) RTP packets are sent.
2) At least three RTCP packets are sent (at least the first RTCP packet may be sent before first RTP packet).
3) The "RTP timestamp" field in the last RTCP packet Sender Information is less than the "RTP timestamp" field in the first RTCP packet Sender Information, when interpreting those fields as unsigned 32-bit integers.

Pass criteria:	0.999≤RiS≤1.001, for all i∈2..n0.999≤RS≤1.001.
0.999≤Qi≤1.001, for all i∈3..n

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.
NTP time calculations require 64-bit integer arithmetic capability.
The pass criteria correspond to a minimum timestamp clock accuracy of 0.1% but a much higher accuracy should in general be expected, however in some cases the test setup can introduce packet delay jitter that makes accurate measurement impossible and the above values are chosen as a trade-off.

#### 6.2.4.6	Packet Count Test

Purpose:	Test if the reported packet count corresponds with sent RTP packets.

Status:	Mandatory

Preconditions:	1) The system under test has passed test 6.2.2.3.
2) The system under test is set to send RTP packets during the test session, e.g. pre-configured or set as send-receive or send-only through test instrument signalling.

Test procedure:	1) Observe the system under test output.
2) Note the "sender’s packet count" field (pi) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets (n) sent from the system under test.
4) Count and note the number of RTP packets (Pi) sent from the system under test between each pair i-1, i of subsequent RTCP packets,
5) For each pair of subsequent RTCP packets, calculate the difference in packet count,
di=pi-pi-1, iϵ2..n.

Stop condition:	1) RTP packets are sent.
2) At least three RTCP packets are sent after the first RTP packet (one or more RTCP packets may be sent before first RTP packet).

Pass criteria:	di= Pi, for all i∈2..n.

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.

#### 6.2.4.7	Wrapped Packet Count Test

Purpose:	Test if the reported packet count corresponds with sent RTP packets, even when the "sender’s packet count" field wraps around.

Status:	Conditionally Mandatory (precondition 3)

Preconditions:	1) The system under test has passed test 6.2.4.6.
2) The system under test is set to send RTP packets during the test session, e.g. pre-configured or set as send-receive or send-only through test instrument signalling.
3) The "sender’s packet count" field in the first RTCP packet Sender Information is set to a value close to 232-1 (when interpreted as an unsigned 32-bit integer), such that the value will wrap during the test.

Test procedure:	1) Observe the system under test output.
2) Note the "sender’s packet count" field (pi) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets (n) sent from the system under test.
4) Count and note the number of RTP packets (Pi) sent from the system under test between each pair i-1, i of subsequent RTCP packets,
5) For each pair of subsequent RTCP packets where pi≥pi-1 when interpreting pi and pi-1 as unsigned 32-bit values, calculate the difference in packet count, di=pi-pi-1, iϵ2..n.
6) For each pair of subsequent RTCP packets where pi<pi-1 when interpreting pi and pi-1 as unsigned 32-bit values, calculate di=232-pi-1+pi=232+pi-pi-1 % 232, i∈2..n.

Stop condition:	1) RTP packets are sent.
2) At least three RTCP packets are sent after the first RTP packet (one or more RTCP packets may be sent before first RTP packet).
3) The "sender’s packet count" field in the last RTCP packet Sender Information is less than the "sender’s packet count" field in the first RTCP packet Sender Information, when interpreting those fields as unsigned 32-bit integers.

Pass criteria:	di= Pi, for all i∈2..n.

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.

#### 6.2.4.8	Octet Count Test

Purpose:	Test if the reported octet count corresponds with sent RTP packets.

Status:	Mandatory

Preconditions:	1) The system under test has passed test 6.2.2.3.
2) The system under test is set to send RTP packets during the test session, e.g. pre-configured or set as send-receive or send-only through test instrument signalling.

Test procedure:	1) Observe the system under test output.
2) Note the "sender’s octet count" field (ci) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets (n) sent from the system under test.
4) Count and note the number of RTP packets (Pi) sent from the system under test between each pair i-1, i of subsequent RTCP packets.
5) Note the RTP payload size (sNi), with Ni∈1..Pi, excluding RTP and lower layer headers and any RTP padding, of each RTP packet sent from the system under test between each pair of subsequent RTCP packets.
6) For each pair of subsequent RTCP packets, calculate the difference in octet count, bi=ci-ci-1, and sum of RTP payload sizes Si=Ni=1PisNi with i∈2..n.

Stop condition:	1) RTP packets are sent.
2) At least three RTCP packets are sent after the first RTP packet (one or more RTCP packets may be sent before first RTP packet).

Pass criteria:	bi= Si, for all i∈2..n.

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.

#### 6.2.4.9	Wrapped Octet Count Test

Purpose:	Test if the reported octet count corresponds with sent RTP packets, even when the "sender’s octet count" field wraps around.

Status:	Conditionally Mandatory (precondition 3)

Preconditions:	1) The system under test has passed test 6.2.4.8.
2) The system under test is set to send RTP packets during the test session, e.g. pre-configured or set as send-receive or send-only through test instrument signalling.
3) The "sender’s octet count" field in the first RTCP packet Sender Information is set to a value close to 232-1 (when interpreted as an unsigned 32-bit integer), such that the value will wrap during the test.

Test procedure:	1) Observe the system under test output.
2) Note the "sender’s octet count" field (ci) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets (n) sent from the system under test.
4) Count and note the number of RTP packets (Pi) sent from the system under test between each pair i-1, i of subsequent RTCP packets,
5) Note the RTP payload size (sNi), with Ni∈1..Pi, excluding RTP and lower layer headers and any RTP padding, of each RTP packet sent from the system under test between each pair of subsequent RTCP packets.
6) For each pair of subsequent RTCP packets, calculate the sum of RTP payload sizes Si=Ni=1PisNi with i∈2..n.
7) For each pair of subsequent RTCP packets where ci≥ci-1 when interpreting ci and ci-1 as unsigned 32-bit values, calculate the difference in octet count, bi=ci-ci-1, iϵ2..n. 
8) For each pair of subsequent RTCP packets where ci<ci-1 when interpreting ci and ci-1 as unsigned 32-bit values, calculate  bi=232-ci-1+ci=232+ci-ci-1 modulo 232, i∈2..n.

Stop condition:	1) RTP packets are sent.
2) At least three RTCP packets are sent after the first RTP packet (one or more RTCP packets may be sent before first RTP packet).
3) The "sender’s octet count" field in the last RTCP packet Sender Information is less than the "sender’s octet count" field in the first RTCP packet Sender Information, when interpreting those fields as unsigned 32-bit integers

Pass criteria:	bi= Si, for all i∈2..n.

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.

#### 6.2.4.10	Synchronization Information Test

Purpose:	Test if the RTCP sender information and corresponding RTP for two or more streams that are sent simultaneously, and that are intended to be synchronized, are consistent enough to allow synchronization.

FFS.

### 6.2.5	Source Description (SDES) Tests

#### 6.2.5.1	Basic SDES Test

Purpose:	Test if sent RTCP SDES packets fulfils a few basic criteria for content and length consistency.

Status:	Mandatory

Preconditions:	1) The system under test has passed test 6.2.2.6.
2) The system under test is set to send RTP packets during the test session, e.g. pre-configured or set as send-receive or send-only through test instrument signalling.

Test procedure:	Observe system under test output.

Stop condition:	1) RTP packets are sent.
2) At least three RTCP packets are sent per used SSRC identifier (at least the first RTCP packet may be sent before first RTP packet for each SSRC).

Pass criteria:	The following is fulfilled for all sent RTCP SDES packets:
1) The source count (SC) header field that matches the number of SDES item chunks included in the respective RTCP SDES packet.
2) Each SDES item chunk ends with at least one octet containing a zero value.
3) Each SDES item in a chunk has a length field that corresponds to the text content length in octets of that SDES item, corresponding to the distance in octets starting from the octet after that length field until the start of the next SDES item or until the terminating zero value octet (see above).
4) Each SDES item in a chunk with non-zero content length has a text content where the last octet value is not zero (is not a zero-terminated string).

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.

#### 6.2.5.2	Canonical End-point Identifier (CNAME) Test

Purpose:	Test if CNAME content is identical for every RTCP packet belonging to the same RTP stream.

Status:	Mandatory

Preconditions:	1) The system under test has passed test 6.2.5.1.
2) The system under test is set to send RTP packets during the test session, e.g. pre-configured or set as send-receive or send-only through test instrument signalling.

Test procedure:	1) Observe system under test output.
2) Note the "SSRC of sender" field (si) and CNAME SDES item field (ci) values of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets (n) sent from the system under test.

Stop condition:	1) RTP packets are sent from the system under test.
2) At least three RTCP packets are sent per used SSRC identifier after sending the first RTP packet for that SSRC identifier (one or more RTCP packets may be sent before first RTP packet for each SSRC).

Pass criteria:	ci= ci-1, for all i∈2..n where si=si-1.

Comments:	The test result is agnostic to RTP packets being received by the system under test or not.

#### 6.2.5.3	3GPPRTP Private Information (PRIV) Test

Purpose:	Test if PRIV 3GPPRTP content to announce conformance to this specification corresponds to clause 7.3.

Status:	Conditionally Mandatory (precondition 2)

Preconditions:	1) The system under test has passed test 6.2.5.1.
2) The system under test is designed or set to announce conformance to this specification in RTCP.

Test procedure:	1) Observe system under test output.
2) Note all the SDES item field values of each RTCP SDES PRIV packet sent from the system under test.

Stop condition:	At least three compound RTCP packets are sent from the system under test.

Pass criteria:	1) SDES PRIV 3GPPRTP item is present in at least the first three compound RTCP packets.
2) For every SDES PRIV 3GPPRTP item:
	a) The content of the prefix string is "3GPPRTP" (not zero-terminated).
	b) The prefix length is 7.
	c) The format of the value string starts with "<#>.<#>.<#>",
	where <#> is a one- or two-digit decimal number
	d) The first character, if any, after c) above is a space (32).

Comments:	The test result is agnostic to RTP packets being sent or received by the system under test.

### 6.2.6	Receiver Report Block Tests

[Editor’s note: Priority 1. Ensure that this document covers:
Incorrect loss when RTP sequence number of a stream does not start at 0 (6.2.6.4)
Incorrect loss when RTP sequence number rolls over (6.2.6.7 and 6.2.6.12)
Incorrect loss / report content when no packets are received in a reporting period (6.2.6.5 and 6.2.6.10)
Incorrect loss when RTP SSRC changes (6.2.6.3 and optionally 6.2.6.4)
Incorrect delay reporting (6.2.6.19)
Incorrect jitter reporting (6.2.6.17)]

#### 6.2.6.1	SSRC Consistency Test

Purpose:	Test if the SSRC value included in a sent RTCP report block corresponds to the SSRC of a received RTP stream.

Status:	Mandatory

Preconditions:	1) The system under test has passed tests 6.2.2.2, 6.2.2.5, 6.2.2.6, and 6.2.2.7.
2) The system under test is set to receive RTP packets during the test session, e.g. pre-configured or set as send-receive or receive-only through test instrument signalling.
3) Data injection is set to send RTP.

Test procedure:	Observe system under test and data injection output.

Stop condition:	An RTCP packet is sent from system under test, containing at least a receiver report block.

Pass criteria:	The SSRC value in the RTCP report block can be found as RTP header SSRC field value in at least one of the RTP packets sent from data injection.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.
A report block containing an SSRC value identical to the SSRC value in the RTP header of received RTP packets is henceforth denoted as "reporting on" the RTP stream those RTP packets belong to.

#### 6.2.6.2	SSRC Completeness Test

Purpose:	Test if all RTP streams (all SSRC) used in the RTP session are reported on, while allowing that not all RTP streams are reported in every RTCP packet (e.g. due to RTCP packet MTU restrictions).

Status:	Conditionally Mandatory (Precondition 1)

Preconditions:	1) The system under test is capable of using multiple simultaneous RTP streams in a single RTP session.
2) The system under test has passed test 6.2.6.1.
3) The system under test is set to receive RTP packets during the test session, e.g. pre-configured or set as send-receive or receive-only through test instrument signalling.
4) Data injection is set to send RTP.
5) The number of simultaneously used, unique, SSRC values to be sent by the data injection in the RTP session (m) is known a-priori (only one in the single-stream case).

Test procedure:	1) Observe system under test and data injection output.
2) Note which SSRC values (Si) and report count (Ci) that are included in RTCP report blocks sent from the system under test.
3) Count the number of RTCP packets (n) sent from the system under test.
4) Note which SSRC values (Rj) that are included in RTP headers of RTP packets sent from data injection.
5) Calculate the number of received report blocks N=i=1nCi.

Stop condition:	1) At least one RTP packet with SSRC value Rj is sent from data injection for all j∈1..m.
2) N=m.

Pass criteria:	 Si=Rj for all i∈1..n and j∈1..m.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.

#### 6.2.6.3	SSRC Change Test

Purpose:	Test if receiving an RTP stream that changes SSRC is correctly reported on, keeping reporting separate per used SSRC value.

FFS.

#### 6.2.6.4	Initial Zero Loss Test

Purpose:	Test if loss-free reception of RTP packets at RTP session start is correctly reported as no loss in RTCP, even if the initial RTP sequence number is non-zero.

Status:	Mandatory

Preconditions:	1) The system under test has passed test 6.2.6.1.
2) The system under test is set to receive RTP packets during the test session, e.g. pre-configured or set as send-receive or receive-only through test instrument signalling.
3) Data injection is set to send a single RTP stream (one SSRC) without loss, with monotonically increasing RTP sequence number (SN), and with the first RTP packet sent containing a non-zero RTP sequence number (SN).

Test procedure:	Observe system under test and data injection output.

Stop condition:	An RTCP packet is sent from system under test, containing at least a receiver report block reporting on the RTP stream from data injection.

Pass criteria:	The report block reporting on the RTP stream from data injection fulfils:
1) The fraction lost report block field is zero.
2) The cumulative number of packets lost report block field is zero.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.

#### 6.2.6.5	Zero Loss Test

Purpose:	Test if loss-free reception of RTP packets during a reporting period is correctly reflected in the RTCP report.

Status:	Mandatory

Preconditions:	1) The system under test has passed test 6.2.6.4.
2) The system under test is set to receive RTP packets during the test session, e.g. pre-configured or set as send-receive or receive-only through test instrument signalling.
3) Data injection is set to send a single RTP stream (one SSRC) without loss and with monotonically increasing RTP sequence number (SN).

Test procedure:	Observe system under test and data injection output.

Stop condition:	Two RTCP packets are sent from system under test, each containing at least a receiver report block reporting on the RTP stream from data injection.

Pass criteria:	The report block in RTCP packets sent from the system under test, reporting on the RTP stream from data injection fulfils:
1) The fraction lost report block field is zero in the second RTCP packet.
2) The cumulative number of packets lost report block field value is identical in the first and second RTCP packet.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.

#### 6.2.6.6	Loss Test

Purpose:	Test if the "fraction lost" and "cumulative number of packets lost" fields in the receiver report are consistent with one another and with the "extended last sequence number received" field.

Status:	Mandatory

Preconditions:	1) The system under test has passed tests 6.2.6.5 and 6.2.6.11.
2) The system under test is set to receive RTP packets during the test session, e.g. pre-configured or set as send-receive or receive-only through test instrument signalling.
3) Data injection is set to send a single RTP stream (one SSRC) with loss according to test procedure (below) and with monotonically increasing RTP sequence number (SN).

[Test procedure:	1) Observe system under test and data injection output.
2) Note the extended last sequence number received (si), fraction lost (fi) and cumulative number of packets lost (ci) for each RTCP report block sent from the system under test reporting on the RTP stream sent from data injection.
3) An RTCP packet, i=1, is sent from the system under test, reporting on the RTP stream from data injection.
4) Drop RTP packets from data injection to the system under test according to (one sub-bullet per repetition of steps 4-9): 
	a) A single RTP packet; i=2, n2=1.
	b) Two RTP packets in succession; i=3, n3=2.
	c) Two RTP packets with three RTP packets in between; i=4, n4=2.
	d) Every 20th RTP packet; i=5, n5=N5203.
	e) Every 10th RTP packet; i=6, n6=N6102.
5) An RTCP packet, i, is sent from the system under test, reporting on the RTP stream from data injection.
6) Calculate number of packets sent Ni=si-si-1.
7) Calculate fraction lost Fi=256niNi, with ni set according to 9a) – 9e) below.
8) Calculate packets lost Li=ci-ci-1.
9) Repeat from step 4.
	a) A single RTP packet; i=2, n2=1.
	b) Two RTP packets in succession; i=3, n3=2.
	c) Two RTP packets with three RTP packets in between; i=4, n4=2.
	d) Every third RTP packet; i=5, n5=N53.
	e) Every other RTP packet; i=6, n6=N62.

Stop condition:	Step 8 is fulfilled for i=6, repetition 4e).

Pass criteria:	1) fi=Fi, for all i∈2..6.
2) Li=ni, for all i∈2..6.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.
It is assumed that the information in extended last sequence number received is correct and enough to correlate with actual, observed RTP packet loss.]

#### 6.2.6.7	Wrapped Loss Test

Purpose:	Test if the "fraction lost" and "cumulative number of packets lost" fields in the receiver report are consistent with one another and with the "extended last sequence number received" field, even when the RTP "sequence number" field wraps around.

Status:	Conditionally Mandatory (Precondition 4)

Preconditions:	1) The system under test has passed tests 6.2.6.6 and 6.2.6.12.
2) The system under test is set to receive RTP packets during the test session, e.g. pre-configured or set as send-receive or receive-only through test instrument signalling.
3) Data injection is set to send a single RTP stream (one SSRC) with loss according to test procedure (below).
4) The "sequence number" (SN) field in the first RTP packet sent from data injection is set to a value close to 216-1 (when interpreted as an unsigned 16-bit integer), such that the value will wrap during the test.

Test procedure:	1) Observe system under test and data injection output.
2) Note the extended last sequence number received (ei), fraction lost (fi) and cumulative number of packets lost (ci) for each RTCP report block sent from the system under test reporting on the RTP stream sent from data injection.
3) An RTCP packet, i=1, is sent from the system under test, reporting on the RTP stream from data injection.
4) Drop RTP packets from data injection to the system under test according to (one sub-bullet per repetition of steps 4-9): 
	a) A single RTP packet; i=2, n2=1.
	b) Two RTP packets in succession; i=3, n3=2.
	c) Two RTP packets with three RTP packets in between; i=4, n4=2.
	d) Every 20th RTP packet; i=5, n5=N520.
	e) Every 10th RTP packet; i=6, n6=N610.
5) An RTCP packet, i, is sent from the system under test, reporting on the RTP stream from data injection.
6) Calculate number of packets sent Ni=ei-ei-1.
7) Calculate fraction lost Fi=256niNi, with ni set according to 9a) – 9e) below.
8) Calculate packets lost Li=ci-ci-1.
9) Repeat from step 4.
	i=2, n2=1
	i=3, n3=2
	i=4, n4=2
	i=5, n5=N53
	i=6, n6=N62

Stop condition:	Step 8 is fulfilled for i=6, repetition 4e).

Pass criteria:	1) fi=Fi, for all i∈2..6.
2) Li=ni, for all i∈2..6.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.
It is assumed that the information in extended last sequence number received is correct and enough to correlate with actual, observed RTP packet loss.

#### 6.2.6.8	Duplicate Loss Test

Purpose:	Test if the "fraction lost" and "cumulative number of packets lost" fields in the receiver report are consistent with one another and with the "extended last sequence number received" field, even when receiving duplicate RTP packets (with duplicate values in "sequence number" field).

Status:	Conditionally Mandatory (Precondition 4)

Preconditions:	1) The system under test has passed test 6.2.6.6.
2) The system under test is set to receive RTP packets during the test session, e.g. pre-configured or set as send-receive or receive-only through test instrument signalling.
3) Data injection is set to send a single RTP stream (one SSRC) with loss according to test procedure (below).
4) Data injection is set to send some RTP packets multiple times according to test procedure (below).

Test procedure:	1) Observe system under test and data injection output.
2) Note the extended last sequence number received (ei), fraction lost (fi) and cumulative number of packets lost (ci) for each RTCP report block sent from the system under test reporting on the RTP stream sent from data injection.
3) An RTCP packet, i=1, is sent from the system under test, reporting on the RTP stream from data injection.
4) Drop and repeat RTP packets from data injection to the system under test according to (one sub-bullet per repetition of steps 4-9): 
	a) Drop a single RTP packet, repeat a single non-dropped RTP packet;
		i=2, n2=1, r2=1.
	b) Drop two RTP packets in succession, repeat a single non-dropped RTP packet;
		i=3, n3=2, r2=1.
	c) Drop two RTP packets with three RTP packets in between, repeat three non-dropped RTP packets;
		i=4, n4=2, r2=3.
	d) Drop every 20th RTP packet, repeat every 20th non-dropped RTP packet;
		i=5, n5=N5320, r5=219N59400.
	e) Drop every 10th RTP packet, repeat every 20th non-dropped RTP packet;
		i=6, n6=N6210, r5=9N56200.
5) An RTCP packet, i, is sent from the system under test, reporting on the RTP stream from data injection.
6) Calculate number of packets sent Ni=ei-ei-1.
7) Calculate fraction lost Fi=256ni-riNi, with ni and ri set according to 4a) – 4e) above.
8) Calculate packets lost Li=ci-ci-1.
9) Repeat from step 4.
	a) Drop a single RTP packet, repeat a single non-dropped RTP packet;
		i=2, n2=1, r2=1.
	b) Drop two RTP packets in succession, repeat a single non-dropped RTP packet;
		i=3, n3=2, r2=1.
	c) Drop two RTP packets with three RTP packets in between, repeat three non-dropped RTP packets;
		i=4, n4=2, r2=3.
	d) Drop every third RTP packet, repeat every third non-dropped RTP packet;
		i=5, n5=N53, r5=2N59.
	e) Drop every other RTP packet, repeat every third non-dropped RTP packet;
		i=6, n6=N62, r5=N56.

Stop condition:	Step 8 is fulfilled for i=6, repetition 4e).

Pass criteria:	1) fi=maxi0, Fi, for all i∈2..6.
2) Li=ni-ri, for all i∈2..6.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.
It is assumed that the information in extended last sequence number received is correct and enough to correlate with actual, observed RTP packet loss.

#### 6.2.6.9	Out-of-sequence Loss Test

Purpose:	Test if the "fraction lost" and "cumulative number of packets lost" fields in the receiver report are consistent with one another and with the "extended last sequence number received" field, even when receiving RTP packets with the "sequence number" field not monotonically incrementing.

Status:	Conditionally Mandatory (Precondition 3)

Preconditions:	1) The system under test has passed test 6.2.6.12.
2) The system under test is set to receive RTP packets during the test session, e.g. pre-configured or set as send-receive or receive-only through test instrument signalling.
3) Data injection is set to send a single RTP stream (one SSRC) where the "sequence number" field in the RTP packet header is not generated with a constant increment of 1 (n, n+1, n+2, …) but with alternating increments of -1 and 3 (n+1, n, n+3, n+2, …).

Test procedure:	1) Observe the system under test and data injection output.
2) Note the extended last sequence number received (ei), fraction lost (fi) and cumulative number of packets lost (ci) for each RTCP report block sent from the system under test reporting on the RTP stream sent from data injection.
3) An RTCP packet, i=1, is sent from the system under test, reporting on the RTP stream from data injection.
4) Drop and repeat RTP packets from data injection to the system under test according to (one sub-bullet per repetition of steps 4-9): 
	a) Drop a single RTP packet;
		i=2, n2=1.
	b) Drop two RTP packets in succession;
		i=3, n3=2.
	c) Drop two RTP packets with three RTP packets in between;
		i=4, n4=2.
	d) Drop every 20th RTP packet;
		i=5, n5=N5320.
	e) Drop every 10th RTP packet;
		i=6, n6=N6210.
5) An RTCP packet, i, is sent from the system under test, reporting on the RTP stream from data injection.
6) Calculate number of packets sent Ni=ei-ei-1.
7) Calculate fraction lost Fi=256niNi, with ni set according to 4a) – 4e) above.
8) Calculate packets lost Li=ci-ci-1.
9) Repeat from step 4.
	a) Drop a single RTP packet;
		i=2, n2=1.
	b) Drop two RTP packets in succession;
		i=3, n3=2.
	c) Drop two RTP packets with three RTP packets in between;
		i=4, n4=2.
	d) Drop every third RTP packet;
		i=5, n5=N53.
	e) Drop every other RTP packet;
		i=6, n6=N62.

Stop condition:	Step 8 is fulfilled for i=6, repetition 4e).

Pass criteria:	1) fi=Fi, for all i∈2..6.
2) Li=ni-ri, for all i∈2..6.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.

#### 6.2.6.10	All Loss Test

Purpose:	Test if receiving no RTP packets during one or more reporting periods is correctly reflected in the RTCP reports sent when receiving RTP packets again.

Status:	Mandatory

Preconditions:	1) The system under test has passed tests 6.2.6.4 and 6.2.6.5.
2) The system under test is set to receive RTP packets during the test session, e.g. pre-configured or set as send-receive or receive-only through test instrument signalling.
3) Data injection is set to send a single RTP stream (one SSRC) with loss according to the test procedure below.

Test procedure:	1) Observe system under test and data injection output.
2) Note the cumulative number of packets lost (ci) for each RTCP report block sent from the system under test reporting on the RTP stream sent from data injection.
3) RTP packets are sent from data injection.
4) An RTCP packet, i=1, is sent from the system under test, reporting on the RTP stream from data injection. 
5) All RTP packets are prevented from reaching the system under test and dropped (not buffered), without modifying the session as seen from the system under test, e.g. through proprietary data injection interaction or through transport network actions.
6) Note the sequence number s1 of the last RTP packet reaching the system under test before the RTP loss period.
6) At least two RTCP packets, i∈2..n, are sent from the system under test, reporting on the RTP stream from data injection (the first one likely covering the beginning of the all-loss period).
7) RTP packets are no longer prevented from reaching the system under test, without modifying the session as seen from the system under test, e.g. through proprietary data injection interaction or through transport network actions.
8) Note the sequence number s2 of the first RTP packet reaching the system under test after the RTP loss period.

Stop condition:	An RTCP packet, i=n+1, is sent from the system under test after step 8 in the test procedure.

Pass criteria:	The report block in RTCP packets sent from the system under test, reporting on the RTP stream from data injection fulfils:
1) RTCP packets with i∈3..n do not contain any report block for the SSRC used by data injection and that was subjected to loss in step 5 of the test procedure.
2) cn+1=c1+s2-s1.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not. s2 must be strictly larger than s1 (i.e., not wrapped around) for the pass criteria to work as currently formulated.

#### 6.2.6.11	Extended Highest Sequence Number Received Test

Purpose:	Test if the "extended last sequence number received" field in the receiver report is consistent with received RTP "sequence number" field.

Status:	Mandatory

Preconditions:	1) The system under test has passed test 6.2.6.1.
2) The system under test is set to receive RTP packets during the test session, e.g. pre-configured or set as send-receive or receive-only through test instrument signalling.
3) Data injection is set to send a single RTP stream (one SSRC).
4) The "sequence number" (SN) field in the first RTP packet sent from data injection is monotonously increasing (see 6.2.6.12 for test of wrapped value).

Test procedure:	1) Observe the system under test and data injection output.
2) Note the "extended last sequence number received" field (ei) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets (n) sent from the system under test.
4) Count the number of sent RTP packets (Pi) sent from data injection between each pair i-1, i of subsequent RTCP packets sent from the system under test.
5) Note the "sequence number" field (si,j) of each RTP packet, with j∈1..Pi.
6) Note the RTP packet number, li=j, where si,j=ei % 216, or if no RTP packet with such si,j exists, set li=0.

Stop condition:	1) RTP packets are sent from data injection.
2) At least three RTCP packets are sent from the system under test after the first RTP packet from data injection (one or more RTCP packets may be sent before first RTP packet).

Pass criteria:	li∈1..Pi, for all i∈2..n.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.

#### 6.2.6.12	Wrapped Extended Highest Sequence Number Received Test

Purpose:	Test if the "extended last sequence number received" field in the receiver report is consistent with received RTP "sequence number" field, even when the "sequence number" field wraps around, and the extended part above 16 bits is properly incremented after wrap.

Status:	Conditionally Mandatory (Precondition 4)

Preconditions:	1) The system under test has passed test 6.2.6.11.
2) The system under test is set to receive RTP packets during the test session, e.g. pre-configured or set as send-receive or receive-only through test instrument signalling.
3) Data injection is set to send a single RTP stream (one SSRC).
4) The "sequence number" (SN) field in the first RTP packet sent from data injection is set to a value close to 216-1 (when interpreted as an unsigned 16-bit integer), such that the value will wrap during the test.

Test procedure:	1) Observe the system under test and data injection output.
2) Note the "extended last sequence number received" field (ei) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets (n) sent from the system under test.
4) Count the number of sent RTP packets (Pi) sent from data injection between each pair i-1, i of subsequent RTCP packets sent from the system under test.
5) Note the "sequence number" field (si,j) of each RTP packet, with j∈1..Pi.
6) Note the RTP packet number, li=j, where si,j=ei % 216, or if no RTP packet with such si,j is received, set li=0.

Stop condition:	1) RTP packets are sent from data injection.
2) At least three RTCP packets are sent from the system under test after the first RTP packet from data injection (one or more RTCP packets may be sent before first RTP packet).
3) s1,1>sn-1,Pn-1 when interpreted as unsigned 16-bit integers.

Pass criteria:	1) li∈1..Pi, for all i∈2..n.
2) en≫16-e1≫16=1.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.

#### 6.2.6.13	Out-of-sequence Extended Highest Sequence Number Received Test

Purpose:	Test if the "extended last sequence number received" field in the receiver report is consistent with received RTP "sequence number" field, even when receiving RTP packets with the "sequence number" field not monotonically incrementing.

Status:	Conditionally Mandatory (Precondition 3)

Preconditions:	1) The system under test has passed test 6.2.6.11.
2) The system under test is set to receive RTP packets during the test session, e.g. pre-configured or set as send-receive or receive-only through test instrument signalling.
3) Data injection is set to send a single RTP stream (one SSRC) where the "sequence number" field in the RTP packet header is not generated with a constant increment of 1 (n, n+1, n+2, …) but with alternating increments of -1 and 3 (n+1, n, n+3, n+2, …).

Test procedure:	1) Observe the system under test and data injection output.
2) Note the "extended last sequence number received" field (ei) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets (n) sent from the system under test.
4) Count the number of sent RTP packets (Pi) sent from data injection between each pair i-1, i of subsequent RTCP packets sent from the system under test.
5) Note the "sequence number" field (si,j) of each RTP packet, with j∈1..Pi.
6) Note the RTP packet number, li=j, where si,j=ei % 216, or if no RTP packet with such si,j is received, set li=0.

Stop condition:	1) RTP packets are sent from data injection.
2) At least three RTCP packets are sent from the system under test after the first RTP packet from data injection (one or more RTCP packets may be sent before first RTP packet).

Pass criteria:	1) li∈1..Pi, for all i∈2..n.
2) ei % 216= maxjsi,j,for all i∈2..n and j∈1..li.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.

#### 6.2.6.14	Interarrival Jitter Test

Purpose:	Test if the "interarrival jitter" field in the receiver report, as described by RTP [2] section 6.4.1, is consistent with received RTP packets.

Status:	Conditionally Mandatory (Precondition 3)

Preconditions:	1) The system under test has passed tests 6.2.4.2, 6.2.4.4, and 6.2.6.1.
2) The system under test is set to receive RTP packets during the test session, e.g. pre-configured or set as send-receive or receive-only through test instrument signalling.
3) Data injection is set to send a single RTP stream (one SSRC) that is subjected to delay variations according to delay and loss profiles from TS 26.132 [11] Annex E.3.

Test procedure:	1) Observe the system under test and data injection output.
2) Retrieve RTP Time Stamp rate information (S), e.g. from "<clock rate>" on the SDP "a=rtpmap:<payload type> <encoding name>/<clock rate>" line, applicable for the RTP stream that the RTCP packets are associated with.
3) For each delay and loss profile k from dly_profile_20msDRX_10pctBLER_e2e and dly_profile_20msDRX_10pctBLER_ue1_to_eNB2 in TS 26.131 [11] Annex E.3, repeat the following steps:
4) Start data injection sending RTP packets subjected to delay variations according to the delay and loss profile.
5) Note the reception time (tk,i) and "interarrival jitter" (jk,i) field in the report block of each RTCP packet sent from the system under test.
6) Count the number of RTCP packets (nk) sent from the system under test.
7) Count the number of sent RTP packets (Pk,i) sent from data injection between each pair i-1, i of subsequent RTCP packets sent from the system under test.
8) Note the reception time (rk,i,j) and "timestamp" field (Tk,i,j) of each RTP packet, with j∈1..Pk,i.
9) Calculate the arrival time ak,i,j=Tk,1,1+rk,i,j-rk,1,1S in timestamp units of each RTP packet, with j∈1..Pk,i, and where rk,1,1 is the reception time of the first received RTP packet with timestamp Tk,1,1.
10) Calculate the difference in packet spacing Dk,i,j=ak,i,j-Tk,i,j-ak,i,j-1-Tk,i,j-1, with j∈2..Pk,i.
11) Calculate (not using integer arithmetic) the filtered packet jitter Jk,i=j=2Pk,isj, where sj=Dk,i,j-sj-116, with s1=Jk,i-1, where Jk,i-1 corresponds to the filtered packet jitter reported in previous RTCP packet, and Jk,1=0.
12) When there are no more delay and loss profile data to apply to RTP packets, stop sending RTP packets to avoid further impact to Jk,i.

Stop condition:	1) RTP packets are sent from data injection.
2) At least one RTCP packet is sent from the system under test after the last RTP packet from data injection, for each delay and loss profile.

Pass criteria:	jk,nk=Jk,nk, for all k, where expected, nominal values based only on delay and loss profile content, excluding any data injection implementation impact, are (for information):

Table 6.2.6.14-1 Examples of delay and loss profile jitter

| Delay and loss profile | k | nk | Jk,nk S=8000 | Jk,nk S=16000 |
| --- | --- | --- | --- | --- |
| dly_profile_20msDRX_10pctBLER_e2e | 1 | 8000 | 93 | 186 |
| dly_profile_20msDRX_10pctBLER_ue1_to_eNB2 | 2 | 8000 | 27 | 55 |

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.

#### 6.2.6.15	Duplicate Interarrival Jitter Test

Purpose:	Test if the "interarrival jitter" field in the receiver report, as described by RTP [2] section 6.4.1, is consistent with received RTP packets, even in the presence of duplicate RTP packets.

FFS.

#### 6.2.6.16	Last Sender Report Timestamp Test

Purpose:	Test if the LSR field in the receiver report from the system under test corresponds to middle 32 bits of "NTP timestamp" 64-bit field for some previous RTCP sender report sender info received from data injection.

Status:	Mandatory

Preconditions:	1) The system under test has passed test 6.2.2.7.
2) The system under test is set to receive RTP packets during the test session, e.g. pre-configured or set as send-receive or receive-only through test instrument signalling.
3) Data injection is set to send a single RTP stream (one SSRC).

Test procedure:	1) Observe the system under test and data injection output.
2) Note the LSR field (Li) in the report block of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets (n) sent from the system under test.
4) Note the "NTP timestamp, most significant word" (mj) and "NTP timestamp, least significant word" (lj) fields in the sender information of each RTCP packet sent from data injection.
5) Count the number of RTCP packets (p) sent from data injection.
6) Calculate the reduced-precision last SR NTP time Nj=mj & 216-1≪216+lj≫216 for each RTCP packet sent from data injection, with j∈1..p.
7) Note the RTCP packet number sent from data injection, qi=j, where Li=Nj, or if no RTCP packet with such Nj exists, set qi=0.

Stop condition:	1) RTP packets are sent from data injection.
2) RTCP packets are sent from data injection.
3) At least three RTCP packets with Li≠0 are sent from the system under test after the first RTCP packet from data injection.

Pass criteria:	qi>0, for at least one i∈1..n.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.

#### 6.2.6.17	Delay Since Last SR Test

Purpose:	Test if the reception time and delay since last SR field in the receiver report from the system under test is consistent with observed reception time and NTP timestamp field of the corresponding RTCP sender report sender info from data injection.

Status:	Mandatory

Preconditions:	1) The system under test has passed test 6.2.6.16.
2) The system under test is set to receive RTP packets during the test session, e.g. pre-configured or set as send-receive or receive-only through test instrument signalling.
3) Data injection is set to send a single RTP stream (one SSRC).

Test procedure:	1) Observe the system under test and data injection output.
2) Note the LSR (Li) and "delay since last LSR" (DLSR) (Di) fields in the report block and reception time (ti) of each RTCP packet sent from the system under test.
3) Count the number of RTCP packets (n) sent from the system under test.
4) Note the "NTP timestamp, most significant word" (mj) and "NTP timestamp, least significant word" (lj) fields in the sender information, and reception time (rj) of each RTCP packet sent from data injection.
5) Count the number of RTCP packets (p) sent from data injection.
6) Calculate the reduced-precision last SR NTP time Nj=mj & 216-1≪216+lj≫216 for each RTCP packet sent from data injection, with j∈1..p.
7) Calculate the data injection reporting interval sj=rj-rj-1, for j∈2..p.
8) Note the RTCP packet number sent from data injection, qi=j, where Li=Nj, or if no RTCP packet with such Nj exists, set qi=0.
9) Calculate the observed round-trip reporting time, di=rqi-ti for all i with qi≠0.
10) Calculate the round-trip time, Ri=rqi-ti-Di65536 for all i with qi≠0.

Stop condition:	1) RTP packets are sent from data injection.
2) RTCP packets are sent from data injection.
3) At least three RTCP packets with Li≠0 and Di≠0 are sent from the system under test after the first RTCP packet from data injection.

Pass criteria:	1) di≥Di65536 for all i with qi≠0.
2) Di65536≤maxjsj for all i with qi≠0 and j∈2..p.

Comments:	The test result is agnostic to RTP packets being sent by the system under test or not.

### 6.2.7	Feedback Report Block Tests

#### 6.2.7.1	Ignoring Unknown Feedback Report Test

Purpose:	Test if receiving an RTCP feedback report of unknown (non-negotiated) type is correctly ignored without negative impact on other RTP/RTCP operation.

FFS.

#### 6.2.7.2	AVPF Transmission Timing Test

Purpose:	Test if sending an RTCP feedback report uses AVPF timing.

FFS.

### 6.2.8	Extended Report Block Tests

#### 6.2.8.1	Ignoring Unknown XR Test

Purpose:	Test if receiving an RTCP XR packet of unknown (non-negotiated) type is correctly ignored without negative impact on other RTP/RTCP operation.

FFS.

### 6.2.9	APP Tests

#### 6.2.9.1	Ignoring Unknown APP Test

Purpose:	Test if receiving an RTCP APP packet with unknown name is correctly ignored without negative impact on other RTP/RTCP operation.

FFS.

### 6.2.10	Reduced-Size Packet Tests

#### 6.2.10.1	Ignoring Unsupported Reduced-Size Test

Purpose:	Test if receiving a reduced-size RTCP packet when its use is not negotiated is correctly ignored without negative impact on other RTP/RTCP operation.

FFS.

End of document