3GPP TSG-SA4 Meeting #107	S4-200215

# Wroclaw, Poland,  Jan 20 - 24, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.234 | CR | <CR#> | rev | <Rev#> | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PSS VR metrics support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | VRQoE |  |  |  |  |  | Date: |  |  | 2020-01-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The work item VRQoE has added metrics definitions into TS 26.118, but there is no support requirement in PSS specifications yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a requirement in PSS that a VR client should support these metrics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VR metrics not supported in PSS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ================ First change =================

### 7.4.5	360 video and 3D audio for VR (Virtual Reality)

#### 7.4.5.1	Video

##### 7.4.5.1.1	Operation Points

If the PSS client supports 360 VR video, it shall include a receiver that complies with

-	the Basic H.264/AVC Operation Point Receiver requirements as specified in TS 26.118 [121], clause 5.1.4.

If the PSS client supports 360 VR video, it should include a receiver that complies with

-	the Main H.265/HEVC Operation Point Receiver requirements as specified in TS 26.118 [121], clause 5.1.5.

If the PSS client supports 360 VR video, it may include a receiver that complies with

-	the Flexible H.265/HEVC Operation Point Receiver requirements as specified in TS 26.118 [121], clause 5.1.6.

##### 7.4.5.1.2	Progressive Download

If the 3GPP PSS client supports 360 VR video for Progressive Download services, it shall include a receiver that complies with

-	the Basic Video Media Profile Receiver requirements for file format signalling and encapsulation as specified in TS 26.118 [121], clause 5.2.2.2.

If the 3GPP PSS client supports 360 VR video for Progressive Download services, it should include a receiver that complies with

-	the Main Video Media Profile Receiver requirements for file format signalling and encapsulation as specified in TS 26.118 [121], clause 5.2.3.2.

If the 3GPP PSS client supports 360 VR video for Progressive Download services, it may include a receiver that complies with

-	the Advanced Video Media Profile Receiver requirements for file format signalling and encapsulation as specified in TS 26.118 [121], clause 5.2.4.2.

NOTE: This profile requires extensions beyond the 3GP file format that are only defined in TS26.118 [121].

##### 7.4.5.1.3	DASH

If the 3GPP PSS client supports 360 VR video for DASH services, it shall include a receiver that complies with

-	the Basic Video Media Profile Receiver requirements for DASH as specified in TS 26.118 [121], clause 5.2.2.3.

If the 3GPP PSS client supports 360 VR video for DASH services, it should include a receiver that complies with

-	the Main Video Media Profile Receiver requirements for DASH as specified in TS 26.118 [121], clause 5.2.3.3.

If the 3GPP PSS client supports 360 VR video for DASH services, it may include a receiver that complies with

-	the Advanced Video Media Profile Receiver requirements for DASH as specified in TS 26.118 [121], clause 5.2.3.4.

#### 7.4.5.2	Audio

##### 7.4.5.2.1	Operation Points

If the PSS client supports 3D/VR audio, it should include a receiver that complies with

-	the 3GPP MPEG-H Audio Operation Point Receiver requirements as specified in TS 26.118 [121], clause 6.1.4.

##### 7.4.5.2.2	Progressive Download

If the 3GPP PSS client supports 3D/VR audio for Progressive Download services, it should include a receiver that complies with

-	the OMAF 3D Audio Baseline Media Profile Receiver requirements for file format signalling and encapsulation as specified in TS 26.118 [121], clause 6.2.2.2.

##### 7.4.5.2.3	DASH

If the 3GPP PSS client supports 3D/VR audio for DASH services, it should include a receiver that complies with

-	the OMAF 3D Audio Baseline Media Profile Receiver requirements for file format signalling and encapsulation as specified in TS 26.118 [121], clause 6.2.2.3.

#### 7.4.5.3	Metrics

If the 3GPP PSS client supports 360 VR video for Progressive Download or DASH services, it should include a receiver that complies with

-	the VR Metrics requirements as specified in TS 26.118 [121], clause 9.

## ================ End of change =================