Source:	Huawei Technologies Co.,Ltd

Title:	Two field issues with EVS - Revised

Document for:	Agreement

Agenda Item:	7.3

1. Introduction

During our field tests with mobile phones running in EVS mode, we have encountered two voice quality issues that can be reproduced. It is our opinion that those might relate to some undiscovered bugs in the reference codes of EVS. It is recommended that SA4 arranges an effort to verify these issues and take corrective action to address them.

2 Description of the Issues

2.1 Issue 1: Noise with fix-point EVS decoder

Using a particular test sequence, we found that the mobile phone, running fixed point EVS decoder, outputs some strong single frequency pulse noises. This problem can be reproduced by feeding the same test sequence to the fixed-point reference code (EVS codec 3GPP ts26.442 Nov 13, 2018. Version 12.12.0 / 13.7.0 / 14.3.0 / 15.1.0), illustrated in Fig 1. In comparison, the floating-point reference code (EVS codec 3GPP ts26.443 APR 03, 2018. Version 12.10.0 / 13.6.0 / 14.2.0 / 15.0.0) does not have this type of output signals using the same test sequence, as shown in Fig 2. The sampling rate of the input data is 32KHz and the format is MIME.

Command Line:

./EVS_dec –mime 32  Stream1_swb_13200.bit output.pcm

Similar problems have occurred three times, three different test sequences can trigger this problem.

2.2 Issue 2: Noise from decoding inactive part

In another test, a loud noise appeared after decoding inactive part with EVS-IO mode . During the test, the network had an abnormality resulting in all the frames sent to the decoder were bad frames,. Using EVS standard reference codes one can reproduce this problem, whether it is the fixed-point or the floating-point reference codes, as shown in Fig 3. In comparison, using AMRWB to decode a similar abnormal bitstream, the result is good, and there is NO loud noise, as in Fig 4. The sampling rate of the input data for this test is 16KHz and the format is MIME.

Command Line:

./EVS_dec –mime 16 Stream2_wb-12650.bitoutput.pcm

3. Conclusion

The noises caused by the two issues are very obvious and disturbing, which seriously impact the user experience. With the rapid evolution of the network, it is expected that the number of calls using EVS will increase dramatically, even such a small probability problem would be amplified to a large scale. Therefore, it is important and necessary for SA4 to arrange an effort to confirm and to fix these issues.

- Bit streams

Stream1_swb_13200.bit

Stream2_wb-12650.bit

Both are included in the zip file.

Annex A: Comparison of the PCM stream In ISSUE 1

Figure 1. Decoding with Fix-point Code

Figure 2. Decoding with Float-point Code

Annex B: Comparison of the PCM stream In ISSUE 2

Figure 3. Decoding with EVS

Figure 4. Decoding with AWRWB