3GPP TSG-SA4 Meeting #108-e	S4-200576

# 2-9 April, 2020, Electronic Meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | <CR#> | rev | <Rev#> | Current version: | 16.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Collaboration Model: Multi-MNO Distribution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The different collaboration scenarios were frequently discussed in SA4, including Multi-CDN scenarios. Today, most content provider offer their content via all available Internet Service Providers, incl mobile networks. The most obvious and most common collaboration, namely the multi MNO distribution collaboration, was verbally discussed, but has not made it into the specification. Further, when the 5GMSd AS for content hosting hosting is located in the external data network, the service access information from the trusted 5GMSd AF does not contain an DASH Manifest or a progressive streaming URL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The service access information is corrected and all parameters are (conditional) optional. Two new multi MNO collaboration scenarios, focusing on using network services from different MNOs, is added to the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Most common and important collaboration scenarios are not considered in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.7, Annex A.8 (new), Annex A.9 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 4.2.3	Service Access Information

The Service Access Information is the set of parameters and addresses, which are needed by the 5GMSd Client to activate the reception of a downlink streaming session or other 5GMSA network features.

The Service Access Information may be provided together with other service announcement information using M8d. Alternatively, the 5GMSd Client fetches the Service Access Information from the 5GMSd AF.

The Service Access Information contains different information, depending on the collaboration model between 5GMSA system and 5GMSd Application Provider and on on offered features.

Table 4.2.3-1: Parameters of baseline service access information

| Parameters | Description |
| --- | --- |
| Media player entry | A document or a pointer to a document that defines a media presentation e.g. MPD for DASH content or URL to a video clip file. |
| URL signing information | …. |
| 5GMSd Application Service Configuration Id | Unique identification of the M1d Provisioning Session. |

When consumption reporting is activated, then the parameters from Table 4.2.3-2 are present.

Table 4.2.3-2: Parameters for consumption reporting configuration

| Parameters | Description |
| --- | --- |
| Report interval | Identify the interval when the consumption report messages are sent by the 5GMS Media Session Handler. |
| Server address | A list of 5GMSd AF addresses where the consumption reporting messages are sent by the 5GMS Media Session Handler. |
| Location reporting | Identify whether the 5GMS Media Session Handler provides the location data to the Media AF (in case of MNO or trusted third parties) |

Table 4.2.3-3: Parameters for dynamic policy invocation configuration

| Parameters | Description |
| --- | --- |
| Server address | A list of 5GMSd AF addresses which offer the APIs for dynamic policy invocation sent by the 5GMS Media Session Handler. . (Opaque URL, following the 5GMSA URL format) |
| Policy Template Ids | A list of Policy Template Ids, which the 5GMSd Client is authorized to use. |

Table 4.2.3-4: Parameters for metrics configuration

| Parameters | Description |
| --- | --- |
| Server address | A list of 5GMSd AF addresses to which metric reports shall be sent. |
| DNN | The DNN which shall be used when sending metrics report. If not specified, the default DN shall be used. |
| Reporting interval | The sending interval between metrics reports. If not specified, a single final report shall be sent after the streaming session has ended. |
| Sample percentage | The percentage of streaming sessions that shall report metrics. If not specified, reports shall be sent for all sessions |
| Streaming source filter | A list of URL patterns for which metrics reporting shall be done. If not specified, reporting shall be done for all sessions. |
| Metrics | A list of metrics which shall be reported. |

Table 4.2.3-5: Parameters for Network Assistance configuration

**** Next Change ****

# A.7.	Collaboration 7

This collaboration scenario represents a MNO CDN scenario (like in Collaboration 2), where the CDN is used for ingest and delivery of the content. Additional 5GMSA features are used which require interactions with the PCF.

Figure A.7-1: Collaboration 7

# A.8.	Collaboration 8

This collaboration scenario represents a multi MNO distribution scenario, where an external CDN (5GMSd AS) is used to delivery content through multiple MNO networks. Additional 5GMSA features are used from the serving Mobile Network which lead to PCF interactions of the serving Network.

Figure A.8-1: Collaboration 8

# A.9.	Collaboration 9

This collaboration scenario represents a multi MNO distribution scenario, where an external CDN (5GMSd AS) is used to delivery content through multiple MNO networks. Additional 5GMSA features are used from the serving Mobile Network which lead to NEF interactions of the serving Network.

Figure A.9-1: Collaboration 9

**** Last Change ****