3GPP TSG-SA4 Meeting #108 	S4-200588

# Electronic,  , 6th–9th April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.512 | CR | TBA | rev | – | Current version: | x.X.X |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Consumption Reporting Procedure API- M1d and M5d |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Enensys |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 2020-03-XX |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add consumption Reporting APIs in M1d and and the support of Sample percentage in Consumption Reporting API in M5D |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fill the section 4.2.5 with the operations used to configure the Consumption Reporting Add a new API for consumption reporting procedure – M1d Add the support of Sample percentage in the M5d APIs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It would be impossible to configure the consumption report in interface M1d |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.5, 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

================================START OF FIRST CHANGE==============================

### 4.2.5	Consumption Reporting Configuration procedures

#### 4.2.5.1	General

These procedures are used by the 5GMSd Application Provider to activate and to configure consumption reporting. They are further elaborated in clause 5.4.2

#### 4.2.5.2	Activate Consumption reporting Configuration

This procedure is used by the 5GMSd Application Provider to activate the Consumption Reporting procedure. The 5GMSd Application Provider shall use the HTTP POST method to activate the Consumption reporting procedure and to transmit the Consumption Reporting Configuration to the 5GMSd AF. Upon successful operation, the 5GMSd AF shall respond with a 201 (Created) response message.

#### 4.2.5.3	Read Provisioning Session properties

This procedure is used by the 5GMSd Application Provider to obtain the Consumption Reporting Configuration from the 5GMSd AF. The 5GMSd Application Provider uses the GET method for this purpose.

#### 4.2.5.4	Update Provisioning Session properties

The update operation is invoked by the 5GMSd Application Provider to modify the Consumption Reporting Configuration. All available parameters may be updated. The HTTP PATCH or HTTP PUT methods shall be used for the update operation.

If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) reflecting the successful update operation.

#### 4.2.5.5	Delete Provisioning Session

This operation is used by the 5GMSd Application Provider to terminate the related consumption reporting procedure. The HTTP DELETE method shall be used for this purpose. As a result, the 5GMSd AF will release any associated resources, purge any cached data, and delete any corresponding configurations.

If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) response message.

=================================END OF FIRST CHANGE===============================

===============================START OF SECOND CHANGE=============================

## 5.4	Consumption Reporting APIs

### 5.4.1	General

The consumption Reporting Procedure is involved in the following interface M1d, M5d and M7d. Each APIs is described in a particular section

Editor Will this specification include M7d APIs?

### 5.4.2	M1d Consumption Reporting APIs

#### 5.4.2.1General

#### This API is used to manage the Consumption Reporting Procedure for a particular session in the interface M1d. The different procedures are described in section 4.2.5.

#### 5.4.2.2 Data Model

#### 5.4.2.2.1 	Consumption Reporting Configuration – M1d

This type represents the Consumption Reporting Configuration..

Table 5.4.2.3-1: Consumption Reporting Configuration type

| Attribute name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| reportInterval | DurationSec | 0..1 | Identify the interval between two consecutive consumption reporting messages. If Absent, the 5GMSd AF can set the value. |
| Sample percentage | PercentType | 0..1 | The proportion of clients that shall report media consumption. If not specified, all clients shall send reports |
| locationType | integer | 0..1 | Identify the UE location type if location reporting is enabled (only for trusted AF). CGI, ECGI and NCGI shall be represented by the values 0, 1 and 2, respectively (See [7]). If not present, the location is unabled. |

### 5.4.2 Resource structure

The Consumption Reporting provisioning API is accessible through this URL path:

{apiRoot}/3gpp m1d/v1/application service providers/{application service provider id}/provisioning sessions/{provisioning session id}/

The following operations and the corresponding HTTP methods are supported:

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Activate Consumption Reporting procedure with a Consumption Reporting Configuration | consumption reporting configuration | POST | This is used to activate the Consumption Reporting procedure and to set the Consumption Reporting Configuration. |
| Fetch Consumption Reporting Configuration | consumption reporting configuration | GET | This operation is used to retrieve an existing Consumption Reporting Configuration. |
| Update Consumption Reporting Configuration | consumption reporting configuration | PUT, PATCH | This operation is used to modify the configuration of an existing Ingest Configuration. |
| Delete Consumption Reporting procedure | consumption reporting configuration | DELETE | This operation is used to desactivate the Consumption Reporting procedure for that particular session. |

### 5.4.2 M5d Consumption Reporting APIs

### 5.4.2.1 General

The Consumption Reporting API is a RESTful API that allows the Media Session Handler to report media consumption to the 5GMSd AF. The API defines data models, resources and the related procedures for the creation and management of the consumption reporting procedures.

#### 5.4.2.2	Data model

##### 5.4.2.2.1	Introduction

This clause defines data structures to be used in consumption reporting procedure.

Table 5.4.2.2.1-1 specifies data types re-used by the Consumption Reporting API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the 5GMS Consumption Reporting API.

Table 5.4.2.2.1-1: 5GMS Consumption Reporting API re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| DurationSec | 3GPP TS 29.122 [x3] | Unsigned integer identifying a period of time in units of seconds. |
| domainNames | 3GPP TS 29.122 [x3] | Indicates an FQDN or a regular expression as a domain name matching criteria. |
| DateTime | 3GPP TS 29.122 [x3] | string with format "date-time" as defined in OpenAPI Specification [x4]. |

##### 5.4.2.2.2	ConsumptionReportingConfiguration resource

This type represents the different parameters that initialise the consumption report.

Table 5.4.2.2.1.2-1: Definition of ConsumptionReportingConfiguration resource

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- |
| reportInterval | DurationSec | 1..1 | Identify the interval between two consecutive consumption reporting messages. |  |
| consumptionReportingServerAddress | domainNames | 1..N | A list of addresses where the consumption reporting message shall be sent by the 5GMS Media Session Handler. |  |
| locationReporting | Boolean | 1..1 | Identify whether the 5GMS Media Session Handler is allowed to provide the location data in the consumption reporting message. |  |
| Sample percentage | PercentType | 0..1 | The proportion of clients that shall report media consumption. If not specified, all clients shall send reports |  |

##### 5.4.2.2.3	ConsumptionReporting resource

This type represents a consumption report data. This structure is used by the Media Session Handler to report the consumption.

Table 5.4.2.2.1.3-1: Definition of ConsumptionReporting resource

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- |
| mediaPlayerEntry | string | 1..1 | Identify the Media player entry. In case of DASH, the media player entry pointer can be a URL of the MPD. |  |
| reportingClientId | string | 1..1 | Identify the identifier of the UE that consumes data.The client ID can be a MSISDN. |  |
| locationType | integer | 0..1 | Identify the UE location type if location reporting is enabled (only for trusted AF). CGI, ECGI and NCGI shall be represented by the values 0, 1 and 2, respectively (See [7]) |  |
| location | string | 0..1 | Identify the UE location where the consumption media if location reporting is enabled (only for trusted AF). |  |
| consumptionReportingUnit | ConsumptionReportingUnit | 1..N | Identify a list of consumption reporting units. |  |

###### 5.4.2.2.1.4	Type: ConsumptionReportingUnit

This type represents each consumption report unit.

Table 5.4.2.2.1.4-1: Definition of type ConsumptionReportingUnit

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- |
| mediaConsummed | String | 1..1 | Identify the media consummed. In case of DASH, the Representation@id may be used. |  |
| startTime | DateTime | 1..1 | Define the time where the consumption starts of this unit. |  |
| duration | DurationSec | 1..1 | Identify the duration of the consumption of the quality of this unit. |  |

#### 5.4.2.3	Resource structure

##### 5.4.2.3.1	Introduction

Editor’s Note: URI definition shall be unified in this document. A global section where the URI is defined should be created. This section may define the URI as a global URI. It may also simplify the URI construction in a way that for instance {apiRoot}/3gpp-5gms-consumption-reporting/v1/ would be replaced by {5GMSAconsumptionReportApiRoot} where is just defined as a global URI by the AF without any constraint on the way this URI is build.
If this section exists, it will impact every Resource structure defined in this section.

All resource URIs of this API shall have the following root:

{apiRoot}/3gpp-5gms-consumption-reporting/v1/

"apiRoot" is set as described in subclause X. "apiName" shall be set to "3gpp-5gms-consumption-reporting " and "apiVersion" shall be set to "v1" for the version defined in the present document. All resource URIs in the subclauses below are defined relative to the above root URI.

The following resources and HTTP methods initiated by the 5GMS Client are supported for this API:

Table 5.4.2.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Meaning |
| --- | --- | --- | --- |
| Consumption Reporting Session | 3gpp-5gms-consumption-reporting/v1/{aspId} /session/ | GET | Forbidden. |
|  |  | PUT | Create a Consumption Reporting Session resource. |
| Consumption Reporting Configuration Acquisition | 3gpp-5gms-consumption-reporting/v1/{aspId} /session/{sessionId}/acquire | GET | Acquire consumption reporting configuration. |
|  |  | POST | Forbidden. |
| Consumption Reporting Data | 3gpp-5gms-consumption-reporting/v1/{aspId} /session/{sessionId} | GET | Forbidden. |
|  |  | POST | Report media consumption. |

##### 5.4.2.3.2	Consumption Reporting Session resource

##### 5.4.2.3.2.1	Introduction

This resource allows the Media Session Handler to create Consumption Reporting Data resources when the Consumption Reporting is used for a Downlink Streaming session.

##### 5.4.2.3.2.2	Resource definition

Resource URI:

{apiRoot}/3gpp-5gms-consumption-reporting/v1/{aspId}/session

This resource shall support the resource URI variables defined in table 5.X.2.3.2.2-1.

Table 5.4.2.3.2.2-1: Resource URI variables for resource "Consumption Reporting Session"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of [x3]. |
| aspId | Identifier of the 5GMS Application Provider. |

##### 5.4.2.3.2.3	Resource methods

###### 5.4.2.3.2.3.1	GET

This HTTP method is not supported for the resource.

###### 5.4.2.3.2.3.2	PUT

The PUT method allows the Media Session Handler create a Consumption Reporting resource. It is initiated by the Media Session Handler and answered by the 5GMSd AF.

This method shall support request and response data structures, and response codes, as specified in the table 5.4.2.3.2.3.2-1.

Table 5.4.2.3.2.3.2-1: Data structures supported by the PUT request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | None |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | ConsumptionReportingConfiguration | 0..1 | 201 OK | The consumption reporting parameters may be returned. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 [x3] also apply. |  |  |  |  |

###### 5.4.2.3.2.3.3	PATCH

This HTTP method is not supported for the resource.

###### 5.4.2.3.2.3.4	POST

This HTTP method is not supported for the resource.

###### 5.4.2.3.2.3.5	DELETE

This HTTP method is not supported for the resource.

##### 5.4.2.3.3	Consumption Reporting Configuration Acquisition resource

###### 5.4.2.3.3.1	Introduction

This method allows the Media Session Handler to acquire Consumption Reporting configuration parameters when the Consumption Reporting is used for a Downlink Streaming session.

###### 5.4.2.3.3.2	Resource definition

Resource URI: {apiRoot}/3gpp-5gms-consumption-reporting/v1/{aspId}/session/{sessionId}/acquire

This method shall support the resource URI variables defined in table 5.4.2.3.3.2-1.

Table 5.4.2.2.3.3.2-1: Resource URI variables for resource "Consumption Reporting Configuration Acquisition"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of TS 29.122 [x3]. |
| aspId | Identifier of the 5GMS Application Provider. |
| sessionId | Identifier of the 5GMS Downlink Streaming session. |

###### 5.4.2.3.3.3	Resource methods

5.4.2.3.3.3.1	GET

The GET method allows the Media Session Handler to acquire Consumption Reporting configuration parameters. It is initiated by the Media Session Handler and answered by the 5GMSd AF.

This method shall support request and response data structures, and response codes, as specified in the table 5.4.2.3.3.3.3-1.

Table 5.4.2.3.3.3.3-1: Data structures supported by the GET request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | none |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | ConsumptionReportingConfiguration | 1..1 | 200 OK | The consumption reporting configuration parameters shall be returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 [x3] also apply. |  |  |  |  |

5.4.2.3.3.3.2	PUT

This HTTP method is not supported for the resource.

5.4.2.3.3.3.3	PATCH

This HTTP method is not supported for the resource.

5.4.2.3.3.3.4	POST

This HTTP method is not supported for the resource.

5.4.2.3.3.3.5	DELETE

This HTTP method is not supported for the resource.

##### 5.4.2.3.4	Consumption Reporting Data resource

###### 5.4.2.3.4.1	Introduction

Editor’s Note: Whether reports should be sent as RESTful resources is still under consideration

This resource allows the Media Session Handler to send the actual consumption data when the Consumption Reporting is used for a Downlink Streaming session.

###### 5.4.2.3.4.2	Resource definition

Resource URI: {apiRoot}/3gpp-5gms-consumption-reporting/v1/{aspId}/session/{sessionId}

This resource shall support the resource URI variables defined in table 5.4.2.3.4.2-1.

Table 5.4.2.3.4.2-1: Resource URI variables for resource "Consumption Reporting Data"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of TS 29.122 [x3]. |
| aspId | Identifier of the 5GMS Application Provider. |
| sessionId | Identifier of the 5GMS Downlink Streaming session. |

###### 5.4.2.3.4.3	Resource methods

5.4.2.3.4.3.1	GET

This HTTP method is not supported for the resource.

5.4.2.3.4.3.2	PUT

This HTTP method is not supported for the resource.

5.4.2.3.4.3.3	PATCH

This HTTP method is not supported for the resource.

5.4.2.3.4.3.4	POST

The POST method allows the Media Session Handler to send consumption data. It is initiated by the 5GMS Media Session Handler and acknowledged by the 5GMSd AF.

This method shall support request and response data structures, and response codes, as specified in the table 5.4.2.3.4.3.3-1.

Table 5.4.2.3.4.3.3-1: Data structures supported by the POST request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | ConsumptionReporting | 1..1 | Consumption Reporting data to send to the Media AF. |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | none |  | 200 OK | The consumption reporting data is received by the Media AF. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 [x3] also apply. |  |  |  |  |

5.4.3.4.3.5	DELETE

This HTTP method is not supported for the resource.

================================END OF SECOND CHANGE==============================