3GPP TSG-S4 Meeting #108-e	S4-200608

# 2020-03-02 – 2020-03-09

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR | <CR#> | rev | - | Current version: | 1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Setting b=AS in presence of an open offer. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fraunhofer IIS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVS_codec |  |  |  |  |  | Date: |  |  | 2020-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For EVS, the open offer requires b=AS set to the maximum for 128kbps EVS modes, as it is not known at the time of the offer what the answer would be. Even in the presence of a preferred RTP payload type with reduced bit rate range, b=AS would need to be set to this high value. This general limitation of b=AS to be the maximum of all offered payload types for a media leads to over-provisioning of the RAN ressources. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allow b=AS for EVS be set to the lower value of a payload type with limited bit rate range if both the limited payload type and an open offer are present. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Over-provisioning of network ressources or ommision of open offer, leading to reduced interoperability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.2.5.2	Speech

If an MTSI client includes an AMR or AMR-WB mode-set, or EVS Primary mode br or br-recv parameter in the SDP offer or answer, the MTSI client shall set the b=AS parameter to a value matching the maximum codec mode in the mode-set or the highest bit-rate in the br or br-recv, the packetization time (ptime), and the intended redundancy level. For example, b=AS for AMR-WB at IPv6 should be set to 38 if mode-set includes {6.60, 8.85, 12.65}, the packetization time is 20, and if no extra bandwidth is allocated for redundancy. Likewise, b=AS for EVS Primary mode at IPv4 should be set to 42 if br=7.2-24.4, the packetization is header-full payload format, ptime=20, and no extra bandwidth is allocated for redundancy.

If an MTSI client does not include an AMR or AMR-WB mode-set, or EVS Primary mode br or br-recv parameter in the SDP offer or answer, the MTSI client shall set the b=AS parameter in the SDP to a value matching the highest AMR/AMR-WB mode, i.e., AMR 12.2 and AMR-WB 23.85, or the highest bit-rate of EVS Primary mode depending on negotiated bandwidth(s), i.e., EVS 24.4 for NB and EVS 128 for WB, SWB and FB, respectively, except if another RTP payload type of the same code with br or br-recv parameter in the SDP offer or answer is present.

NOTE 1:	When no mode-set is defined, then this should be understood as that the offerer or answerer is capable of sending and receiving all codec modes of AMR or AMR-WB. An MTSI client in terminal will not include the mode-set parameter in SDP offer in the initial offer-answer negotiation. See Clause 6.2.2.2, Tables 6.1 and 6.2. It is however expected that the mode-set is defined when an SDP offer is received from an MTSI MGW inter-working with CS GERAN/UTRAN, see Clause 6.2.2.3, Table 6.5.

The bandwidth to use for b=AS for AMR and AMR-WB, and EVS Primary mode should be computed as shown in Annexes K and Q respectively. Tables 6.7 and 6.8 shows the bandwidth for the respective AMR and AMR-WB codec when the packetization time is 20 and no extra bandwidth is allocated for redundancy. The b=AS value is computed without taking statistical variations, e.g., the effects of DTX, into account. Such variations can be considered in the scheduling and call admission control. Detailed procedures to compute b=AS of AMR and AMR-WB, and EVS Primary mode can be found in Annexes K and Q.

NOTE 2:	For any payload format, b=AS of EVS Primary mode at 5.9 kbps source controlled variable bit-rate (SC-VBR) coding is computed as the b=AS of its highest component bit-rate, 8 kbps.

NOTE 3:	b=AS of EVS AMR-WB IO mode can be computed as in the octet-aligned payload format of AMR-WB as shown in Annex K.

b=AS of EVS shall be equal to the maximum of b=AS of the highest included EVS primary mode and b=AS of the highest included EVS AMR-WB IO mode, regardless of the presence and configuration of evs-mode-switch. If EVS primary modes with EVS payload types having the br or br-recv parameter in the SDP offer or answer are complemented by an EVS open offer, the b=AS of EVS should be set for the EVS primary mode's br or br-recv parameter.

NOTE 4:	Setting the b=AS for the payload types with a bit rate range avoids over-provising of network ressources but may limit the flexibility of the open offer by a reduced b=AS.

Table 6.7: b=AS for each codec mode of AMR when ptime is 20

| Payload format |  | Codec mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 4.75 | 5.15 | 5.9 | 6.7 | 7.4 | 7.95 | 10.2 | 12.2 |
| Bandwidth-efficient | IPv4 | 22 | 22 | 23 | 24 | 24 | 25 | 27 | 29 |
|  | IPv6 | 30 | 30 | 31 | 32 | 32 | 33 | 35 | 37 |
| Octet-aligned | IPv4 | 22 | 22 | 23 | 24 | 25 | 25 | 28 | 30 |
|  | IPv6 | 30 | 30 | 31 | 32 | 33 | 33 | 36 | 38 |

Table 6.8: b=AS for each codec mode of AMR-WB when ptime is 20

| Payload format |  | Codec Mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 6.6 | 8.85 | 12.65 | 14.25 | 15.85 | 18.25 | 19.85 | 23.05 | 23.85 |
| Bandwidth-efficient | IPv4 | 24 | 26 | 30 | 31 | 33 | 35 | 37 | 40 | 41 |
|  | IPv6 | 32 | 34 | 38 | 39 | 41 | 43 | 45 | 48 | 49 |
| Octet-aligned | IPv4 | 24 | 26 | 30 | 32 | 33 | 36 | 37 | 40 | 41 |
|  | IPv6 | 32 | 34 | 38 | 40 | 41 | 44 | 45 | 48 | 49 |

Table 6.9: b=AS for each bit-rate of EVS Primary mode when ptime is 20

| Payload format |  | Bit-rate |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7.2 | 8 | 9.6 | 13.2 | 16.4 | 24.4 | 32 | 48 | 64 | 96 | 128 |
| Header-full | IPv4 | 24 | 25 | 27 | 30 | 34 | 42 | 49 | 65 | 81 | 113 | 145 |
|  | IPv6 | 32 | 33 | 35 | 38 | 42 | 50 | 57 | 73 | 89 | 121 | 153 |

Tables 6.10-1 to 6.10-3 describe the setting of the bandwidth properties that should be used for the ‘a=bw-info’ attribute for a few possible combinations of codec, codec rate, packetization schemes and redundancy levels. The Minimum Supported Bandwidth does not prevent encoding the speech with an even lower bitrate, for example when EVS is used in the 5.9 kbps VBR mode or during DTX periods when SID frames are encoded with a very low bit rate and are generated with a reduced frame rate. Bit rates lower than the Minimum Supported Bandwidth may also be used when sending DTMF. Additional combinations and corresponding bandwidth properties are found in Annex K for AMR, AMR-WB and EVS AMR-WB IO mode and in Annex Q for EVS primary mode.

Table 6.10-1: Recommended bandwidth properties for AMR to be used with the ‘a=bw-info’ attribute when codec modes up to 12.2 are negotiated

| Parameter | Assumed setting |
| --- | --- |
| Negotiated codec modes | 4.75, 5.9, 7.4, 12.2 |
| Codec mode used without redundancy | 12.2 |
| Codec mode used with redundancy | 5.9 |
| Payload format | AMR/AMR-WB bandwidth-efficient |
| Minimum frame aggregation | 1 frame per packet |
| Maximum frame aggregation | 4 frames per packet |
| Maximum redundancy level | 100% |
| Redundancy offset | 0 |
| IP version | 6 |
| Bandwidth property | Value |
| Maximum Supported Bandwidth | 37 (see NOTE 1) |
| Maximum Desired Bandwidth | 37 |
| Minimum Desired Bandwidth | 31 |
| Minimum Supported Bandwidth | 13 (see NOTE 2) |
| NOTE 1: If redundancy is needed for higher codec modes, if additional redundancy is needed, or if redundancy offset is needed then the Maximum Supported Bandwidth needs to be set to a higher value. NOTE 2: The Minimum Supported Bandwidth is calculated based on the lowest codec rate when maximum frame aggregation is used. |  |

Table 6.10-2: Recommended bandwidth properties for AMR-WB to be used with the ‘a=bw-info’ attribute when codec modes up to 12.65 are negotiated

| Parameter | Assumed setting |
| --- | --- |
| Negotiated codec modes | 6.6, 8.85, 12.65 |
| Codec mode used without redundancy | 12.65 |
| Codec mode used with redundancy | 6.6 |
| Payload format | AMR/AMR-WB bandwidth-efficient |
| Minimum frame aggregation | 1 frame per packet |
| Maximum frame aggregation | 4 frames per packet |
| Maximum redundancy level | 100% |
| Redundancy offset | 0 |
| IP version | 6 |
| Bandwidth property | Value |
| Maximum Supported Bandwidth | 38 (see NOTE 1) |
| Maximum Desired Bandwidth | 38 |
| Minimum Desired Bandwidth | 32 |
| Minimum Supported Bandwidth | 13 (see NOTE 2) |
| NOTE 1: If redundancy is needed for higher codec modes, if additional redundancy is needed, or if redundancy offset is needed then the Maximum Supported Bandwidth needs to be set to a higher value. NOTE 2: The Minimum Supported Bandwidth is calculated based on the lowest codec rate when maximum frame aggregation is used. |  |

Table 6.10-3: Recommended bandwidth properties for EVS to be used with the ‘a=bw-info’ attribute when codec modes up to 13.2 are negotiated

| Parameter | Assumed setting |
| --- | --- |
| Negotiated codec modes | 5.9 – 13.2 |
| Codec mode used without redundancy | 13.2 |
| Codec mode used with redundancy | 7.2 |
| Payload format | EVS |
| Minimum frame aggregation | 1 frame per packet |
| Maximum frame aggregation | 4 frames per packet |
| Maximum redundancy level | 100% |
| Redundancy offset | 0 |
| IP version | 6 |
| Bandwidth property | Value |
| Maximum Supported Bandwidth | 40 (see NOTE 1 and NOTE 2) |
| Maximum Desired Bandwidth | 38 |
| Minimum Desired Bandwidth | 32 (see NOTE 2) |
| Minimum Supported Bandwidth | 14 (see NOTE 2) |
| NOTE 1: If redundancy is needed for higher codec modes, if additional redundancy is needed, or if redundancy offset is needed then the Maximum Supported Bandwidth needs to be set to a higher value. NOTE 2: The bandwidth is calculated assuming EVS 7.2 kbps when maximum frame aggregation is used. |  |

SDP examples using the ‘a=bw-info’ attribute for speech are shown in annex A.6.3.