3GPP TSG-SA4#109-e meeting	S4-200849

# 20th May – 3rd June 2020, Electronic meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.511 | CR | <CR#> | rev | - | Current version: | 1.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5GMS3: TV profiles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The 3GPP TS 26.511 specification is a collection of 5GMS profiles with dedicated media requirements. Each profile is associated to a service scenario. The source would like to ensure that simple requirements are associated to each scenario. Avoiding the toolbox approach for device requirements can be solved by spliting service scenarios that have different requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to split the TV profile into: - an HDTV profile in which AVC fullHD is mandatory and HEVC fullHD is recommended - a UHDTV profile in which only HEVC is specfified and mandatory, HDR extensions being recommended. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of clarity on what is supported for operators and service providers wanting to deploy TV services (live, On demand…) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4, 5.4a (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

## 5.4	HDTV (HD Television) profile

### 5.4.1	Video decoding

#### 5.4.1.1	H.264 (AVC)

If the 5GMS client supports the HDTV profile, it shall support the H.264/AVC Full HD Operation Point Receiver requirements as specified in 3GPP TS 26.116 [12], clause 4.4.3

NOTE:	As defined in 3GPP TS 26.116 [12], clause 4.4.3.6, receivers conforming to the H.264/AVC Full HD Operation Point shall also support decoding and displaying H.264/AVC 720p HD Bitstreams.

#### 5.4.1.2	H.265 (HEVC)

If the 5GMS client supports the Television (TV) profile, it should support H.265/HEVC Full HD Operation Point Receiver requirements as specified in 3GPP TS 26.116 [12], clause 4.5.3.

NOTE:	As defined in 3GPP TS 26.116 [12], clause 4.5.3.7, receivers conforming to the H.265/HEVC Full HD Operation Point shall also support decoding and displaying H.265/HEVC 720p HD Bitstreams.

### 5.4.2	Audio decoding

If the 5GMS client supports the HDTV profile, it should support eAAC+ stereo Operation point as defined in 3GPP TS 26.117 [4], clause 6.3.3.

### 5.4.3	Subtitle decoding

If the 5GMS client supports the HDTV profile, it should support the IMSC-1.1-TEXT-DEC decoding capabilities as defined in clause 4.5.1.1.

### 5.4.4	Decapsulation and decryption

#### 5.4.4.1	Introduction

This clause defines the requirements in terms of decapsulation and decryption of media for an HDTV Profile Media Player in combination with codecs. Media Encapsulation in 5G Media Streaming for downlink is defined based on the MPEG Common Media Application Format (CMAF) [7]. However, 3GPP TS 26.116 [12] defines file format encapsulation and DASH Mapping in clause 5.

This clause addresses decapsulation and decryption requirements in a sense that a 5GMS client supporting the HDTV profile requires to be able to process content that conforms to 3GPP TS 26.116 [12] encapsulation and CMAF encapsulation.

Editor’s NOTE: To be resolved: The CMAF Track and Switching Set constraints in this specification and the Representation and Adaptation Set constraints in 3GPP TS 26.116 [12] are considered almost identical, but it still needs to be checked in detail. In the absence of both, it is expected that content offered for HDTV Profile receivers conform to both specifications.

#### 5.4.4.2	Video media profiles

If the 5GMSd client supports supports the HDTV profile, then the following applies:

-	the AVC-HD playback requirements as defined in clause 4.2.1.3.1.4 and the H.264/AVC 720p HD Operation Point Receiver requirements as specified in 3GPP TS 26.116 [12], clause 4.4.2 shall be supported.

-	the AVC-FullHD playback requirements as defined in clause 4.2.1.3.2.4 and the H.264/AVC Full HD Operation Point Receiver requirements as specified in 3GPP TS 26.116 [12], clause 4.4.3 shall be supported.

-	the HEVC-HD playback requirements as defined in clause 4.2.2.3.1.4 and the H.265/HEVC 720p HD Operation Point Receiver requirements as specified in 3GPP TS 26.116 [12], clause 4.5.2 should be supported.

-	the HEVC-FullHD playback requirements as defined in clause 4.2.2.3.2.4 and the H.265/HEVC Full HD Operation Point Receiver requirements as specified in 3GPP TS 26.116 [12], clause 4.5.3 should be supported.

#### 5.4.4.3	Audio media profiles

If the 5GMSd client supports the HDTV profile, then the following should be supported:

-	the eAAC+ stereo playback requirements as defined in 3GPP TS 26.117 [4] clause 7.5.2.4.

#### 5.4.4.4	Subtitle media profiles

If the 5GMSd client supports the HDTV profile, then the following should be supported:

-	the IMSC1.1 text track playback requirements as defined in clause 4.5.1.5.

#### 5.4.4.5	Encrypted content

If the 5GMSd client supports the HDTV profile the same requirements apply as in clause 5.2.7.6 for the default profile.

### 5.4.5	Capability discovery

If the 5GMSd client supports the HDTV profile it is expected to support capability discovery such that 5GMS-aware applications can identify if a specific media profile is supported.

The same requirements apply as in clause 5.2.8 for the default profile.

A 5GMSd client that supports the HDTV profile may support additional capability methods that permits to identify additional capabilities of media profile playback taking into account information provided in the MPD for TV Video Profiles. Examples include the W3C Media Capability APIs [18]. More details are FFS.

=====  CHANGE  =====

## 5.4a	UHDTV (Ultra-HD Television) profile

### 5.4a.1	Video decoding

#### 5.4a.1.1	H.265 (HEVC)

If the 5GMS client supports the UHDTV profile, it shall support H.265/HEVC UHD Operation Point Receiver requirements as specified in 3GPP TS 26.116 [12], clause 4.5.4.

NOTE:	As defined in 3GPP TS 26.116 [12], clause 4.5.4.7, receivers conforming to the H.265/HEVC UHD Operation Point shall also support decoding and displaying H.265/HEVC 720p HD and H.265/HEVC Full HD Bitstreams.

If the 5GMS client supports the UHDTV profile, it should support H.265/HEVC UHD HDR Operation Point Receiver requirements as specified in 3GPP TS 26.116 [12], clause 4.5.6.

NOTE:	As defined in 3GPP TS 26.116 [12], clause 4.5.6.8, receivers conforming to the H.265/HEVC UHD HDR Operation Point shall also support decoding and processing H.265/HEVC 720p HD, H.265/HEVC Full HD, H.265/HEVC UHD and H.265/HEVC Full HD HDR Bitstreams

If the 5GMS client supports the UHDTV profile, it should support H.265/HEVC UHD HDR HLG Operation Point Receiver requirements as specified in 3GPP TS 26.116 [12], clause 4.5.8.

NOTE:	As defined in 3GPP TS 26.116 [12], clause 4.5.8.7, receivers conforming to the H.265/HEVC UHD HDR HLG Operation Point shall also support decoding and processing H.265/HEVC 720p HD, H.265/HEVC Full HD, H.265/HEVC UHD and H.265/HEVC Full HD HDR HLG Bitstreams.

### 5.4a.2	Audio decoding

If the 5GMS client supports the UHDTV profile, it should support eAAC+ stereo Operation point as defined in 3GPP TS 26. 117 [4], clause 6.3.3.

### 5.4a.3	Subtitle decoding

If the 5GMS client supports the UHDTV profile, it should support the IMSC-1.1-TEXT-DEC decoding capabilities as defined in clause 4.5.1.1.

### 5.4a.4	Decapsulation and decryption

#### 5.4a.4.1	Introduction

This clause defines the requirements in terms of decapsulation and decryption of media for a UHDTV Profile Media Player in combination with codecs. Media Encapsulation in 5G Media Streaming for downlink is defined based on the MPEG Common Media Application Format (CMAF) [7]. However, 3GPP TS 26.116 [12] defines file format encapsulation and DASH Mapping in clause 5.

This clause addresses decapsulation and decryption requirements in a sense that a 5GMS client supporting the UHDTV profile requires to be able to process content that conforms to 3GPP TS 26.116 [12] encapsulation and CMAF encapsulation.

Editor’s NOTE: To be resolved: The CMAF Track and Switching Set constraints in this specification and the Representation and Adaptation Set constraints in TS 26.116 [12] are considered almost identical, but it still needs to be checked in detail. In the absence of both, it is expected that content offered for UHDTV Profile receivers conform to both specifications.

#### 5.4a.4.2	Video media profiles

If the 5GMSd client supports supports the UHDTV profile, then the following applies:

-	the HEVC-HD playback requirements as defined in clause 4.2.2.3.1.4 and the H.265/HEVC 720p HD Operation Point Receiver requirements as specified in 3GPP TS 26.116 [12], clause 4.5.2.7 shall be supported.

-	the HEVC-FullHD playback requirements as defined in clause 4.2.2.3.2.4 and the H.265/HEVC Full HD Operation Point Receiver requirements as specified in 3GPP TS 26.116 [12], clause 4.5.3.7 shall be supported.

-	the HEVC-UHD playback requirements as defined in clause 4.2.2.3.3.4 and the H.265/HEVC UHD Operation Point Receiver requirements as specified in 3GPP TS 26.116 [12], clause 4.5.4.7 shall be supported.

-	the HEVC-FullHD playback requirements as defined in clause 4.2.2.3.2.4 and the H.265/HEVC Full HD HDR Operation Point Receiver requirements as specified in 3GPP TS 26.116 [12], clause 4.5.5.7 should be supported.

-	the HEVC-FullHD playback requirements as defined in clause 4.2.2.3.2.4 and the H.265/HEVC Full HD HDR HLG Operation Point Receiver requirements as specified in 3GPP TS 26.116 [12], clause 4.5.7.7 should be supported.

-	the HEVC-UHD playback requirements as defined in clause 4.2.2.3.3.4 and the H.265/HEVC UHD HDR Operation Point Receiver requirements as specified in 3GPP TS 26.116 [12], clause 4.5.5.7 should be supported.

-	the HEVC-UHD playback requirements as defined in clause 4.2.2.3.3.4 and the H.265/HEVC UHD HDR HLG Operation Point Receiver requirements as specified in 3GPP TS 26.116 [12], clause 4.5.8.7 should be supported.

#### 5.4a.4.3	Audio media profiles

If the 5GMSd client supports the UHDTV profile, then the following should be supported:

-	the eAAC+ stereo playback requirements as defined in 3GPP TS 26.117 [4] clause 7.5.2.4.

#### 5.4a.4.4	Subtitle media profiles

If the 5GMSd client supports the UHDTV profile, then the following should be supported:

-	the IMSC1.1 text track playback requirements as defined in clause 4.5.1.5.

#### 5.4a.4.5	Encrypted content

If the 5GMSd client supports the UHDTV profile the same requirements apply as in clause 5.2.7.6 for the default profile.

### 5.4a.5	Capability discovery

If the 5GMSd client supports the UHDTV profile it is expected to support capability discovery such that 5GMS-aware applications can identify if a specific media profile is supported.

The same requirements apply as in clause 5.2.8 for the default profile.

A 5GMSd client that supports the UHDTV profile may support additional capability methods that permits to identify additional capabilities of media profile playback taking into account information provided in the MPD for UHDTV Video Profiles. Examples include the W3C Media Capability APIs [18]. More details are FFS.