3GPP TSG-SA4#109-e meeting	S4-200867

# 20th May – 3rd June 2020, Electronic meeting										   rev of S4-200848

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.511 | CR | <CR#> | rev | - | Current version: | 1.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5GMS3: Default downlink profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, Deutsche Telekom AG, Orange, SK Telecom, Telecom Italia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2020-05-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The 26.511 specification is a collection of 5GMS profiles with dedicated media requirements. Each profile is associated to a service scenario. In the absence of indication of a particular profile, the default profile requirements are applicable. - Discussions happened on the level of requirement associated to HEVC for which HDR support was proposed as the pre-condition to mandate the codec. - The source observe that - Video-based services are occupying the major part of mobile data traffic - Full HD is becoming a generalized distribution format for which coding efficiency is the most critical. - Coding gains of HEVC vs AVC are well known and significant (40-50%) - Video-based services deployed over 5GMS are likely to address devices over LTE as well, where bandwidth constrains are even more critical. - HDR services are progressively increasing but in a marginal proportion compared to SDR. - Mandating the support of HDR as a precondition is a too strong requirement for ensuring the use of HEVC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to indicate in the default downlink profile, based on what is suggested in S4-200760: - a default support for AVC HD as mandatory (no change) - a conditional support of Full HD for which HEVC is is mandatory. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Including HDR and WCG as a precondition to support HEVC creates confusion to service providers intending to deploy video services using HEVC but not neccesarily requesting the support of HDR/WCG. Not ensuring the HEVC decoding capability for Full HD services may lead to unecessary network load with a less efficient video codec, as the default one. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2; 5.2.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

If the 5GMS client supports the reception of video, then

-	AVC-HD-Dec decoding capability shall be supported as defined in clause 4.2.1.1.

-	HEVC-HD-Dec decoding capability should be supported as defined in clause 4.2.2.1.

If the 5GMS client supports the reception of Full HD video, then

-	AVC-FullHD-Dec decoding capability shall be supported as defined in clause 4.2.1.1.

-	HEVC-FullHD-Dec decoding capability shall be supported as defined in clause 4.2.2.1.

=====  CHANGE  =====

#### 5.2.7.2	Video media profiles

If the 5GMSd client supports the reception of video, then the following applies:

-	the AVC-HD playback requirements as defined in clause 4.2.1.3.1.4 shall be supported.

-	the HEVC-HD playback requirements as defined in clause 4.2.2.3.1.4 should be supported.

If the 5GMSd client supports the reception of Full HD video, then the following applies:

-	the AVC-FullHD playback requirements as defined in clause 4.2.1.3.2.4 shall be supported.

-	the HEVC-FullHD playback requirements as defined in clause 4.2.2.3.2.4 shall be supported.

-	the HEVC-UHD playback requirements as defined in clause 4.2.2.3.3.4 may be supported.