Source:	Qualcomm Incorporated

Title:	Proposals for data collection of HaNTE – test methods

## Document for:	Agreement

## Agenda Item:	9.6

## Abstract

This contribution proposes test methods that participating companies can use for a data collection round robin in support of the work item on Handsets Featuring Non-Traditional Earpieces (HaNTE), approved at SA#86 [1].

## Background

The objectives of the HaNTE work item, as described in [1] are the following:

-	Update 3GPP TS 26.132 with reference to the appropriate version of ITU-T P.64 that addresses HaNTE devices.

-	Review performance of HaNTE devices and update requirements in 3GPP TS 26.131 if necessary, to ensure an adequate user experience.

-	Establish guidelines for mounting of HaNTE devices to ensure a repeatable and reproducible measurement method in 3GPP TS 26.132.

- 	Investigate testing RFR/RLR at a single position and at different positions, and report the results; based on those results, and if judged necessary by SA4, new requirements/test methods may be specified.

-	Address privacy aspects identified during the study phase and develop requirements/test method if necessary.

-	Address distortion aspects identified during the study phase and update requirements/test method if necessary.

-	Address out of band noise aspects identified during the study phase and develop requirements/test method if necessary.

At SA4#107, a contribution [2] proposed changes to TS 26.131 and TS 26.132 that address some of the objectives of the work item. The source believes that inter-lab repeatability and reproducibility is a key aspect and must be assessed in connection with new test method proposals prior to agreement. The source then proposed [3], and SQ agreed in principle [4] to:

- Conduct a round-robin testing of HaNTE devices with willing participating labs.

- The following tentative time-plan for the round robin:

| March 3rd, 2020 | SQ SWG Telco | - Agree in principle on conduction of round robin test. - Identify 3 test labs interested in participating in the round robin. |
| --- | --- | --- |
| March, 2020 | Over email among interested member companies | - Agree on a selection of 3 to 5 HaNTE devices. - Work collectively over reflector on draft test methodology, including mounting guidelines. |
| April, 6th 2020 | SA4#108 e-meeting SQ SWG Telco session 1 | - Review inputs on detailed test methodologies for iii.a., iii.b., iii.c. and iii.d., including choice of codecs and connection instructions. |
| April, 8th 2020 | SA4#108 e-meeting SQ SWG Telco session 2 | - Agree on core test methodologies - Identify test Lab 1 for first round of measurements |
| April, 2020 |  | - Further test methodology refinements (reflector) - Review a spreadsheet template for the data collection. (reflector) |
| May, 2020 |  | - Test Lab 1 conducts preliminary measurements |
| May, 25th 2020 | SA4#109, Erlangen | - Review preliminary results of Test Lab 1 - Agree on further refinements and revise test plan |
| Jun, 2020 |  | - Test Lab 1 conducts additional measurements - Test Lab 2 conducts measurements |
| End of June, 2020, date [TBD] | SQ SWG Telco [TBD] | - Review results of Test Lab 1 and 2. |
| July/August, 2020 |  | - Test Lab 3 conducts measurements - Test Lab 4 conducts measurements |
| 24th-28th August, 2020 | SA4#110, US | - Review results of Test Lab 4 - Discuss next steps. |

Editor’s note: Time plan is tentative and lab availability may shift in time due to the COVID-19 situation. Testing may have to be postponed.

## Participating labs

As of the writing of this document, the following labs have expressed interest in participating in the round robin test effort:

| Tentative lab order | Lab | Contact Person | Contact email |
| --- | --- | --- | --- |
| 1 | Qualcomm Incorporated | Andre Schevciw | aschevci@qti.qualcomm.com |
| 2 | HEAD Acoustics GmbH | Jan Reimes | jan.reimes@head-acoustics.de |
| 3 | Orange | Alain Curti | alain.curti@orange.com |
| 4 | Huawei Technologies | Antero Tossavainen | antero.tossavainen@huawei.com |

## List of Devices

As of the writing of this document, the following devices have been identified as available for the round robin test effort:

Editor’s Note: There are eight devices assigned to this effort. Manufacturer and model information has been removed upon request of one of the manufacturers.

## Test Methods for HaNTE

The descriptions of the test methods to be used for the round-robin test are provided in Appendix A.

## 6. References

- TD SP-191212 (WID NEW) New WID on Handsets Featuring Non-Traditional Earpieces (HaNTE), Source: SA4

- Tdoc S4-200173,  Proposals for HaNTE, Source: Orange

- Tdoc S4-AHQ146, Proposals for data collection of HaNTE, Source: Qualcomm Incorporated

- Tdoc S4-AHQ147, Report from SA4 SQ SWG conf. call on HaNTE (3rd March 2020), Source: SA4 SQ SWG interim Chairman

Appendix A: Test Methods

# A. Objective listening speech quality MOS-LQO in receive direction

# A.1. Wideband

NOTE: This test method was originally described in clause 6.4.2 of Final draft ETSI ES 202 739 V1.8.1 (2020-03) Speech and multimedia Transmission Quality (STQ); Transmission requirements for wideband VoIP terminals (handset and headset) from a QoS perspective as perceived by the user. It is adapted here for the purposes of the HaNTE work item. https://www.etsi.org/deliver/etsi_es/202700_202799/202739/01.08.01_50/es_202739v010801m.pdf

The listening speech quality tests are conducted under clean network conditions.

Requirement

NOTE: Original requirements from ETSI are provided here only for reference when conducting measurements. There is no working assumption that the requirements below are appropriate for HaNTE purposes.

The requirements for the listening speech quality and the delay under clean network conditions are according to Table A.

Table A

| Speech coder | MOS-LQOF (P.863) |
| --- | --- |
| AMR-WB | > 4,0 |
| NOTE: Recommendation ITU-T P.863 is using a fullband scale. Not sufficient experience is available so far with this method. Therefore, the numbers given for MOS-LQOF are provisional and may be updated with a later revision of the present document |  |

Measurement method

Objective listening speech quality is measured using Recommendation ITU-T P.863 in fullband mode.

The handset terminal or the headset terminal is setup as described in clause 5.1.1 of 3GPP TS 26.132, except that the version of ITU-T Recommendation P.64 to be referenced is the 06/2019 version. The criteria for determining the ECRP follows this order:

1st: At the manufacturer defined position (MECRP), if provided.

2nd: At the centre of the earpiece, if the handset features a traditional earpiece.

3rd: Following a graphical user interface showing the location of optimal sound radiation, if provided in a handset featuring a non-traditional earpiece.

4th: After a subjective determination by the test operator of the optimal holding position.

The test signal to be used for the measurements shall be 4 sentence pairs (male/female) fulfilling the requirements of Recommendation ITU-T P.863.1. The 4 sentence pairs are taken from Recommendation ITU-T P.501. It shall be stated, which sentence pairs were used. The test signal level is averaged over all sentence pairs (4 sentence pairs).

Measurement is operated at nominal and maximum value of volume control.

The measurement is done 4 times, every time using another pair of the speech sentences. The result of the measurement is the averaged value of all 4 measurements.

NOTE 3: For the use of P.863 the following applies (see Recommendation ITU-T P.863.1):

- Fullband Context (MOS-LQOF):

  - Reference Signal Fullband flat filtered 20 Hz to 20 kHz.

  - Test Signal Wideband flat low pass filtered 7,8 kHz.

# Privacy in Receive Direction (max volume RLR)

Requirement

The differences in the handset mode RLR1 (RLRA), and the far-field RLRs measured at a radius of 42 centimetres from the HATS EEP in points A, B, C, D and E of Figure B1 and Table B (RLRB), shall be less than [TBD].

Note 1: The handset mode RLR is measured according to 3GPP TS 26.132 Clause 8.2.2.2, except that the short male/female British-English single talk sequence described in ITU-T Recommendation P.501 Annex D [22] is used for efficiency purposes.

Figure B1

Table B

| Measuring position | Measuring angle (starting from E) |
| --- | --- |
| A | -180° |
| B | -135° |
| C | -90° |
| D | -45° |
| E | 0° |

Figure B2

42cm

Measurement method

a)	The test signal to be used for the measurements shall be the short male/female British-English single talk sequence described in ITU-T Recommendation P.501 Annex D [22]. The test signal level shall be -16 dBm0 measured at the digital reference point or the equivalent analogue point. The test signal level is calculated over the complete test signal sequence.

b)	The handset terminal or the headset terminal is setup as described in clause 5.1.1 of 3GPP TS 26.132, except that the version of ITU-T Recommendation P.64 to be referenced is the 06/2019 version. The criteria for determining the ECRP follows this order:

1st: At the manufacturer defined position (MECRP), if provided.

2nd: At the centre of the earpiece, if the handset features a traditional earpiece.

3rd: Following a graphical user interface showing the location of optimal sound radiation, if provided in a handset featuring a non-traditional earpiece.

4th: After a subjective determination by the test operator of the optimal holding position.

c)	The volume control setting of the handset, if any, is adjusted to its maximum volume setting.

d) 	To determine RLRA, the receiving sensitivity is measured as described in 3GPP TS 26.132 Clause 8.2.2.2, except that the short speech sequence described in step a) is used instead.

e)	To determine RLRB for the different orientations of HATS, the receiving sensitivity is measured at each of the orientations in A, B, C, D and E of Figure B1 with a measurement microphone positioned at the HATS Reference Point (HRP) height (ear height) and located 42cm from the HRP (see Figure B2). A low noise microphone is recommended. If a second HATS is used as a measurement microphone, then it is free-field equalized as described in ITU-T Recommendation P.581. The equalized output signal of each artificial ear is power-averaged over the total duration of the analysis; the right and left artificial ear signals are voltage-summed for each 1/3-octave frequency band; these 1/3-octave band data are considered as the input signal to be used for calculations or measurements.-

The receiving sensitivity shall be calculated from each band of the 20 frequencies given in table A.2 of ITU-T Recommendation P.79 Annex A, bands 1 to 20. For the calculation, the averaged measured level at each frequency band is referred to the averaged test signal level measured in each frequency band.

The sensitivity is expressed in terms of dBPa/V and the RLR shall be calculated according to ITU-T Recommendation P.79, formula (A-23c), over bands 1 to 20, using m = 0,175 and the receiving weighting factors from table A.2 of ITU-T Recommendation P.79 Annex A.

No leakage correction shall be applied. The hands-free correction as described in ITU-T Recommendation P.340 shall be applied. When using the combination of left and right artificial ear signals from the HATS the HFLE has to be 8 dB, instead of 14 dB. For further information see ITU-T Recommendation P.581.

f)	For each of the orientations A, B, C, D and E of HATS, the RLR = (RLRA - RLRB) is computed.

# Receive Frequency Response Robustness to Holding Position (Wideband)

Requirement

[TBD]

Measurement method

The test method is the same as in 3GPP TS 26.132 Clause 9.4.1.2 (super-wideband), except that:

- The call is conducted with the AMR-WB codec with the 12.65kbps bitrate.

- The device shall be measured in a 2x2 “grid” of dimensions 20x20mm[TBD] around the ECRP and should be measured in a 3x3 “grid” of dimensions 20x20mm around the ECRP. See Figure C.

Figure C

The criteria for determining the ECRP follows this order:

1st: At the manufacturer defined position (MECRP), if provided.

2nd: At the centre of the earpiece, if the handset features a traditional earpiece.

3rd: Following a graphical user interface showing the location of optimal sound radiation, if provided in a handset featuring a non-traditional earpiece.

4th: After a subjective determination by the test operator of the optimal holding position.

- Additionally, the device is measured at ECRP with the three handset holding point configurations as defined in Appendix B [TBD].

Note: The test method to be used is the super-wideband test method to allow for observation of potential out-of-band distortion and noise components.

The

Appendix B: Handset positioning instructions

Editor’s note: This clause will be populated with handset positioning instructions for each device.