3GPP TSG-SA4 Meeting #109-e 	S4-200980

# Electronic meeting, Telco, May 25-29 and June 2-3, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.346 | CR | 0632 | rev | 2 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Partial File Handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Enensys |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eDASH, TEI15 |  |  |  |  |  | Date: |  |  | 2020-05-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | MBMS partial file handling operation is described in the sub-clauses of clause 7.9. An MBMS application that can make use of a partial file from the MBMS client, in the event the full file could not be fully recovered by the MBMS client, signals its willingness to accept such partial file. Under the assumption that the MBMS application communicates with the MBMS client using HTTP methods for file retrieval, the application will send a partial-file-accept request as the HTTP GET request, which allows the MBMS client to return the sought content either in its entirety (if successfully retrieved via nominal MBMS transmission and/or Associated Delivery Procedures), or a portion of that file if the entire content could not be successfully recovered, via a partial file response. The partial file handling mechanism today is exclusively initiated by client request for such a partial file. However, we believe that the most appropriate solution should enable the server to indicate partial file capability, independent of the client request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Spec text change to permit a partial file handling capable MBMS application to initially request strictly the full file, and upon receiving a signal from the MBMS client that only a partial file is available, may lead to a subsequent file request that contains indication of its acceptance of partial file content. Correction of existing typos such as removing the word “Request” in the title of clause 7.9.2.2 (“HTTP Request Response Format for Partial Files”), and similar word deletion of the phrase in clause 7.9.1 “…HTTP Request Response format as defined in clause 7.9.2.2…”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inability to leverage partial file delivery to a compatible application to the fullest possible extent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.9.1, 7.9.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 26.247 (Rel-15), CR S4-200982 |  |  |
|  |  | X |  | Other core specifications |  |  |  | TS 26.347 (Rel-15), CR S4-200984 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S4-200868 |  |  |  |  |  |  |  |  |

1st CHANGE: Modifications to clause 7.9.1

### 7.9.1	General

Files may be lost entirely or partially during the transmission and the MBMS Client may not be able to recover the lost files. This clause provides recommended procedures for handling partial files in the MBMS client.

A partial file is defined as follows:

-	an FDT Instance is received that contains a File entry with a specific TOI.

-	the object associated to the TOI was not recovered by any recovery procedure (FEC, file repair, etc.). The MBMS client determines that a file will not be recovered. A file is not recovered if the starting time of the ADPD as defined in 9.3.2 is reached for that file and if present, the file repair for that file has not succeeded,

The partial file is the collection of all FDT Instance data that is assigned to the file, i.e.

-	Content-Location (URI of a file).

-	Content-Length (source file length in bytes)

-	Content-Type (content MIME type)

as well as all received bytes including their position in the original file. In addition, the mbms2015:IndependentUnitPositions attribute may be present.

If the application supports the handling of partial files, then the MBMS client should provide the partial file to the application including the position of all received bytes. The partial file handling capability information from the application to the MBMS client as well as the handing of the partial file data is generally out of scope of this specification.

However, if the application communicates with the MBMS client using HTTP methods then

-	the MBMS client may signal the availability of a partial file to a request according to the Response format as defined in clause 7.9.2.2.

-	The application may signal the capability to handle partical files by using a partial-file-accept request as defined in clause 7.9.2.1. If a partial-file-accept request is sent,

-	the MBMS client should respond. If it responds it shall use the HTTP Response format as defined in clause 7.9.2.2 and shall include all received bytes in the response.

-	Appropriate examples in the context of DASH over MBMS is provided in clause 7.9.2.3.

Independent units are defined in clause 7.9.3. The MBMS client may also support handling of independent units. If an MBMS client supports handling of independent units, it shall also support partial file handling.

END OF 1st CHANGE

2nd CHANGE: Modifications to clause 7.9.2.2

#### 7.9.2.2	HTTP Response Format for Partial Files

If the MBMS client receives a regular HTTP GET request that includes the ‘Accept’ header but which does not contain media type 'application/3gpp-partial', and the MBMS client has received a partial file with at least one byte, it should respond with HTTP/1.1 404 Not Found, and include in the response the ‘Content-Type’ header set to 'application/3gpp-partial'.

If the MBMS client receives a regular HTTP GET request that includes an ‘Accept’ header along with media type ‘application/3gpp-partial’ , i.e. a partial-file-accept request, and the MBMS client has received a partial file with at least one byte, then the MBMS client may respond with a partial file response defined as follows:

-	The response code shall be set to 200 OK

-	The Content-Type header shall be set to application/3gpp-partial.

-	The message body is a multipart message body shall be exactly the same format as the multipart/byteranges media type as described in RFC 7233 [5], Annex A. The multipart/byteranges media type includes one or more body parts, each with its own Content-Type and Content-Range fields as the means to convey the byte range(s) of the partial file being returned.  Each Content-Type header shall be set to the value of the Content-Type provided in the FDT Instance for this file. An extension header may be added 3gpp-access-position providing a byte position at which the handler of assigned to the Content-Type of the file may access the file. The value may be created from the mbms2015: IndependentUnitPositions, if present.

-	A cache directive should be included in the response to prevent any intermediate proxies from storing an incomplete file and serving it to another application. Example for such cache directive are "Cache-Control: max-age=0" or "Cache-Control: no-cache"

If the MBMS client receives a partial-file-accept request, and the MBMS client has received a partial file with no bytes (i.e. only the FDT Instance describing the file metadata is received), then the MBMS client may respond with a partial file response defined as follows:

-	The response code shall be set to 416 Requested Range Not Satisfiable

-	The Content-Type header shall be set to the value of the Content-Type provided in the FDT Instance for this file.

-	The Content-Range shall be set to bytes */Content-Length with Content-Length the value of the attribute Content-Length provided in the FDT Instance for this file.

END OF 2nd CHANGE