3GPP TS 26.511 V1.3.0 (2020-04)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

5G Media Streaming (5GMS);

Profiles, Codecs and Formats

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	5GMS codecs and formats capabilities	10

4.1	Introduction	10

4.2	Video	10

4.2.1	H.264 (AVC)	10

4.2.1.1	Decoding	10

4.2.1.2	Encoding	10

4.2.1.3	Media Profiles: Mapping to 5GMS Delivery	11

4.2.1.3.1	AVC-HD	11

4.2.1.3.1.1	ISO BMFF File Format	11

4.2.1.3.1.2	CMAF Track Definition	11

4.2.1.3.1.3	CMAF Switching Set Definition	11

4.2.1.3.1.4	Playback Requirements	11

4.2.1.3.1.5	Content Generation Requirements	12

4.2.1.3.2	AVC-FullHD	12

4.2.1.3.2.1	ISO BMFF File Format	12

4.2.1.3.2.2	CMAF Track Definition	12

4.2.1.3.2.3	CMAF Switching Set Definition	12

4.2.1.3.2.4	Playback Requirements	12

4.2.1.3.2.5	Content Generation Requirements	13

4.2.1.3.3	AVC-UHD	13

4.2.1.3.3.1	ISO BMFF File Format	13

4.2.1.3.2.2	CMAF Track Definition	13

4.2.1.3.3.3	CMAF Switching Set Definition	13

4.2.1.3.3.4	Playback Requirements	13

4.2.1.3.3.5	Content Generation Requirements	14

4.2.2	H.265 (HEVC)	14

4.2.2.1	Decoding	14

4.2.2.2	Encoding	14

4.2.2.3	Media Profiles: Mapping to 5G Media Streaming	15

4.2.2.3.1	HEVC-HD	15

4.2.2.3.1.1	ISO BMFF File Format	15

4.2.2.3.1.2	CMAF Track Definition	15

4.2.2.3.1.3	CMAF Switching Set Definition	15

4.2.2.3.1.4	Playback Requirements	15

4.2.2.3.1.5	Content Generation Requirements	16

4.2.2.3.2	HEVC-FullHD	16

4.2.2.3.2.1	ISO BMFF File Format	16

4.2.2.3.2.2	CMAF Track Definition	16

4.2.2.3.2.3	CMAF Switching Set Definition	16

4.2.2.3.2.4	Playback Requirements	17

4.2.2.3.2.5	Content Generation Requirements	17

4.2.2.3.3	HEVC-UHD	17

4.2.2.3.3.1	ISO BMFF File Format	17

4.2.2.3.2.2	CMAF Track Definition	17

4.2.2.3.3.3	CMAF Switching Set Definition	17

4.2.2.3.3.4	Playback Requirements	18

4.2.2.3.3.5	Content Generation Requirements	18

4.3	Audio	18

4.3.1	Decoding	18

4.3.2	Encoding	18

4.4	Speech	19

4.4.1	Decoding	19

4.4.2	Encoding	19

4.5	Subtitles	19

4.5.1	IMSC1.1. Text Track	19

4.5.1.1	Decoding Capability	19

4.5.1.2	ISO BMFF File Format	19

4.5.1.3	CMAF Track Definition	19

4.5.1.4	CMAF Switching Set Definition	19

4.5.1.5	Playback Requirements	19

5	5GMS Profiles	20

5.1	Introduction	20

5.2	Downlink streaming default profile	20

5.2.1	Introduction	20

5.2.2	Video decoding	20

5.2.3	Audio decoding	21

5.2.4	Speech decoding	21

5.2.5	Subtitles	21

5.2.6	Presentation format	21

5.2.7	Decapsulation and decryption	21

5.2.7.1	Introduction	21

5.2.7.2	Video media profiles	21

5.2.7.3	Speech media profiles	22

5.2.7.4	Audio media profiles	22

5.2.7.5	Subtitle media profiles	22

5.2.7.6	Encrypted content	22

5.2.8	Capability discovery	23

5.2.8.1	General	23

5.2.8.2	Video media profiles	23

5.2.8.3	Audio media profiles	24

5.2.8.4	Subtitle media profiles	24

5.2.8.5	Encryption mode discovery	24

5.3	Uplink streaming default profile	25

5.3.1	Introduction	25

5.3.2	Video encoding	25

5.3.4	Audio encoding	25

5.3.5	Speech encoding	25

5.3.6	Encapsulation	25

5.3.6.1	Introduction	25

5.3.6.2	Video media profile	25

5.3.6.3	Speech media profile	25

5.3.6.4	Audio media profile	26

5.4	Television (TV) profile	26

5.4.1	Video decoding	26

5.4.1.1	H.264 (AVC)	26

5.4.1.2	H.265 (HEVC)	26

5.4.1.1	H.264 (AVC)	26

5.4.1.2	H.265 (HEVC)	27

5.4.2	Audio decoding	27

5.4.3	Subtitle decoding	27

5.4.4	Decapsulation and decryption	27

5.4.4.1	Introduction	27

5.4.4.2	Video media profiles	27

5.4.4.3	Audio media profiles	28

5.4.4.4	Subtitle media profiles	29

5.4.4.5	Encrypted content	29

5.4.5	Capability discovery	29

5.5	Downlink 360 Virtual Reality (VR) profile	29

5.5.1	Video	29

5.5.1.1	Operation Points	29

5.5.1.2	DASH encapsulation	29

5.5.2	Audio	29

5.5.2.1	Operation Points	29

5.5.2.2	DASH encapsulation	30

5.6	Uplink 360 Virtual Reality (VR) profile	30

5.6.1	Video	30

5.6.1.1	Operation Points	30

5.6.1.2	Encapsulation	30

5.6.2	Audio	30

5.6.2.1	Operation Points	30

5.6.2.2	Encapsulation	30

Annex A (informative): Change history	31

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the profiles, codecs and formats for 5G Media Streaming (5GMS) services based on the 5G Media Streaming Architecture (5GMSA) including both unicast downlink and unicast uplink. The codec and format recommendations defined for each profile apply to the 5GMS client components in the user equipment (UE) as well as the Media Application Server (AS). Profiles are defined to address specific service scenarios. A default profile is defined in the case no other profile is claimed to be supported.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ITU-T Recommendation H.264 (06/2019): "Advanced video coding for generic audiovisual services".

[3]	ITU-T Recommendation H.265 (02/2018): "High efficiency video coding".

[4]	3GPP TS 26.117: "5G Media Streaming (5GMS); Speech and audio profiles".

[5]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[6]	3GPP TS 26.307: "Presentation Layer for 3GPP Services".

[7]	ISO/IEC 23000-19: "Information Technology Multimedia Application Format (MPEG-A) – Part 19: Common Media Application Format (CMAF) for segmented media".

NOTE:	This references the second edition which in FDIS stage is available as MPEG output document N18636.

[8]	ISO/IEC 23001-7: "MPEG systems technologies - Part 7: Common encryption in ISO base media file format files".

[9]	CTA-5003: "Web Application Video Ecosystem (WAVE): Device Playback Capabilities Specification" available here https://cdn.cta.tech/cta/media/media/resources/standards/pdfs/cta-5003-final.pdf.

[10]	3GPP TS 26.512: " 5G Media Streaming (5GMS); Protocols".

[11]	IETF RFC 6381: The 'Codecs' and 'Profiles' Parameters for "Bucket" Media Types.

[12]	3GPP TS 26.116: "Television (TV) over 3GPP Services; Video Profiles".

[13]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[14]	ISO/IEC 14496-12: "Information technology — Coding of audio-visual objects — Part 12: ISO base media file format".

[15]	ISO/IEC 14496-15: "Information technology — Coding of audio-visual objects — Part 15: Carriage of network abstraction layer (NAL) unit structured video in the ISO base media file format".

[16]	W3C IMSC1.1, "TTML Profiles for Internet Media Subtitles and Captions 1.1", http://www.w3.org/TR/ttml-imsc1.1.

[17]	ISO/IEC 14496-30: "Information technology — Coding of audio-visual objects — Part 30: Timed text and other visual overlays in ISO base media file format".

[18]	W3C Media Capabilities, "Media Capabilities", https://w3c.github.io/media-capabilities/, March 2020.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GMS	5G Media Streaming

5GMSA	5G Media Streaming Architecture

AMR	Adaptive Multi Rate

AMR-WB	Adaptive Multi Rate – Wide Band

API	Application Programming Interface

AS	Application Server

AVC	Advanced Video Coding

AVC-HD	Advanced Video Codec – High Definition

CMAF	Common Media Application Format

DASH	Dynamic Adaptive Streaming over HTTP

EVS	Enhanced Voice Services

HD	High Definition

HDR	High Dynamic Range

HEVC	High Efficiency Video Coding

HLG	Hybrid Log-Gamma

IMSC	Internet Media Subtitles and Captions

MPEG	Moving Picture Experts Group

MPEG-H	It is a group of international standards under development by the ISO/IEC Moving Picture Experts Group (MPEG) - formally known as ISO/IEC 23008 - High efficiency coding and media delivery in heterogeneous environments

OMAF	Omnidirectional Media Application Format

PSS	Packet-switched Streaming Service

TTML	Timed Text Markup Language

TV	Television

UE	User Equipment

UHD	Ultra High Definition

VR	Virtual Reality

VCL	Varnish Configuration Language

# 4	5GMS codecs and formats capabilities

## 4.1	Introduction

This clause defines codecs and formats capabilities for video, audio, speech and subtitles.

## 4.2	Video

### 4.2.1	H.264 (AVC)

#### 4.2.1.1	Decoding

The following H.264 (AVC) media decoding capabilities are defined:

-	AVC-HD-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 3.1 [2] bitstreams, for which the maximum VCL Bit Rate is constrained to be 14Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1000 and 1200, respectively.

-	AVC-FullHD-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 4.0 [2] bitstreams.

-	AVC-UHD-Dec: the capability to decode H.264/AVC Progressive High Profile Level 5.1 [2] bitstreams for H.264/AVC with the following additional restrictions and requirements:

-	the maximum VCL Bit Rate is constrained to be 120Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively.

-	the bitstream does not contain more than 10 slices per picture.

NOTE:	High Profile for H.264/AVC excludes Flexible macro-block order, Arbitrary slice ordering, Redundant slices, Data partition.

#### 4.2.1.2	Encoding

The following H.264 (AVC) media encoding capabilities are defined:

-	AVC-HD-Enc: the capability to encode a video signal with

-	up to 108,000 macroblocks per second, and

-	up to a frame size of 3,600 macroblocks, and

-	up to 120 frames per second, and

-	the Chroma format being 4:2:0, and

-	the bit depth being 8 bit,

to a bitstream that is decodable by a decoder that is AVC-HD-Dec capable as defined in clause 4.2.1.1.

-	AVC-FullHD-Enc: the capability to encode a video signal with

-	up to 245,760 macroblocks per second, and

-	up to a frame size of 8,192 macroblocks, and

-	up to 240 frames per second, and

-	the Chroma format being 4:2:0, and

-	the bit depth being 8 bit,

to a bitstream that is decodable by a decoder that is AVC-FullHD-Dec capable as defined in clause 4.2.1.1.

-	AVC-UHD-Enc: the capability to encode a video signal with

-	up to 983,040 macroblocks per second, and

-	up to a frame size of 36,864 macroblocks, and

-	up to 480 frames per second, and

-	the Chroma format being 4:2:0, and

-	the bit depth being 8 bit,

to a bitstream that is decodable by a decoder that is AVC-UHD-Dec capable as defined in clause 4.2.1.1.

#### 4.2.1.3	Media Profiles: Mapping to 5GMS Delivery

##### 4.2.1.3.1	AVC-HD

###### 4.2.1.3.1.1	ISO BMFF File Format

If AVC-HD media is provided in a bitstream that is decodable by a decoder capable of the AVC-HD-Dec decoding capabilities as defined in clause 4.2.1.1 and the media is encapsulated in an ISO BMFF Track [14], then the file format track shall conform to the requirements of the codec entry 'avc1' or 'avc3' as defined in ISO/IEC 14496-15 [15].

###### 4.2.1.3.1.2	CMAF Track Definition

If AVC-HD media is provided in an CMAF track, then the CMAF track shall conform to

-	the requirements of the ISO BMFF File format track defined in clause 4.2.1.3.1.1,

-	the general CMAF Track constraints in ISO/IEC 23000-19, clause 7, as well as

-	the general video track constraints defined in ISO/IEC 23000-19, clause 9.

###### 4.2.1.3.1.3	CMAF Switching Set Definition

If AVC-HD media is provided in an CMAF Switching Set, then

-	every CMAF track in the CMAF Switching Set shall conform to the requirements of CMAF Track in clause 4.2.1.3.1.2,

-	the general CMAF Switching Set constraints in ISO/IEC 23000-19 [27], clause 7, as well as

-	the general CMAF video track Switching Set constraints defined in ISO/IEC 23000-19 [7], clause 9.

###### 4.2.1.3.1.4	Playback Requirements

For a receiver supporting the AVC-HD media profile the following applies:

-	It shall support the AVC-HD-Dec decoding capabilities as defined in clause 4.2.1.1.

-	It shall support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [9] for any content conforming to a CMAF Switching Set as defined in clause 4.2.1.3.1.3 namely:

-	8.2 Sequential Track Playback

-	8.3	Random Access to Fragment

-	8.4 Random Access to Time

-	8.5 Switching Set Playback

-	8.6 Regular Playback of Chunked Content

-	8.7 Regular Playback of Chunked Content, non-aligned append

-	It should support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [9] for any content conforming to a CMAF Switching Set as defined in clause 4.2.1.3.1.3 namely:

-	8.9 Out-Of-Order Loading

-	8.10 Overlapping Fragments

-	8.12 Playback of Encrypted Content

###### 4.2.1.3.1.5	Content Generation Requirements

For a transmitter supporting the AVC-HD media profile the following applies:

-	It shall support the AVC-HD-Enc encoding capabilities as defined in clause 4.2.1.2.

-	It shall support the generation of a CMAF Track that conforms to the requirements in clause 4.2.1.3.1.2.

-	If used for Adaptive Bit Rate (ABR) distribution, it shall support the generation of a CMAF Switching Set as defined in clause 4.2.1.3.1.3.

##### 4.2.1.3.2	AVC-FullHD

###### 4.2.1.3.2.1	ISO BMFF File Format

If AVC-FullHD media is provided in a bitstream that is decodable by a decoder capable of the AVC-FullHD-Dec decoding capabilities as defined in clause 4.2.1.1 and the media is encapsulated in an ISO BMFF Track [14], then the file format track shall conform to the requirements of the codec entry 'avc1' or 'avc3' as defined in ISO/IEC 14496-15 [15].

###### 4.2.1.3.2.2	CMAF Track Definition

If AVC-FullHD media is provided in an CMAF track, then the CMAF track shall conform to

-	the requirements of the ISO BMFF File format track defined in clause 4.2.1.3.2.1,

-	the general CMAF Track constraints in ISO/IEC 23000-19, clause 7, as well as

-	the general video track constraints defined in ISO/IEC 23000-19, clause 9.

###### 4.2.1.3.2.3	CMAF Switching Set Definition

If AVC-FullHD media is provided in an CMAF Switching Set, then

-	every CMAF track in the CMAF Switching Set shall conform to the requirements of CMAF Track in clause 4.2.1.3.2.2,

-	the general CMAF Switching Set constraints in ISO/IEC 23000-19 [27], clause 7, as well as

-	the general CMAF video track Switching Set constraints defined in ISO/IEC 23000-19 [7], clause 9.

###### 4.2.1.3.2.4	Playback Requirements

For a receiver supporting the AVC-FullHD media profile the following applies:

-	It shall support the AVC-FullHD-Dec decoding capabilities as defined in clause 4.2.1.1.

-	It shall support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [9] for any content conforming to a CMAF Switching Set as defined in clause 4.2.1.3.2.3 namely:

-	8.2 Sequential Track Playback

-	8.3	Random Access to Fragment

-	8.4 Random Access to Time

-	8.5 Switching Set Playback

-	8.6 Regular Playback of Chunked Content

-	8.7 Regular Playback of Chunked Content, non-aligned append

-	It should support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [9] for any content conforming to a CMAF Switching Set as defined in clause 4.2.1.3.2.3 namely:

-	8.9 Out-Of-Order Loading

-	8.10 Overlapping Fragments

-	8.12 Playback of Encrypted Content

###### 4.2.1.3.2.5	Content Generation Requirements

For a transmitter supporting the AVC-FullHD media profile the following applies:

-	It shall support the AVC-FullHD-Enc encoding capabilities as defined in clause 4.2.1.2.

-	It shall support the generation of a CMAF Track that conforms to the requirements in clause 4.2.1.3.2.2.

-	If used for Adaptive Bit Rate (ABR) distribution, it shall support the generation of a CMAF Switching Set as defined in clause 4.2.1.3.2.3.

##### 4.2.1.3.3	AVC-UHD

###### 4.2.1.3.3.1	ISO BMFF File Format

If AVC-UHD media is provided in a bitstream that is decodable by a decoder capable of the AVC-UHD-Dec decoding capabilities as defined in clause 4.2.1.1 and the media is encapsulated in an ISO BMFF Track [14], then the file format track shall conform to the requirements of the codec entry 'avc1' or 'avc3' as defined in ISO/IEC 14496-15 [15].

###### 4.2.1.3.2.2	CMAF Track Definition

If AVC-UHD media is provided in an CMAF track, then the CMAF track shall conform to

-	the requirements of the ISO BMFF File format track defined in clause 4.2.1.3.3.1,

-	the general CMAF Track constraints in ISO/IEC 23000-19, clause 7, as well as

-	the general video track constraints defined in ISO/IEC 23000-19, clause 9.

###### 4.2.1.3.3.3	CMAF Switching Set Definition

If AVC-UHD media is provided in an CMAF Switching Set, then

-	every CMAF track in the CMAF Switching Set shall conform to the requirements of CMAF Track in clause 4.2.1.3.3.2,

-	the general CMAF Switching Set constraints in ISO/IEC 23000-19 [27], clause 7, as well as

-	the general CMAF video track Switching Set constraints defined in ISO/IEC 23000-19 [7], clause 9.

###### 4.2.1.3.3.4	Playback Requirements

For a receiver supporting the AVC-UHD media profile the following applies:

-	It shall support the AVC-UHD-Dec decoding capabilities as defined in clause 4.2.1.1.

-	It shall support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [9] for any content conforming to a CMAF Switching Set as defined in clause 4.2.1.3.3.3 namely:

-	8.2 Sequential Track Playback

-	8.3	Random Access to Fragment

-	8.4 Random Access to Time

-	8.5 Switching Set Playback

-	8.6 Regular Playback of Chunked Content

-	8.7 Regular Playback of Chunked Content, non-aligned append

-	It should support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [9] for any content conforming to a CMAF Switching Set as defined in clause 4.2.1.3.3.3 namely:

-	8.9 Out-Of-Order Loading

-	8.10 Overlapping Fragments

-	8.12 Playback of Encrypted Content

###### 4.2.1.3.3.5	Content Generation Requirements

For a transmitter supporting the AVC-UHD media profile the following applies:

-	It shall support the AVC-UHD-Enc encoding capabilities as defined in clause 4.2.1.2.

-	It shall support the generation of a CMAF Track that conforms to the requirements in clause 4.2.1.3.3.2.

-	If used for Adaptive Bit Rate (ABR) distribution, it shall support the generation of a CMAF Switching Set as defined in clause 4.2.1.3.3.3.

### 4.2.2	H.265 (HEVC)

#### 4.2.2.1	Decoding

The following H.265 (HEVC) media decoding capabilities are defined:

-	HEVC-HD-Dec: the capability to decode H.265 (HEVC) Main Profile, Main Tier, Level 3.1[3] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

-	HEVC-FullHD-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 4.1[3] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

-	HEVC-UHD-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 5.1[3] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

#### 4.2.2.2	Encoding

The following H.265 (HEVC) media encoding capabilities are defined:

-	HEVC-HD-Enc: the capability to encode a video signal with

-	up to 33,177,600 luma samples per second, and

-	up to a luma picture size of 983,040 samples, and

-	up to 120 frames per second, and

-	the Chroma format being 4:2:0, and

-	the bit depth being 8 bit,

to a bitstream that is decodable by a decoder that is HEVC-HD-Dec capable as defined in clause 4.2.2.1.

-	HEVC-FullHD-Enc: the capability to encode a video signal with

-	up to 133,693,440 luma samples per second, and

-	up to a luma picture size of 2,228,224 samples, and

-	up to 240 frames per second, and

-	the Chroma format being 4:2:0, and

-	the bit depth being either 8 or 10 bit,

to a bitstream that is decodable by a decoder that is HEVC-FullHD-Dec capable as defined in clause 4.2.2.1.

-	HEVC-UHD-Enc: the capability to encode a video signal with

-	up to 534,773,760 luma samples per second, and

-	up to a luma picture size of 8,912,896 samples, and

-	up to 480 frames per second, and

-	the Chroma format being 4:2:0, and

-	the bit depth being either 8 or 10 bit,

to a bitstream that is decodable by a decoder that is HEVC-UHD-Dec capable as defined in clause 4.2.2.1.

#### 4.2.2.3	Media Profiles: Mapping to 5G Media Streaming

##### 4.2.2.3.1	HEVC-HD

###### 4.2.2.3.1.1	ISO BMFF File Format

If HEVC-HD media is provided in a bitstream that is decodable by a decoder capable of the HEVC-HD-Dec decoding capabilities as defined in clause 4.2.1.1 and the media is encapsulated in an ISO BMFF Track [14], then the file format track shall conform to the requirements of the codec entry 'hvc1' or 'hev1' as defined in ISO/IEC 14496-15 [15].

###### 4.2.2.3.1.2	CMAF Track Definition

If HEVC-HD media is provided in an CMAF track, then the CMAF track shall conform to

-	the requirements of the ISO BMFF File format track defined in clause 4.2.2.3.1.1,

-	the general CMAF Track constraints in ISO/IEC 23000-19, clause 7, as well as

-	the general video track constraints defined in ISO/IEC 23000-19, clause 9.

###### 4.2.2.3.1.3	CMAF Switching Set Definition

If HEVC-HD media is provided in an CMAF Switching Set, then

-	every CMAF track in the CMAF Switching Set shall conform to the requirements of CMAF Track in clause 4.2.2.3.1.2,

-	the general CMAF Switching Set constraints in ISO/IEC 23000-19 [27], clause 7, as well as

-	the general CMAF video track Switching Set constraints defined in ISO/IEC 23000-19 [7], clause 9.

###### 4.2.2.3.1.4	Playback Requirements

For a receiver supporting the HEVC-HD media profile the following applies:

-	It shall support the HEVC-HD-Dec decoding capabilities as defined in clause 4.2.1.1.

-	It shall support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [9] for any content conforming to a CMAF Switching Set as defined in clause 4.2.2.3.1.3 namely:

-	8.2 Sequential Track Playback

-	8.3	Random Access to Fragment

-	8.4 Random Access to Time

-	8.5 Switching Set Playback

-	8.6 Regular Playback of Chunked Content

-	8.7 Regular Playback of Chunked Content, non-aligned append

-	It should support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [9] for any content conforming to a CMAF Switching Set as defined in clause 4.2.2.3.1.3 namely:

-	8.9 Out-Of-Order Loading

-	8.10 Overlapping Fragments

-	8.12 Playback of Encrypted Content

###### 4.2.2.3.1.5	Content Generation Requirements

For a transmitter supporting the HEVC-HD media profile the following applies:

-	It shall support the HEVC-HD-Enc encoding capabilities as defined in clause 4.2.2.2.

-	It shall support the generation of a CMAF Track that conforms to the requirements in clause 4.2.2.3.1.2.

-	If used for Adaptive Bit Rate (ABR) distribution, it shall support the generation of a CMAF Switching Set as defined in clause 4.2.2.3.1.3.

##### 4.2.2.3.2	HEVC-FullHD

###### 4.2.2.3.2.1	ISO BMFF File Format

If HEVC-FullHD media is provided in a bitstream that is decodable by a decoder capable of the HEVC-FullHD-Dec decoding capabilities as defined in clause 4.2.1.1 and the media is encapsulated in an ISO BMFF Track [14], then the file format track shall conform to the requirements of the codec entry 'hvc1' or 'hev1' as defined in ISO/IEC 14496-15 [15].

###### 4.2.2.3.2.2	CMAF Track Definition

If HEVC-FullHD media is provided in an CMAF track, then the CMAF track shall conform to

-	the requirements of the ISO BMFF File format track defined in clause 4.2.2.3.2.1,

-	the general CMAF Track constraints in ISO/IEC 23000-19, clause 7, as well as

-	the general video track constraints defined in ISO/IEC 23000-19, clause 9.

###### 4.2.2.3.2.3	CMAF Switching Set Definition

If HEVC-FullHD media is provided in an CMAF Switching Set, then

-	every CMAF track in the CMAF Switching Set shall conform to the requirements of CMAF Track in clause 4.2.2.3.2.2,

-	the general CMAF Switching Set constraints in ISO/IEC 23000-19 [27], clause 7, as well as

-	the general CMAF video track Switching Set constraints defined in ISO/IEC 23000-19 [7], clause 9.

###### 4.2.2.3.2.4	Playback Requirements

For a receiver supporting the HEVC-FullHD media profile the following applies:

-	It shall support the HEVC-FullHD-Dec decoding capabilities as defined in clause 4.2.1.1.

-	It shall support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [9] for any content conforming to a CMAF Switching Set as defined in clause 4.2.2.3.2.3 namely:

-	8.2 Sequential Track Playback

-	8.3	Random Access to Fragment

-	8.4 Random Access to Time

-	8.5 Switching Set Playback

-	8.6 Regular Playback of Chunked Content

-	8.7 Regular Playback of Chunked Content, non-aligned append

-	It should support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [9] for any content conforming to a CMAF Switching Set as defined in clause 4.2.2.3.2.3 namely:

-	8.9 Out-Of-Order Loading

-	8.10 Overlapping Fragments

-	8.12 Playback of Encrypted Content

###### 4.2.2.3.2.5	Content Generation Requirements

For a transmitter supporting the HEVC-FullHD media profile the following applies:

-	It shall support the HEVC-FullHD-Enc encoding capabilities as defined in clause 4.2.2.2.

-	It shall support the generation of a CMAF Track that conforms to the requirements in clause 4.2.2.3.2.2.

-	If used for Adaptive Bit Rate (ABR) distribution, it shall support the generation of a CMAF Switching Set as defined in clause 4.2.2.3.2.3.

##### 4.2.2.3.3	HEVC-UHD

###### 4.2.2.3.3.1	ISO BMFF File Format

If HEVC-UHD media is provided in a bitstream that is decodable by a decoder capable of the HEVC-UHD-Dec decoding capabilities as defined in clause 4.2.1.1 and the media is encapsulated in an ISO BMFF Track [14], then the file format track shall conform to the requirements of the codec entry 'hvc1' or 'hev1' as defined in ISO/IEC 14496-15 [15].

###### 4.2.2.3.2.2	CMAF Track Definition

If HEVC-UHD media is provided in an CMAF track, then the CMAF track shall conform to

-	the requirements of the ISO BMFF File format track defined in clause 4.2.2.3.3.1,

-	the general CMAF Track constraints in ISO/IEC 23000-19, clause 7, as well as

-	the general video track constraints defined in ISO/IEC 23000-19, clause 9.

###### 4.2.2.3.3.3	CMAF Switching Set Definition

If HEVC-UHD media is provided in an CMAF Switching Set, then

-	every CMAF track in the CMAF Switching Set shall conform to the requirements of CMAF Track in clause 4.2.2.3.3.2,

-	the general CMAF Switching Set constraints in ISO/IEC 23000-19 [27], clause 7, as well as

-	the general CMAF video track Switching Set constraints defined in ISO/IEC 23000-19 [7], clause 9.

###### 4.2.2.3.3.4	Playback Requirements

For a receiver supporting the HEVC-UHD media profile the following applies:

-	It shall support the HEVC-UHD-Dec decoding capabilities as defined in clause 4.2.1.1.

-	It shall support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [9] for any content conforming to a CMAF Switching Set as defined in clause 4.2.2.3.3.3 namely:

-	8.2 Sequential Track Playback

-	8.3	Random Access to Fragment

-	8.4 Random Access to Time

-	8.5 Switching Set Playback

-	8.6 Regular Playback of Chunked Content

-	8.7 Regular Playback of Chunked Content, non-aligned append

-	It should support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [9] for any content conforming to a CMAF Switching Set as defined in clause 4.2.2.3.3.3 namely:

-	8.9 Out-Of-Order Loading

-	8.10 Overlapping Fragments

-	8.12 Playback of Encrypted Content

###### 4.2.2.3.3.5	Content Generation Requirements

For a transmitter supporting the HEVC-UHD media profile the following applies:

-	It shall support the HEVC-UHD-Enc encoding capabilities as defined in clause 4.2.2.2.

-	It shall support the generation of a CMAF Track that conforms to the requirements in clause 4.2.2.3.3.2.

-	If used for Adaptive Bit Rate (ABR) distribution, it shall support the generation of a CMAF Switching Set as defined in clause 4.2.2.3.3.3.

## 4.3	Audio

### 4.3.1	Decoding

Audio decoding capabilities are defined in 3GPP TS 26.117 [4] clause 5.3.

### 4.3.2	Encoding

Audio encoding capabilities are defined in 3GPP TS 26.117 [4] clause 5.3.

## 4.4	Speech

### 4.4.1	Decoding

Speech decoding capabilities are defined in 3GPP TS 26.117 [4] clause 5.2.

### 4.4.2	Encoding

Speech encoding capabilities are defined in 3GPP TS 26.117 [4] clause 5.3.4.5.

## 4.5	Subtitles

### 4.5.1	IMSC1.1. Text Track

#### 4.5.1.1	Decoding Capability

The IMSC1.1. Text Track decoding capability is defined as IMSC1.1-TEXT-DEC. Support for IMSC1.1-TEXT-DEC requires the support of an IMSC1.1 text processor as defined in W3C IMSC1.1 [16].

#### 4.5.1.2	ISO BMFF File Format

If an IMSC1.1. Text Track is provided in a bitstream that is decodable by a decoder capable of the IMSC1.1-TEXT-DEC decoding capabilities as defined in clause 4.5.1.1 and the media is encapsulated in an ISO BMFF Track [14], then the file format track shall conform to the requirements of the codec entry 'imt2' as defined in ISO/IEC 14496-30 [17].

#### 4.5.1.3	CMAF Track Definition

If IMSC1.1-TEXT-DEC media is provided in an CMAF track, then the CMAF track shall conform to

-	the requirements of the ISO BMFF File format track defined in clause 4.5.1.2,

-	the general CMAF Track constraints in ISO/IEC 23000-19, clause 7, as well as

-	the general subtitle track constraints defined in ISO/IEC 23000-19, clause 11.

-	the IMSC1.1. text track constraints defined in ISO/IEC 23000-19, Annex L.2.

#### 4.5.1.4	CMAF Switching Set Definition

If IMSC1.1-TEXT-DEC media is provided in an CMAF Switching Set, then

-	only a single CMAF track according to the requirements in clause 4.5.1.3 shall be present.

#### 4.5.1.5	Playback Requirements

For a receiver supporting the IMSC1.1. Text Track media profile the following applies:

-	It shall support the IMSC1.1-TEXT-DEC decoding capabilities as defined in clause 4.5.1.1.

-	It shall support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [9] for any content conforming to a CMAF Switching Set as defined in clause 4.5.1.4, namely:

-	8.2 Sequential Track Playback

-	8.3	Random Access to Fragment

-	8.4 Random Access to Time

-	It should support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [9] for any content conforming to a CMAF Switching Set as defined in clause 4.2.1.4 namely:

-	8.9 Out-Of-Order Loading

-	8.10 Overlapping Fragments

-	8.12 Playback of Encrypted Content

# 5	5GMS Profiles

## 5.1	Introduction

A 5GMS profile describes a set of capability requirements associated to a service scenario. A default profile is defined for minimum media requirements to be supported in case no other profile is claimed to be supported. The Television (TV) profile covers live and on demand streaming of audio-visual services. The 360 Virtual Reality (VR) profile covers the live and on demand streaming of 360 content.

## 5.2	Downlink streaming default profile

### 5.2.1	Introduction

This profile defines selected UE-based 5GMS Client functionalities as defined in TS26.501 [5] and shown in Figure 4.2.2-1 in TS 26.501 [5]. The following functions are defined in this clause:

-	Media Decapsulation

-	Media Decryption

-	Media Decoding

-	Media Presentation and Rendering

### 5.2.2	Video decoding

[

Option A (PSS capabilities)

If the 5GMS client supports the reception of video,

-	AVC-HD-Dec decoding capability shall be supported as defined in clause 4.2.1.1.

-	HEVC-HD-Dec decoding capability should be supported as defined in clause 4.2.2.1.

End of option A

Option B (upgraded capabilities)

If the 5GMS client supports the reception of video,

-	AVC-FullHD-Dec decoding capability shall be supported as defined in clause 4.2.1.1.

-	HEVC-FullHD-Dec decoding capability shall be supported as defined in clause 4.2.2.1.

End of option B

]

### 5.2.3	Audio decoding

If the 5GMS client supports the reception of audio:

-	eAAC+ decoding capability should be supported as defined in 3GPP TS 26.117 [4] clause 5.3.

-	AMR-WB+ decoding capability may be supported as defined in 3GPP TS 26.117 [4] clause 5.3.

### 5.2.4	Speech decoding

If the 5GMS client supports the reception of audio:

-	the EVS decoding capability shall be supported as defined in 3GPP TS 26.117 [4] clause 5.2.

-	the AMR-WB decoding capability should be supported as defined in 3GPP TS 26.117 [4] clause 5.2.

-	the AMR decoding capability may be supported as defined in 3GPP TS 26.117 [4] clause 5.2.

### 5.2.5	Subtitles

If the 5GMS client supports the reception of subtitles:

-	the IMSC1.1 Text Track decoding capabilities IMSC1.1-TEXT-DEC as defined in clause 4.5.1.1 should be supported.

### 5.2.6	Presentation format

If 5GMS client supports presentation, then 3GPP HTML5 Profile as defined in 3GPP TS 26.307 [6] should be supported.

### 5.2.7	Decapsulation and decryption

#### 5.2.7.1	Introduction

This clause defines the requirements in terms of decapsulation and decryption of media for a 5GMSd Media Player in combination with codecs. Media Encapsulation in 5G Media Streaming for downlink is defined based on the MPEG Common Media Application Format (CMAF) [7]. Based on this, this clause documents the requirements and recommendations for the support of media profiles by the definition of media player requirements.

#### 5.2.7.2	Video media profiles

If the 5GMSd client supports the reception of video, then the following applies:

[

Option A (PSS capabilities)

-	the AVC-HD playback requirements as defined in clause 4.2.1.3.1.4 shall be supported.

-	the HEVC-HD playback requirements as defined in clause 4.2.2.3.1.4 should be supported.

End of option A

Option B (upgraded capabilities)

-	the AVC-HD playback requirements as defined in clause 4.2.1.3.1.4 shall be supported.

-	the AVC-FullHD playback requirements as defined in clause 4.2.1.3.2.4 shall be supported.

-	the AVC-UHD playback requirements as defined in clause 4.2.1.3.3.4 may be supported.

-	the HEVC-HD playback requirements as defined in clause 4.2.2.3.1.4 shall be supported.

-	the HEVC-FullHD playback requirements as defined in clause 4.2.2.3.2.4 shall be supported.

-	the HEVC-UHD playback requirements as defined in clause 4.2.2.3.3.4 may be supported.

End of option B]

#### 5.2.7.3	Speech media profiles

If the 5GMSd client supports the reception of speech, then the following shall be supported:

-	the EVS playback requirements as defined in 3GPP TS 26.117 [4] clause 7.4.2.4.

If the 5GMSd client supports the reception of speech, then the following should be supported:

-	the AMR-WB playback requirements as defined in 3GPP TS 26.117 [4] clause 7.3.2.4.

If the 5GMSd client supports the reception of speech, then the following may be supported:

-	the AMR playback requirements as defined in 3GPP TS 26.117 [4] clause 7.2.2.4.

#### 5.2.7.4	Audio media profiles

If the 5GMSd client supports the reception of audio, then the following should be supported:

-	the eAAC+ stereo playback requirements as defined in 3GPP TS 26.117 [4] clause 7.5.2.4.

If the 5GMSd client supports the reception of audio, then the following may be supported:

-	the AMR-WB+ playback requirements as defined in 3GPP TS 26.117 [4] clause 7.6.2.4.

#### 5.2.7.5	Subtitle media profiles

If the 5GMSd client supports the reception of subtitle, then the following should be supported:

-	the IMSC1.1 text track playback requirements as defined in clause 4.5.1.5.

#### 5.2.7.6	Encrypted content

If the 5GMSd client supports encrypted content and any of the video playback requirement as defined in clause 5.2.7.2, then the 5GMSd client shall support the playback requirements for encrypted content as documented in clause 8 of CTA-WAVE 5003 [9], clause 8.12 for either:

-	video content encrypted according to [7] clause 8, using the 'cenc' AES-CTR subsample pattern encryption scheme, as specified in [8], clause 10.1; or

-	video content encrypted according to [7] clause 8, using the 'cbcs' AES-CBC subsample pattern encryption scheme, as specified in [8], clause 10.4, with the following restrictions Pattern Block length of 10 and an encrypt:skip pattern of 1:9 as defined in clause 9.6 of [8].

If the 5GMSd client supports decrypted content and any of the video playback requirement in clause 5.2.7.2, then the 5GMSd client should support the playback requirements for encrypted content as documented in clause 8 of CTA-WAVE 5003 [9], clause 8.12 for both:

-	video content encrypted according to [7] clause 8, using the 'cenc' AES-CTR subsample pattern encryption scheme, as specified in [8], clause 10.1, and

-	video content encrypted according to [7] clause 8, using the 'cbcs' AES-CBC subsample pattern encryption scheme, as specified in [8], clause 10.4, with the following restrictions Pattern Block length of 10 and an encrypt:skip pattern of 1:9 as defined in clause 9.6 of [8].

If the 5GMSd client supports encrypted content and any of the speech and audio playback requirement in clause 5.2.7.3 and clause 5.2.7.4, then the 5GMSd client shall support the playback requirements for encrypted content as documented in clause 8 of CTA-WAVE 5003 [9], clause 8.12 for either:

-	audio content encrypted according to [7] clause 8, using the 'cenc' AES-CTR subsample pattern encryption scheme, as specified in [8], clause 10.1, or

-	audio content encrypted according to [7] clause 8, using the 'cbc1' AES-CBC subsample pattern encryption scheme, as specified in [8], clause 9.7.

If the 5GMSd client supports encrypted content and any of the speech and audio playback requirement in clause 5.2.7.3 and clause 5.2.7.4, then the 5GMSd client should support the playback requirements for encrypted content as documented in clause 8 of CTA-WAVE 5003 [9], clause 8.12 for both:

-	audio content encrypted according to [7] clause 8, using the 'cenc' AES-CTR subsample pattern encryption scheme, as specified in [8], clause 10.1, and

-	audio content encrypted according to [7] clause 8, using the 'cbc1' AES-CBC subsample pattern encryption scheme, as specified in [8], clause 9.7.

Any subtitle track, if present, should not be encrypted.

### 5.2.8	Capability discovery

#### 5.2.8.1	General

A 5GMSd client is expected to support capability discovery such that 5GMS-aware applications can identify if a specific media profile is supported. In order to identify if a media profile is supported, the 5GMS client may provide an API as defined in TS 26.512 [10] in via the M7d interface, for which the client can be queried with a specific MIME type string, if the media profile is supported.

The MIME types follow RFC 6381 [11].

A 5GMSd client should support at least one of the following capability discovery mechanisms for media profiles:

-	If isTypeSupported() for the media profile with argument <profiles> results in a yes, then the respective media profile is supported with the requirements defined in a specific clause.

-	If isTypeSupported() for the media profile with argument <codecs> results in a yes, then the respective media profile is supported with the requirements defined in a specific clause.

-	If a conforming CMAF header is provided for playback initialization and 5GMS Client does not throw an error response, then the respective media profile is supported with the requirements defined in a specific clause.

For each media profile mentioned in clause 5.2.6, the <profiles> parameter and the <codecs> parameter are provided in the following. These parameters should be used in the capability exchange.

#### 5.2.8.2	Video media profiles

For AVC-HD,

-	the <profiles> parameter is compatible to video/mp4 profiles='cmfc'

-	the <codecs> parameter is compatible to either 'avc1.640028' or 'avc3.640028'

For AVC-FullHD,

-	the <profiles> parameter is compatible to video/mp4 profiles='cmfc'

-	the <codecs> parameter is compatible to either 'avc1.640029' or 'avc3.640029'

For AVC-UHD,

-	the <profiles> parameter is compatible to video/mp4 profiles='cmfc'

-	the <codecs> parameter is compatible to either 'avc1.640028' or 'avc3.640028'

For HEVC-HD,

-	the <profiles> parameter is compatible to video/mp4 profiles='cmfc'

-	the <codecs> parameter is compatible to either 'hvc1.1.2.L93.B0' or 'hev1.1.2.L93.B0'

For HEVC-FullHD,

-	the <profiles> parameter is compatible to video/mp4 profiles='cmfc'

-	the <codecs> parameter is compatible to either 'hvc1.2.4.L123.B0' or 'hev1.2.4.L123.B0'

For HEVC-UHD,

-	the <profiles> parameter is compatible to video/mp4 profiles='cmfc'

-	the <codecs> parameter is compatible to either 'hvc1.2.4.L153.B0' or 'hev1.2.4.L153.B0'

#### 5.2.8.3	Audio media profiles

For AMR,

-	the <profiles> parameter is defined in TS26.117, clause 7.2.2.4 as audio/mp4 profiles='camr'

-	the <codecss> parameter is defined in TS26.117, clause 7.2.2.4 as 'samr'

For AMR-WB,

-	the <profiles> parameter is defined in TS26.117, clause 7.3.2.4 as audio/mp4 profiles='camw'

-	the <codecs> parameter is defined in TS26.117, clause 7.3.2.4 as 'samw'

For EVS,

-	the <profiles> parameter is defined in TS26.117, clause 7.4.2.4 as audio/mp4 profiles='cevs'

-	the <codecs> parameter is defined in TS26.117, clause 7.4.2.4 as 'sevs'

For eAAC+ stereo,

-	the <profiles> parameter is defined in TS26.117, clause 7.6.2.4 as audio/mp4 profiles='ceac'

-	the <codecs> parameter is defined in TS26.117, clause 7.6.2.4 as 'mp4a'

For AMR-WB+,

-	the <profiles> parameter is defined in TS26.117, clause 7.7.2.4 as audio/mp4 profiles='camp'

-	the <codecs> parameter is defined in TS26.117, clause 7.7.2.4 as 'sawp'

#### 5.2.8.4	Subtitle media profiles

For IMSC1.1 Text Tracks,

-	the <profiles> parameter is defined in ISO/IEC 23000-19 [7], clause 11.3.3 as application/mp4 profiles='im2t'.

-	the <codecs> parameter is defined in ISO/IEC 23000-19 [7], clause 11.3.3 as 'stpp.ttml.im2t'.

#### 5.2.8.5	Encryption mode discovery

Encryption mode discovery is FFS.

## 5.3	Uplink streaming default profile

### 5.3.1	Introduction

This profile defines selected UE-based 5GMS Media Streamer functionalities as defined in TS 26.501[5] and shown in Figure 4.3.2-1 in TS 26.501 [5]. The following functions are defined in this clause,

-	Media Encoding

-	Media Upstream Client Encapsulation

### 5.3.2	Video encoding

If the 5GMS UE supports the transmission of video:

-	the real-time encoding capabilities for HEVC-FullHD-Enc as defined in clause 4.2.2.2 defined as the sender requirements for HEVC-FullHD-Enc Operation Point shall be supported.

### 5.3.4	Audio encoding

If the 5GMS UE supports the transmission of audio:

-	the sender requirements for the eAAC+ stereo Operation Point as defined in 3GPP TS 26.117 [4] clause 6.3.2.3 should be supported.

### 5.3.5	Speech encoding

If the 5GMS UE supports the transmission of speech:

-	the sender requirements for the EVS Operation Point as defined in 3GPP TS 26.117 [4] clause 6.2.4.3 shall be supported.

-	the sender requirements for the AMR-WB Operation Point as defined in 3GPP TS 26.117 [4] clause 6.2.3.3 should be supported.

-	the sender requirements for the AMR Operation Point as defined in 3GPP TS 26.117 [4] clause 6.2.2.3 may be supported.

### 5.3.6	Encapsulation

#### 5.3.6.1	Introduction

This clause defines the requirements in terms of encapsulation of media for a 5GMSu Media Streamer in combination with codecs. Media Encapsulation in 5G Media Streaming for uplink is defined based on the MPEG Common Media Application Format (CMAF) [7].  Based on this, this clause documents the requirements and recommendations for the support of media profiles by the definition of media streamer requirements.

#### 5.3.6.2	Video media profile

If the 5GMS UE supports the streaming of video, then the following shall be supported:

-	the content generation requirements for the HEVC-FullHD Media Profile as defined in clause 4.2.2.3.2.5

-	the sender requirements for the HEVC-FullHD Operation Point as defined in clause 5.3.2.

#### 5.3.6.3	Speech media profile

If the 5GMS client supports the transmission of speech, then the following shall be supported:

-	the EVS content generation requirements as defined in 3GPP TS 26.117 [4] clause 7.4.2.5.

If the 5GMS client supports the transmission of speech, then the following should be supported:

-	the AMR-WB content generation requirements as defined in 3GPP TS 26.117 [4] clause 7.3.2.5.

If the 5GMS client supports the transmission of speech, then the following may be supported:

-	the AMR content generation requirements as defined in 3GPP TS 26.117 [4] clause 7.2.2.5.

#### 5.3.6.4	Audio media profile

If the 5GMS client supports the transmssion of audio, then the following should be supported:

-	the eAAC+ stereo content generation requirements as defined in 3GPP TS 26.117 [4] clause 7.6.2.5.

## 5.4	Television (TV) profile

### 5.4.1	Video decoding

[

Option A (PSS capabilities)

#### 5.4.1.1	H.264 (AVC)

If the 5GMS client supports the Television (TV) profile, it shall support the H.264/AVC 720p HD Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.4.2.6.

If the 5GMS client supports the Television (TV) profile, it should support the H.264/AVC Full HD Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.4.3.6.

#### 5.4.1.2	H.265 (HEVC)

If the 5GMS client supports the Television (TV) profile, it should support:

-	H.265/HEVC 720p HD Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.2.7,

-	H.265/HEVC Full HD Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.3.7,

-	H.265/HEVC UHD Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.4.7,

-	H.265/HEVC Full HD HDR Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.5.8,

-	H.265/HEVC UHD HDR Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.6.8.

-	H.265/HEVC Full HD HDR HLG Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.7.7, and

-	H.265/HEVC UHD HDR HLG Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.8.7.

End of option A

Option B (upgraded capabilities)

#### 5.4.1.1	H.264 (AVC)

If the 5GMS client supports the Television (TV) profile, it shall support the H.264/AVC Full HD Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.4.3.6.

#### 5.4.1.2	H.265 (HEVC)

If the 5GMS client supports the Television (TV) profile, it shall support:

-	H.265/HEVC Full HD Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.3.7,

If the 5GMS client supports the Television (TV) profile, it should support:

-	H.265/HEVC UHD Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.4.7,

-	H.265/HEVC Full HD HDR Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.5.8, and

-	H.265/HEVC UHD HDR Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.6.8.

-	H.265/HEVC Full HD HDR HLG Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.7.7, and

-	H.265/HEVC UHD HDR HLG Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.8.7.

End of option B

]

### 5.4.2	Audio decoding

If the 5GMS client supports the Television (TV) profile, it /should support eAAC+ stereo Operation point as defined in 3GPP TS 26. 117 [4], clause 6.3.3.

### 5.4.3	Subtitle decoding

If the 5GMS client supports the Television (TV) profile, it should support the IMSC-1.1-TEXT-DEC decoding capabilities as defined in clause 4.5.1.1.

### 5.4.4	Decapsulation and decryption

#### 5.4.4.1	Introduction

This clause defines the requirements in terms of decapsulation and decryption of media for a Television (TV) Profile Media Player in combination with codecs. Media Encapsulation in 5G Media Streaming for downlink is defined based on the MPEG Common Media Application Format (CMAF) [7]. However, TS 26.116 [7] defines file format encapsulation and DASH Mapping in clause 5.

This clause addresses decapsulation and decryption requirements in a sense that a 5GMS client supporting the Television (TV) profile requires to be able to process content that conforms to TS26.116 [7] encapsulation and CMAF encapsulation.

Editor’s NOTE: To be resolved: The CMAF Track and Switching Set constraints in this specification and the Representation and Adaptation Set constraints in TS26.116 [7] are considered almost identical, but it still needs to be checked in detail. In the absence of both, it is expected that content offered for TV Profile receivers conform to both specifications.

#### 5.4.4.2	Video media profiles

If the 5GMSd client supports supports the Television (TV) profile, then the following applies:

[

Option A (PSS capabilities)

-	the AVC-HD playback requirements as defined in clause 4.2.1.3.1.4 and the H.264/AVC 720p HD Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.4.2.6 shall be supported.

-	the AVC-FullHD playback requirements as defined in clause 4.2.1.3.1.4 and the H.264/AVC Full HD Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.4.3.6 should be supported.

-	the HEVC-HD playback requirements as defined in clause 4.2.2.3.1.4 and the H.265/HEVC 720p HD Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.2.7 should be supported.

-	the HEVC-FullHD playback requirements as defined in clause 4.2.2.3.2.4 and the H.265/HEVC Full HD Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.3.7 should be supported.

-	the HEVC-FullHD playback requirements as defined in clause 4.2.2.3.2.4 and the H.265/HEVC Full HD HDR Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.5.7 should be supported.

-	the HEVC-FullHD playback requirements as defined in clause 4.2.2.3.2.4 and the H.265/HEVC Full HD HLG Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.7.7 should be supported.

-	the HEVC-UHD playback requirements as defined in clause 4.2.2.3.3.4 and the H.265/HEVC UHD Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.4.7 may be supported.

-	the HEVC-UHD playback requirements as defined in clause 4.2.2.3.3.4 and the H.265/HEVC UHD HDR Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.5.7 may be supported.

-	the HEVC-UHD playback requirements as defined in clause 4.2.2.3.3.4 and the H.265/HEVC UHD HLG Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.8.7 may be supported.

End of option A

Option B (upgraded capabilities)

-	the AVC-HD playback requirements as defined in clause 4.2.1.3.1.4 and the H.264/AVC 720p HD Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.4.2.6 shall be supported.

-	the AVC-FullHD playback requirements as defined in clause 4.2.1.3.1.4 and the H.264/AVC Full HD Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.4.3.6 shall be supported.

-	the HEVC-HD playback requirements as defined in clause 4.2.2.3.1.4 and the H.265/HEVC 720p HD Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.2.7 shall be supported.

-	the HEVC-FullHD playback requirements as defined in clause 4.2.2.3.2.4 and the H.265/HEVC Full HD Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.3.7 shall be supported.

-	the HEVC-FullHD playback requirements as defined in clause 4.2.2.3.2.4 and the H.265/HEVC Full HD HDR Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.5.7 shall be supported.

-	the HEVC-FullHD playback requirements as defined in clause 4.2.2.3.2.4 and the H.265/HEVC Full HD HLG Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.7.7 shall be supported.

-	the HEVC-UHD playback requirements as defined in clause 4.2.2.3.3.4 and the H.265/HEVC UHD Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.4.7 may be supported.

-	the HEVC-UHD playback requirements as defined in clause 4.2.2.3.3.4 and the H.265/HEVC UHD HDR Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.5.7 may be supported.

-	the HEVC-UHD playback requirements as defined in clause 4.2.2.3.3.4 and the H.265/HEVC UHD HLG Operation Point Receiver requirements as specified in TS 26.116 [12], clause 4.5.8.7 may be supported.

End of option B]

#### 5.4.4.3	Audio media profiles

If the 5GMSd client supports the Television (TV) profile, then the following should be supported:

-	the eAAC+ stereo playback requirements as defined in 3GPP TS 26.117 [4] clause 7.5.2.4.

#### 5.4.4.4	Subtitle media profiles

If the 5GMSd client supports the Television (TV) profile, then the following should be supported:

-	the IMSC1.1 text track playback requirements as defined in clause 4.5.1.5.

#### 5.4.4.5	Encrypted content

If the 5GMSd client supports the Television (TV) profile the same requirements apply as in clause 5.2.7.6 for the default profile.

### 5.4.5	Capability discovery

If the 5GMSd client supports the Television (TV) profile it is expected to support capability discovery such that 5GMS-aware applications can identify if a specific media profile is supported.

The same requirements apply as in clause 5.2.8 for the default profile.

A 5GMSd client that supports the Television (TV) profile may support additional capability methods that permits to identify additional capabilities of media profile playback taking into account information provided in the MPD for TV Video Profiles. Examples include the W3C Media Capability APIs [18]. More details are FFS.

## 5.5	Downlink 360 Virtual Reality (VR) profile

### 5.5.1	Video

#### 5.5.1.1	Operation Points

If the 5GMS UE supports 360 VR video, it shall include a receiver that complies with:

-	the Basic H.264/AVC Operation Point Receiver requirements as specified in TS 26.118 [13], clause 5.1.4.

If the 5GMS UE supports 360 VR video, it should include a receiver that complies with:

-	the Main H.265/HEVC Operation Point Receiver requirements as specified in TS 26.118 [13], clause 5.1.5.

If the 5GMS UE supports 360 VR video, it may include a receiver that complies with:

-	the Flexible H.265/HEVC Operation Point Receiver requirements as specified in TS 26.118 [13], clause 5.1.6.

#### 5.5.1.2	DASH encapsulation

If the 5GMS UE supports 360 VR video for DASH services, it shall include a receiver that complies with:

-	the Basic Video Media Profile Receiver requirements for DASH as specified in TS 26.118 [13], clause 5.2.2.3.

If the 5GMS UE supports 360 VR video for DASH services, it should include a receiver that complies with:

-	the Main Video Media Profile Receiver requirements for DASH as specified in TS 26.118 [13], clause 5.2.3.3.

If the 5GMS UE supports 360 VR video for DASH services, it may include a receiver that complies with:

-	the Advanced Video Media Profile Receiver requirements for DASH as specified in TS 26.118 [13], clause 5.2.3.4.

### 5.5.2	Audio

#### 5.5.2.1	Operation Points

If the 5GMS UE supports 3D/VR audio, it should include a receiver that complies with:

-	the 3GPP MPEG-H Audio Operation Point Receiver requirements as specified in TS 26.118 [13], clause 6.1.4.

#### 5.5.2.2	DASH encapsulation

If the 5GMS UE supports 3D/VR audio for DASH services, it should include a receiver that complies with:

-	the OMAF 3D Audio Baseline Media Profile Receiver requirements for file format signalling and encapsulation as specified in TS 26.118 [13], clause 6.2.2.3.

## 5.6	Uplink 360 Virtual Reality (VR) profile

### 5.6.1	Video

#### 5.6.1.1	Operation Points

If the 5GMS UE supports 360 VR video, it shall include an encoder that can encode a bitstream at a minimum of 1080p resolution 60 fps that can be decoded by a receiver that complies with:

-	the Basic H.264/AVC Operation Point Receiver requirements as specified in TS 26.118 [13], clause 5.1.4.

If  the 5GMS UE supports 360 VR video, it should include an encoder that can encode a bitstream at a minmum of 2160p resolution 60 fps that can be decoded by a receiver that complies with:

-	the Main H.265/HEVC Operation Point Receiver requirements as specified in TS 26.118 [13], clause 5.1.5.

If the 5GMS UE supports 360 VR video, it may include an encoder that can encode a bitstream at a minimum of 2160p resolution 60 fps that can be decoded by a receiver that complies with:

-	the Flexible H.265/HEVC Operation Point Receiver requirements as specified in TS 26.118 [13], clause 5.1.6.

#### 5.6.1.2	Encapsulation

[TBD]

### 5.6.2	Audio

#### 5.6.2.1	Operation Points

If the 5GMS UE supports 3D/VR audio, it should include an encoder that can encode a bitstream that can be decoded by a receiver that complies with:

-	the 3GPP MPEG-H Audio Operation Point Receiver requirements as specified in TS 26.118 [13], clause 6.1.4.

#### 5.6.2.2	Encapsulation

[TBD]

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-12 | SA#86 | SP=19xxxx |  |  |  | Presented to TSG SA#86 (for information) | 1.0.0 |
| 2020-01 | SA4#107 | S4-200314 |  |  |  | Integration of S4-200033 and S4-200272 | 1.1.0 |
| 2020-04 | SA4#108-e | S4-200510 |  |  |  | Integration of S4-AHI936 | 1.2.0 |
| 2020-04 | SA4#108-e | S4-200631 |  |  |  | Integration of support for subtitles in S4-200626 | 1.3.0 |