| 3GPP TS 26.512 V1.2.0 (2020-05) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 5G Media Streaming (5GMS); Protocols (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	9

3.3	Abbreviations	9

4	Procedures for Downlink Streaming	9

4.1	General	9

4.2	APIs relevant to Downlink Streaming	10

4.3 	Procedures of the M1d (5GMS Provisioning) interface	10

4.3.1	General	10

4.3.2	Provisioning Session procedures	11

4.3.2.1	General	11

4.3.2.2	Create Provisioning Session	11

4.3.2.3	Read Provisioning Session properties	11

4.3.2.4	Update Provisioning Session properties	11

4.3.2.5	Delete Provisioning Session	11

4.3.3	Content Hosting Configuration procedures 	11

4.3.3.1	General	11

4.3.3.2	Create Content Hosting Configuration	11

4.3.3.3	Read Content Hosting Configuration properties	11

4.3.3.4	Update Content Hosting Configuration properties	12

4.3.3.5	Delete Content Hosting Configuration	12

4.3.4	Ingest Protocols procedures	12

4.3.4.1	General	12

4.3.4.2	Create Ingest Protocols	12

4.3.4.3	Read Ingest Protocols	12

4.3.4.4	Update Ingest Protocols	12

4.3.4.5	Delete Ingest Protocols	12

4.3.5	Content Preparation Template procedures	12

4.3.5.1	General	12

4.3.5.2	Create Content Preparation Template	12

4.3.5.3	Read Content Preparation Template	13

4.3.5.4	Update Content Preparation Template	13

4.3.5.5	Delete Content Preparation Template	13

4.3.6	Server Certificate procedures	13

4.3.6.1	General	13

4.3.6.2	Create Server Certificate	13

4.3.6.3	Read Server Certificate	13

4.3.6.4	Update Server Certificate	13

4.3.6.5	Delete Server Certificate	13

4.3.7	Dynamic Policy Configuration procedures	13

4.3.8	Consumption Reporting Configuration procedures	13

4.3.8.1	General	13

4.3.8.2	Create Consumption Reporting Configuration	14

4.3.8.3	Read Provisioning Session properties	14

4.3.8.4	Update Provisioning Session properties	14

4.3.8.5	Delete Provisioning Session	14

4.4	Procedures of the M2d (5GMS Ingest) interface	14

4.5	Procedures of the M3d interface	14

4.6	Procedures of the M4d (Media Streaming) interface	14

4.7	Procedures of the M5d (Media Session Handling) interface	14

4.7.1	Introduction	14

4.7.2	Procedures for dynamic policy invocation	14

4.7.3.	Procedures for consumption reporting	15

4.7.4.	Procedures for metrics reporting	16

4.8	Procedures of the M6d (UE Media Session Handling) interface	16

4.9	Procedures of the M7d (UE Media Player) interface	16

4.9.1	General	16

4.9.2.	Metrics reporting procedures	16

4.10 Procedures of the M8d interface	17

5	Procedures for Uplink Streaming	17

5.1	General	17

5.2	APIs relevant to Uplink Streaming	17

6	General aspects of APIs for 5G Media Streaming	17

6.1	HTTP resource URIs and paths	18

6.2	Usage of HTTP	18

6.3	HTTP response codes	18

6.4	Data types	18

7	Provisioning (M1) APIs	18

7.1	General	18

7.x	API name	18

7.x.1	General	18

7.x.2	Resource structure	18

7.x.3	Data model	18

7.x.3.1	Notifications	18

7.2	Provisioning Sessions API	19

7.2.1	Overview	19

7.2.2	Resource structure	19

7.2.3	Data model	19

7.3	Server Certificates Provisioning API	19

7.3.1	Overview	19

7.3.2	Resource structure	19

7.3.3	Data model	19

7.4	Content Processing Templates Provisioning API	19

7.4.1	Overview	19

7.4.2	Resource structure	19

7.4.3	Data model	19

7.5	Ingest Protocols API	19

7.5.1	Overview	19

7.5.2	Resource structure	19

7.5.3	Data model	19

7.5.4	Media ingest protocols	19

7.5.4.1	Introduction	19

7.5.4.2	DASH-based media ingest protocol	20

7.6	Content Hosting Configuration API	20

7.6.1	Overview	20

7.6.2	Resource structure	20

7.6.3	Data model	21

7.6.3.1 ContentHostingConfiguration resource	21

7.6.4	Operations	24

7.6.4.1	Overview	24

7.6.4.2	Content caching	24

7.6.4.3	Cache purging	24

7.6.4.4	Content processing	25

7.6.4.5	URL signing	25

7.6.4.6	Geofencing	26

7.7	Consumption Reporting Configuration API	26

7.7.1	Overview	26

7.7.2	Resource structure	26

7.7.3	Data model	27

7.7.3.1	Re-used data types	27

7.7.3.2	ConsumptionReportingConfiguration resource	27

7.8	Metrics Reporting Provisioning API	27

7.8.1	Overview	27

7.8.2	Resource structure	27

7.8.3	Data model	27

7.9	Policy Templates Provisioning API	27

8	Media Ingest and Publish (M2) APIs	27

9	Internal (M3) APIs	27

10	Media Streaming (M4) APIs	28

11	Media Session Handling (M5) APIs	28

11.1	General	28

11.2	Service Access Information API	28

11.3	Consumption Reporting API	28

11.3.1	General	28

11.3.2	Data model	28

11.3.2.1	Introduction	28

11.3.2.2	ConsumptionReportingConfiguration resource	28

11.3.2.3	ConsumptionReporting resource	29

11.3.24	Type: ConsumptionReportingUnit	29

11.3.3	Resource structure	30

11.3.3.1	Introduction	30

11.3.3.2	Consumption Reporting Session resource	30

11.3.3.2.1	Introduction	30

11.3.3.2.2	Resource definition	30

11.3.3.2.3	Resource methods	30

11.3.3.2.3.1	GET	30

11.3.3.2.3.2	PUT	31

11.3.3.2.3.3	PATCH	31

11.3.3.2.3.4	POST	31

11.3.3.2.3.5	DELETE	31

11.3.3.3	Consumption Reporting Configuration Acquisition resource	31

11.3.3.3.1	Introduction	31

11.3.3.3.2	Resource definition	31

11.3.3.3.3	Resource methods	31

11.3.3.3.3.1	GET	31

11.3.3.3.3.2	PUT	32

11.3.3.3.3.3	PATCH	32

11.3.3.3.3.4	POST	32

11.3.3.3.3.5	DELETE	32

11.3.3.4	Consumption Reporting Data resource	32

11.3.3.4.1	Introduction	32

5.453.4.2	Resource definition	32

11.3.3.4.3	Resource methods	32

11.3.3.4.3.1	GET	32

11.3.3.4.3.2	PUT	32

11.3.3.4.3.3	PATCH	33

11.3.3.4.3.4	POST	33

11.3.3.4.3.5	DELETE	33

11.4	 Metrics Reporting API	33

11.5	Dynamic Policies API	34

5.4.1	Overview	34

5.4.2	Data model	34

5.4.2.1	DynamicPolicy resource type	34

5.4.3	Resource structure	34

5.4.3.1	General	34

5.4.3.3	Dynamic Policies resource	34

5.4.3.3.1	Introduction	34

5.4.3.3.2	Resource definition	34

5.4.3.3.3	Resource Method GET	35

5.4.3.2.4	Resource Method POST	35

5.4.3.4	Dynamic Policy resource	35

5.4.3.4.1	Introduction	35

5.4.3.4.2	Resource definition	35

5.4.3.4.3	Resource Method GET	36

5.4.3.4.4	Resource Method PUT	36

5.4.3.4.5	Resource Method PATCH	36

5.4.3.4.6	Resource Method DELETE	36

11.6	Network Assistance API	36

12	UE Media Session Handling (M6) APIs for uplink and downlink	36

13	UE Media Stream Handler (M7) APIs for uplink and downlink	36

14	Application (M8) APIs for uplink and downlink	36

Annex <X> (informative): Change history	37

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the set of protocols and APIs for 5G Media Streaming (5GMS) services based on the 5G Media Streaming Architecture (5GMSA). The 5GMSA supports services including MNO and third-party Downlink Media Streaming Services, and MNO and third-party Uplink Media Streaming Services.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[3]		DASH Industry Forum, "Specification of Live Media Ingest", 
https://dashif-documents.azurewebsites.net/Ingest/master/DASH-IF-Ingest.pdf

[4]	3GPP TS 26.247: "Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[5]	Standard ECMA-262, 5.1 Edition, "ECMAScript Language Specification", June 2011.

[6]	IETF RFC 6234: "US Secure Hash Algorithms (SHA and SHA-based HMAC and HKDF)".

[7]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[8]	ITU-T Recommendation X.509 (2005) | ISO/IEC 9594-8:2005: "Information Technology – Open Systems Interconnection – The Directory: Public-key and attribute certificate frameworks".

[9]	IETF RFC 7230: "Hypertext-Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[10]	IETF RFC 4648: "The Base16, Base32, and Base64 Data Encodings".

[11]	IEEE Standard 1003.1, Issue 7: "The Open Group Base Specifications", 2018.
https://pubs.opengroup.org/onlinepubs/9699919799/

[12]	3GPP TS 29.122, “T8 reference point for Northbound APIs”

[13]	3GPP TS 38.321, "NR; Medium Access Control (MAC) protocol specification".

[14]	3GPP TS 36.321, "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[15]		3GPP TS 27.007, "AT Command set for User Equipment (UE) – (Release 16)".

[16]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3", August 2018.

[17]	IETF RFC 7468: "Textual Encodings of PKIX, PKCS, and CMS Structures", April 2015.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GMS	5G Media Streaming

5GMSd	5GMS downlink

5GMSu	5GMS uplink

5GMSA	5GMS Architecture

CDN	Content Delivery Network / Content Distribution Network

CRUD	Create, Read, Update, Delete

MNO	Mobile Network Operator

QoE	Quality of Experience

QoS	Quality of Service

AF		Application Function

AS		Application Server

# 4	Procedures for Downlink Streaming

## 4.1	General

Editor’s Note: This clause gives a general introduction to the 5G Media Streaming Reference Points.

The architecture in Figure 4.1-1 represents the media architecture connecting UE internal functions and related network functions.

This clause provides an overview of the procedures on each interface, following the stage 2 description in TS 26.501 [2].

Figure 4.1-1: Media Architecture for unicast media downlink streaming (identical to TS26.501 [2], Figure 4.2.1-2)

Editor's Note: The diagram may be removed in the final specification to avoid any inconsistencies with stage 2 specifications

## 4.2	APIs relevant to Downlink Streaming

Table 4.2-1 below summarises the APIs used to provision and use the various downlink streaming features specified in TS 26.501 [2].

Table 4.2-1: Summary of APIs relevant to downlink streaming features

| 5GMSd feature | Abstract | Relevant APIs |  |  |
| --- | --- | --- | --- | --- |
|  |  | Interface | API name | Clause |
| Content Hosting | Content is ingested, hosted and distributed by the 5GMSd AS according to a Content Hosting Configuration associated with a Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Server Certificates Provisioning API | 7.3 |
|  |  |  | Content Processing Templates Provisioning API | 7.4 |
|  |  |  | Ingest Protocols API | 7.5 |
|  |  |  | Content Hosting Provisioning API | 7.6 |
|  |  | M2d |  |  |
|  |  | M4d | DASH (TS 26.247) or 3GP (TS 26.244) |  |
| Metrics reporting | The 5GMSd Client uploads metrics reports to the 5GMSd AF according to a provisioned Metrics Reporting Configuration it obtains from the Service Access Information for its Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Metrics Reporting Provisioning API |  |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  |  | Metrics Reporting API | 11.3 |
| Consumption Reporting | The 5GMSd Client provides feedback reports on currently consumed content according to a provisioned Consumption Reporting Configuration it obtains from the Service Access Information for its Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Consumption Reporting Provisioning API |  |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  |  | Consumption Reporting API | 11.4 |
| Dynamic Policy invocation | The 5GMSd Client activates different traffic treatment policies selected from a set of Policy Templates configured in its Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Policy Templates Provisioning API |  |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  |  | Dynamic Policies API | 11.5 |
| Network Assistance | The 5GMSd Client queries the 5GMSd AF for information about the network according to a provisioned configuration it obtains from the Service Access Information for its Provisioning Session. | M5d | Service Access Information API | 11.2 |
|  |  |  | Network Assistance API | 11.6 |

## 4.3 	Procedures of the M1d (5GMS Provisioning) interface

### 4.3.1	General

A 5GMSd Application Provider may use the procedures in this clause to provision the network for downlink media sessions that are operated by the 5GMSd Application Provider. These sessions may be DASH streaming sessions, progressive download sessions, or any other type of media streaming or distribution (e.g. HLS) sessions.

The M1d interface offers three different sets of procedures:

-	Configuration of content ingest at M2d for onward distribution over M4d by the 5GMSd AS: designed as an API that is equivalent to the functionality of a public CDN. The resource types involved in content hosting configuration are Ingest Protocols (clause 4.2.X), Content Preparation Template (clause 4.2.Y), Server Certificate (clause 4.2.Z) and Provisioning Session (4.2.2).

-	Configuration of dynamic policies: allows the configuration of Policy Templates at M5d that can be applied to M4d downlink sessions.

-	Configuration of reporting: permits the MNO to collect at M5d QoE and consumption reports about M4d downlink sessions.

A 5GMSd Application Provider may use any of these procedures, in any combination, to support its downlink media sessions.

### 4.3.2	Provisioning Session procedures

#### 4.3.2.1	General

Prior to configuring content hosting, dynamic policies, or reporting, the 5GMSd Application Provider shall create a new Provisioning Session. The following CRUD operations are used to manage a provisioning session.

#### 4.3.2.2	Create Provisioning Session

This procedure is used by the 5GMSd Application Provider to create a new Provisioning Session. The 5GMSd Application Provider shall use the HTTP POST method to create a new Provisioning Session. Upon successful creation, the 5GMSd AF shall respond with a 201 (Created) response message that includes the resource identifier of the newly created Provisioning Session in the body of the reply and the URL of the resource, including its resource identifier, shall be returned as part of the HTTP Location header field.

Editor’s Note: Terminology alignment with Application Service Configuration Id missing.

#### 4.3.2.3	Read Provisioning Session properties

This procedure is used by the 5GMSd Application Provider to obtain the properties of the Provisioning Session from the 5GMSd AF. The 5GMSd Application Provider uses the GET method for this purpose.

#### 4.3.2.4	Update Provisioning Session properties

The Update operation is not allowed on Provisioning Sessions.

#### 4.3.2.5	Delete Provisioning Session

This procedure is used by the 5GMSd Application Provider to delete a Provisioning Session. The 5GMSd AF will release any associated resources, purge any cached data, delete all QoS and reporting configurations associated with this Provisioning Session. The 5GMSd AF shall use the HTTP DELETE method for this purpose.

### 4.3.3	Content Hosting Configuration procedures

#### 4.3.3.1	General

These procedures are used by the 5GMSd Application Provider and the 5GMSd AF on M1d to configure the content hosting feature for downlink streaming. They are further elaborated in clause 5.2.

#### 4.3.3.2	Create Content Hosting Configuration

This procedure is used by the 5GMSd Application Provider to create a new Content Hosting Configuration. The 5GMSd Application Provider shall use the HTTP POST method for this purpose.

If the ingest uses the Push method, i.e. the pull attribute is set to False, then the path and entryPoint shall not be provided and are read-only. The targetDomain is assigned by the 5GMSd AF and is read-only.

If the procedure is successful, the 5GMSd AF shall generate a resource id representing the new Content Hosting Configuration. In this case, the 5GMSd AF shall respond with a 201 (Created) HTTP response message, and provide the URL to the newly created resource in the Location header field.

#### 4.3.3.3	Read Content Hosting Configuration properties

This procedure is used by the 5GMSd Application Provider to obtain the properties of an existing Content Hosting Configuration resource from the 5GMSd AF. The HTTP GET method shall be used for this purpose.

If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) response message that includes the Content Hosting Configuration in the response message body.

#### 4.3.3.4	Update Content Hosting Configuration properties

The update operation is invoked by the 5GMSd Application Provider to modify the properties of an existing Content Hosting Configuration resource. All available parameters, except the originDomain and targetDomain, may be updated. The HTTP PATCH or HTTP PUT methods shall be used for the update operation.

If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) and provide the content of the resource in the response, reflecting the successful update operation.

Editor’s Note: Check the HTTP Status Codes.

#### 4.3.3.5	Delete Content Hosting Configuration

This operation is used by the 5GMSd Application Provider to destroy an Content Hosting Configuration resource and to terminate the related distribution. The HTTP DELETE method shall be used for this purpose. As a result, the 5GMSd AF will release any associated network resources, purge any cached content, and delete any corresponding configurations.

If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) response message.

Editor’s Note: Check the HTTP Status Codes for DELETE.

### 4.3.4	Ingest Protocols procedures

#### 4.3.4.1	General

The set of content ingest protocols supported by the 5GMSd AS at interface M2d is described by the Ingest Protocols resource at M1d, as specified in clause 5.2.2.2.

#### 4.3.4.2	Create Ingest Protocols

The Create operation is not permitted for the Ingest Protocols resource.

#### 4.3.4.3	Read Ingest Protocols

Editor’s Note: Missing specification.

#### 4.3.4.4	Update Ingest Protocols

The Update operation is not permitted for the Ingest Protocols resource.

#### 4.3.4.5	Delete Ingest Protocols

The Delete operation is not permitted for the Ingest Protocols resource.

### 4.3.5	Content Preparation Template procedures

#### 4.3.5.1	General

The 5GMSd AS is able to process content ingested at interface M2d before serving it at interface M4d, as specified in clause 5.2.4.4. The content processing operations are specified in a Content Preparation Template resource, as specified in clause 5.2.2.3.

#### 4.3.5.2	Create Content Preparation Template

Editor’s Note: Missing specification.

If the MIME content type indicated in Content-Type is not understood by the 5GMSd AF, the creation of the Content Preparation Template resource shall fail with HTTP error response status code 422 (Unprocessable entity).

If the 5GMSd AF is unable to provision the resources indicated in the supplied Content Preparation Template, the creation operation shall fail with an HTTP response status code of 503 (Service Unavailable).

#### 4.3.5.3	Read Content Preparation Template

Editor’s Note: Missing specification.

#### 4.3.5.4	Update Content Preparation Template

Editor’s Note: Missing specification.

#### 4.3.5.5	Delete Content Preparation Template

Editor’s Note: Missing specification.

### 4.3.6	Server Certificate procedures

Each X.509 server certificate presented by the 5GMSd AS at interface M4d is represented by a Server Certificate resource at M1d.

#### 4.3.6.1	General

Editor’s Note: Missing specification.

#### 4.3.6.2	Create Server Certificate

Editor’s Note: Missing specification.

#### 4.3.6.3	Read Server Certificate

Editor’s Note: Missing specification.

#### 4.3.6.4	Update Server Certificate

Editor’s Note: Missing specification.

#### 4.3.6.5	Delete Server Certificate

Editor’s Note: Missing specification.

### 4.3.7	Dynamic Policy Configuration procedures

### 4.3.7.1	General

These procedures are used by the 5GMS Application Provider to configure the Policy Templates for streaming sessions of a particular Provisioning Session.

Since Policy Templates require 5GMS System operator verification, a Policy Template that is newly created cannot be used immediately. Upon creation, a Policy Template shall be in the pending state. Once it has been validated by the 5GMS System operator, it shall take the ready state, indicating that it may be applied to streaming sessions. If it is subsequently updated by the 5GMS Application Provider, a Policy Template shall return to the pending state, awaiting revalidation by the operator of the 5GMS System. Finally, a Policy Template may be suspended by the 5GMS System operator, e.g. in case of a violation of the usage terms or for some other reasons, which renders it unusable.

The 5GMSd/5GMSu AF shall verify the status of a Policy Template prior to allowing a Dynamic Policy Instance to instantiate it. Only Policy Templates in the ready state are eligible to be instantiated in this way.

Figure 4.3.7.1-1 below is a state diagram showing the life-cycle of a Policy Template:

Figure 4.3.7.1-1: Policy Template State Diagram

### 4.3.7.2	Create Policy Template

This procedure is used by the 5GMS Application Provider to create a new Policy Template. The HTTP POST method shall be used for this purpose.

If the procedure is successful, the 5GMSd/5GMSu AF shall generate a resource identifier to uniquely identify the newly created Policy Template. In that case, it shall respond with a 201 (Created) HTTP response message and provide the URL to the newly created resource in the Location header field.

The default state of a newly created Policy Template is pending.

### 4.3.7.3	Read Policy Template

This procedure is used by the 5GMS Application Provider and other 5GMSd/5GMSu AFs to query the properties of an existing Policy Template resource from the 5GMSd/5GMSu AF. The HTTP GET method shall be used for this purpose.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response that includes the Policy Template in the response message body.

### 4.3.7.4	Update Policy Template

The update operation is invoked by the 5GMS Application Provider to modify the properties of an existing Policy Template. All available properties except state may be updated. The HTTP PATCH or HTTP PUT methods shall be used for the update operation.

Any update to the policy template will change the state back into the pending state, which makes it temporarily unusable.

Editor’s Note: It is FFS, whether ANY update or SOME updates of the Policy Template will trigger a state change to pending.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response message that includes the Policy Template in the response message body. Modifications to read-only properties, such as changes to the state of a Policy Template, shall be rejected with a 403 (Forbidden) HTTP response.

### 4.3.7.5	Delete Policy Template

This operation is used by the 5GMS Application Provider to destroy a Policy Template resource. The HTTP DELETE method shall be used for this purpose. As a result, the 5GMSd/5GMSu AF will remove the Policy Template from any Provisioning Sessions that reference it.

Currently active streaming sessions using the destroyed Policy Template, if any exist, shall be stopped by the removal of the Policy Template.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response message.

### 4.3.8	Consumption Reporting Configuration procedures

4.3.8.1	General

These procedures are used by the 5GMSd Application Provider to activate and to configure consumption reporting. They are further elaborated in clause 5.4.2.

#### 4.3.8.2	Create Consumption Reporting Configuration

This procedure is used by the 5GMSd Application Provider to activate consumption reporting for a particular Provisioning Session. The 5GMSd Application Provider shall use the HTTP POST method to activate the consumption reporting procedure and to transmit the Consumption Reporting Configuration to the 5GMSd AF. Upon successful operation, the 5GMSd AF shall respond with a 201 (Created) response message and the same resource URL shall be returned in the Location header field.

#### 4.3.8.3	Read Provisioning Session properties

This procedure is used by the 5GMSd Application Provider to obtain the current Consumption Reporting Configuration from the 5GMSd AF. The 5GMSd Application Provider uses the GET method for this purpose.

#### 4.3.8.4	Update Provisioning Session properties

The update operation is invoked by the 5GMSd Application Provider to modify the current Consumption Reporting Configuration. All available parameters may be updated. The HTTP PATCH or HTTP PUT methods shall be used for the update operation.

If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) reflecting the successful update operation.

#### 4.3.8.5	Delete Provisioning Session

This operation is used by the 5GMSd Application Provider to terminate the related consumption reporting procedure. The HTTP DELETE method shall be used for this purpose. As a result, the 5GMSd AF will release any associated resources, purge any cached data, and delete any corresponding configurations.

If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) response message.

## 4.4	Procedures of the M2d (5GMS Ingest) interface

Editor’s Note: This clause should contain content ingestion procedures between Network External Media Application Servers and the 5GMSd AS. This Clause may be removed, in case only external referenceable content ingest procedures are used.

## 4.5	Procedures of the M3d interface

Interface M3d is internal and no procedures on this interface are specified.

## 4.6	Procedures of the M4d (Media Streaming) interface

Editor’s Note: This clause should contain content ingestion procedures between Network External Media Application Servers and the 5GMSd AS. This Clause may be removed, in case only external referenceable content ingest procedures are used.

## 4.7	Procedures of the M5d (Media Session Handling) interface

### 4.7.1	Introduction

Editor’s Note: This clause should contain the API related procedures for the Media Session Handling API. The Media Session Handling API is used for consumption reporting, for QoE reporting, for requesting different policy and charging treatments or for other network assistance services.

### 4.7.2	Procedures for Service Access Information

#### 4.7.2.1	General

Service Access Information is the set of parameters and addresses needed by the 5GMSd Client to activate reception of a downlink streaming session. This procedure specifies the case where the 5GMSd Client fetches the Service Access information from the 5GMSd AF using parameters it previously obtained via M8d.

#### 4.7.2.2	Create Service Access Information

The Create operation is not allowed on Service Access Information.

#### 4.7.2.3	Read Service Access Information properties

This procedure shall be used by the Media Session Handler to acquire Service Access Information from the 5GMSd AF. The Media Session Handler uses the GET method for this purpose.

The downlink streaming session for which the Media Session Handler is requesting data is identified by a unique reference contained in the path of the URL, as specified in clause 5.x.3.

#### 4.7.2.4	Update Service Access Information properties.

The Update operation is not allowed on Service Access Information.

#### 4.7.2.5	Delete Service Access Information properties

The Delete operation is not allowed on Service Access Information.

### 4.7.3	Procedures for dynamic policy invocation

Provisioning for Dynamic Policy usage is defined in section XX.

This procedure is used by a Media Session Handler to manage Dynamic Policy Instance resources via the M5d interface. A dynamic policy invocation consists of a Policy Template Id, flow description(s), a 5GMSd Application Service Configuration Id and potentially other parameters, according to TS 26.501 clause 5.7.

A Policy Template Id identifies the desired Policy Template to be applied to an application flow. A Policy Template includes properties such as specific QoS (e.g. background data) or different charging treatments. The 5GMSd AF combines the information from the Policy Template with dynamic information from the Media Session Handler to gather a complete set of parameters to invoke the N33 or N5 API call. The Policy Template may contain for example the AF identifier. Example values for a Policy Template Id are defined in Clause [].

The flow description allows the identification and classification of the media traffic, such as the packet filter sets [TS 23.501 Clause 5.7.6].

In order to instantiate a new dynamic policy, the Media Session Handler shall first create a resource for the Dynamic Policy Instance on the 5GMSd AF. When the Media Session Handler needs several dynamic policies, it repeats the step as often as needed.

The Media Session Handler creates a new Dynamic Policy Instance by sending an HTTP POST message to the 5GMSd AF. The body of the HTTP POST message shall include 5GMSd Application Service Configuration Id, the Policy Template Id and the traffic descriptor. The traffic descriptor identifies the actual application flow(s) to be policed according to the Policy Template. If the operation is successful, the 5GMSd AF creates a new resource URL representing the Dynamic Policy Instance. In this case, the 5GMSd AF shall respond to the Media Session Handler with a 201 Created HTTP response message, including the URL for the newly created Dynamic Policy Instance resource as the value of the Location header field.

Editor’s Note: At minimum, the N5 and N33 API requires the UE IP Address at time of API invocation. The full Flow Description is an optional element, when more fine-grained traffic flow identification is required.  It needs to be studied, how to enable usage of other traffic filtering parameters, such as an application id.

The Media Session Handler can modify the parameters of an existing Dynamic Policy Instance resource using either the HTTP PUT or PATCH methods, as appropriate to the desired update. The 5GMSd AF shall trigger the appropriate actions towards other Network Functions like PCF or NEF when all information is set.

Editor’s Note: It is not clear, what triggers the 5GMSd AF to start the PCF / NEF interactions.

The Media Session Handler can destroy a Dynamic Policy Instance resource using the HTTP DELETE method. As a result, the 5GMSd AF shall trigger the appropriate actions towards other Network Functions like PCF or NEF to remove the associated PCC rule.

Editor’s Note: Notification subscription will be added in the next version of the pCR.

### 4.7.4.	Procedures for consumption reporting

These procedures are used by the Media Session Handler and the Consumption Reporting functions of the 5GMSd Client to submit a consumption report via the M5d interface if Consumption Reporting is applied for a Downlink Streaming session. The metadata indicating that Consumption Reporting is applied for the Downlink Streaming session is described in clause X.

In order to create a resource for the consumption reporting procedure, the Media Session Handler shall send an HTTP POST message to the 5GMSd AF to create a new Consumption Reporting Instance resource. After receiving the HTTP POST message, if the Media Session Handler is authorized, the 5GMSd AF shall create a Consumption Reporting Instance. This Consumption Reporting Configuration resource is addressed by a URI that is described in clause X. The 5GMSd AF shall respond to the Media Session Handler with a 201 (Created) message including a Location header field containing the URI of the created resource and a message body. The Media Session Handler shall use the URI in the Location header received in the 201 (Created) response message in the subsequent exchanges with the 5GMSd AF. The 5GMSd AF may include the relevant Consumption Reporting Configuration in the message body.

If the Consumption Reporting Configuration is not present in the message body of the 201 (Created) message, the Media Session Handler shall send an HTTP GET message including the URI of the Consumption Reporting resource to the 5GMSd AF to fetch the consumption reporting parameters. The 5GMSd AF shall respond with a 200 (OK) message that includes the relevant Consumption Reporting Configuration in the message body. If the Consumption Reporting Configuration present in the message body of the 201 (Created) response message, the Media Session Handler should not send an HTTP GET message to the 5GMSd AF to acquire the consumption reporting parameters.

Whenever a consumption report event is triggered, the Media Session Handler shall send an HTTP POST message to the 5GMSd AF. If several 5GMSd AF instances are listed in the consumption reporting parameters as valid consumption reporting endpoints for the Consumption Reporting Instance, the Media Session Handler shall choose one and send the message to the selected. The 5GMSd AF shall respond with a 200 (OK) message to signal successful processing of the Consumption Reporting request. If the 5GMSd AF has updated the consumption reporting parameters for the associated Consumption Reporting Instance, the 5GMSd AF shall notify the Media Session Handler by including a Consumption Reporting Configuration in the body of the 200 (OK) response message containing the current consumption reporting parameters.

The Consumption Reporting API defining the resources, data models and related procedures for the creation and management of consumption reporting are described in clause X.

### 4.7.5.	Procedures for metrics reporting

These procedures shall be used by the Media Session Handler to control metrics reporting when such reporting is configured by the OAM via the 5G control channel.

The Media Session Handler shall subscribe to metrics configurations from the OAM according to TS 26.247 Annex L.1. When a metrics configuration is received, the Media Session Handler shall store this configuration and use it for all subsequent streaming sessions.

When a streaming session is started the Media Session Handler shall determine whether metrics from this session shall be reported. The determination shall be based on the sample percentage and streaming source filter specified in the stored metrics configuration, according to TS 26.247 Annex F.

If metrics are reported for the session, the Media Session Handler shall request the Media Player to create a metrics collection job. The Media Player shall return a reference to the created job, which the Media Session Handler shall use in all subsequent actions related to this job.

The Media Session Handler shall configure the metrics collection job with the set of metrics that shall be collected during the session. The format of the configuration shall be according to TS 26.247 Annex L.2, but note that only the metrics attribute in the configuration shall be used for this purpose.

The Media Session Handler shall regularly request the collected metrics from the Media Player according to the reporting interval specified in the metrics configuration. The metrics returned by the Media Player shall use the format as described in TS 26.247 clause 10.6, and the Media Session Handler shall forward these to the OAM according to TS 26.247 Annex L.1.

When the session is finished the Media Session Handler shall delete the metrics collection job.

## 4.8	Procedures of the M6d (UE Media Session Handling) interface

Editor’s Note: This clause should contain procedures for the interaction of 5GMSd Aware application and the Media Session Handler

## 4.9	Procedures of the M7d (UE Media Player) interface

Editor’s Note: This clause should contain procedures for the interaction of 5GMSd Aware application and the Media Player

### 4.9.1	General

### 4.9.2	Metrics reporting procedures

These procedures shall be used by the Media Session Handler function to control metrics reporting when such reporting is configured via metadata sent in-band via the media manifest.

When a streaming session is started, the Media Session Handler shall check if the manifest contains any metrics configuration, as specified in TS 26.247 clauses 10.4 and 10.5. If such a configuration is found, the Media Session Handler shall use it for the current streaming session.

The Media Session Handler shall first determine whether metrics from this session shall be reported. The determination shall be based on the sample percentage attribute specified in the metrics configuration, according to TS 26.247 clause 10.5.

If metrics are reported for the session, the Media Session Handler shall request the Media Player to create a metrics collection job. The Media Player shall return a reference to the created job, which the Media Session Handler shall use in all subsequent actions related to this job.

The Media Session Handler shall configure the metrics collector job with the set of metrics which shall be collected during the session. The format of the configuration shall be according to TS 26.247 Annex L.2, but note that only the metrics attribute in the configuration shall be used for this purpose.

The Media Session Handler shall regularly request the collected metrics from the Media Player according to the reporting interval specified in the metrics configuration. The metrics returned by the Media Player shall use the format as described in TS 26.247 clause 10.6, and the Media Session Handler shall forward these to the server address using the specified DNN according to the procedures described in TS 26.247 clause 10.6.

When the session is finished the Media Session Handler shall delete the metrics collection job.

### 4.9.3	Consumption reporting procedures

Before a streaming session is started, the Media Session Handler shall check if the Service Access Information contains any Consumption reporting configuration, as specified in clauses 4.7.3. If such a configuration is present, the Media Session Handler shall initiate consumption reporting based on this configuration for the current streaming session.

The Media Session Handler shall first determine whether consumption reporting is active for the session. The determination shall be based on the sample percentage attribute specified in the consumption reporting configuration. When the samplePercentage is not present or its value is 100, consumption reporting is active for the session. If the samplePercentage is less than 100, the Media Session Handler generates a random number which is uniformly distributed in the range 0 to100; consumption reporting is active for the session when the generated random number is of a lower value than the samplePercentage value.

If consumption reporting for this session is active, the Media Session Handler shall regularly determine the consumption reporting parameters defined in clause 11.3.2.4 from the Media Player through the M7d interface and shall report these values according to the reporting interval specified in the Client Consumption Reporting Configuration.

## 4.10 	Procedures of the M8d interface

Editor’s Note: This clause should contain procedures for the application.

# 5	Procedures for Uplink Streaming

## 5.1	General

## 5.2	APIs relevant to Uplink Streaming

Editor’s Note: Table, listing the relevant APIs for Uplink Streaming (Clause and apiName)

# 6	General aspects of APIs for 5G Media Streaming

<This section intends to contain the apiRoot definition, usage of HTTP 1.1 & HTTP2 and other common aspects>

## 6.1	HTTP resource URIs and paths

## 6.2	Usage of HTTP

## 6.3	HTTP response codes

## 6.4	Data types

### 6.4.1	General

### 6.4.2	Simple data types

Table 6.4.2-1 below specifies common simple data types used within the 5GMS APIs, including a short description of each. In cases where types from other specifications are reused, a reference is provided.

Table 6.4.2-1: Simple data types

| Type name | Type definition | Description | Reference |  |
| --- | --- | --- | --- | --- |
| Percentage | number | A percentage expressed as a floating point value between 0.0 and 100.0 (inclusive). |  |  |
| DurationSec | integer | An unsigned integer identifying a period of time expressed in units of seconds. | TS 29.122 [12] Table 5.2.1.3.2-2 |  |
| DateTime | string | An absolute date and time expressed using the OpenAPI date-time string format. | TS 29.122 [12] Table 5.2.1.3.2-2 |  |

# 7	Provisioning (M1) APIs

## 7.1	General

[

Editor’s Note: The following is a section template for API definitions, following TS 29.522.

## 7.x	API name

## 7.x.1	General

### 7.x.2	Resource structure

### 7.x.3	Data model

### 7.x.3.1	Notifications

Editor’s Note: Some APIs may contain notifications

]

## 7.2	Provisioning Sessions API

### 7.2.1	Overview

### 7.2.2	Resource structure

### 7.2.3	Data model

## 7.3	Server Certificates Provisioning API

### 7.3.1	Overview

The Server Certificates Provisioning API is used to provide X.509 [8] server certificates that can be referenced by a Content Hosting Configuration and subsequently presented by the 5GMSd AS when it distributes content to 5GMSd Clients at interface M4d using Transport Layer Security [12]. Server Certificates are provisioned within the scope of a Provisioning Session.

### 7.3.2	Resource structure

The Server Certificates Provisioning API is accessible through the following URL base path:

{apiRoot}/3gpp-m1d/v1/provisioning-sessions/{provisioningSessionId}/

Table 7.3.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column shall be appended to the URL base path.

Table 7.3.2-1: Operations supported by the Server Certificates Provisioning API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Reserve Server Certificate | certificates | POST | Invoked on the Server Certificates collection associated with a Provisioning Session to solicit a Certificate Signing Request for a new Server Certificate. The request message body shall be empty. If the operation succeeds, the URL of the reserved Server Certificate resource shall be returned in the Location header of the response and this shall comply with the sub-resource path specified below for manipulating Server Certificate resources in the collection. The body of the response shall be a PEM-encoded X.509 Certificate Signing Request generated by the 5GMSd AF, as specified in clause 7.3.3 below. |
| Retrieve Server Certificate | certificates/{certificateId} | GET | Used to retrieve a previously uploaded Server Certificate. If a Server Certificate resource has been reserved but not yet uploaded, this operation shall return 404 (Not Found). |
| Upload Server Certificate |  | PUT | Used by the 5GMSd Application Provider to supply a new Server Certificate in response to a solicited Certificate Signing Request, or to replace an existing Server Certificate. The body of the request message shall be a PEM-encoded X.509 certificate signed with the public key of the Certificate Signing Request, as specified in clause 7.3.3 below. The 5GMSd AF shall associate the Server Certificate with the private key it generated alongside the Certificate Signing Request. |
| Destroy Server Certificate |  | DELETE | Removes the specified Server Certificate from the set of certificates associated with the Provisioning Session. |

NOTE:	The Server Certificate resource identifier {certificateId} differs from the serial number of the X.509 certificate.

### 7.3.3	Data model

The Certificate Signing Request and the Server Certificate resource shall comply with the Privacy-Enhanced Mail (PEM) textual format specified in RFC 7468 [13], i.e. a Base64-encoded DER certificate request or certificate, including leading and trailing encapsulation boundary lines.

Certificate Signing Requests and Server Certificate resources shall have the MIME content type application/x-pem-file.

### 7.3.4	Operations

Under no circumstances shall the 5GMSd AF reveal the private key associated with the Certificate Signing Request to the 5GMSd Application Provider.

## 7.4	Content Preparation Templates Provisioning API

### 7.4.1	Overview

Content Preparation Templates are used to specify manipulations applied by a 5GMSd AS to media resources ingested at interface M2d for distribution at interface M4d. The Content Preparation Templates API is used to provision a Content Preparation Template within the scope of a Provisioning Session that can subsequently be referenced from a Content Hosting Configuration.

### 7.4.2	Resource structure

The Content Preparation Templates Provisioning API is accessible through the following URL base path:

{apiRoot}/3gpp-m1d/v1/provisioning-sessions/{provisioningSessionId}/

Table 7.4.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column shall be appended to the URL base path.

Table 7.4.2-1: Operations supported by the Content Preparation Templates Provisioning API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Content Preparation Template | content-preparation-templates | POST | Invoked on a Content Preparation Templates collection when supplying a new Content Preparation Template resource. If the operation succeeds, the URL of the newly created Content Preparation Template resource shall be returned in the Location header of the response and this shall comply with the sub-resource path specified below for manipulating Content Preparation Templates. |
| Retrieve Content Preparation Template | content-preparation-templates/‌{contentPreparationTemplateId} | GET | Used to retrieve a Content Preparation Template resource. |
| Update Content Preparation Template |  | PUT, PATCH | Used to modify an existing Content Preparation Template resource. |
| Destroy Content Preparation Template |  | DELETE | Used to destroy an existing Content Preparation Template resource. |

7.4.3	Data model

The data model of the Content Preparation Template resource shall be determined by its MIME content type.

### 7.4.4	Operations

Editor’s Note: Missing specification.

## 7.5	Ingest Protocols Discovery API

### 7.5.1	Overview

The Ingest Protocols Discovery API is used by a 5GMSd Application Provider to find out which content ingest protocols are supported by the 5GMSd AS(s) associated with a 5GMSd AF. One of the supported ingest protocols is subsequently indicated in a Content Hosting Configuration for downlink streaming.

### 7.5.2	Resource structure

The Ingest Protocols Discovery API is accessible through the following URL base path:

{apiRoot}/3gpp-m1d/v1/provisioning-sessions/{provisioningSessionId}/

Table 7.5.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column shall be appended to the URL base path.

Table 7.5.2-1: Operations supported by the Ingest Protocols Discovery API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Fetch list of supported ingest protocols | protocols | GET | This operation is used to retrieve a list of supported ingest protocols. |

### 7.5.3	Data model

#### 7.5.3.1	IngestProtocols resource

The data model for the IngestProtocols resource is specified in table 7.6.3.1-1 below:

Table 7.6.3.1-1: Definition of IngestProtocols resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| downlinkIngestProtocols | [URI String] | 1..1 | An array of URI strings, each one uniquely identifying a media ingest protocol supporting at interface M2d by the 5GMSd AS(s), as specified in clause 7.5.4. |

### 7.5.4	Media ingest protocols

#### 7.5.4.1	Introduction

Every media ingest protocol shall be uniquely identified by a URI string allocated within the scope of the name space urn:3gpp:5gms:ingest-protocol.

This specification defines a single DASH-based media ingest protocol.

#### 7.5.4.2	DASH-based media ingest protocol

If IngestConfiguration[protocol] in the Content Hosting Configuration is set to urn:3gpp:5gms:ingest-protocol:dash-if-ingest, media resources shall be ingested by the 5GMSd AS as specified by [3]. The IngestConfiguration[pull] shall be set to False, indicating that a Push-based protocol is used. The IngestConfiguration[entryPoint] parameter shall be set to the URL that will be used to upload the DASH segments and MPD to the 5GMSd AS at interface M2d. This entry point URL shall not contain a path: the path for the URL shall instead be specified by the IngestConfiguration[Path] parameter.7.6	Content Hosting Configuration API

## 7.6	Content Hosting Configuration API

### 7.6.1	Overview

This clause specifies the API that a 5GMSd Application Provider uses at interface M1d to provision and manage 5GMSd AS Content Hosting Configurations by interacting with a 5GMSd AF. Each such configuration is represented by a ContentHostingConfiguration, the data model for which is specified in clause 7.6.3 below. The RESTful resources for managing Content Hosting Configurations are specified in clause 7.6.2 and the operations on these resources are further elaborated in clause 7.6.4.

### 7.6.2	Resource structure

The Content Hosting Configuration API is accessible through this URL base paths:

{apiRoot}/3gpp-m1d/v1/provisioning-sesions/{provisioningSessionId}/

Table 7.6.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column shall be appended to the URL base path:

Table 7.6.2-1: Operations supported by the Content Hosting Configuration API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Content Hosting Configuration | content-hosting-configuration | POST | Used to create a Content Hosting Configuration resource. |
| Retrieve Content Hosting Configuration |  | GET | Used to retrieve an existing Content Hosting Configuration. |
| Update Content Hosting Configuration |  | PUT, PATCH | Used to modify an existing Content Hosting Configuration. |
| Delete Content Hosting Configuration |  | DELETE | Used to delete an existing Content Hosting Configuration. |
| Purge Content Hosting Configuration cache | content-hosting-configuration/purge | POST | This operation is used to invalidate some or all cached media resources associated with this Content Hosting Configuration. |

### 7.6.3	Data model

#### 7.6.3.1 	ContentHostingConfiguration resource

The data model for the ContentHostingConfiguration resource is specified in table 7.6.3.1-1 below:

Table 7.6.3.1-1: Definition of ContentHostingConfiguration resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| IngestConfiguration | Object | 1..1 | Describes the 5GMSd Application Provider’s origin server from which media resources will be ingested via interface M2d. |
| name | String | 1..1 | A name associated with this origin server. |
| path | String | 1..1 | The relative path which will be used to address the media resources at interface M2d. This path is provided by the 5GMSd AF in the case of Push. |
| pull | Boolean | 1..1 | Indicates whether to the 5GMSd AS shall use Pull or Push for ingesting the content. |
| protocol | String | 1..1 | Identifies the media ingest protocol. The set of supported protocols is defined in clause 7.5.4. |
| entryPoint | String | 1..1 | An entry point to ingest the content. The semantics of the entry point are dependent on the selected ingest protocol. In the case of Push ingest (pull flag is set to False), this parameter is returned by the 5GMSd AF to the 5GMSd Application Provider and indicates the entry point for pushing the content. In case of Pull (pull flag is set to True), the entryPoint shall be provided by the origin to the 5GMSd AF to indicate the location from which content is to be pulled. In this case, the entryPoint shall be used as the base URL. A request received by the 5GMSd AS is mapped to a URL using the provided base URL to fetch the content from the origin server. |
| DistributionConfiguration | [Object] | 1..n | Specifies the distribution method and configuration for the ingested content. More than one distribution may be configured for the ingested content, e.g. to offer different distribution configurations such as DASH and HLS. |
| contentPreparationTemplateId | String | 0..1 | Indicates that content preparation prior to distribution is requested by the 5GMSd Application Provider. It identifies the Content Preparation Template that shall be used as defined in clause 7.4 |
| targetDomain | String | 1..1 | All resources of the current distribution shall be accessible through this targetDomain FQDN. This FQDN is assigned by the 5GMS AF for the current distribution. There is a one-to-one association between the originDomain and the targetDomain for this distribution. |
| originDomain | String | 1..1 | The 5GMSd Application Provider assigns an originDomain for the distribution, which is for example used by the 5GMSd AS to select an appropriate certificate to present at M4d, and to set appropriate CORS HTTP response headers at M4d. The originDomain is associated with a targetDomain that is assigned by the 5GMSd AF. The 5GMSd Application Provider may choose to publish the content using its own originDomain (e.g. to use its own certificates or to avoid CORS issues). In such case, the 5GMSd Application Provider creates a CNAME DNS record to point from the originDomain to the targetDomain. |
| PathRewriteRules | [Object] | 0..n | An ordered list of rules for rewriting the ingest URL path to the distribution URL path. If multiple rules match a particular resource’s path, only the first rule, in order of appearance, shall be applied. |
| originPathPattern | String | 1..1 | A pattern that shall be used to match the path part of the M2d ingest URL for the resource. If the pattern matches, then the path mapping rule is applied. The path part of the ingest URL used for the matching shall include the leading “/”. The originPathPattern shall be provided as a regular expression as specified in [5]. |
| distributionPath | String | 1..1 | If the pathPattern is a match for the current resource, then the ingest path, starting with the “/” and ending with the last “/” in the path, excluding the last segment of the path, shall be replaced by this distributionPath in the URL exposed by the 5GMSd AS at M4d. |
| CachingConfiguration | [Object] | 0..1 | Defines a configuration of the 5GMSd AS cache for a matching subset of media resources ingested in relation to this Content Hosting Configuration. |
| urlPatternFilter | String | 1..1 | A pattern that will be used to match media resource URLs to determine whether a given media resource is eligible for caching by the 5GMSd AS. The format of the pattern shall be a regular expression as specified in [5]. |
| CachingDirectives | Object | 1..1 | If a urlPatternFilter applies to a resource, then the provided CachingDirectives shall be applied by the 5GMSd AS at M4d, potentially overwriting any origin caching directives ingested at M2d. |
| statusCodeFilters | [Integer] | 0..1 | The set of HTTP origin response status codes to which these CachingDirectives apply. The filter shall be provided as a regular expression as specified in [5]. If the list is empty, the CachingDirectives shall apply to all HTTP origin response status codes at M2d. |
| noCache | Boolean | 1..1 | If set to True, this indicates that the media resources matching the filters shall not be cached by the 5GMSd AS and shall be marked as not to be cached when served by the 5GMSd AS at M4d. |
| maxAge | Integer | 0..1 | The caching time-to-live period that shall be set on ingested media resources matching the filters. This determines the minimum period for which the 5GMSd AS shall cache matching media resources as well as the time-to-live period signalled by the 5GMSd AS at interface M4d when it serves such media resources. The time-to-live for a given media resource shall be calculated relative to the time it was ingested. |
| GeoFencing | Object | 0..1 | Limit access to the content to the indicated geographic areas. |
| locationType | String | 1..1 | The type of the location information. |
| locations | [String] | 1..1 | Array of locations from which access to the resources is to be allowed. |
| UrlSignature | Object | 0..1 | Defines the URL signing scheme. Only correctly signed and valid URLs will be allowed to access the content resource at M4d. |
| urlPattern | String | 1..1 | A pattern that shall be used to match M4d media resource URLs. The 5GMSd AS shall not serve a matching media resource at M4d unless it includes a valid authentication token. The format of the pattern shall be a regular expression as specified in [5]. |
| tokenName | String | 1..1 | The name of the M4d request query parameter that the Media Player must use to present the authentication token when required to do so. |
| passphraseName | String | 1..1 | The name of the query parameter that is used to refer to the passphrase when constructing the authentication token. Note that the token is not included in the cleartext part of the M4d URL query component. |
| passphrase | String | 1..1 | The shared secret between the 5GMSd Application Provider and the 5GMSd AS for this DistributionConfiguration. The passphrase is used in the computation and verification of the M4d authentication token but is never sent in-the-clear over that interface. |
| tokenExpiryName | String | 1..1 | The name of the M4d request query parameter that the Media Player must use to present the token expiry field. |
| useIPAddress | Boolean | 1..1 | If set to True, the IP address of the UE is included in the computation of the authentication token for resources that match urlPattern and access to matching media resources shall be allowed by the 5GMSd AF only when the M4d request is made from a UE with this IP address. |
| ipAddressName | String | 0..1 | The name of the M4d request query parameter that is encoded as part of the authentication token if the useIPAddress flag is set to True. Note that the IP address is not passed in the cleartext part of the M4d URL query component. |
| certificateId | String | 0..1 | When content is distributed using TLS [16], the X.509 [8] certificate for the origin domain is shared with the 5GMSd AF so that it can be presented by the 5GMSd AS in the TLS handshake at M4d. This attribute indicates the identifier of the certificate to use. |

### 7.6.4	Operations

#### 7.6.4.1	Overview

This clause defines the behaviour that is expected from the 5GMSd AS when the Content Hosting Configuration has been successfully provisioned. The main operations that are performed affect the caching and purging of cached content as well as the processing for media preparation and at the edge.

#### 7.6.4.2	Content caching

A Content Hosting Configuration may specify caching rules to be applied to media resources when they are distributed by the 5GMSd AS over interface M4d. The distribution shall use the urlPatternFilter in the CachingConfiguration object to determine which caching directives apply to that object. In case a media resource’s URL matches the pattern filter of more than one CachingConfiguration, the first match shall apply. In case no CachingConfiguration is identified as a match, the 5GMSd AS shall apply the caching directives that were received from the origin. In the case where no match is found and the origin server does not supply caching directives at M2d, then default caching directives based on the media resource type shall be applied.

A caching directive shall either indicate that a matching media resource is not to be cached by the 5GMSd AS, nor by downstream M4d clients (noCache set to True), or that the 5GMSd AS and downstream M4d clients are to cache it for maxAge seconds. The maxAge value applies relative to the time when a media resource was ingested, t_ingest. For an HTTP-based ingest, this corresponds to the Date header field in the HTTP request/response that carries the media resource at M2d. At the time t_ingest + maxAge, the object is considered stale and should not be served at M4d from the 5GMSd AS cache. The 5GMSd AS shall compensate for any synchronization skew between the origin and its own clock. This can be for instance done by including the max-stale HTTP cache directive in its M4d responses.

The maxAge value may be signalled at M4d by the 5GMSd AS using the Expires HTTP response header or the HTTP Cache-Control directives max-age or s-maxage.

When distributing a media resource using HTTP, a no-cache request may be translated into a no-cache and no-store HTTP Cache-Control directive and/or a max-age=0 HTTP Cache-Control directive.

By default, all origin HTTP header fields shall be assumed as not forwarded by the 5GMSd AS, unless specified otherwise by setting the flag originCacheHeaders to True.

#### 7.6.4.3	Cache purging

The 5GMSd Application Provider may perform a purge operation to invalidate some or all cached media resources of a particular Content Hosting Configuration. A regular expression describing the set of media resource URLs to be purged from the 5GMSd AS cache for the Content Hosting Configuration in question shall be supplied in the body of the request. The body shall be encoded using the application/x-www-form-urlencoded MIME type as a key–value pair, with the key being the string pattern and the value being the regular expression.

On receiving a purge request, the 5GMSd AF shall immediately invalidate all media resources in the 5GMSd AS cache matching the regular expression by declaring them as stale. Any request at interface M4d for a purged media resource will trigger the fetching (and possible caching) of the current version from the origin via M2d in case of a Pull-based ingest. For Push-based ingest, the request shall be responded to with a 404 (Not Found) HTTP response, until a new version of the object is pushed by the origin to the 5GMSd AS via M2d.

#### 7.6.4.4	Content processing

The 5GMSd AF can perform various content processing tasks (such as repackaging, encryption, ABR transcoding) on media resources ingested at M2d prior to serving them at M4d. These processing tasks shall be specified in a Content Preparation Template resource referenced from the Content Hosting Configuration object.

#### 7.6.4.5	URL signing

The URL signing procedure allows the 5GMSd Application Provider to prevent deep linking and unauthorized access to M4d media resources. It works by cryptographically signing some elements of the M4d request URL and then appending this authentication token to the URL as an additional query parameter. The token is generated by the 5GMSd Application Provider and supplied to the player, for example as part of an initial URL. When it receives a request that requires URL signing, the 5GMSd AS verifies the presence and validity of the token in the M4d request URL before allowing access to the requested media resource. The 5GMSd AS(s) and the origin share a secret that is encoded as part of the query parameter hash, but not shared with the 5GMSd Media Player.

The validity of the authentication token can also be limited to a single UE. If useIPAddress is set to True, then the public IP address of the UE as viewed by the 5GMSd AS, ue_public_ip_address, shall be incorporated into the token calculation. The parameter name shall be indicated by ipAddressName.

The shared secret shall be provided in UrlSignature[passphrase] as a string of length between 6 and 50 characters. The parameter name for the passphrase shall be provided by passphraseName.

The expiry time of the signed URL, tokenExpiry, shall be included as an additional query parameter in the URL exposed at M4d with the name indicated in tokenExpiryName. The expiry time shall be the string representation of the number of seconds from 1970-01-01T00:00:00Z UTC until the specified UTC date/time, ignoring leap seconds, as defined in section 4.16 of POSIX.1 [11].

Given the above, the authentication token shall be calculated as:

token = SHA512(url&UrlSignature[tokenExpiryName]=token_expiry&UrlSignature[ipAddressName]= ue_public_ip_address&UrlSignature[passphraseName]=passphrase)

where the SHA512 function shall be the SHA-512 hash [6] of the enclosed string. The url parameter shall be the original M4d media resource request URL, including the scheme, authority and path components but excluding any query and fragment components.

The resulting token value shall be “base64url” encoded, as specified in section 5 of [10],prior to inclusion in the M4d URL.

The query part of the signed URL presented by the 5GMSd Media Player at M4d as proof of authenticity shall be composed as follows:

query= UrlSignature[tokenExpiryName]=token_expiry &UrlSignature[tokenName]=base64url(token)

For all media resources requested at reference point M4d that match the regular expression specified in UrlSignature[urlPattern], the 5GMSd AS shall validate the query presented in the request URL according to the following steps:

- If the parameter indicated by UrlSignature[tokenName] is absent from query, or if the supplied token value is malformed, the 5GMSd AS shall respond with a 403 (Forbidden) error response message and terminate further processing of the M4d request.

- If the parameter indicated by UrlSignature[tokenExpiryName] is absent from query, or if the supplied token_expiry value has expired, or if the supplied token_expiry is malformed, the 5GMSd AS shall respond with a 403 (Forbidden) error response message and terminate further processing of the M4d request.

- The 5GMSd AS shall compute the authentication token according to the token production specified above using the requesting UE’s public IP address as the value of ue_public_ip_address if required by UrlSignature[useIPAddress] being set to True. After applying “base64url” encoding, the 5GMSd AS shall compare this with the value supplied in the URL query parameter whose name is UrlSignature[tokenName]. If the two values differ, the 5GMSd AS shall respond with a 403 (Forbidden) error response message and terminate further processing of the M4d request.

- Otherwise, the presented authentication token is valid. The 5GMSd AS shall either return the media resource in a 200 (OK) response message (if it is able to serve that media resource), or else return an appropriate error response, such as 404 (Not Found) or 503 (Service Unavailable).

#### 7.6.4.6	Geofencing

The 5GMSd Application Provider may wish to limit the access to its media content to certain areas or to non-roaming UEs. Geofencing is used to configure the zone from which content is accessible.

Editor’s note: Missing specification

## 7.7	Consumption Reporting Provisioning API

### 7.7.1	Overview

The Consumption Reporting Provisioning API is a RESTful API that allows a 5GMSd Application Provider to configure the Consumption Reporting Procedure for a particular Provisioning Session at interface M1d. The different procedures are described in section 4.2.5. The Consumption Reporting Configuration is represented by a ConsumptionReportingConfiguration, the data model for which is specified in clause 7.7.3 below. The RESTful resources for managing the Consumption Reporting Configuration is specified in clause 7.7.2.

### 7.7.2	Resource structure

The Consumption Reporting Provisioning API is accessible through the following URL base path:

{apiRoot}/3gpp-m1d/v1/provisioning-sessions/{provisioningSessionId}/

Editor’s Note: to be updated according to the last version of the Spec

Table 7.7.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column shall be appended to the URL base path.

t

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Activate Consumption Reporting procedure with a Consumption Reporting Configuration | consumption-reporting-configuration | POST | This is used to activate the consumption reporting procedure and to set the Consumption Reporting Configuration. |
| Fetch Consumption Reporting Configuration | consumption-reporting-configuration | GET | This operation is used to retrieve an existing Consumption Reporting Configuration. |
| Update Consumption Reporting Configuration | consumption-reporting-configuration | PUT, PATCH | This operation is used to modify the configuration of an existing Ingest Configuration. |
| Delete Consumption Reporting Configuration | consumption-reporting-configuration | DELETE | This operation is used to deactivate the consumption reporting procedure for that particular session. |

### 7.7.3	Data model

#### 7.7.3.1	ConsumptionReportingConfiguration resource

The data model for the ConsumptionReportingConfiguration resource is specified in table 7.7.3.1-1 below:

Table 7.7.3.1-1: ConsumptionReportingConfiguration resource (alternative)

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| reportingInterval | DurationSec | 0..1 | Identifies the interval between two consecutive consumption reports. The value shall be greater than zero. If absent, a single final report shall be sent immediately after the streaming session has ended. Note: DurationSec is defined in TS 29.122, Clause 5.2.1.3.2. |
| samplePercentage | Percentage | 0..1 | The proportion of clients that shall report media consumption, expressed as a floating point value between 0.0 and 100.0. If not specified, all clients shall send consumption reports |
| locationType | LocationType | 0..1 | Identifies the UE location type if location reporting is enabled (only for trusted AF). CGI, ECGI and NCGI shall be represented by the values 0, 1 and 2, respectively (See [7]). If not present, location reporting is disabled. |

## 7.8	Metrics Reporting Provisioning API

### 7.8.1	Overview

### 7.8.2	Resource structure

### 7.8.3	Data model

## 7.9	Policy Templates Provisioning API

### 7.9.1	Overview

The Policy Templates Provisioning API allows a 5GMS Application Provider to configure a set of Policy Templates within the scope of a Provisioning Session that can subsequently be applied to media streaming sessions belonging to that Application Provider using the Dynamic Policies API specified in clause 11.5. A Policy Template is used to specify the traffic shaping and charging policies to be applied to these media streaming sessions.

A Policy Template, identified by its policyTemplateId, represents a set of PCF/NEF API parameters which defines the service quality and associated charging for the media streaming sessions. The Policy Template is configured as part of the Provisioning procedures with the 5GMS AF and is then used by the 5GMS AF to request specific QoS and charging policies for that session from the PCF or NEF.

### -

7.9.2	Resource structure

The Policy Template Provisioning API is accessible through the following URL base path:

{apiRoot}/3gpp-m1d/v1/provisioning-sessions/{provisioningSessionId}/

Table 7.9.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column shall be appended to the URL base path.

Table 7.9.2-1: Operations supported by the Policy Template Provisioning API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create a new Policy Template | policy-templates | POST | Used to create a new Policy Template resource. |
| Fetch a Policy Template | policy-templates/‌{policyTemplateId} | GET | Used to retrieve an existing Policy Template resource. |
| Update a Policy Template |  | PUT, PATCH | Used to modify the configuration of an existing Policy Template. |
| Delete a Policy Template |  | DELETE | Used to delete an existing Policy Template resource. |

### 7.9.3	Data model

#### 7.9.3.1	PolicyTemplate resource

The data model for the PolicyTemplate resource is specified in Table 7.9.3-1 below:

Table 7.9.3-1: Definition of PolicyTemplate resource

| Property | Type | Cardinality | Visibility | Description |
| --- | --- | --- | --- | --- |
| policyTemplateId | Integer | 1..1 |  | Unique identifier of this Policy Template within the scope of the Provisioning Session. |
| state | Enumeration of Strings | 1..1 |  | A Policy Template may be in the pending, ready, or suspended state. Only a Policy Template in the ready state may be instantiated as a Dynamic Policy Instance and applied to streaming sessions. |
| apiEndPoint | String | 1..1 | MNO Admin | The API endpoint that should be invoked when activating a Dynamic Policy Instance based on this Policy Template. |
| apiType | Enumeration of Strings | 1..1 | MNO Admin | N5: Npcf Policy Authorization Service. N33: AsSessionWithQoS or CHargableParty. |
| externalReference | String | 1..1 |  | Additional identifier for this Policy Template, unique within the scope of its Provisioning Session, that can be cross-referenced with external metadata about the streaming session. |
| QoSSpecification | Object | 0..1 |  | Specifies the network quality of service to be applied to streaming sessions at this Policy Template. |
| marBwUl | BitRate | 0..1 |  | As defined in clause 8.6.2.7 of TS 29.514. |
| marBwDl | BitRate | 0..1 |  |  |
| maxPacketLossRateDl | PacketLossRateRm | 0..1 |  |  |
| maxPacketLossRateUl | PacketLossRateRm | 0..1 |  |  |
| maxSuppBwDl | BitRate | 0..1 |  |  |
| maxSuppBwUl | BitRate | 0..1 |  |  |
| minDesBwDl | BitRate | 0..1 |  |  |
| minDesBwUl | BitRate | 0..1 |  |  |
| mirBwUl | BitRate | 0..1 |  |  |
| mirBwDl | BitRate | 0..1 |  |  |
| ApplicationSessionContext | Object | 1..1 |  | Specifies information about the application session context to which this Policy Template can be applied. |
| afAppId | AfAppId | 0..1 | Read-Only | As defined in clause 5.6.2.3 of TS 29.514. |
| sliceInfo | Snssai | 0..1 |  |  |
| dnn | Dnn | 0..1 |  |  |
| aspId | AspId | 0..1 |  |  |
| ChargingSpecification | Object | 0..1 |  | Provides information about the charging policy to be used for this Policy Template. |
| sponId | SponId | 0..1 |  | As defined in clause 5.6.2.3 of TS 29.514. |
| sponStatus | SponsoringStatus | 0..1 |  |  |
| gpsi | [Gpsi] | 0..* |  | List of UEs permitted to instantiate this Policy Template. |

Editor’s Note: The parameter externalReference is for further study. It may be a provisioning parameter of the Media Player and / or a Media Session Handler to assist mapping of external references to a policyTemplateId.

Editor’s Note: The ChargingSpecification object may contain any charging related information, such as sponId or afChargeId.

# 8	Media Ingest and Publish (M2) APIs

# 9	Internal (M3) APIs

APIs of this reference point are not specified within this release.

# 10	Media Streaming (M4) APIs

# 11	Media Session Handling (M5) APIs

## 11.1	General

## 11.2	Service Access Information API

### 11.2.1	General

### 11.2.2	Resources

The Service Access Information API is accessible through the following URL base path:

{apiRoot}/3gpp-m5d/v1/service-access-information/{saiSubresource}

The following operations and the corresponding HTTP methods are supported. In each case, the sub-resource path specified in the second column shall be substituted into {sai-subresource} in the above URI template:

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Fetch Service Access Information | { saiSubresource } | GET | Used to acquire the Service Access Information resource for the specified Provisioning Session. |

### 11.2.3	Data model

#### 11.2.3.1	ServiceAccessInformation resource type

The data model for the ServiceAccessInformtion resource is specified in Table 11.2.3.1-1 below:

Table 11.2.3.1-1: Definition of ServiceAccessInformation resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| provisioningSessionId | String | 1..1 | Unique identification of the M1d Provisioning Session. |
| StreamingAccess | Object | 0..1 |  |
| mediaPlayerEntry | URL String | 1..1 | A document or a pointer to a document that defines a media presentation e.g. MPD for DASH content or URL to a video clip file. |
| ClientConsumptionReportingConfiguration | Object | 0..1 |  |
| reportingInterval | DurationSec | 0..1 | The time interval, expressed in seconds, between consumption report messages being sent by the Media Session Handler. The value shall be greater than zero. When this property is omitted, a single final report shall be sent immediately after the streaming session has ended. |
| serverAddresses | [URL String] | 1..1 | A list of 5GMSd AF addresses (URLs) where the consumption reporting messages are sent by the Media Session Handler. (Opaque URL, following the 5GMS URL format.) |
| locationReporting | Boolean | 1..1 | Stipulates whether the Media Session Handler is required to provide location data to the 5GMSd AF in consumption reporting messages (in case of MNO or trusted third parties). |
| samplePercentage | Percentage | 1..1 | The percentage of streaming sessions that shall send consumption reports, expressed as a floating point value between 0.0 and 100.0. |
| DynamicPolicyInvocationConfiguration | Object | 0..1 |  |
| serverAddresses | [URL String] | 1..1 | A list of 5GMSd AF addresses (URLs) which offer the APIs for dynamic policy invocation sent by the Media Session Handler. (Opaque URL, following the 5GMS URL format.) |
| validPolicyTemplateIds | [String] | 1..n | A list of Policy Template identifiers which the 5GMSd Client is authorized to use. |
| ClientMetricsReportingConfiguration | Object | 0..1 |  |
| serverAddresses | [URL String] | 1..1 | A list of 5GMSd AF addresses to which metrics reports shall be sent. (Opaque URL, following the 5GMS URL format.) |
| dataNetworkName | String | 0..1 | The DNN which shall be used when sending metrics reports. If not specified, the name of the default DN shall be used. |
| reportingInterval | DurationSec | 0..1 | The time interval, expressed in seconds, between metrics reports being sent by the Media Session Handler. The value shall be greater than zero.When this property is omitted, a single final report shall be sent immediately after the streaming session has ended. |
| samplePercentage | Percentage | 1..1 | The percentage of streaming sessions that shall report metrics, expressed as a floating point value between 0.0 and 100.0. |
| urlFilters | [String] | 1..1 | A list of URL patterns for which metrics reporting shall be done. The format of each pattern shall be a regular expression as specified in [5]. If not specified, reporting shall be done for all sessions. |
| metrics | [String] | 1..1 | A list of metrics which shall be reported. |

### 11.2.4	Operations

This clause defines the behaviour that is expected from the 5GMSd AF when a Service Access Information resource is acquired from the Media Session Handler. The main operation that is performed is to look up or generate the Service Access Information.

11.3	Consumption Reporting API

### 11.3.1	General

The Consumption Reporting API is a RESTful API that allows the Media Session Handler to report media consumption to the 5GMSd AF. The API defines data models, resources and the related procedures for the creation and management of the consumption reporting procedures.

### 11.3.2	Data model

#### 11.3.2.1	Introduction

This clause defines data structures to be used in consumption reporting procedure.

Table 11.3.2.1-1 specifies data types re-used by the Consumption Reporting API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the 5GMS Consumption Reporting API.

Note: Should we gather all these imported Data types in one section?

Table 11.3.2.1-1: 5GMS Consumption Reporting API re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| domainNames | 3GPP TS 29.122 [12] | Indicates an FQDN or a regular expression as a domain name matching criteria. |

#### 11.3.2.2	ConsumptionReportingConfiguration resource

This type represents the different parameters that initialise the consumption report.

Editor’s Note: Maybe this type should be only defined in section 5.x	Acquiring Service Access Information as proposed in S4-200577

Table 11.3.2.1.2-1: Definition of ConsumptionReportingConfiguration resource

| Attribute name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| reportInterval | DurationSec | 1..1 | Identify the interval between two consecutive consumption reporting messages. |
| consumptionReportingServerAddress | [domainNames] | 1..1 | A list of addresses where the consumption reporting message shall be sent by the 5GMS Media Session Handler. |
| locationReporting | Boolean | 1..1 | Identify whether the 5GMS Media Session Handler is allowed to provide the location data in the consumption reporting message. |
| samplePercentage | Percentage | 0..1 | The proportion of clients that shall report media consumption. If not specified, all clients shall send consumption reports |

#### 11.3.2.3	ConsumptionReporting resource

This type represents a consumption report data. This structure is used by the Media Session Handler to report the consumption.

Table 11.3.2.3-1: Definition of ConsumptionReporting resource

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- |
| mediaPlayerEntry | String | 1..1 | Identify the Media player entry. In case of DASH, the media player entry pointer can be a URL of the MPD. |  |
| reportingClientId | String | 1..1 | Identify the identifier of the UE that consumes data. The client ID can be a MSISDN. |  |
| locationType | integer | 0..1 | Identify the UE location type if location reporting is enabled (only for trusted AF). CGI, ECGI and NCGI shall be represented by the values 0, 1 and 2, respectively (See [7]) |  |
| location | string | 0..1 | Identify the UE location where the consumption media if location reporting is enabled (only for trusted AF). |  |
| consumptionReportingUnit | ConsumptionReportingUnit | 1..N | Identify a list of consumption reporting units. |  |

#### 11.3.24	Type: ConsumptionReportingUnit

This type represents each consumption report unit.

Table 11.3.2.4-1: Definition of type ConsumptionReportingUnit

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- |
| mediaConsummed | string | 1..1 | Identify the media consummed. In case of DASH, the Representation@id may be used. |  |
| startTime | DateTime | 1..1 | Define the time where the consumption starts of this unit. |  |
| duration | DurationSec | 1..1 | Identify the duration of the consumption of the quality of this unit. |  |

### 11.3.3	Resource structure

#### 11.3.3.1	Introduction

Editor’s Note: URI definition shall be unified in this document. A global section where the URI is defined should be created. This section may define the URI as a global URI. It may also simplify the URI construction in a way that for instance {apiRoot}/3gpp-5gms-consumption-reporting/v1/ would be replaced by {5GMSAconsumptionReportApiRoot} where is just defined as a global URI by the AF without any constraint on the way this URI is built.
If this section exists, it will impact every Resource structure defined in this section.

All resource URIs of this API shall have the following root:

{apiRoot}/3gpp-5gms-consumption-reporting/v1/

"apiRoot" is set as described in subclause X. "apiName" shall be set to "3gpp-5gms-consumption-reporting " and "apiVersion" shall be set to "v1" for the version defined in the present document. All resource URIs in the subclauses below are defined relative to the above root URI.

The following resources and HTTP methods initiated by the 5GMS Client are supported for this API:

Table 11.4.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Meaning |
| --- | --- | --- | --- |
| Consumption Reporting Session | 3gpp-5gms-consumption-reporting/v1/{aspId} /session/ | GET | Forbidden. |
|  |  | PUT | Create a Consumption Reporting Session resource. |
| Consumption Reporting Configuration Acquisition | 3gpp-5gms-consumption-reporting/v1/{aspId} /session/{sessionId}/acquire | GET | Acquire consumption reporting configuration. |
|  |  | POST | Forbidden. |
| Consumption Reporting Data | 3gpp-5gms-consumption-reporting/v1/{aspId} /session/{sessionId} | GET | Forbidden. |
|  |  | POST | Report media consumption. |

#### 11.3.3.2	Consumption Reporting Session resource

#### 11.3.3.2.1	Introduction

This resource allows the Media Session Handler to create Consumption Reporting Data resources when the Consumption Reporting is used for a Downlink Streaming session.

#### 11.3.3.2.2	Resource definition

Resource URI:

{apiRoot}/3gpp-5gms-consumption-reporting/v1/{aspId}/session

This resource shall support the resource URI variables defined in table 5.X.2.3.2.2-1.

Table 11.3.3.2.2-1: Resource URI variables for resource "Consumption Reporting Session"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of [x3]. |
| aspId | Identifier of the 5GMS Application Provider. |

#### 11.3.3.2.3	Resource methods

##### 11.3.3.2.3.1	GET

This HTTP method is not supported for the resource.

##### 11.3.3.2.3.2	PUT

The PUT method allows the Media Session Handler create a Consumption Reporting resource. It is initiated by the Media Session Handler and answered by the 5GMSd AF.

This method shall support request and response data structures, and response codes, as specified in table 11.3.3.2.3.2-1.

Table 11.3.3.2.3.2-1: Data structures supported by the PUT request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | none |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | ConsumptionReportingConfiguration | 0..1 | 201 OK | The consumption reporting parameters may be returned. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 [x3] also apply. |  |  |  |  |

##### 11.3.3.2.3.3	PATCH

This HTTP method is not supported for the resource.

##### 11.3.3.2.3.4	POST

This HTTP method is not supported for the resource.

##### 11.3.3.2.3.5	DELETE

This HTTP method is not supported for the resource.

#### 11.3.3.3	Consumption Reporting Configuration Acquisition resource

##### 11.3.3.3.1	Introduction

This method allows the Media Session Handler to acquire Consumption Reporting configuration parameters when the Consumption Reporting is used for a Downlink Streaming session.

##### 11.3.3.3.2	Resource definition

Resource URI: {apiRoot}/3gpp-5gms-consumption-reporting/v1/{aspId}/session/{sessionId}/acquire

This method shall support the resource URI variables defined in table 11.4.3.3.2-1.

Table 11.3.2.3.3.2-1: Resource URI variables for resource "Consumption Reporting Configuration Acquisition"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of TS 29.122 [x3]. |
| aspId | Identifier of the 5GMS Application Provider. |
| sessionId | Identifier of the 5GMS Downlink Streaming session. |

##### 11.3.3.3.3	Resource methods

###### 11.3.3.3.3.1	GET

The GET method allows the Media Session Handler to acquire Consumption Reporting configuration parameters. It is initiated by the Media Session Handler and answered by the 5GMSd AF.

This method shall support request and response data structures, and response codes, as specified in the table 11.3.3.3.3.3-1.

Table 11.3.3.3.3.3-1: Data structures supported by the GET request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | none |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | ConsumptionReportingConfiguration | 1..1 | 200 OK | The consumption reporting configuration parameters shall be returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 [x3] also apply. |  |  |  |  |

###### 11.3.3.3.3.2	PUT

This HTTP method is not supported for the resource.

###### 11.3.3.3.3.3	PATCH

This HTTP method is not supported for the resource.

###### 11.3.3.3.3.4	POST

This HTTP method is not supported for the resource.

###### 11.3.3.3.3.5	DELETE

This HTTP method is not supported for the resource.

#### 11.3.3.4	Consumption Reporting Data resource

##### 11.3.3.4.1	Introduction

Editor’s Note: Whether reports should be sent as RESTful resources is still under consideration

This resource allows the Media Session Handler to send the actual consumption data when the Consumption Reporting is used for a Downlink Streaming session.

##### 5.453.4.2	Resource definition

Resource URI: {apiRoot}/3gpp-5gms-consumption-reporting/v1/{aspId}/session/{sessionId}

This resource shall support the resource URI variables defined in table 11.3.3.4.2-1.

Table 11.3.3.4.2-1: Resource URI variables for resource "Consumption Reporting Data"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of TS 29.122 [x3]. |
| aspId | Identifier of the 5GMS Application Provider. |
| sessionId | Identifier of the 5GMS Downlink Streaming session. |

##### 11.3.3.4.3	Resource methods

###### 11.3.3.4.3.1	GET

This HTTP method is not supported for the resource.

###### 11.3.3.4.3.2	PUT

This HTTP method is not supported for the resource.

###### 11.3.3.4.3.3	PATCH

This HTTP method is not supported for the resource.

###### 11.3.3.4.3.4	POST

The POST method allows the Media Session Handler to send consumption data. It is initiated by the 5GMS Media Session Handler and acknowledged by the 5GMSd AF.

This method shall support request and response data structures, and response codes, as specified in the table 11.3.3.4.3.3-1.

Table 11.3.3.4.3.3-1: Data structures supported by the POST request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | ConsumptionReporting | 1..1 | Consumption Reporting data to send to the Media AF. |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | none |  | 200 OK | The consumption reporting data is received by the Media AF. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 [x3] also apply. |  |  |  |  |

###### 11.3.3.4.3.5	DELETE

This HTTP method is not supported for the resource.

## 11.4		Metrics Reporting API

## 11.5	Dynamic Policies API

### 11.5.1	Overview

The Dynamic Policies API allows the Media Session Handler to request a specific policy and charging treatment to be applied to a particular application data flow by invoking RESTful operations on the 5GMSd AF at interface M5d. The API defines a set of data models, resources and the related procedures for the creation and management of the dynamic policy request. The corresponding JSON schema for the representation of the resources and operations defined by the API is provided in Annex ZZ.

### 11.5.2	Data model

#### 11.5.2.1	DynamicPolicy resource type

Table 11.5.2.1-1: Definition of Dynamic Policy resource

| Attribute name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| policyTemplateId | String | 1 | Identifies the Policy Template which should be applied to the application flow(s). |
| flowDescription | Object | 1 | Refer to subclause 5.3.8 of 3GPP TS 29.214 for encoding. |
| applicationServiceConfigurationId | String | 1 | Uniquely Identifies 5GMSd Application Service Configuration, which is linked to the Application Service Provider. |
| enforcementMethod | String | 1 | Description of the Policy Enforcement Method. The parameter is set by the 5GMSd AF. |

### 11.5.3	Resource structure

#### 11.5.3.1	General

All resource URIs of this API should have the following root:

{apiRoot}/3gpp-dynamicpolicies/v1/

All sub-resource paths in the subclauses below are defined relative to the above root URI.

The following resources and HTTP methods are supported for the Dynamic Policies API:

Table 11.5.3.1-1: Resources and methods overview

| Resource name | Sub-resource path | Allowed HTTP methods | Meaning |
| --- | --- | --- | --- |
| Dynamic Policies | policies | GET | Forbidden. The 5GMSd AF shall return an error code. |
|  |  | POST | Create a new Dynamic Policy resource. |
| Dynamic Policy | policies/{policy-id} | GET | Read a Dynamic Policy resource |
|  |  | PUT | Replace an existing Dynamic Policy resource. |
|  |  | PATCH | Modify an existing Dynamic Policy resource. |
|  |  | DELETE | Delete an existing Dynamic Policy resource. |

#### 11.5.3.3	Dynamic Policies resource

##### 11.5.3.3.1	Introduction

Editor’s Note: To be filled in

##### 11.5.3.3.2	Resource definition

Resource URI: {apiRoot}/3gpp-dynamicpolicies/v1/policies/

This resource shall support the resource URI variables defined in Table 5.x.3.3.2-1.

Table 11.5.3.3.2-1: Resource URL variables for resource "Dynamic Policies"

| Name | Definition |
| --- | --- |
| apiRoot | Base URL to the 5GMSd AF. |

##### 11.5.3.3.3	Resource Method GET

The 5GMSd AF shall return an Error code.

##### 11.5.3.2.4	Resource Method POST

The POST method creates a new Dynamic Policy resource for a given Media Session Handler. The Media Session Handler shall initiate the HTTP POST request message and the 5GMSd AF shall respond to the message. The 5GMSd AF shall construct the URI of the created resource using that URI.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in the Table 5.x.3.2.4-1 and Table 5.x.3.2.4-2.

Table 11.5.3.2.4-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |

Table 11.5.3.2.4-2: Data structures supported by the POST request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | DynamicPolicy | 1 | Parameters to create a Dynamic Policyinstance. |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  |  |  | 201 Created | The Dynamic Policy instance resource was created successfully. The URI of the created resource shall be returned in the Location HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table xxx also apply. |  |  |  |  |

#### 11.5.3.4	Dynamic Policy resource

##### 11.5.3.4.1	Introduction

<some Intro>

##### 11.5.3.4.2	Resource definition

Resource URI: {apiRoot}/3gpp-dynamicpolicies/v1/policies/{policyId}

This resource shall support the resource URI variables defined in Table 5.x.3.3.2-1.

Table 11.5.3.4.2-1: Resource URL variables for resource "Dynamic Policy"

| Name | Definition |
| --- | --- |
| apiRoot | Base URL of the 5GMSd AF. |
| {policyId} | Unique identifier, created by the 5GMSd AF during the create procedure (5.x.3.3.4). |

##### 11.5.3.4.3	Resource Method GET

##### 11.5.3.4.4	Resource Method PUT

##### 11.5.3.4.5	Resource Method PATCH

##### 11.5.3.4.6	Resource Method DELETE

## 11.6	AF-based Network Assistance API

## 11.7	RAN Signaling-based Network Assistance API

If RAN Signaling-based Network Assistance is supported, the Media Session Handler uses an interface to the RAN Modem (specifically, the UE MAC entity in the modem) to send and receive bit rate recommendation messages. The interface to the modem may be based on AT commands.

Furthermore, messaging across that interface corresponds to the logical translations of the Bit Rate Recommendation and/or Bit Rate Recommendation Query messages, carried by the Recommended bit rate MAC CE, exchanged between the RAN Modem and the RAN, as specified in [13] for 5G NR and [14] for LTE. The association between the LCID for which the recommendation applies and the actual flow (including the intermediate RLC channel) is performed by the modem. The input parameters used by the Media Session Handler to send and receive bit rate recommendation messages are FFS.

Editor’s note:	The internal interface to the modem may be based on AT commands. The AT command +CGEQREQ as defined in [15] may be used for the exchange of bit rate recommendations between the Media Session Handler and the RAN Modem. CT1 has been requested to define appropriate AT commands for bit rate recommendation. Upon definition of the appropriate AT commands for bit rate recommendation messaging, this clause will be updated to reflect that.

When used for requesting a bit rate boost, the query shall not request a bit rate that may exceed the MFBR for the corresponding QoS Flow. Failure to ensure this may result in unexpected congestion-induced packet delays and dropping.

The Bit Rate Recommendation Query shall indicate the bit rate desired by the application, as described by [13] and [14]. This request may be used by the 5GMSd Media Session Handler to request for a temporary increase in bit rate for the corresponding flow (bit rate boost). The RAN responds with a Bit Rate Recommendation message that confirms the recommended bitrate after the boost grant. Once the bit rate drops again after a boost grant, the network shall inform the Media Session Handler about the new recommended bit rate by means of an ANBR message.

Whenever the Media Session Handler receives a message from the RAN Modem, corresponding to the logical translation of the Bit Rate Recommendation message for the associated RAN uplink or downlink, it shall indicate the associated bit rate recommendation to either the Media Player (via M7d, in the case of downlink streaming) or Media Streamer (via M7u, in the case of uplink streaming) function of an affiliated PDU session. Furthermore, whenever the Media Session Handler receives a request for a bit rate boost from either the Media Player (via M6d in the case of downlink streaming) or the Media Streamer (via M6u, in the case of uplink streaming) function of an affiliated PDU session, it may send a bit rate boost message to the RAN Modem. That bit rate boost request is logically translated by the modem to the Bit Rate Recommendation Query message which is then sent to the RAN on the associated RAN uplink or downlink.

It is left to the implementer of the media player to decide how to best use the bit rate recommendation and the bit rate recommendation query information for the media streaming sessions.

# 12	UE Media Session Handling (M6) APIs for uplink and downlink

The M6 and M7 API may be real APIs, i.e. likely not using Swagger for definition but e.g. IDL.

# 13	UE Media Stream Handler (M7) APIs for uplink and downlink

The M6 and M7 API may be real APIs, i.e. likely not using Swagger for definition but e.g. IDL.

# 14	Application (M8) APIs for uplink and downlink

APIs of this reference point are not specified within this release.

# Annex A (Informative):	AF QoS Mapping

### X.1	Overview

This annex defines the mapping from a media streaming description into Dynamic Policy Instance properties for different types of media streaming description.

### X.2	DASH mapping

Editor’s Note: This clause should describe the mapping from DASH to Dynamic Policy Instances.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 25.6.2019 | SA4#104 | S4-190649 |  |  |  | Initial Version | 0.0.1 |
| 23.1.2020 | SA4#107 | S4-200077, S4-200238, S4-200239, S4-200318 |  |  |  | Updates during SA4#107 | 0.3.0 |
| 07.02.2020 | ConfCall | S4-AHI931, S4-AHI932 |  |  |  | Scope, editorial improvements and online edits from Conf Call (6th Feb 2020) | 0.3.1 |
| 11.02.2020 | offline |  |  |  |  | Editorial updates according to offline email discussions | 0.3.2 |
| 2020-02 | ConfCall | S4-AHI950 |  |  |  | Editorial updates from Conf Call (Online, 13th Feb 2020) | 0.4.0 |
| 2020-03 | - | SP-200237 |  |  |  | Specification to TSG: 5G Media Streaming (5GMS); Protocols TS 26.512, Version 1.0.0 | 1.0.0 |
|  |  | S4-AHI953 |  |  |  |  | 1.0.1 |
| 2020-04 | SA4#108e | S4-200513, S4-200514, S4-200633 |  |  |  | Renaming entities in the 5GMS Provisioning API, Additional clauses to specify procedures for manipulating Ingest Protocols, Content Preparation Templates and Server Certificates, Consumption Reporting Procedure API- M1d and M5d | 1.0.2 |
| 2020-05 | Conf Call | S4-AHI989 |  |  |  | New Structure | 1.1.0 |
| 2020-06 | SA4#109e | S4-200920, S4-200886, S4-200889, S4-200883 |  |  |  | 920: Consumption reporting in M7d interface, 886: RAN Signaling-based Network Assistance, 889: API for Service Access information acquisition, 883: APIs for Server Certificates, Content Preparation Templates and Ingest Protocols | 1.2.0 |