3GPP TSG-S4 Meeting #109e	S4-200876

# Electronic, The Internet,  20.5. - 3.6.2020									revision of S4-200865

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of collaboration scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 18.5.2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The principles of Multi-MNO Distribution Scenarios were agreed in S4-200575. When a Media Streaming Service is offered through multiple different MNO networks and MNO specific network features like Dynamic Policy or Consumption Reporting is needed, a 5GMSd Client needs to acquire MNO specific service access information, i.e. service access information, which are applicable for the serving MNO network. This CR adds a missing procedure to acquire the service access information, based on the current serving MNO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section around aquiring MNO specific service access information is added to Clause 5. The new section contains a callflow for DNS based and HTTPS based resolution. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5G Media Streaming Architecture does not provide support for the most common collaboration scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3, 5.10 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

## Annex B (informative):	MNO specific Service Access Information acquisition

### B.1	General

A 5GMSd Application Provider may offer its content via multiple Access Networks from different operators. Each access network can consist of a 5G System and one or more 5G Media Streaming Architecture components, in particular its own 5GMSd AFs. The 5GMSd AFs of that access network use own FQDNs and potentially own URL root path elements. When a 5GMSd-Aware Application is deployed in different 5G Systems the 5GMSd Client needs to acquire Service Access Information that resolves to the 5GMSd AF endpoint address(es) appropriate for the serving 5G System.

When the 5GMSd Application Provider uses 5GMS features from different providers, then the 5GMSd Client needs find the service access information of the 5GMS feature provider, which is currently providing connectivity for the 5GMSd Client.

The service access information contains the URLs and API parameters of the configured 5GMSd AFs and ASs of that 5G Media Streaming System.

There are different ways to resolve the matching service access information. This section focuses on two solutions, where the 5GMSd Client fetches the service access information from a 5GMSd AF within the trusted DN of the serving mobile network.

### B.2	Deployment with DNS based resolution

Figure B.2-1 depicts the deployment for the DNS based resolution of the 5GMSd AF in the Trusted DN of the MNO, which is currently providing connectivity to the 5GMSd Client. The DNS function are defined in RFC 1034 [X].

Figure B.2-1: DNS based resolution of 5GMSd AF in trusted DN

For the DNS base resolution mechanisms, the 5GMSd Client is provisioned with a Service Access Information URL, for example dynamically via M8d or statically within the code of the 5GMSd Aware Application. The service access information URL contains an FQDN from 5GMSd Application Provider or a global URL from GSMA domain (i.e. from “3gppnetworks.org” domain).

Figure B.2-2: Message Sequence Chart for DNS based resolution

Steps.

1: 	The 5GMSd aware Application is provisioned (among other parameters) with an URL to the 5GMSd AF to acquire service access information. The 5GMSd aware Application provides this URL to the 5GMSd client upon start.

2:	The 5GMSd Client uses DNS to resolve the FQDN of the URL. The 5GMSd Client sends a DNS request to the DNS server in the trusted DN.

3:	The DNS server in trusted DN is not responsible for the FQDN and the DNS server forwards the DNS request to the DNS server in the external DN, which is responsible for the FQDN. This may be the DNS server of the 5GMSd Application Provider or a GSMA DNS server.

4:	Based on information from the originating network, the external DNS creates a response. The response may be a CNAME redirect (i.e. FQDN from the serving mobile network responsibility) or an IP Address (preferably an IP Anycast Address).

5:  The DNS server forwards the DNS response to the 5GMSd Client.

6: 	When the 5GMSd Client got another FQDN with the response (a CNAME), then the 5GMSd Client resolves the FQDN to an IP address. The resolved IP address should be an IP address of an 5GMSd AF in the trusted DN.

7:	The 5GMSd Client issues a request to the resolved 5GMSd AF in order to acquire the service access information.

8:	The 5GMSd AF provides the service access information with the response. The service access information contains URLs and parameters according to provisioned 5GMS features.

9:	When needed, the 5GMSd Client uses the information from the acquired service access information to activate the needed 5GMSd feature.

### B.3	Deployment with HTTPS based resolution

Figure B.3-1 depicts the deployment for the HTTPS based resolution of the 5GMSd AF in the Trusted DN of the MNO, which is currently providing connectivity to the 5GMSd Client. The process of resolving IP addresses from FQDNs is not depicted in this deployment architecture.

Figure B.3-1: HTTPS based resolution of 5GMSd AF in trusted DN

For the HTTPS base resolution mechanisms, the 5GMSd Client is provisioned with a Service Access Information URL, for example dynamically via M8d or statically within the code of the 5GMSd Aware Application. The service access information URL contains an FQDN to an 5GMSd AF within the 5GMSd Application Provider domain, which acts as a request redirector.

Figure B.3-2: Message Sequence Chart for HTTPS based resolution

Steps.

1: 	The 5GMSd aware Application is provisioned (among other parameters) with an URL to the 5GMSd AF to quired service access information. The 5GMSd aware Application passes this URL to the 5GMSd client upon start.

2:	The 5GMSd Client uses DNS to resolve the FQDN of the URL.

3:	The 5GMSd Client issues a request to the resolved 5GMSd AF in order to acquire the service access information.

4:	Based on information from the originating network (e.g. visible IP of the 5GMSd Client), the 5GMSd AF in the external DN creates a redirection response. The 5GMSd AF looks up the according URL of the 5GMSd AF in the trusted DN and sends a redirection response to the 5GMSd Client.
When the network, which is currently serving the UE, does not offer a 5GMS features or the 5GMS features are not provisioned (i.e. the 5GMSd AF in the external DN), the 5GMSd AF provides a response containing an Error message.

5:  The 5GMSd Client issues a request to the resolved 5GMSd AF in order to acquire the service access information.

6:	The 5GMSd AF provides the service access information with the response. The service access information contains URLs and parameters according to provisioned 5GMS features.

7:	When needed, the 5GMSd Client uses the information from the acquired service access information to activate the needed 5GMSd feature.

NOTE: Instead of redirecting a 5GMSd Client to the 5GMSd AF in the trusted DN, the external 5GMSd AF may directly provide the service access information in Step 4. In the case the 5GMSd Application Provider needs to update its 5GMSd AF according to changes within the trusted DN, e.g. changes of FQDNs.

**** Last Change ****