# ISO/IEC JTC 1/SC 29/WG 2 N 10

ISO/IEC JTC 1/SC 29/WG 2
MPEG Technical requirements 
Convenorship: SFS (Finland)

Document type:	Output Document

Title:	5G activities relevant to MPEG

Status:	Approved

Date of document:	2020-10-16

Source:	ISO/IEC JTC 1/SC 29/WG 2

# Expected action:	None

# Action due date:	None

No. of pages:	24 (with cover page)

Email of Convenor:	igor.curcio@nokia.com

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29wg2

INTERNATIONAL ORGANIZATION FOR STANDARDIZATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 2

MPEG TECHNICAL REQUIREMENTS

ISO/IEC JTC 1/SC 29/WG 2 N 10

Online - October 2020

| Title | 5G Activities relevant to MPEG |
| --- | --- |
| Source | WG 2, MPEG Technical requirements |
| Status | Approved |
| Serial Number | 19835 |

# Introduction

MPEG & 5G is an activity that was created in Geneva at the MPEG 126. The AhG mailing list has over 100 subscribers (at MPEG 131). To subscribe to the mailing list, please visit

https://lists.aau.at/mailman/listinfo/mpeg-and-5g.

This report updates the output document [1] in sections 2-4 with the developments at the MPEG 132 meeting. The delta compared to [1] is:

- Updates on the 3GPP SA1 activities in section 2.1 (sources [2, 4, 6])

- Updates of finalized 3GPP SA4 work and study items in section 2.2.1 (source [3]);

- Updates in section 2.2.2 with new 3GPP SA4 activities potentially of MPEG interest.

- Addition of section 5 with use cases and initial requirements on indoor spatial information storage (source [7]);

- Update of references.

Information and investigation about typical bit rates of XR traffic characteristics would be very valuable. This would help in developing 5G system and radio technologies. In 3GPP some information is available in TR 26.925. A new mandate for the AHG could be formulated with this purpose.

A LS from 3GPP SA4 was also received in document m54761 about 5G video codec characterization and related metrics. This was handled by JVET and the Test Sub-Group.

  - Mandate 1: Continue the study of 5G use cases, requirements, specifications and new work items that could be relevant to MPEG work.

Selected activities in 3GPP are listed below as well as references to overviews on immersive media standards are given. In this document, two types of “Items” will be referenced:

- Work Items (WI): this is 3GPP work that triggers new normative Technical Specifications (TSs), or work that targets to update existing TSs.

- Study Items (SI): this is 3GPP work that is informative and exploratory in nature. It has no normative status, but may contain the seeds for future work which is normative. The typical output of Study Items is Technical Reports (TRs). TRs often contain open problems or technology foresights which could be used as a hint of future developments.

3GPP works in Releases within which all Work Items belong. A Release is a master project that spans all 3GPP Working Groups (WGs) and has a given timeline. Work Items are usually launched and framed within a given 3GPP Release. For releases up to 16 it is not possible to add new features, but only bug fixes are possible (via Change Requests (CRs)). The timeline for the currently active 3GPP Releases is as follows:

- Release 17 (Stage 3) freeze: June 2021.

  - Study and Work items in 3GPP SA1

Information was collected from discussion with 3GPP SA1 chairman, delegates, and SA1 documents. On a general note, it was explained that 3GPP SA1 regularly discusses with 3GPP SA4 on media aspect via liaisons and sometimes joint meeting. 3GPP SA4 translates use cases into range of bitrates, and other KPIs for 3GPP SA1 to elaborate relevant requirements. In the context of 5G, use cases such as 8K video, VR/AR, etc., were of interest for 3GPP SA1 in terms of bandwidth and latency requirements.

From the information collected, it appears that the main media services discussed in SA1 in the context of 5G are:

- Broadcast TV services

- AR/VR for many sectors like medical and not only entertainment

- AV production for large TV recorded events (e.g. stadium), small rooms like operating room or TV studios. In this scenario, small scale private or public networks can be used where 5G low latency radio is a big advantage for wireless devices.

Regarding 5G low latency, important new use cases for media encoding and streaming are:

- Public safety

- Medical, real-time recording of patient with X-Ray while a surgeon navigates with an intra body tool.

Another category of 5G use cases is the device-to-device communication:

- relaying streams in the home downstream, e.g. by a TV set or modem, to the 5G device (the relaying would act as “small antenna” in the home)

- Study item on interactivity between devices (interaction between devices and between device and cloud in home or in the operator network).

The use cases and topics mentioned above have been studied and are being normatively specified in the following items by 3GPP SA1:

- Audio-Visual Service Production (AVPROD)

  - Use cases below as well as other findings of the study item were reported in TR 22.827 (link).

    - On-site Live Audio Presentation

    - Audio Streaming in Live Performances

    - Live production with integrated audience services

    - Intercom system for large live events

    - Single-source uncompressed Outside Broadcast Contribution

    - Single-source compressed Outside Broadcast Contribution

    - Professional TV Production Contributions from an Off-Site, Remotely-Produced, Multi-Camera Outside Broadcast

    - Simple Live Sports Commentary

    - Video Streaming of Live Events using an Airborne Relay

    - Live Immersive Media Service

    - Video Streaming in Professional Coverage of Live Performances

    - Authentication of heterogeneous AVPROD devices on a shared non-public network

    - Onboarding of audio-visual IoT devices onto a non-public network.

  - In June 2019, the work moved into normative phase and a new Rel-17 work item was launched in SP-190304. The work objective is to specify requirements for using the 5G system for the production and contribution of audio-visual content and services as presented in TR 22.827. Specifically, this work item addresses:

    - New KPIs for macro network & non-public networks (NPN)

    - New service requirements for NPN

    - KPIs for Multicast and Broadcast Services

    - Network Exposure Requirements

    - Clock synchronization

    - AVPROD application Specific Requirements

    - Mobile & airborne base stations for NPNs

    - Service continuity.

The normative requirements resulting from this work item are in TS 22.263 (link).

- Network Controlled Interactive Service in 5GS (NCIS)

  - Use cases below as well as other findings of the study item were reported in TR 22.842 (link)

    - Interactive Service Supporting

    - VR Based Interactive Service

    - Cloud/Edge/Split Rendering for Games

    - Potential Impacts or Interactions with Existing Services/Features

    - Absolute high but relative low speed

    - IoE based social networking services

    - Communication within NCIS Group

  - In June 2019, the work moved into normative phase and a new Rel-17 work item was launched in SP-190303. The aim of this work is to specify the interactive service requirements including:

    - KPIs for interactive service (including data rate, latency, reliability, user number, etc.) required for either ProSe Communication path or 5GC path

    - Group support and management for interactive service required for either ProSe Communication or 5GC path;

    - Requirements related to suitable Hosted Service discovery.

The normative requirements resulting from this work item are in TS 22.261 (link).

- Communication Services for Critical Medical Applications (CMED)

  - Use cases below as well as other findings of the study item were reported in TR 22.826 (link)

    - Duplicating Video on additional monitors

    - Augmented Reality Assisted Surgery

    - Robotic Aided Surgery

    - Emergency care – Ultrasound examination and remote interventional support

    - Communication QoS requirement for robotic telesurgery

    - Patient monitoring inside ambulances

    - Cardiac telemetry outside the hospital.

  - In June 2019, the work moved into normative phase and a new Rel-17 work item was launched in SP-190306. The objective of this work item is to provide normative requirements to enable a 3GPP system to adequately provide the connectivity between medical devices and critical medical applications. This specifically involves the requirements relating to the following:

    - 5G system performances (KPIs) required to enable use cases involving high quality and augmented imaging systems in hybrid ORs

    - 5G system performances (KPIs) required to enable use cases with tele-diagnosis

    - 5G system performances (KPIs) required to enable use cases with tele or robotic-aided operations

    - Security management in 5G systems for the sharing of medical data between care providers or with patients while still fulfilling national regulatory requirements

The normative requirements resulting from this work item are in TS 22.263 (link).

- Enhanced Relays for Energy eFficiency and Extensive Coverage  (REFEC)

  - Use cases below as well as other findings of the study item were reported in TR 22.866 (link)

    - Container Use case

    - Wagon Use case

    - Enhancing indoor coverage for the fire brigade

    - Use case for elderly healthcare

    - Use case for connected ambulance

    - Use case for rural areas connected healthcare

    - Micro-environment services

  - In June 2019, the work moved into normative phase and a new Rel-17 work item was launched in SP-190307. The objective of the work is to develop Stage 1 normative service requirements for multi-hop relay by enhancing the work done for “Indirect 3GPP Communication” in TS 22.278. This work will consider the potential requirements and KPIs in different domains (e.g. inHome, SmartCities, SmartFarming, SmartFactories, Smart Energy, Public Safety, Logistics) identified and concluded in TR 22.866.

The normative requirements resulting from this work item are in TS 22.261 (link).

In Dec 2019, SA1 launched a new study item on traffic characteristics and performance requirements for AI/ML model transfer in 5GS. The aim of this work is to study the use cases and to propose potential service and performance requirements for 5G system support of Artificial Intelligence (AI)/Machine Learning (ML) model distribution and transfer (download, upload, updates, etc.). Media-related use cases around video/speech processing are among the targeted use cases. In particular, the objectives include:

- Studying the use cases and potential service and performance requirements for identifying traffic characteristics of AI/ML model distribution, transfer and training for various applications, e.g. video/speech processing, automotive, other verticals, including:

  - Traffic characteristics and performance requirements for AI/ML operation splitting between AI/ML endpoints;

  - Traffic characteristics and performance requirements for AI/ML model/data distribution and sharing over 5G system;

  - Traffic characteristics and performance requirements for Distributed/Federated Learning over 5G system.

- Gap analysis on performance requirements for AI/ML model distribution and transfer, e.g.

  - Data rate, latency, reliability, coverage and capacity, etc. for AI/ML model downloading/uploading.

A Techical Report will be the output of this work. The TR number is 22.874 (5GS; Study on Traffic Characteristics and Performance Requirements for AI/ML Model Transfer). The expected completion time is by March 2021 (Release 18).

- Study on evolution of IMS Multimedia telephony service (FS_MMTELin5G)

  - The prime objective of this study is to analyze the scenarios and identify potential service requirements for AR/VR communication

  - Use cases below as well as other findings of the study item are reported in draft TR 22.873 (link)

Study Item description

The study item description is available at the following link:

https://www.3gpp.org/ftp/tsg_sa/WG1_Serv/TSGS1_87_SophiaAntipolis/Docs/S1-192721.zip

Justification

With the evolution and capability enhancement of network system, telephony service has evolved from CS voice call to IMS multimedia telephony service. Nowadays 3GPP has introduced new network capabilities and new type devices (e.g. AR/VR devices, robot, etc.), which can bring promising improvements for IMS multimedia telephony service.

As a fundamental service of 5G, multimedia telephony service is expected to leverage cutting-edge technologies to meet the new demands from both consumer and vertical markets.A typical example could be a call-centre agent offers online step-by-step remote assistance with AR/VR elements to a remote client via a 5G voice/video call in a customer service scenario.

Objectives

The first objective of this SID is to study new scenarios and identify potential service requirements for AR/VR communication.

The second objective is to do a gap analysis to existing requirements in 3GPP (e.g. in 22.228 and 22.173) to identify new requirements for introduction into 3GPP.

Progress

15% as of October 2020

Completion date

Expected March 2021

Draft Technical report TR 22.873

Latest version

Version 0.3.0

Use cases

- Conference call with AR holography

- The use case refers to multi-party media-conference in which participants can virtually meet using AR glasses in their respective conference room.

- controlled environment (virtual participant is meant to be placed in an empty seat).

- real time and live 3D model capture/transmission/rendering meeting the usual end-to-end latencies for video conferencing.

- challenge of relative positioning and eye catching when dealing with more than 2 users.

- the 3D model is required able to accurately represent a user from any angle in a conference room (likely 180 degrees).

- Real-time speech recognition

- The use case describes the real-time speech recognition and translation during a call (audio or audio +video) in which the translated speech is displayed as text or subtitle to the distant user.

- dedicated metadata identifying the language and the voice properties (e.g. male/female, voice tone…) may help the translation engine particularly in the case in which a text-to-speech engine is used rather than just subtitles being displayed.

- Study on traffic characteristics and performance requirements for Artificial Intelligence (AI).Machine Learning (ML) model transfer in 5GS (FS_AMMT)

  - The aim of this work is to study the use cases and to propose potential service and performance requirements for 5G system support of Artificial Intelligence (AI)/Machine Learning (ML) model distribution and transfer (download, upload, updates, etc.).

  - Use cases below as well as other findings of the study item are reported in draft TR 22.874 (link)

Study Item description

The study item description is available at the following link:

https://www.3gpp.org/ftp/tsg_sa/TSG_SA/TSGS_86/Docs/SP-191040.zip

Justification

Artificial Intelligence (AI)/Machine Learning (ML) is taking more and more important tasks, releasing huge productivity gains. In order to effectively support AI/ML applications on 5G terminals (e.g. smartphone, smart car, mobile robot, UAV), three major challenges need to be dealt with:

- How to do the AI inference if a 5G UE does not have the necessary on-board computation/battery resource?

- In case a 5G UE can partially do the on-board AI inference, how does it obtain the updated AI/ML model and intermediate output based on the changing task and environment?

- How to train a global AI model across 5G network?

For Challenge 1), a solution could be to offload the AI inference operation from UE to 5G cloud/edge. The UE sends the sensing/perception data to 5G cloud/edge and executes the commands from the control server on 5G cloud/edge after the server performs the AI inference. This solution can be realized in manner of “Cyber-physical control” service as identified in SA1 spec [3]. However, the solution relies on ultra-high end-to-end reliability and very low round-trip latency for sensing/perception data uploading and controlling command downloading, which requires not only a full-coverage URLLC deployment (still very challenging in FR2), but also a mature MEC deployment.

Hence performing at least a part of AI inference on-board is still desired, if the UE has sufficient capability, i.e. a split inference between network and UE. Then Challenge 2) becomes critical. AI applications in multi-functional or mobile terminals usually need to update the AI/ML model (e.g. Deep Neural Network (DNN) in case of deep machine learning) adaptive to task and environment. Due to the limited storage at 5G UE and unpredictable change of environments, it is unfeasible to pre-load all possible AI/ML models on-board, and model downloading and/or transfer learning is needed. Hence online downloading AI model from 5G cloud needs to be supported. Compared to “Offloading AI/ML inference” solution, “Downloading AI/ML model” may require much higher data rate (probably higher than eMBB capability), but can substantially relax the latency requirement.

For either “Offloading AI/ML inference” or “Downloading AI/ML model”, Challenge 3) would be critical in case one AI/ML model will be used in global scenarios rather than only used in a local scenario. Firstly, the AI server will take a huge computation burden if the model training is centralized to the server.  Secondly, privacy protection rules (e.g. GDPR) may not allow local training data to be uploaded to cloud. In this case, distributed/federated learning over 5G network can be adopted, in which UEs train their own NN based on a NN architecture downloaded from cloud and the local training data, and upload the partially-trained NNs for the AI server to form the global NN by, e.g. “Federated averaging”.

Challenge 3) becomes more challenging if a collaborative ML is performed over different types of intelligent agents. In this case, the training data can be shared between different types of intelligent agents over 5G Uu interface or sidelink.

In any of the above three use cases, AI/ML models or even training data will become a new traffic type conveyed in 5G network, which may be substantially different from legacy services and call for enhancements to 5G. The service requirements of AI/ML model transfer in the 5G system needs to be studied.

Objectives

The aim of this work is to study the use cases and to propose potential service and performance requirements for 5G system support of Artificial Intelligence (AI)/Machine Learning (ML) model distribution and transfer (download, upload, updates, etc.).

The objectives include:

- Studying the use cases and potential service and performance requirements for identifying traffic characteristics of AI/ML model distribution, transfer and training for various applications, e.g. video/speech processing, automotive, other verticals, including:

  - Traffic characteristics and performance requirements for AI/ML operation splitting between AI/ML endpoints;

  - Traffic characteristics and performance requirements for AI/ML model/data distribution and sharing over 5G system;

  - Traffic characteristics and performance requirements for Distributed/Federated Learning over 5G system.

- Gap analysis on performance requirements for AI/ML model distribution and transfer, e.g.

  - Data rate, latency, reliability, coverage and capacity, etc. for AI/ML model downloading/uploading.

NOTE: Studying AI/ML models themselves are not in the scope of the SI.

Progress

40% as of October 2020

Completion date

Expected March 2021

Draft Technical report TR 22.874

Latest version

Version 0.1.0

Use cases

- Split AI/ML operations between the end device and the edge/cloud

  - Split AI/ML image recognition

      - The use case describes the principle to share the inference workload between a mobile device and remote cloud resources so as to relax computational/memory pressure on the mobile device. The convolutional neural network (CNN) made of several layers is split into 2 parts, the first one being processed by the mobile device and the second part by the cloud resource. The selection of the split point depends on the complexity of the remaining tasks to be done and the amount of intermediate data to be transferred.

      - Requirements are set on latency and bit rate for intermediate data upload on reliable communication networks

  - Enhanced media recognition: Deep Learning Based Vision Applications

      - This is a use case in which object recognition is processed from the video captured by a user. Feature Pyramid Network is suggested for feature extraction for object recognition. This approach differs from the previous case in the sense that the captured object is recognized from the dedicated library. Moreover, the use cases highlight the capability to receive a 3D model of the recognized object.

  - Media quality enhancement: Video streaming upgrade

      - Use case in which the streamed video is in a “low” resolution and then upscaled locally in the user device using AI. The AI-based up-scaler receives the low-resolution video + metadata from autoencoder.

  - Split control for robotics

      - Use case in which ultra-low latency is required for robot control. Operations exclusively done in the robot are too complex and energy consuming whereas exclusively done remotely (edge/cloud) cannot meet the latency requirements.

  - Session-specific model transfer split computation operations

      - Use case of AR-glass split rendering. Depending on the network conditions, 2 different split points are defined favoring either the uplink data-rate or the computation load of the user device. Potential requirements may be defined as the capability to QoS parameters as input of the decision engine for the split points selection.

- AI/ML model distribution and sharing

  - AI/ML model distribution for image recognition

      - Use case in which the DNN model stored on a mobile device is not the most efficient one and requires the download of a better one. Requirements to minimize the size of the DNN model.

      - Aligned with ISO/IEC CD 15938-17 Compression of neural networks for multimedia content description and analysis.

  - Real time media editing with on-board AI inference

      - This use case addresses the use of dedicated ML model for post processing the media captured by the device (e.g. specific ML model for live indoor concert available in the venue, optimized post processing functions for dark images enhancement, noise sound cleaning…).

  - AI/ML model distribution for speech recognition

      - AI/ML models are downloaded by a device so as to run Automatic Speech Recognition (ASR) locally. The selection of the appropriate model is done based on external conditions (e.g. noisy background…) or presets (e.g. language…)

- Distributed/federated learning

  - Federated Learning for image recognition

      - Use case in which a user device takes part of the learning phase of an image recognition AI/ML model. The resulting training report is uploaded to a Federated aggregator and the updated global model is returned to the user device as an improved version. Multiple user devices can be included as part of the learning phase of a common model.

  - Compressed Federated Learning for image/video processing

      - As an extension of the previous use case, in order to minimize the traffic load, not the whole model is uploaded to the Federated aggregator, but only a Compressed Federated Learning (CFL) for which only significant value weight gradients are sent.

  - Data Transfer Disturbance in Multi-agent multi-device ML Operations

      - This is a use case in which model transfers can be optimized through prioritization or data exchange between servers. Very related to network conditions.

  - Study and Work items in 3GPP SA4

2.2.1 Finalized Work Items

The following Work Items are related to already finalized 3GPP Releases that may have relation to MPEG activities. This work will not evolve, unless a new WI with new features is proposed to a current release:

- Server and Network Assisted DASH for 3GPP Multimedia Services (SAND).

- Enhanced QoE Reporting for MTSI (EQoE_MTSI).

- SAND for MBMS (SAND4M).

- Addition of HDR to TV Video Profiles (HDR).

- Virtual Reality Profiles for Streaming Media (VRStream).

- Media Handling Aspects of 5G Conversational Services (5G_MTSI_Codecs).

- 5G Media streaming architecture (5GMSA)

  - The main objective is to create a new 5G Media Streaming (5GMSA) architecture specification which supports:

    - MNO and 3rd party Media Downlink Streaming Services; MNO and 3rd party Media Uplink Streaming Services; Corresponding UE functions and APIs; Backwards compatibility for EUTRAN deployments (with and without MBMS); Usage of 5G specific features such as network slicing and edge computing.

    - The new 5G Media Streaming architecture should be functionally decomposed into independent components enabling different deployments with various degrees of integration between 5G MNOs and Content Providers

  - Comment: Specification TS 26.501 completed, but corrections expected.

- QoE metrics for VR (FS_QoE_VR)

  - The objective of this Study Item is to investigate the QoE parameters and metrics which may need to be reported by the client to the network for evaluation of VR user experience.

  - Comment: Study Completed. Alignment with MPEG solicited.

- WID on VR QoE metrics (VRQoE)

  - Release 16.

  - Completion date: March 2020.

  - Impacted specifications:

    - TS 26.118 VR profiles for streaming applications.

    - TS 26.247 Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH).

  - For 2D (DASH) streaming there are already existing QoE metrics defined, which can help service providers and operators to measure aspects related to the service quality. As discussed in the FS_VR study (TR 26.918), and recommended in FS_QoE_VR study (TR 26.929), there is a need to define QoE metrics relevant for VR services, so that at least the basic and most important aspects of the VR experience can be measured. In concrete:

    - Define VR metrics observation points within the VR Client Reference Model in TS 26.118.

    - Metrics describing the characteristics of the VR device, such as resolution, FOV, etc.

    - Metrics describing the interaction delay, related to the time between head movements and update of the visible content.

    - Extensions to the DASH Metrics configuration and reporting to support the new metrics.

  - Comment: this work is pretty aligned with MPEG-I activities on metrics and is relevant for MPEG.

- Study Item on V2X Media Handling and Interaction (FS_mV2X)

  - Release 16, completed in December 2019.

  - Target document: TR 26.985 Vehicle-to-everything (V2X); Media handling and interaction. Related documents: TR 22.886, TS 22.186, TR 23.795.

  - The overall objective of this work item is to analyse the use cases of Video data sharing for assisted and improved automated driving (VaD) explained in TR 22.886 and their requirements in TS 22.186 in detail, to clarify the operation of advanced driving using networked visual information. The required procedures for media capture, compression, and transmission are investigated. We also study the possibility of using other types of media-related information 3GPP services can supply to vehicles, such as voice, audio, image, or graphics

  - Comment: Relevance to MPEG object detection and/or coding?

- Service Interactivity (SerInter)

  - Release 16, completed in December 2019.

  - The main target specification for this WI is TS 26.247.

  - Objectives: this work targets mainly the specification of notifications to interactivity applications and has relation with DASH-IF activities.

- Media Handling Extensions for 5G Conversational Services (5G_MEDIA_MTSI_ext)

  - Release 16.

  - Completion date: March 2020.

  - The target existing specifications for this WI are: TS 26.114, TS 26.131, TS 26.132, TS 26.223 and TR 26.919.

  - Objectives: it covers conversational media rate adaptation (based on the RTP protocol) and introduction of additional IETF protocols, such as the SCTP and DTLS.

- Enhancements to Framework for Live Uplink Streaming (E-FLUS)

  - Release 16.

  - Completion date: March 2020.

  - The main specifications for this WI are: TR 26.939 (contains use cases) and TS 26.238.

  - Objectives: this work item covers low-delay conversational uplink streaming (based on the RTP protocol) and in particular these topics: QoS requirements, upload strategies, downlink distribution, network assistance, network-based media processing, professional media applications, drone mounted cameras.

- 5G Media Streaming Stage 3 (5GMS3)

  - Release 16.

  - Completion date: September 2020.

  - Target specifications: some existing specs are updated (TS 26.234 and TS 26.247) and new ones are created:

    - TS 26.511 5G Media Streaming (5GMS); Profiles, codecs and formats

    - TS 26.512 5G Media Streaming (5GMS); Protocols (it includes Session control, DRM, QoE metrics, Network Assistance, Edge computing, slicing and Capability Exchange aspects).

    - TS 26.117 5G Media Streaming (5GMS); Speech and Audio Profiles.

  - Objectives: this WI is based on the new streaming architecture that supports the 5G evolution (5GMSA), and extends the work with Stage 3 specifications.

  - Comments: this work has relevance for MPEG, since the protocols make use of DASH.

- Work Item on Removal of MPEG-4 Visual and H.263 from 3GPP Services (RM_H263_MP4V)

Impacted specifications: TS 26.114, TS 26.140, TS 26.234, TS 26.346

Release: 16.

Expected completion date: September 2020.

Objectives:

The objective of this work item are as follows:

- remove any normative (shall, should, may) statements related to MPEG-4 Visual, if applicable, from Rel-16 service specifications and related unnecessary or confusing statements.

- remove any normative (shall, should, may) statements related to H.263 from Rel-16 service specifications and related unnecessary or confusing statements.

- Identify other 3GPP service specifications (under SA4 control and outside SA4) that reference H.263 or MPEG-4 Visual and provide relevant updates, if necessary.

- Communicate with relevant organizations on this update, if necessary.

- Study Item on eXtended Reality (XR) in 5G (FS_XR5G)

  - Release 16.

  - Completion date: March 2020.

  - Target documents: TR 26.928 Extended Reality (XR) in 5G. Other references include: TR 26.891  section 6.3, TR 22.981, Permanent document on FS_XR5G.

  - Objectives: among the others, investigate the relevance of Augmented and Extended Reality in the context of 3GPP by:

    - Analysing technologies and equipment in place that provide Extended Reality experiences.

    - Collecting use cases

    - Analysing and identifying the media formats (including audio and video), metadata, accessibility features, interfaces and delivery procedures between client and server required to offer such an experience

    - Identifying relevant client and network architectures and APIs that support XR use cases

    - Studying the processing requirements (both audio and video) and associated issues such as spatial resolutions, frame rate, latency, etc.

  - Comment: Any plan for a converged architecture for cloud XR? This work is very relevant in the SA4 and it is likely that will continue in some form. There is relevance for MPEG too.

- Study Item on Typical Traffic Characteristics of Media Services (FS_TyTraC)

  - Release 16.

  - Completion date: March 2020.

  - Target document: TR 26.925 Typical traffic characteristics of media services on 3GPP networks.

  - Objectives:

    - Collect current media centric Third-Party and Operator services characteristics;

    - Collect the typical deployment characteristics today, such as bandwidth requirements;

    - Provide an overview on technological developments;

    - Provide a summary on typical characteristics and requirements for different media services on 3GPP networks;

    - Identify the applicability of existing 5QIs for such services and potentially identify requirements for new 5QIs or QoS related parameters.

  - Comment: Status of Study: Input on typical traffic for MPEG based media?

Comments: the above work items make use of DASH, QoE metrics, OMAF, CICP, ISO/IEC 14496-15 (Carriage of NAL unit structured video in ISOBMFF). The QoE metrics have been developed by 3GPP. The Virtual Reality Profiles for Streaming Media work is based on OMAF technologies.

2.2.2 Currently Active Work and Study Items

- EVS Codec Extension for Immersive Voice and Audio Services (IVAS_Codec)

  - The overall objective of this work item is to develop a single general-purpose audio codec for immersive 4G and 5G services and applications including the VR use cases envisioned in 3GPP TR 26.918 (Virtual Reality (VR) media services over 3GPP).

  - Comment: Status >=30% complete. Conversational codec relevant for MPEG-I.

- 5G Media Streaming Stage 3 (5GMS3)

  - Release 16.

  - Completion date: September 2020.

  - Target specifications: some existing specs are updated (TS 26.234 and TS 26.247) and new ones are created:

    - TS 26.511 5G Media Streaming (5GMS); Profiles, codecs and formats

    - TS 26.512 5G Media Streaming (5GMS); Protocols (it includes Session control, DRM, QoE metrics, Network Assistance, Edge computing, slicing and Capability Exchange aspects).

    - TS 26.117 5G Media Streaming (5GMS); Speech and Audio Profiles.

  - Objectives: this WI is based on the new streaming architecture that supports the 5G evolution (5GMSA), and extends the work with Stage 3 specifications.

  - Comments: this work has relevance for MPEG, since the protocols make use of DASH.

- Support of Immersive Teleconferencing and Telepresence for Remote Terminals (ITT4RT)

  - Release 17.

  - Completion date: June 2021.

  - Target specifications: TS 26.114 and TS 26.233. (TR 26.919 and TR 26.918 are also references for this WI).

  - Objectives: specify VR support in RTP-based conversational services to enable support of an immersive experience for remote terminals joining teleconferencing and telepresence sessions. The work is expected to enable scenarios with two-way audio and one-way immersive video, e.g., a remote single user wearing an HMD participates to a conference will send audio and optionally 2D video (e.g., of a presentation, screen sharing and/or a capture of the user itself), but receives stereo or immersive voice/audio and immersive video captured by an omnidirectional camera in a conference room connected to a fixed network.

  - Work currently progresses through a permanent document that includes use cases, requirements, architecture considerations, gap analysis and potential solutions. Latest version of the permanent document can be accessed here.

- Work Item on ”Operation Points for 8K VR 360 Video over 5G” (8K_VR_5G)

  - Release 17

  - Approved WID found here: https://www.3gpp.org/ftp/tsg_sa/TSG_SA/TSGs_89E_Electronic/Docs/SP-200667.zip

  - Completion date: March 2021

  - Target specifications: TS 26.118, TS 26.511, TR 26.925

  - Objectives: The work item will specify operation points in VR streaming specification TS 26.118 as well as new media decoding capabilities for 5GMS in TS 26.511 in order to enable support for up to 8K video. More specifically, this work item aims to conduct the following normative work in TS 26.118 and TS 26.511, toward fulfilling this objective:

    - Define new 360-degree video operation point(s) for VR streaming to be used in conjunction with the Main Video Profile with conforming bitstream requirement based on H.265/HEVC Main-10 Profile Main Tier Profile beyond the existing level 5.1 decoding capability, supporting spatial resolutions up to 8K as well as frame rates up to 120fps.

    - Provide implementation guidelines for deploying such new profiles and operation points.

    - Include the newly defined decoding capabilities and associated profiles and operation points into 5G Media Streaming.

    - Document typical traffic characteristics of such 8K VR 360 video services.

- Work Item on Removal of MPEG-4 Visual and H.263 from 3GPP Services (RM_H263_MP4V)

The WID is available here.

Impacted specifications: TS 26.114, TS 26.140, TS 26.234, TS 26.346

Release: 16.

Expected completion date: September 2020.

Objectives:

The objective of this work item are as follows:

- remove any normative (shall, should, may) statements related to MPEG-4 Visual, if applicable, from Rel-16 service specifications and related unnecessary or confusing statements.

- remove any normative (shall, should, may) statements related to H.263 from Rel-16 service specifications and related unnecessary or confusing statements.

- Identify other 3GPP service specifications (under SA4 control and outside SA4) that reference H.263 or MPEG-4 Visual and provide relevant updates, if necessary.

- Communicate with relevant organizations on this update, if necessary.

- Study Item on Feasibility Study on VR Streaming Conformance and Guidelines (FS_VR_CoGui)

  - Release 17. New Study Item just launched.

  - Completion date: December 2020.

  - Target document: TR 26.A00 Study on VR Streaming Conformance and Guidelines.

  - Objective: provide content generation and usage guidelines for promoting the technologies defined in TS 26.118 (VR Streaming), some if the aspects to be addressed include:

    - Develop usage scenarios for the application of technologies defined in TS 26.118 for live and on-demand streaming of VR360 content.

    - Document content generation guidelines that address the usage scenarios.

    - Collect relevant source content material for the usage scenarios.

    - Define objective quality measures for the relevant usage scenarios and provide information on certain parameter settings such as bitrates, quality options, etc. for each media type.

    - Provide bitstreams and media presentations that follow the content generation guidelines and conform to the requirements in TS 26.118.

    - Conduct subjective tests for the relevant usage scenarios and characterize the applicability of the technologies in TS 26.118 and certain parameter settings such as bitrates, quality options, etc. for each media.

    - Document existing test results, if they exist, for the relevant usage scenarios.

  - Comments: coordination with MPEG is expected, whenever needed.

- FS_XRTraffic (Feasibility Study on Typical Traffic Characteristics for XR Services and other Media)

Not relevant for MPEG.

- FS_5GMS_Multicast (Feasibility Study on Multicast Architecture Enhancements for 5GMSA)

Not relevant for MPEG.

- FS_EMSA (New SID on Streaming Architecture extensions For Edge processing)

Not relevant for MPEG.

- FS_5GVideo (Feasibility Study on 5G Video Codec Characteristics)

The SID is available here.

Impacted specification: new TR 26.9xx (Video Codec Characteristics for 5G-based Services and Applications).

Release: 17.

Expected completion date: December 2020.

Objectives:

- Collect a subset of relevant scenarios for video codecs in 5G-based services and applications, including video formats (resolution, frame rates, color space, etc.), encoding and decoding requirements, adaptive streaming requirements, predominantly based on scenarios defined for 5G media streaming as well as for TR 26.925 and TR 26.928.

- Collect relevant and exemplary test conditions and material for such scenarios, including test sequences.

- Define performance metrics for such scenarios with focus on objective performance metrics.

- Collect relevant interoperability functionalities and enabling elements for video codecs in different 5G services such as MTSI and Telepresence (i.e. RTP based conversational communications), or 5G media streaming (e.g. based on DASH/CMAF) supporting the identified scenarios.

- Collect relevant criteria and key performance indicators for the integration of video codecs in 5G processing platforms, taking into account factors such as encoding and decoding complexity in the context of the defined scenarios.

- Characterize the existing codecs H.264/AVC and H.265/HEVC in the context of the above scenarios and document the findings in a consistent manner.

- Identify gaps and deficiencies of existing codecs in such use cases and derive requirements for potential new codecs.

- Collect initial information on how new codecs under development in ISO/IEC SC29 WG11 (MPEG)/JVET (in particular including VVC and EVC) may meet the above criteria based on the characterization results provided for example by ISO/IEC SC29 WG11 (MPEG)/JVET.

- FS_FLUS_NBMP (Study on the use of NBMP in EFLUS)

The SID is available here.

Impacted specification: TR 26.939.

Release: 17.

Expected completion date: December 2020.

Objectives:

- Develop a detailed workflow of the establishment of FLUS session and NBMP workflow based on TS26.238.

- Investigate if any signalling, format or protocol is missing from the TS26.238 and NBMP specifications to successfully establish the above workflow.

- Collect relevant and exemplary use-cases for the above environment.

- Map 3GPP network QoS parameters to the NBMP QoS parameters and identify the possible missing QoS parameters in the NBMP specification.

- Investigate the possible improvements and extensions of the workflow by enhancing TS26.238 and NBMP specifications.

- FS_5GSTAR (Study on the 5G Glass-Type AR/MR Devices)

The SID is available here.

Impacted specification: New TR 26.9XY (Support of 5G Glass-type AR/MR Devices).

Release: 17.

Expected completion date: June 2021.

Objectives:

The objective of this study item includes the following:

- Provide formal definitions for the functional structures of AR glasses, classified as device types of XR5G-A4 (standalone) and XR5G-A2, A5 (wirelessly tethered) in TR 26.928, including their capabilities and constraints with respect to communication, computing and graphics processing, tracking, sensors, display, and power consumption

NOTE 1: Device type of XR5G-A3 (video see-through HMD) are not the primary scope of this study, but are not excluded per se.

- Describe key use cases for AR services over 5G based on those in Table 4.10 of TR 26.928, map each use case to suitable device types, and define relevant processing functions and reference architectures for the devices that are required to support the use cases, taking into account the constraints and capabilities identified under the first objective.

NOTE 2: The preferred cases will be those capable of delivering experiences previous or existing media services could not support, e.g., those requiring features for AR/MR 3D download, communication or conversational (related to use cases 1, 2, 7, 8, 11, 15, 16, 17, 19, 20, or 23). The study also allows new use cases not covered in TR 26.928.

- Describe the architecture for media flow relevant to the use cases identified in the second objective. Identify media (exchange) formats and profiles relevant to the use cases identified in the second objective that can be processed on AR platforms as defined under the first objective. Identify where media processing functions occur and which type of media formats are used for exchange between these elements to the described architecture.

NOTE 3: Exchange formats include both, formats and signals consumed on AR devices (e.g. overlays, scenes, animations) as well as those that are generated (and shared) on/by those (for example to support spatial localization, object recognition and tracking).

- Identify necessary content delivery transport protocols and capability exchange mechanisms, as well as suitable 5G system functionalities (e.g., device, edge, network) and QoS (including radio access and core network technologies) required for the use cases. If existing technologies and protocols cannot serve the cases sufficiently, describe the necessary functionalities and features.

- Identify key performance indicators and quality of experience factors (such as immersiveness, presence, localization, and world tracking accuracy) for the AR use cases based on the initial considerations in TR26.928, clause 4.2 with additional emphasis on the use cases, device platforms and exchanges formats for AR as identified in earlier objectives.

NOTE 4: As there can be several approaches for the optical implementation of displays, this study does not prefer any display types, nor will it define any detailed criteria or quality requirements.

- Identify relevant radio and system parameters (required bitrates, latencies, loss rates, range, etc.) to support the identified AR use cases and the required QoE, in particular when the AR device is connected via 5G sidelink interfaces.

NOTE 5: Depending on the use cases, other types of peripheral devices, such as external cameras and sensors, can also be connected to the UEs in a similar fashion.

- For each of the identified use cases and AR device platforms, provide a detailed overall power analysis for media AR related processing and communication building on the information in TR26.928, clause 4.8. Different design options should be considered, for example if media processing is carried out on the device, on a puck/smartphone or in the edge/cloud. It is recommended to contact relevant 3GPP WGs, in particular, and RAN4, on expected modem power consumption for certain use cases.

NOTE 6: Support other 3GPP WGs and other SDOs on relevant aspects related to their work and responsibilities for these new design options and form factors on a need basis, in particular when requested for processing functions in scope of SA4.

- Identify potential areas for normative work as the next phase and communicate with other 3GPP WGs on relevant aspects related to the study.

  - Summary of the Joint 3GPP/VRIF/AIS 2nd Workshop on VR Ecosystems and Standards: “Immersive Media meets 5G”, 15-16 April 2019 Culver City, U.S.A.

Source: presentations available here:

https://www.3gpp.org/ftp/tsg_sa/WG4_CODEC/Joint%203GPP%20SA4%20-%20VRIF-AIS%20Workshop%202019/Docs/

Presentation on key takeaways from the workshop:

https://www.3gpp.org/ftp/tsg_sa/TSG_SA/TSGS_85/Docs/SP-190638.zip

During the workshop, among the others, the importance of the following use cases and technologies (relevant for MPEG) was highlighted:

- VR for training

- Enterprise/Social VR/AR

- VR/AR overlays

- Volumetric capture, formats and streaming

- Edge/Cloud computing for AR/VR

- Cloud rendering

- >=8K resolutions for 360-degree video

Comment: one of the potential reported gaps was content and tools for Extended Reality.

  - Overviews of immersive media standards

- M50778 “The Overview of standards activities related to immersive media (v8.1)” provides an overview of standards activities related to immersive media, including 3GPP, DVB, VRIF, ITU-R, QoE (QUALINET, VQEG, ITU-T), DASH IF, Khronos, WebXR, CTA, IETF/IRTF, SMPTE, ETSI, SVA, and IEEE P2048.

  - http://wg11.sc29.org/doc_end_user/documents/128_Geneva/wg11/m50778-v1-m50778.zip

- Presentation “3GPP achievements on VR & ongoing developments on XR over 5G” by 3GPP SA4 vice chair at the 2nd Workshop on VR Ecosystems and Standards, April 15-16, 2019, organized by 3GPP, VRIF & AIS.

  - https://www.vr-if.org/wp-content/uploads/3GPP-achievements-on-VR-ongoing-developments-on-XR-over-5G.pdf

  - Other 5G-related standards and organizations

- Liaison statement from ITU-T FG ML5G on Results of ITU’s work on machine learning for 5G networks

    - http://wg11.sc29.org/doc_end_user/documents/128_Geneva/wg11/m50729-v1-m50729.zip

- EBU 5G-MAG. New organization that links broadcasters, 3GPP, DVB:

    - https://tech.ebu.ch/publications/5g-mag-a-short-introduction-at-ibc-2019

    - www.5g-mag.com

  - Mandate 2: To identify potential new MPEG activities that can help the industry deploy applications, including media services, over 5G.

During MPEG 130, the LS reply from 3GPP SA4 in [5] and the MPEG & 5G AhG discussions triggered the following:

- AHG recommendation: Investigate to what extent activities on (indoor) spatial information (storing, sharing, mapping, localization, etc.) have been pursued within the MPEG.

Based on this investigation, further work on this area may be started in the MPEG. A specific AHG mandate has been added at MPEG 130, and the progress is reported in section 4.

During MPEG 129 the outcome of the AhG was the following (it includes only the currently open actions):

    - AHG recommendation: Cross check MPEG NBMP requirements against the 3GPP TR 22.827 (Study on Audio-Visual Service Production) section 5.10 and verify whether there are any gaps in the MPEG requirements. There was a joint meeting between Systems (NMBP) and Requirements (MPEG & 5G), and it was agreed that this work deserves further attention and analysis which will be done by the NBMP experts. In particular, they will identify possible gaps within the NBMP specification and propose new requirements for the identified gaps.

- AHG recommendation: Study NBMP feasibility and gaps for real-time voice/video communications in the context of the 3GPP TR 26.928 (Extended Reality (XR) in 5G, section 6) and ITT4RT use cases described in the ITT4RT Permanent Document (S4-191256). These activities will be further analyzed by the NBMP experts as the previous item.

  - Mandate 3: Investigate to what extent activities on (indoor) spatial information (storing, sharing, mapping, localization, etc.) have been pursued within MPEG.

During MPEG 131 a preliminary investigation has been done in [6]. The findings are reported here for both the MPEG organization and other organizations.

SC29/MPEG

- MPEG-I Scene Description

- MPEG-I V-PCC and G-PCC

- MPEG CDVS and CDVA

- MPEG ARAF

- MPEG BIFS

- MPEG V

SC24

- X3D

Open Geospatial Consortium

- OGC ARML

- OGC GML

- OGC CityGML

- OGC IndoorGML

- OGC KML

- OGC WMS

Please see [6] for references to the above standards. The AHG recommendation is to continue studying this mandate in relation to the above standards activities (also listed in section 2 of [6]), since some of them may not be totally relevant in this context (or outdated), and possibly identify gaps for which mandate 4 of the MPEG & 5G AHG would be activated (i.e., “Provide initial requirements for the identified new MPEG activities”). For example, it is relevant to investigate the relation of ARAF and BIFS and the current work on SD in MPEG.

The following mandate has been introduced at MPEG 131.

  - Mandate 4: Define focused use cases in the area of indoor spatial information storage, sharing, mapping, localization, etc. and define requirements in order to identify relevant standards and missing technologies.

5.1 XR multimedia streaming use case with high-level architecture

The Study Item on eXtended Reality (XR) in 5G (XR5G) has been completed in March 2020 (3GPP Release 16) with its output technical report TR 26.928 [2]. The XR multimedia streaming use case from such TR provides a good description of the areas where MPEG standards could contribute and benefit from the advances of 5G networks. This high-level use case could be a starting point from which the MPEG community could identify the gaps that would require normative work.

For convenience we provide the use case description below:

“This category covers live and on-demand streaming of XR multimedia streams, which include 2D or Volumetric video (i.e., Constrained 6DoF) streams that are rendered in XR with binaural audio as well as 3DOF and 3DOF+ immersive A/V streams. It is illustrated in Figure 5.4-1, where UE is a device capable of receiving and rendering the type of stream in use. It is also capable of controlling the playback of these streams using input from handheld controllers, hand gestures, biometric readings, body and head movements etc., which is communicated to the content server. Control signals include pause, rewind, viewpoint selection or, in case of emotional streaming, adaptive content selection.

In another system instance, the content server can provide Inter-destination Multimedia Synchronization (n) for a group experience. A Spatial Computing Server is used by XR capable devices to register, compute, update and recall the spatial configuration of their surroundings. The service is meant for indoor spaces like a shared room or building. Appropriate surfaces may be selected for display of XR streams and saved in the Spatial Computing Server. The configuration can be shared amongst authorized users to enhance group experience when multiple users are physically sharing the same space.

A social aspect may be added to XR multimedia streaming by receiving live social media feeds, in addition to the XR media stream, that can be rendered as an overlay. Furthermore, the users may be able to see the avatars of other remote located users consuming the same media (additionally from the same viewpoint in case of multiple viewpoints) and have audio conversations with them by establishing one or more communication channels. The social aspects are added by the cloud in the Figure 5.4-1. The XR Media storage in the cloud is for fetching XR objects, e.g., avatars. The location is shared with the cloud to establish colocation within the XR. For instance, users viewing from the same viewpoint may be considered collocated when consuming synchronized media” [2].

2

1

Figure 5.4-1 XR Multimedia Streaming [2] and potential standardization areas for MPEG

After studying the use case, we identify the following areas that would be relevant to current or future work conducted by the MPEG community:

- Formats for storing and sharing of XR spatial configuration;

- Formats for storing and sharing of XR media augmentation;

The remainder of this document will focus on the first item.

5.2 Further information about spatial configuration

The use case presented in section 2 includes elements of social AR that requires spatial mapping and localization to create a shared experience for multiple users in real-time. TR 26.928 defines spatial mapping in section 4.1.4 as follows [2]:

“Spatial mapping, i.e. creating a map of the surrounding area, and localization, i.e. establishing the position of users and objects within that space, are some of the key areas of XR and in particular AR. Multiple sensor inputs are combined to get a better localization accuracy, e.g., monocular/stereo/depth cameras, radio beacons, GPS, inertial sensors, etc.

Some of the methods involved are listed below:

- Spatial anchors are used for establishing the position of a 3D object in a shared AR/MR experience, independent of the individual perspective of the users. Spatial anchors are accurate within a limited space (e.g. 3m radius for the Microsoft® Mixed Reality Toolkit). Multiple anchors may be used for larger spaces.

- Simultaneous Localization and Mapping (SLAM) is used for mapping previously unknown environments, while also maintaining the localization of the device/user within that environment.

- Visual Localization, e.g., vSLAM, Visual Positioning System (VPS), etc., perform localization using visual data from, e.g., a mobile camera, combined with other sensor data.

Spatial mapping and localization can be done on the device. However, network elements can support the operations in different ways:

- Cloud services may be used for storing, retrieving and updating spatial data. For larger public spaces, crowdsourcing may be used to keep the data updated and available to all.

- A Spatial Computing Server that collects data from multiple sources and processes it to create spatial maps including, but not limited to, visual and inertial data streamed from XR devices. The service can then provide this information to other users and also assist in their localization based on the data received from them.

Indoor and outdoor mapping and localization are expected to have different requirements and limitations. Privacy concerns need to be explored by the service provider when scanning indoor spaces and storing spatial features, especially when it is linked to global positioning.

Tracking adds the concept of continuous localization over time.”

5.3 High-level requirement categories

Based on the information included in sections 2 and 3 above, we recommend that the following high-level requirements should be considered for standardizing common interchangeable data formats for XR applications and network-based (cloud-based) services. In this document the description of such requirements is as simple as categories. More specific technical requirements should be studied further by the MPEG community.

#1 XR spatial configuration for environment capturing, mapping, and localization for both indoor and outdoor situations;

There have been 3D reconstruction and localization technologies available long time in the research and industrial domains, such as Structure from Motion (SfM) or photogrammetric technologies for off-line reconstruction of large (e.g. city scale) physical scenes; and simultaneous localization and mapping (SLAM) technologies for real-time 3D point cloud mapping and device tracking and localization (e.g. in relative small areas, typically in-door environments) due to its visual feature-based nature. Modern AR devices like Oculus Quest, Microsoft HoloLens, and HP’s Reverb G2, all uses inside-out SLAM tracking techniques to understand the physical environment and real-time on-device tracking. Intel RealSense uses Lidar technique to build the understanding of the world. The needs for a standardized spatial configuration become very clear and import to harmonize the different formats about the captured digital features of the physical world (both indoor and outdoor); and supports tracking and localization in a large scale with good interoperability.

High-level requirements (draft)

Definitions:

XR space: virtual and physical space.

Spatial Computing Server (SCS): a network-based server that provides spatial computing service to devices with XR capabilities (XR devices).

XR spatial data: all relevant visual or non-visual data captured by the sensors on XR devices for the creation of XR spatial configuration.

XR spatial configuration: A description of the XR space with the information about the position and orientation (pose estimation) of XR devices.

- The SCS shall be able to create and update the XR spatial configuration of the environment, for the XR space.

- The SCS shall be able to accept the XR spatial data captured by XR devices over the network

- The SCS shall be able to stream the XR configuration to XR devices over the network

- The XR spatial configuration shall have a data structure to represent the XR space with appropriate scaling factor to the physical world

- The XR spatial configuration shall allow the definition of the XR space from small scale space like indoor environment to large scale outdoor space

- The XR spatial configuration shall support visual and/or non-visual features of the XR space for real-time localization and tracking (pose estimation)

- The XR spatial configuration shall allow navigation of the XR client from one space to another when multiple XR spaces are used

#2 XR media augmentation with anchors as placements for both live and on-demand social XR experience;

The concept of XR anchors have been in use in the industry, for instance, Microsoft Azure’s Spatial Anchor and services [3], Google’s Cloud Anchor for ARCore technology [4], and Apple’s AR Anchor by ARKit [5]. They share the basic common function of adding virtual objects to an AR scene. Multiple users can then view and interact with these objects simultaneously from different positions in a shared physical space (AR context). We recommend extending the function from AR to XR and bringing good interoperability among different devices and platforms, with focused media centric augmentation use cases.

High-level requirements (draft)

Definitions:

XR anchor: A point of interest contains spatial properties like the position and orientation in the XR space, and geometric properties from a single spatial point to a complex geometry; and other supplementary attributes such as representation or user interactions.

- The position in the XR space shall be defined by an XR anchor.

- There shall be an association defined for logic links between XR anchors and live streaming media content

- There shall be an association defined for logic links between XR anchors and media on demand content.

- References

[1] WG11 N19510, 5G Activities Relevant to MPEG, MPEG 131 output document, June 2020, online. [2] m55262, [MPEG&5G] Additional media-related activities in 3GPP, MPEG 132 meeting, October 2020, online. [3] m54979, [MPEG&5G] Further information on 3GPP activities related to MPEG, MPEG 132, October 2020, online. [4] m55536, [MPEG&5G] Evolution of multimedia telephony services, MPEG 132, October 2020, online. [5] m53875, Liaison statement from 3GPP SA WG4 on Extended Reality (XR), April 2020, Alpbach, Austria (Virtual). [6] m55537, [MPEG&5G] AI-ML model transfer scenarios, MPEG 131, October 2020, online. [7] m55264, [MPEG&5G] Spatial Information for XR Services, MPEG 132, October 2020, online.

https://en.wikipedia.org/wiki/Structure_from_motion

https://en.wikipedia.org/wiki/Simultaneous_localization_and_mapping

https://xinreality.com/wiki/Inside-out_tracking

https://en.wikipedia.org/wiki/Lidar