Source:	Qualcomm Incorporated

Title:	HEIF Image Format

## Document for:	Agreement

## Agenda Item:	11.5

# 1. Introduction

ITT4RT has a requirement to support still images as overlays. The transmission of still images is unspecified. In this contribution, we propose the usage of the HEIF image format.

# 2. HEIF Image Format

Still images and image sequences, encoded using HEVC, as described in [1] Annex B shall be supported.

# 3. Carriage of HEIF Images and Image Sequences

HEIF image items and image sequences shall be described as regular HEVC media streams. The HEVC Payload format as described in RFC 7798 shall be used for the delivery of the HEVC images, image collections, and image sequences. All NAL units of an HEVC image stored as meta items shall be extracted and transmitted as HEVC access units with the same presentation time stamp.

# 4. Proposal

Incorporate the text in clauses 2 and 3 into the Permanent Document.

# 5. References

[1] ISO/IEC 23008-12: Information technology – MPEG systems technologies – Part 12: Image File Format