F3GPP TSG SA WG4  113-e	S4-210478

06-14 April 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | <CR#> | rev | - | Current version: | 0.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5GMS-EXT] Updated text for Content Preparation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMS-EXT |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The study item description identifies the key topic “Content Preparation”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding potential open issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Key topic not addressed |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

### 5.2.7	Potential open issues

### 5.2.7.1	TS 26.512 current support

TS 26.512 section 7.6.3.1 defines the content hosting configuration resource. The partial table is shown in the following figure:

Table 5.2.7.1: Definition of ContentHostingConfiguration resource

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| name | String | 1..1 | A name for this Content Hosting Configuration. |
| IngestConfiguration | Object | 1..1 | Describes the 5GMSd Application Provider's origin server from which media resources will be ingested via interface M2d. |
| path | String | 1..1 | The relative path which will be used to address the media resources at interface M2d. This path is provided by the 5GMSd AF in the case of Push-based ingest. |
| pull | Boolean | 1..1 | Indicates whether to the 5GMSd AS shall use Pull or Push for ingesting the content. |
| protocol | URI String | 1..1 | A fully-qualified term identifier allocated in the name space urn:3gpp:5gms:content-protocol that identifies the content ingest protocol. The set of supported protocols is defined in clause 8. |
| entryPoint | String | 1..1 | An entry point to ingest the content. The semantics of the entry point are dependent on the selected ingest protocol. In the case of Push ingest (pull flag is set to False), this parameter is returned by the 5GMSd AF to the 5GMSd Application Provider and indicates the entry point for pushing the content. In case of Pull (pull flag is set to True), the entryPoint shall be provided to the 5GMSd AF to indicate the location from which content is to be pulled. In this case, the entryPoint shall be used as the base URL. A request received by the 5GMSd AS is mapped to a URL using the provided base URL to fetch the content from the origin server. |
| DistributionConfigurations | Array(Object) | 1..1 | Specifies the distribution method and configuration for the ingested content. More than one distribution may be configured for the ingested content, e.g. to offer different distribution configurations such as DASH and HLS. |
| contentPreparationTemplateId | String | 0..1 | Indicates that content preparation prior to distribution is requested by the 5GMSd Application Provider. It identifies the Content Preparation Template that shall be used as defined in clause 7.4 |
| canonicalDomainName | String | 1..1 | All resources of the current distribution shall be accessible through this default FQDN assigned by the 5GMSd AF. |

As is shown in this table, the IngestConfiguration defines the input format for the content preparation. Currently, two protocols are supported: HTTP pull-based ingest and DASH-IF push-based (TS 26.512 8.2). In the case of HTTP push-based, if a request is received in M4d that can’t be satisfied, an HTTP pull request is made through M2.

### 5.2.7.2	Open issues in collaboration scenario 1: Content preparation before downlink streaming

Open issue 21: Support of other protocols

The current specification only support HTTP pull and DASH-IF ingest. Other protocols may need to be added. Two possible protocols are the ones that are specified for ingest.

Open issue 2: HTTP Pull request through M2

In the case where a content preparation template exists, then the URL rewrite process must be modified to address more complex use-case:

- When a simple rewrite of path is not adequate due to complex content preparation process, and it requires more complex algorithm to calculate the requesting URL.

- When the content preparation process requires more than one input. Then the reverse address mapping requires two or more address calculation.

- When (and if) the content preparation process can be cascaded.

Open issue 3: Multiple inputs for the Content protection Template

Following the call flow of section 5.2.6.1. the following steps are need:

- Provisioning

- Creating Content Preparation Template (CPT)

- Creating Content Hosting Configuration Template (HCT)

Each Content Hosting Configuration Template (HCT) may include one or more CPTs, each defined in one of its Distribution Configurations. However, the HCT is allowed to include only one Ingest Configurations. Therefore, with the current design, any CPT may have multiple outputs but only one input.

In the most common use-case of a multimedia streaming, at least one audio switching set and one video switching set are generated. If (packager) CPT needs to create a manifest that have multiple content types, then the output of two or more CPT needs to be provided to the packager CPT to create the manifest. This example is shown in the following figure.

Cache/

Storage

Content Preparation Process 1 (video)

M2d

CPP1

CPT1

Content Preparation Process 2 (audio)

M2d

CPP2

CPT2

TCC2

Content Preparation Process 3 (manifest generation)

CPP3

CPP4

CPP5

CPT3

TCC2

Figure 5.2.7.2-1: Example of a packager CPT for creating streaming manifest for audiovisual content

Therefore, we need to define a mechanism that allow content preparation template uses multiple inputs.

Another use-case is that when the metadata track provides splicing point. For a packager to create the periods, the corresponding CPT needs to have two inputs: media/manifest input and the metadata input.

One possible solution is to allow multiple ingest configurations in HCT and the included CPTs to tied to one or more ingest configurations.

Open issue 4: Content Preparation Template information

TS26.512 only mentions that the CPT is identified with its MIME type. At least one format needs to be defined for the instruction in CPT for common services such as multi-rate streaming.

### 5.2.7.2	Open issues in collaboration scenario 2: content preparation after uplink streaming

In addition of the above issues of the collaboration scenario 1, the following issues may be considered.

In this case, the content is delivered to the 5GMSu Application Provider through M2u.

Open issue 5: Egest protocols

TS 26.512 Table 8.1-1 defines the ingest protocols at M2d. But the egest protocols for M2u are not defined. One possibility is use defined ingest protocols also as possible egest protocols. However it must be investigated whether the current definitions of these protocols are adequate for egest.

Editor’s Note: We recommend addressing this issue in uplink streaming topic of this study.

Open issue 6: Egest Configuration

TS 26.512 Content Hosting Configuration resource defines Ingest Configuration. It is not clear whether the same resource can be used for Egest configuration and whether the parameters are adequate.

Editor’s Note: We recommend addressing this issue in uplink streaming topic of this study.

### 5.2.7.2	Open issues in collaboration scenario 3: content prepraration between uplink and downlink

Since both uplink and downlink is used, the following issues should be considered:

Open issue 7: Signalling the connection between the uplink and downlink

Assuming the previous issues are addressed, then the Content Hosting Configuration Template (HCT) resource for uplink and downlink would be separately used in M1u and M1d. The uplink HCT’s egest configuration should be set aligned with the downlink HCT’s ingest configuration as is shown in the following figure.

The following issues must be addressed:

- Whether the protocol is left to the network operator and only direct connection between two AS is signaled?

- Is any (optional) standard protocol needed for the connection? If so, which protocls?

- Are the current Ingest Configuration’s (and its counterpart for Egest) parameters adequate for setting up the connection between uplink and downlink?

- Do the connection configuration and 5GMS Application Server’s addresses need to be hidden from the 5GMS Application Provider?