Source:	Samsung Electronics Co., Ltd.

Title:	FS_5GSTAR: pCR on Mapping to 5G System Architecture

Agenda Item:	11.9

Document for:	Discussion and Agreement

# Introduction & Proposal

It was agreed that the current TR26.998 includes clause 6: “Mapping to 5G System Architecture”, which covers architectures, call flows, exchange formats, and KPIs per each scenarios. Each sub-clause from 6.2 to 6.6 will specify each scenario in detail and tsub-clause 6.1 will provide an introduction. This contribution provides the text and table for 6.1 (currently empty), connecting the list of use cases in clause 5 and the subsequent sub-clauses.

Thus, it is proposed to include the text and table in section 2 of this document into TR26.998

# Proposed change

*** Change 1 ***

# 2	References

[6.1.a]	3GPP TS 26.501: “5G Media Streaming (5GMS); General description and architecture”

[6.1.b]	3GPP TR 26.803: “5G Media Streaming (5GMS); Architecture extensions”

*** End of Change 1 ***

*** Change 2 ***

## 6.1	General

Based on the identified use cases in clause 5, this clause documents how AR/MR service scenarios can be supported in 5G system architecture.

There already exists developed 5G system architectures relevant to deliver immersive media depending on the underlying functionalities, such as real-time communications, adaptive delivery, QoS guarantee, and a support of network node processing. An architecture of 5G Media Streaming (5GMS) for both downlink and uplink is specified in TS26.501[6.1.a] and is being further extended to support the edge media processing in TR26.803[6.1.b]. In addition, MTSI architecture extended to 5G system [4.3.d] may be applied to AR/MR conversational scenarios to guarantee the specific service QoS. In the following clauses, these relevant architectures will be analysed to identify potential standardisation areas for each scenario.

Note that only STAR UE and EDGAR UE in Table 4.1 are taken into account, as WLAR UE as well as WTAR UE have similar functionalities with STAR UE from a 5G system perspective. Specifically, STAR UE (and WLAR/WTAR UEs) possibly has an on-device decoding and rendering capability for immersive media and may rely on support from 5G cloud/edge for a certain condition. On the other hand, EDGAR UE always requires 5G cloud/edge for immersive media decoding and rendering, and the conventional 2D media is exchanged in Uu interface.

Table 6.1 provides a list of AR/MR service scenarios and the associated use cases for each. Note that some use cases may be duplicated as they address multiple features.

Table 6.1. List of service scenario mapping to use cases

| Service Scenario | Clause | Relevant Use Case |
| --- | --- | --- |
| Immersive media downlink streaming | 6.2 | 2. AR Sharing1) 14. AR Streaming with Localization Registry 17. AR remote advertising 18. Streaming of volumetric video for glass-type MR Devices |
| 5G interactive immersive service | 6.3 | 1. 3D Image Messaging 2. AR Sharing1) 4. AR guided assistant at remote location (industrial services) 1) 5. Police Critical Mission with AR1) 15. 5G Shared Spatial Data 16. AR remote cooperation1) |
| 5G cognitive immersive service | 6.4 | 5. Police Critical Mission with AR1) 14. AR Streaming with Localization Registry1) 20. AR IoT control |
| AR two-party calls | 6.5 | 3. Real-time 3D Communication 4. AR guided assistant at remote location (industrial services) 1) 7. Real-time communication with the shop assistant 11. AR animated avatar calls 16. AR remote cooperation1) |
| AR conferencing | 6.6 | 8. 360-degree conference meeting 9. XR Meeting 10. Convention / Poster Session 12. AR avatar multi-party calls 13. Front-facing camera video multi-party calls 19. AR Conferencing |
| 1) may be duplicated into multiple scenarios |  |  |

*** End of Change 2 ***