3GPP TSG-SA4 Meeting SA4#113-e 	S4-210534

April 6th- April 14th  2021

Agenda item	12.5

Source: 	Tencent

Title: 	Signaling for Audio mixing gain

Document for	Agreement

# Introduction

This contribution relates to the signaling of audio mixing gain for immersive and overlay streams so that the receiving device mixes the audio streams correctly and in a meaningful way. This is a follow-up of document S4-210202 (Update to Audio mixing of multiple streaming in ITT4RT), accepted in the SA4#112e Meeting.

# Proposal

The ITT4RT-Tx client may indicate the audio mixing gain value to the ITT4RT-Rx client via SDP or RTP header-extension based solution.

## SDP based solution

### Solution 1: Describing each audio gain separately and per stream

The ITT4RT-Tx client may, in its SDP offer, indicate the mixing gain of the 360-degree audio and overlay audio by including a media-level attribute “3gpp_audio_mix_gain”, in the description of each media stream, for describing the audio mixing gain, with the following ABNF:

a=3gpp_audio_mix_gain:<audio-mixing-gain>

### Solution 2: Describing all audio gains together

The ITT4RT-Tx client may in its SDP offer, signal an array of mixing gain of the 360-degree video and all the overlays in its media attribute, in the description of the 360-degree audio stream, by following the below ABNF

a=3gpp_audio_mix_gain: 3gpp_360audio:<360_mixing_gain> <overlay_id1>:<overlay1_mixing_gain> <overlay_id2>:<overlay2_mixing_gain>

When the value of mixing gain changes and needs to be updated, the ITT4RT-Tx client may only send the media attribute “a=3gpp_audio_mix_gain:” with updated values of the mixing gain mapped to the 360audio and the overlay ids identifiers without the need to send the entire media line. These identifiers may be used to map the updated audio mixing value to the respective audio streams.

## RTP based solution

### Solution 1: Header extension carrying audio grain in each audio stream

For RTP based solution, the packet of each RTP audio stream can indicate, in an RTP header extension, the mixing level of that audio sample, carried in the RTP packet with the following RTP extension (RFC 8285).

0                   1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | len=0 |audio-mixing-gain|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Audio mixing gain using One-Byte Header Format

The 4-bit ID is the local identifier of this element. The 4-bit length is the number, minus one, of data bytes of this header extension element following the one-byte header. The URI for declaring the audio mixing gain header extension in a Session Description Protocol (SDP) extmap attribute and mapping it to a local extension header identifier is

urn:3gpp:rtp-hdrext:audio-mixing-gain

An example attribute line in the SDP for a conference is shown below. The presence of such an attribute in the SDP description indicates that RTP packets in that stream containing the header extension will be carrying such an extension with an ID 7.

a=extmap:7 urn:3gpp:rtp-hdrext:audio-mixing-gain

The header extension may be present only in the first/ first few packets of the RTP audio stream and may be repeated when the mixing gain needs to be updated for optimality.

### Solution 2: Header extension carrying all audio gains together in a single stream

For RTP-based solutions, the packet of the 360-degree RTP audio streams may indicate in an RTP header extension the mixing level of the 360-degree audio streams along with the mixing level of the overlay audios. The mixing gain of the 360-degree/overlay audio streams may be added as an extension element to the header extension, as shown below (RFC 8285).

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length=3            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | L=0   |audio-mixing-gain|  ID   |  L=0  |audio-mixing-gain|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | L=0   |audio-mixing-gain|  ID   |  L=0  |audio-mixing-gain|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | L=0   |audio-mixing-gain|  ID   |  L=0  |audio-mixing-gain|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

An example header extension, with three extension elements

4-bit ID is the local identifier for the audio-mixing-gain of the 360-degree audio or the overlay audio. The 4-bit length is the size of this header extension element following the one-byte header. Additionally, having these extensions added to a single header extension; reduces the overhead/complexity of the ITT4RT-Rx client parsing multiple cross-stream packets and then putting them together.

The above extension is applicable when streams of the 360-degree audio and overlay audio are sent in a single RTP session.

# Proposal

The proposal is to add the above text to section 3.9 in the PD.

# References

- IETF RFC 8285 (2017) “A General Mechanism for RTP Header Extensions”

- IETF RFC 6465 (2011) “A Real-time Transport Protocol (RTP) Header Extension for Client-to-Mixer Audio Level Indication”