Source: 	Nokia Corporation

Title: 	Proposed changes to draft CR on RTCP Viewport Feedback

Document for	Agreement

Agenda item: 	12.5

# Introduction

This document presents changes related to RTCP reporting, to be incorporated to the informative section X.9 of draft CR (S4-210430).

# X.10	Viewport RTCP Feedback

This section provides guidance for the implementation of the RTCP feedback message type ‘Viewport’ described in clause X.7.2.

Figure X.10.1 illustrates the impact of the viewport feedback message on motion-to-high-quality delay. The figure shows a sender (ITT4RT-Tx client) delivering viewport-dependent content to a receiver (ITT4RT-Rx client). After a change in viewport, an RTCP viewport feedback is sent by the receiver to the sender, which responds with updating the viewport in the viewport-dependent content. The motion-to-high quality delay includes the RTT, processing delays and also any delay in generating the RTCP viewport feedback. Therefore, the viewport feedback mechanism requires careful consideration.

An ITT4RT-Rx client may include a predicted viewport (using application-level viewport prediction techniques) in the viewport feedback if the viewport control is set to device_controlled during SDP negotiation. An ITT4RT-Rx client may use event-based viewport feedback or fixed interval RTCP feedback. A more flexible approach is to use fixed interval with early feedback.

Figure x.10.1. Motion to High Quality Delay

## X.10.1 Event-based Viewport Feedback

As per RFC 4585, the client can send event-based immediate feedback as long as:

Events per interval <= RTCP allocated bandwidth / Avg RTCP packet size

where,

Events per interval= Avg no of events reported/ Time interval.

For an RTCP viewport feedback, the event-based feedback interval can be calculated using the trigger angle by:

Events per interval = HMD Speed / Trigger Angle

where HMD Speed is the speed of the head mounted display. Therefore, to respect the 5% bandwidth allocated for the RTCP traffic allowed by the RTP standard, it is recommended that the receiver does not trigger event-based immediate feedbacks for trigger angle smaller than TAmin, and

TAmin >= Average RTCP packet size / RTCP allocated bandwidth  *  HMD speed

With a changing trigger angle, receivers continue to operate within immediate feedback mode, where a feedback is sent as soon as an event is detected. However, the receiver redefines the event, which is the minimum trigger angle, based on current head speed.

Editor’s Note: Trigger angle should be defined more clearly.

## X.10.2 Fixed Interval with Early RTCP Feedback

Alternatively, receivers may evaluate an average time interval, T, between RTCP feedback messages based on the allocated RTCP bandwidth. The value T can be used by the receiver to limit RTCP viewport feedbacks from exceeding the allocated bandwidth in case of high motion, as done for the early RTCP feedback mode in RFC 4585. Using the interval, it is possible to use a combination of regular and event-based feedback messages.

An ITT4RT-Rx client may define a trigger that initiates an early RTCP feedback. When viewport margins are used, the early viewport feedback trigger should take them into account. Figure X.10.2 shows an image of an ERP of a 360-degree video with the current viewport and margins. As the viewport changes, the distance of the viewport, in degrees, from the boundary of the margins (shown as D_left, D_top, D_right, D_bottom and lastly D_direct, which is the distance in the direction of motion) decreases. An implementation may use the velocity of the head (causing the change in viewport), V, or the distance, D, to trigger the early viewport feedback i.e, an early viewport feedback is triggered when the V is above a certain threshold, V',  and D is below a certain threshold, D'.

Figure x.10.2. Moving viewport shown on an ERP of 360-degree video when viewport margins are used

Alternatively, the ITT4RT-Rx client may compute the time till the changing viewport will breach the boundary of the viewport margin, T_breach. An early viewport feedback is triggered when T_breach is below a certain threshold. Regardless of whether D, V or T_breach is used to trigger an early RTCP feedback, if the time to the next scheduled RTCP viewport feedback is small, the early feedback should be suppressed and the regular scheduled feedback is sent. As an example, Figure x.10.3 shows a timeline with T, the interval between two scheduled viewport feedbacks and T_breach, the time at which the viewport is expected to breach the viewport margin i.e., the high-quality region of the 360-degree video. An ITT4RT-Rx client may trigger an early RTCP viewport feedback as long as T-T_breach < T', where T' is the suppression threshold for an early RTCP feedback.

Figure x.10.3: A timeline showing T, T’and T_breach in reference to the scheduled RTCP viewport feedback. Time increases left to right.

When viewport margins are not used, an ITT4RT-Rx client may trigger an early RTCP viewport feedback if the current viewport has deviated more than a threshold from the last reported viewport. This solution is independent of the received 360-degree content and the current high-quality region. Figure x.10.4 shows an illustration for the spherical distance D, in degrees, between the center of the viewport in the last RTCP viewport feedback and the center of the current viewport at the ITT4RT-Rx client. An ITT4RT-Rx client may trigger an early RTCP feedback message in this case when D > D', where D' is some threshold. However, if the time to next scheduled RTCP feedback is less than the suppression threshold T', the early RTCP feedback is not sent.

Figure x.10.4 Changing viewport orientation (dotted blue arrow) shown on an ERP

Editor’s Note: Whether there is a need for signalling any of the thresholds for triggering or suppressing an early RTCP feedback message, for triggering an event-based feedback or for defining periodic feedbacks is FFS.

- ****End changes****

# Proposal

The proposal is to include the proposed changes to the draft CR on viewport-dependent delivery, Tdoc S4-210430.

Contact: Igor Curcio, Saba Ahsan, Nokia, Finland. Emails: íigor.curcio, saba.ahsaný@nokia.com.