F3GPP TSG SA WG4  113-e	S4-210561

06-14 April 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.803 | CR |  | rev |  | Current version: | 1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to call flows and new gap analysis |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_EMSA |  |  |  |  |  | Date: |  |  | 31-04-2021 |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. Some steps in the first call-flow need clarification. 2. Gap Analysis is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Minor revisions of the first call flow - Gap analysis for both call flows |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclarify of call flow and empty (FFS) gap analysis. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 6.3.2	Client-driven edge discovery

Figure 8 outlines a detailed call flow for client-driven session establishment.

Figure 8: Client-driven session establishment

The Edge Computing Provisioning phase is a provisioning phase, that may be repeated several times (e.g. to extend edge processing coverage to new geographical areas or to increase the capacity of an already provisioned area). All steps in this phase are optional and performed on need basis. The steps are:

1.	Spawn ECS: In this step, a new ECS is instantiated to manage new or increased demand for edge processing.

2.	Spawn 5GMS AF: In this step, a new 5GMS AF that is edge-enabled is instantiated to handle new or increased demand for media sessions with edge processing.

3.	EES Configuration: The EES is configured for a specific Edge Data Network.

4.	EES Registration with ECS: The EES registers with the ECS that is in authority over the target EDN.

The 5GMS Application Provider Provisioning phase is performed prior to the establishment of any related media streaming sessions by the 5GMS Application Provider. Subsequent updates to the provisioning session are possible.

5.	Create Provisioning Session: In this step, the 5GMS Application Provider creates a new provisioning session.

6.	Provision 5GMS features: In this step, the 5GMS Application Provider may create different configurations such as Content Hosting, Reporting, Edge Processing, etc.

During the UE Edge Computing Discovery phase, the UE discovers an EAS that capable of serving 5G media streaming session and running the edge applications.

7.	Application Initialization: The user launches the 5GMS-Aware Application. The application performs any required initialization steps.

8.	Locate EAS/5GMS AS: The Application Client requests allocating one or more suitable EAS "5GMS AS" that will be able to serve the application.

9.	Locate local EES: The EEC queries the ECS for a suitable EES.

10. Register with EES: The EEC registers with the selected EES.

11.	Locate EAS/5GMS AS: The EEC contacts the EES to query, using the EAS discovery filters provided by the Application client, for one or more EAS "5GMS AS"’s that can serve the session.

The optional subflow is for provisioning an additional 5GMS AS instance if a suitable EAS "5GMS AS" cannot be located. The steps are:

12.	Check resource template: The 5GMS AF checks the provisioned edge processing resource template for the related application to determine the requirements of the application.

13.	Instantiate new EAS/5MGS AS: The 5GMS AF requests the MnS to instantiate a new EAS "5GMS AS" with the specified requirements and considering parameters provided in the query by EEC.

14.	Spawn 5GMS AS instance: The MnS creates a new instance of the EAS "5GMS AS" with the requested placement and resources.

15.	EAS configuration: The newly instantiated EAS "5GMS AS" is configured.

16.	Register EAS with EES: The newly instantiated EAS registers itself with the triggering EES.

17.	Configure provisioned features: This may include configuring and launching the server-side application in the 5GMS AS.

Completion of UE Edge Computing Discovery phase:

18.	5GMS AS location response: The EES responds to the EEC with the list of suitable EAS "5GMS AS"’s.

19.	5GMS AS location response: The location of the EAS "5GMS AS" is passed back to the application.

After successful discovery (and possibly selection of EAS by the Application if multiple EASs are provided), the actual streaming session may start in the 5GMS Session phase:

20.	Start session: The 5GMS-Aware Application invokes the Media Streamer with appropriate streaming access parameters (e.g. a Media Player Entry such as a DASH MPD URL).

21.	Session starting event: The application informs the Media Session Handler about the start of a new 5GMS session.

22.	Retrieve service access information: The Media Session Handler retrieves Service Access Information from the 5GMS AF appropriate to the 5GMS session.

23.	Media transfer: The 5GMS-Aware Application connects to the selected EAS "5GMS AS" and the streaming starts.

24.	Method calls and notifications: Supporting information about the 5GMS session is passed from the Media Stream Handler to the Media Session Handler.

25.	Reporting, network assistance, and dynamic policy: The Media Session Handler exchanges supporting information about the 5GMS session with the 5GMS AF.

26.	End session: the 5GMS-Aware Application informs the Media Session Handler that the 5GMS session has ended.

27.	Session ending event: The Media Streamer informs the Media Session Handler about the end of the 5GMS session.

28.	Final reporting: The Media Session Handler performs any final reporting to the 5GMS AF.

### 6.3.3	AP-driven management of 5GMS edge processing

Figure 9 outlines a detailed call flow for the AP-driven management of edge processing. In the previous sequence, the optional provisioning of an additional 5GMS AS instance occurs in response to an explicit call from the Application Client to an on-board EEC, whereas in this sequence it occurs, if needed, as part of and in response to Application Provider provisioning. It may also be repeated at any time during the lifetime of the 5GMS Provisioning Session in response to satisfy demand levels (not illustrated for brevity).

Figure 9: AP-driven management of 5GMS edge processing

The Edge Computing Provisioning phase is a provisioning phase, that may be repeated several times (e.g. to extend edge processing coverage to new geographical areas or to increase the capacity of an already provisioned area). All steps in this phase are optional and performed on need basis. Steps 1–4 are identical to those described in clause 6.3.2 above.

The 5GMS Application Provider Provisioning phase is performed prior to the establishment of any related media streaming sessions by the 5GMS Application Provider. Subsequent updates to the provisioning session are possible. Steps 5–6 are identical to those described in clause 6.3.2 above.

The optional sub-flow to provision an additional 5GMS AS instance may be repeated multiple times on need basis to add new capacity or to increase existing capacity for edge processing. The edge processing capacity is tailored for the specific 5GMS Application Provider based on the information in the Provisioning Session. Steps 7–12 are identical to steps 11–16 described in clause 6.3.2 above.

After successful discovery, the actual streaming session may start in the 5GMS Session phase. Steps 13–21 are identical to steps 19–27 described in clause 6.3.2 above.

NOTE:	In this call flow, the Application Client (AC) and EEC are not used to discover the 5GMS AS location. Instead, a Media Player Entry may be provided to the Media Session Handler by the 5GMS AF in the Service Access Information at M5 (step 15), or otherwise the location of the 5GMS AS is provided directly to the 5GMS-Aware Application via (out of scope) interface M8.

## End of the first change

## Second change

## 6.4	Identified Gaps in Architecture and Procedures

## 6.4.1 	Gaps in Client Driven discovery

1. In step 12: “The 5GMS AF checks the provisioned edge processing resource template for the related application to determine the requirements of the application.”

It must be investigated how 5GMS AF access to the edge processing resource template, and whether M1 supports providing this template during the “6. Provision 5GMS features”. Otherwise, the API that 5GMS AF access this template needs to be identified and possibly defined.

2. In step 18, if only one EAS “5GMS AS” is expected to be provided in the response, then the 5GMS AF/EES needs to select the best one.

This process is not defined, i.e. if 5GMS AF selects the best EES based on the template provided by the Application Provider during the “provision 5GMS features” step, and based on the EAS discovery filters provided by EEC, then this process must be indicated and required by “5GMS AF/EEC”.

## 6.4.2 Gaps in AP-driven management

1. In step 6 “Provision 5GMS features”, the desired characteristics of the EAS, such as geographical service area, service continuity support, and service features as is indicated by the EAS discovery filter by the Application in the client-driven call flow must be signalled to 5GMS AF. It is not clear whether an API and/or resource for such signalling is supported by M1 in TS26.512. Additionally, the first gap in 6.4.1 is also observed in step 6 of this call flow.

## End of the second change

Annex <A> (normative):
Deployment Scenarios

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 12th February 2021 | 112-e | S4-210313 |  |  |  | Documentation of relocation solutions |  |
| 12th February 2021 | 112-e | S4-210314 |  |  |  | Updates to EAS discovery |  |
| 12th February 2021 | 112-e | S4-210320 |  |  |  | EMSA architecture |  |
| 4th March | MBS Adhoc | S4aI201139 |  |  |  | EMSA call flow updates from S4aI201140 | 0.5.2 |
| 4th March | MBS Adhoc | S4aI201144 |  |  |  | Online updates | 0.6.0 |