3GPP SA4#114-e	S4-210765

19-28 May 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | <CR#> | rev | - | Current version: | 0.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5GMS-EXT] Candidate solution for Content Preparation format |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMS-EXT |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The study item description identifies the key topic “Content Preparation”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding some candidate solutions for simple CMAF content preparation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Key topic not addressed |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===== CHANGE  1 =====

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Akamai Blog, "A QUICk Introduction to HTTP/3", April 2020, https://developer.akamai.com/blog/2020/04/14/quick-introduction-http3

[3]	Fielding, R., Nottingham, M., and J. Reschke, "HTTP/1.1", Work in Progress, Internet-Draft, draft-ietf-httpbis-messaging-13, 14 December 2020, http://www.ietf.org/internet-drafts/draft-ietf-httpbis-messaging-13.txt

[4]	Belshe, M., Peon, R., and M. Thomson, Ed., "Hypertext Transfer Protocol Version 2 (HTTP/2)", RFC 7540, May 2015, https://www.rfc-editor.org/info/rfc7540

[5]	draft-ietf-quic-http-33, "Hypertext Transfer Protocol Version 3 (HTTP/3)", 15 December 2020

[6]	D. Bhat, A. Rizk, and M. Zink, "Not so QUIC: A Performance Study of DASH over QUIC," NOSSDAV'17: Proceedings of the 27th Workshop on Network and Operating Systems Support for Digital Audio and VideoJune 2017 Pages 13–18 https://doi.org/10.1145/3083165.3083175

[7]	AWS, "Achieving Great Video Quality Without Breaking the Bank", Streaming Media June 2019, https://pages.awscloud.com/rs/112-TZM-766/images/GEN elemental-wp-achieving-great-video-quality-without-breaking-the-bank.pdf

[8]	Netflix, "Optimized shot-based encodes: Now Streaming!", Netflix Blog, May 2018, https://netflixtechblog.com/optimized-shot-based-encodes-now-streaming-4b9464204830

[9]	DASH-IF/DVB Report on Low-Latency Live Service with DASH, July 2017, available here: https://dash-industry-forum.github.io/docs/Report%20on%20Low%20Latency%20DASH.pdf

[10]	DASH-IF IOP Guidelines v5, Low-latency Modes for DASH, available here: https://dash-industry-forum.github.io/docs/CR-Low-Latency-Live-r8.pdf

[11]	ISO/IEC 23009-1, "Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats"

[12]	IETF RFC 8673, "HTTP Random Access and Live Content".

[13]	3GPP TR 26.939: "Guidelines on the Framework for Live Uplink Streaming (FLUS)".

[14]	3GPP TS 26.238: "Uplink Streaming".

[15]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture"

[16]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols"

[17]		ISO/IEC 13818-1:2019 Information technology — Generic coding of moving pictures and associated audio information — Part 1: Systems

[18]	SCTE STANDARD SCTE 35 2020 Digital Program Insertion Cueing Message https://www.scte.org/pdf-redirect/?url=https://scte-cms-resource-storage.s3.amazonaws.com/SCTE-35-2020_notice-1609861286512.pdf

[19]	ISO/IEC 23000-19:2020 Information technology — Multimedia application format (MPEG-A) —Part 19: Common media application format (CMAF) for segmented media

[20]	ISO/IEC 23009-1:2019/DAMD1 Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats — Amendment 1: CMAF support, events processing model and other extensions [21]	VSF TR-06-01, RIST Simple Profile, https://www.videoservicesforum.org/download/technical_recommendations/VSF_TR-06-1_2018_10_17.pdf

[22]	VSF TR-06-02, RIST Main Profile, https://www.videoservicesforum.org/download/technical_recommendations/VSF_TR-06-2_2020_03_24.pdf

[23]	3GPP TS 23.501, System architecture for the 5G System (5GS)

[24]	3GPP TS 23.502, Procedures for the 5G System (5GS)

[25]	3GPP TS 29.517, 5G System; Application Function Event Exposure Service; Stage 3

[26]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes; Stage 3"

[27]	IETF RFC 6733: "Diameter Base Protocol".

[28]				3GPP TS 29.514, "5G System; Policy and Charging Control over Rx reference point; Stage 3"

[29]		IETF RFC 7657: "Differentiated Services (Diffserv) and Real-Time Communication", November 1995.

[30]	IETF RFC 3168: "The Addition of Explicit Congestion Notification (ECN) to IP", September 2001.

[31] 	ETSI TS 103 799 V1.1.1, Publicly Available Specification (PAS); DASH-IF Content Protection Information Exchange Format

[32]	ISO/IEC JTC1/SC29/WG11/N19062	MPEG-I: Network-based Media Processing 23090-8 FDIS Network-Based Media Processing Specification

===== CHANGE  2 =====

### 5.2.8	Candidate Solutions

Editor’s Note: Provide candidate solutions (including call flows) for each of the identified issues.

### 5.2.8.1	Content Preparation Format

### 5.2.8.1.1 Unencrypted single CMAF track to single unencrypted CMAF switching set

The content preparation must define the following parameters:

- The address/location of the input CMAF segments

- The output CMAF switching set information

  - Output parameters: the characteristics that are typically described in a manifest such as MPD or DASH. While these parameters are per track, they can be described one if one or more of them are common cross tracks. Examples:

    - Container profile, Codec/profile/level, bandwidth, container profiles, maximum SAP period, start with SAP,

    - Width, height, sample aspect ratio, frame rate,

    - Audio sampling rate, audio channel configuration

  - Encoding parameters: the parameters used for encoding each track that are not presented in the output manifest, such as bit-rate control, motion search area, and algorithm, CBR/VBR/Capped VBR encoding, use of specific quality metrics

    - Common encoding parameters (usually common in a codec/profile/level)

    - Vendor-defined (implementation specific) parameters

### 5.2.8.1.1 CMAF input format candidate 1: DASH MPD manifest

### The characteristics of the input can be defined by the DASH MPD manifest. The manifest can define the characteristics of the CMAF track. It can even optionally define the locations of CMAF segments when HTTP pull protocol is used to egest those segments.

### 5.2.8.1.2 CMAF input format candidate 2: A new document

### A new document format can be used for describing the input CMAF segments. The advantage of such a solution is when the same format is used for describing the output CMAF formats.

### 5.2.8.1.3 CMAF output format candidate 1: Extended manifest format

In this approach, a standard manifest format is used for output parameters, but it is extended for carrying encoding parameters, both common as well as vendor-defined parameters.

An example is to use MPD format and add descriptors to the adaptation set and/or representations for the encoding parameters. Two classes of descriptors can be added:

- The common encoding descriptor per codec, carrying common parameters

- Vendor-specific descriptors, carrying vendor-defined parameters.

Since the MPD essential and supplemental descriptor syntax allows defining different scheme URIs, both of the above features can be expressed using the same descriptor data type.

### 5.2.8.1.4 CMAF output format candidate 2: Manifest with external encoding document

In this approach, a standard manifest is used to describe the output parameters and a separate document is used to describe the encoding parameters.

An example is the following elements:

- MPD format for output parameters

### A JSON array with object array element which includes a representation id, referencing an representation in the MPD of 1 the encoding parameters for that representation. The encoding parameter a set of common parameters and then can be extended with vendor-specific parameters inside an object that can be identified with a URI identifier for any specific vendor.

### 5.2.8.1.5 CMAF output format candidate 3: A document defining both manifest and encoding parameters

In this approach, a new document is used for describing both manifest and encoding parameters.

An example of such a solution would be a JSON array, in with object array element including the following information:

- The output parameters commonly used in a manifest

- The common encoding parameters

- The encoder specific parameters with specific vendor’s identifier (such as URI)

Another alternative would be to use the DASH-IF’s Content Protection Information Exchange Format (CPIX) [X] and possibly extend it to carry additional parameters that are needed.

### 5.2.8.1.6 Creating template by combining the candidate solutions

Since both input, outputs, and encoding information should be provided as the content preparation template, the following solutions are possible for the overall template by combining the candidate solutions of 5.2.8.1.1-5.2.8.1.5:

- Single MPD:

  - One adaptation set with one input representation describing the input according to 5.2.8.1.1

  - One adaptation set with multiple input representation describing the output tracks according to 5.2.8.1.3

- A document consisting of 2 MPDs with possibly an extra document:

  - One MPD describing the input according to 5.2.8.1.1 and

  - One of the followings:

    - One MPD describing the outputs and encoding format according to 5.2.8.1.3, or

    - One MPD describing the outputs and one document describing the encoding parameters according to 5.2.8.1.4

- Single JSON document:

  - One item describing the input representation according to 5.2.8.1.2,

  - An array of object according to 5.2.8.1.5, each of which describes

    - one output

    - The encoding parameter for that output

### 5.2.8.1.1 Combined CMAF input and output formats combined candidate: NBMP Workflow Description Document

The NBMP specification [X] can describe the entire workflow using the NBMP Workflow Description Document (WDD). In this use case, the WDD describe the input format, as well as the array of tasks/function instances, each of which defines the CMAF output track as well as the encoding parameters for that track, as is shown in the following figure:

5GMSd AS

Content Preparation

Task 1:

Transcoder 1

Task 2:

Transcoder 2

Task n-1:

Transcoder n-1

M2d

Distribution

NBMP WDD and/or function template

Figure 5.2.8-1: Using NBMP WDD to describe CMAF content preparation

The NBMP WDD in this case describes the input CMAF as the input of workflow and each task’s function, configurations, and output. Since many features of NBMP specification are not used in this specific workflow, the WDD features can be profiled to be a simple WDD not using several descriptors defined by NBMP.

The NBMP specification allows defining function templates. So, one way to simplify the support for NBMP by 5GMSd AS is to define a function template for CMAF content preparation. The NBMP CMAF function template can define among other things:

- Input CMAF media profile using explicit description, MPD or HLS m3u8.

- The push and pull protocols for input CMAF

- The CMAF outputs format

- The transocoder’s common and vendor-specific configuration parameters

- Multiple codec output

- Reporting, monitoring and notification parameters for each transcoding functions

Another advantage of this solution is that the WDD format can be used for describing other use-cases and therefore one single format may be able to address several use-cases.