3GPP TSG-SA4 Meeting #114e	S4-210775

Electronic meeting, Telco, May 19-28, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | - | rev | - | Current version: | 0.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updated text on uplink streaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMS-EXT |  |  |  |  |  | Date: |  |  | 2021-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The study item description identifies the key topic “Uplink Media Streaming”. Metrics reporting configuration and reporting functionality are under-described and under-specified in Rel-16 TS 26.501 and TS 26.512. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposed new text for clause 5.5.1.2 and reorganization of that clause, addition of clause 5.5.1.3, and renumbering of existing clause 5.5.1.3 as 5.5.1.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Key topic :Uplink Media Streaming” is not poperly addressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.5.1.1, 5.5.1.2, 5.5.1.3, (new via renumbering) 5.5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===== 1st CHANGE  =====

# 2	References

The following documents The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

<------------- snipped ------------>

[38]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

===== 2nd CHANGE  =====

## 5.5	Uplink media streaming

### 5.5.1	Description

#### 5.5.1.1	Overview

Uplink media streaming functionality is currently under-specified in TS 26.501 [15] and TS 26.512 [16]. This part of the study investigates the gaps and potential solutions for completing the uplink streaming procedures, and associated protocols and APIs.

#### 5.5.1.2	Gap analysis of TS 26.501

The original focus of Rel-16 3GPP TS 26.501 [15] is on the overall system architecture, description of high-level procedures and call flows pertaining to downlink media streaming services.

In TS 26.501, there is significant imbalance in the scope and details of the described procedures for downlink vs. uplink media streaming. Key issues include the following:

1.	The procedures for downlink media streaming include session establishment, provisioning of various types of configuration information, metrics reporting, consumption reporting, dynamic policy and network assistance. In comparison, the described procedures for uplink media streaming are limited to session management, remote control, and network assistance. It is unclear for uplink streaming whether and how the configurations for content preparation, content protocols discovery, dynamic policy, metrics reporting, etc., should be defined and how the associated functions will operate.

-	Absent in clause 6 are procedural description and call flow regarding QoE metrics collection and reporting in uplink media streaming as compared to the presence of such text  in clause 5.5 on metrics collection and reporting in downlink media streaming.

2.	For downlink streaming, it is described that access to Service Access Information by the 5GMSd Client may be provided either over M8d by the Application Provider, or else fetched by the Client via M5u. For uplink streaming, the solely described method for the 5GMSu Client to obtain Service Access Information is via application metadata delivery over M8u. The only exception to this rule is the alternative method for provisioning Service Access Information to the 5GMSu Client by the 5GMSu AF, associated with remote control sessions in reference to remote control use cases and operational mechanisms in the context of FLUS (Framework for Live Uplink Streaming) as described in TR 26.939 [13] and TS 26.238 [14]. However, it should be noted that due to the limited description of the relationship between the uplink streaming framework and use cases, defined procedures and APIs in TS 26.512, it is unclear whether or how remote control sessions associated with uplink streaming delivery can make use of those interface procedures and APIs.

-	Clause 4.3 is missing the description of Service Access Information parameters for a metrics configuration set pertaining to uplink media streaming, as compared to the presence of such information for downlink media streaming in clause 4.2.3. In particular, the “Metrics” parameter of the downlink streaming metrics configuration set is explicitly bound to the 3GPP “metrics” scheme and corresponds to one or more of the QoE metrics for either a progressive download or 3GP-DASH streaming service as defined in TS 26.247 [26]. On the other hand, given the lack of definition by SA4 of uplink streaming QoE metrics collection and reporting related functionality for existing application service specifications, there is no associated list of metrics on collection and reporting by the 5GMSu Client that can be referenced.

#### 5.5.1.3	Gap analysis of TS 26.512

Similar to TS 26.501, in TS 26.512, the originally-defined features, protocols and APIs for 5GMS services mainly pertain to downlink media streaming services. More recently, as part of the specification maintenance process, additional descriptive, clarification and corrective text towards supporting uplink streaming services has been incorporated. However, remaining shortcoming include:

1.	The latter information is centered on identifying relevance of existing and downlink streaming centric interface functionality for uplink streaming. For instance, provisioning and subsequent execution of content protocol discovery, dynamic policy invocation, and metrics reporting are regarded as applicable to support for uplink media streaming services. Also, content preparation via M1-based configuration of Content Preparation Templates may be considered a means for defining manipulations by a 5GMSu AS of media content uploaded from the 5GMSu Client. Such content manipulation may be associated with network-based media processing (NBMP) of uplink-delivered streaming media content (e.g., user-generated content of social media services, professionally-produced uplink streaming of sports events or concerts) as defined in FLUS [13] and further evaluated in the FS_FLUS_NBMP study item. However, further assessment of the potential linkage between content preparation and NBMP should be conducted.

2.	Although the Metrics Reporting Configuration resource as part of M1 provisioning procedures and described in clause 4.3.9 is intended to apply to either downlink or uplink media streaming, the corresponding data model of that resource (see clause 7.8.3) contains no explicitly-referenceable metrics for collection and reporting associated with uplink streaming services. This is due to the absence of a 3GPP-defined metrics scheme for uplink media streaming as it exists for downlink media streaming (e.g., the 3GPP scheme urn:‌3GPP:‌ns:‌PSS:‌DASH:‌QM10).

-	It would be desirable in this study to identify a candidate set of QoE metrics associated with uplink media streaming services, and as specified by a 3GPP-defined metrics scheme, to be collected by the 5GMSu Client and reported to the 5GMSu AF.

-	Up to now, in the development of the 5GMS architecture and associated protocols and APIs, very little attention has been given to defining control plane functionality that can offer unique/beneficial features to the end-user, network operator or Application Provider in the context of uplink media streaming service operation. Such value-added functionality could be enabled by leveraging the information available in the 5GMS network (e.g., subscription class, service characteristics, user/device mobility, network conditions) and which may be static or dynamic in nature. This is also an area that should be further studied.

#### 5.5.1.4	Gap analysis between TS 26.238 (FLUS) and TS 26.512 (5G Media Streaming)

TS 26.238 provides the following features:

1.	The FLUS Control Source may discover multiple FLUS sinks.

2.	The FLUS Control Source may discover the capabilities of each discovered FLUS Sink, including its network-based media processing capabilities.

3.	The FLUS Control Source may also request a FLUS Sink to perform media processing.

4.	The UE capabilities (formats, connectivity protocol, remote control) may be discovered by a FLUS Control Sink.

TS 26.512 uplink streaming currently lacks the above features.