3GPP TSG-S4 Meeting #114e	S4-210822

# Electronic Meeting,  19th May – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | <CR#> | rev | <Rev#> | Current version: | 0.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of Traffic Identification description and addition of identified open issues |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMS_EXT |  |  |  |  |  | Date: |  |  | 2021-05-12 |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Different terms are used to describe the general traffic identification procedures in different SA2 and CT3 specifications. The description for the Traffic Identification aspect was confusing, since different terminology was used. The “open issue” around traffic identification was still empty. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A short summary of terminology is added and the text is adjusted using terms more consistently. Some subsections are re-ordered to put the different technologies into a better sequence. Finds with respect to missing parameters on different interfaces is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 5.3.1	Description

For different features within the 5G Media Streaming Architecture, it is necessary for the 5G System to identify the traffic flows. The increased usage of transport encryption (e.g. HTTPS) increases the difficulty of detecting the packets for certain application flows. Existing detection methods, such as using “significant parts of the URL to be matched” (contained in a Packet Flow Description, see below), are impractical for HTTPS traffic, since the URL is carried in the encrypted payload.

Multimedia streaming applications might not be able to uniquely identify the 5-tuple of the streaming session, since the 5-tuples are often changing. This is due to factors such as load balancing, CDN distribution, multiple concurrent requests for different types of resources, etc. This study will address how to properly configure the 5G System to enable efficient detection of application flows (service data flows) e.g. for event reporting, and QoS profile usage, etc.

Note, that TS 23.50x specifications use a slightly different terminology than TS 29.xxx specifications. Further, TS 23.503 uses a bit different terminology than TS 23.501 and TS 23.502. The two common terms are defined in TS 23.503

- Packet flow: A specific user data flow from and/or to the UE, and

- Service data flow: An aggregate set of packet flows carried through the UPF that matches a service data flow template.

The terms traffic detection (cf TS 23.501) or application detection (cf 23.501) refer to the process of finding matching service data flows among all packet flows. This logic is defined in TS 23.503 as application detection filter.

The procedures in TS 23.502 uses the term flow description, which is only a subset of an IP Packet Filter Set (as defined in TS 23.501).

Figure 5.3.1-1 depicts the chain of functions (taken from TS 29.244 [26], Figure 5.2.1-1) within an UPF. The UPF always first looks up the PFCP session context to which a packet belongs. The PFCP session context is an individual PDU session or a standalone PFCP session not tied to any PDU session. Then there are so-called Packet Detection Rules (PDR), which implement traffic detection of the service data flows with respect to different conditions. Note that a PDR is direction specific, thus, an Uplink (UL) and a Downlink (DL) PDR is needed to detect a bidirectional service data flow.

Figure 5.3.1-1: Packet processing flow in the UP function (Figure 5.2.1-1 from TS 29.244 [26])

Based on the PDR result, the next rules are executed, namely Multi-Access Rule (MAR), Forward Action Rule (FAR), QoS Enforcement Rule (QER), and Usage Reporting Rule (URR).

Only the Forward Action Rule (FAR) is mandatory. The QoS Enforcement Rule (QER) is only present for QoS Flows. The Usage Reporting Rule (URR) is only available when traffic volume measurements (e.g. for charging) are needed.

The Packet Detection Rule (PDR) is based on Service Data Flow Templates, which contain one or more Service Data Flow (SDF) Filters or an Application Identifiers. An Application Identifier refers to one or more Packet Flow Descriptions (PFDs).

A Service Data Flow (SDF) Filter contains for IP PDU Sessions a single IP Packet filter, i.e. any combination of

-	Source/destination IP address or IPv6 prefix.

-	Source / destination port number.

-	Protocol ID of the protocol above IP/Next header type.

-	Type of Service (TOS) (IPv4) or Traffic class (IPv6) and Mask.

-	Flow Label (IPv6).

-	Security parameter index.

-	Packet Filter direction.

A PFD includes a PFD ID; and one or more of the following:

-	3-tuple(s) including protocol, server-side IP address and port number.

-	the significant parts of the URL to be matched, e.g. host name.

-	a domain name matching criterion and information about applicable protocol(s).

The application detection filter can be configured in the SMF and the SMF then provides it in the Service Data Flow Template to the UPF. Or, the Service Data Flow Template for traffic handling in the UPF received from the dynamic PCC Rule.

Besides, the Management of Packet Flow Descriptions enables the UPF to perform accurate application detection when PFD(s) are provided by an Application Service Provider (ASP) and then to apply enforcement actions as instructed in a PCC Rule.

The operator is able to configure pre-defined PCC Rules in the SMF or dynamic PCC Rules in the PCF. A PCC rule include either a list of service data flow filters or an application identifier for service data flow detection. The PCC rule further includes charging control information, i.e. charging key and optionally a Sponsor identifier or an ASP identifier or both.

The application identifier references one or more PFDs, which are managed using the PFD Management API. Depending on the service level agreements between the operator and the Application Server Provider, it may be possible for the ASP to provide to the SMF individual PFDs or the full set of PFDs for each application identifier maintained by the ASP via the PFD Management service in the NEF (PFDF). The PFDs become part of the application detection filters in the SMF/UPF and are thereafter used as part of the logic to detect traffic generated by an application. The ASP may remove or modify some or all of the PFDs which have been provided previously for one or more application identifiers. The SMF may report the application stop to the PCF for an application instance identifier as defined in clause 5.8.2.8.4 of TS 23.501 [5] if the removed/modified PFD in SMF/UPF would result in an inability to detect traffic for that application instance.

The ASP manages (i.e. provisions, updates, deletes) the PFDs through the NEF (PFDF). The PFD(s) are transferred to the SMF through the NEF (PFDF). The PFDF is a logical functionality in the NEF which receives PFD(s) from the ASP through the NEF, stores the PFD(s) in the UDR and provides the PFD(s) to the SMF(s) either on the request from ASP PFD management through NEF (PFDF) (push mode) or on the request from SMF (pull mode). Finally, the PFDF functionality is a service provided by the NEF.

The ASP may provide/update/remove PFDs with an allowed delay to the NEF (PFDF). Upon reception of the request from the ASP, the NEF (PFDF) checks if the ASP is authorized to provide/update/remove those PFD(s) and request the allowed delay. The NEF (PFDF) may be configured with a minimum allowed delay based on SLA to authorize the allowed delay provided by the ASP. When both the requesting ASP and the requested allowed delay are successfully authorized, the NEF (PFDF) translates each external Application Identifier to the corresponding Application Identifier known in the core network. The NEF (PFDF) stores the PDF(s) into the UDR.

The Application Identifier is simply an index to a set of application detection rules configured in the UPF. It is an identifier that can be mapped to a specific application traffic detection rule.

The procedure is depicted Figure 5.3.1-2 below:

Figure 5.3.1-2:

The PFD (Packet Flow Description) is a set of information enabling the detection of application traffic.

Each PFD may be identified by a PFD ID. A PFD ID is unique in the scope of a particular Application Identifier. Conditions for when a PFD ID is included in the PFD are described in TS 29.551 [6]. There may be different PFD types associated with an Application Identifier.

A PFD includes a PFD ID; and one or more of the following:

-	3-tuple(s) including protocol, server-side IP address and port number.

-	the significant parts of the URL to be matched, e.g. host name.

-	a domain name matching criterion and information about applicable protocol(s).

### 5.3.2	Collaboration Scenarios

The 5GMSd Application Provider negotiates with the MNO an SLA to provide differentiated treatment, including network QoS and charging for its 5GMSd-Aware Application. The Application Provider provides the necessary information to the MNO to detect the traffic, to ensure its correct and exclusive identification. The MNO detects the traffic correctly and applies the agreed traffic treatment.

Due to privacy concerns, the content hosting is provided by the Application Provider in an external Data Network. However, the 5GMSd Application Provider leverages the network features either via a 5GMSd AF in the trusted Data Network (Figure 5.9.2-1) or via a 5GMSd AF in the external Data Network (Figure 5.9.2-2).

Figure 5.9.2-1: Collaboration 1 (Collaboration 3 of TS 26.501)

Figure 5.9.2-2: Collaboration 2 (Collaboration 4 of TS 26.501)

In order to use flow-based network features (such as different QoS classes or different charging policies), the 5G System needs to detect the relevant traffic.

### 5.3.3	Deployment Architectures

The following figure depicts a potential architecture design for the realization of traffic detection. The architecture shows the involved network functions in the traffic detection.

Figure 5.3.3-1: Relevant architecture components

### 5.3.4	Mapping to 5G Media Streaming and High-Level Call Flows

#### 5.3.4.1	General

Editor’s Note: Short introduction into the different Traffic Identification schemes.

#### 5.3.4.2	Usage of 5-tuples for Traffic Identification

The application detection filters in the UPF can be configured based on a pre-configured PCC rule (i.e. in the SMF and provided to the UPF) or a dynamic PCC rule (i.e. provided from the PCF). The 5GMS AF is able to provision, update and remove a dynamic PCC rule which contains Service Data Flow description parameters for traffic handling and traffic detection in the UPF. When using 5-tuples for traffic detection, the following fields of an IP Packet Filter Sets are used:

-	Source/destination IP address or IPv6 prefix.

-	Source/destination port number.

-	Protocol ID of the protocol above IP/Next header type.

-	Packet Filter direction.

NOTE:	These fields are encoded in the Flow Description field, defined in clause 5.3.8 of TS 29.514 [28].

As shown in figure 5.3.4.2-1 (below), the 5GMSd AF in the external DN can send a request using Nnef_AFsessionWithQos API to provision, update or remove a request to reserve resources for a specific application/flow with specific flow descriptions. After the AF request authorization, NEF interacts with the PCF, providing the flow description together with the QoS reference, the optional other parameters like Alternative Service Requirements, period of time or traffic volume, etc.

Figure 5.3.4.2-1:Flow description usage for traffic flow identification

If the request is authorised, the PCF determines the required QoS parameters based on the information provided by NEF/AF. After the Nnef_AFsessionWithQoS_Create Procedure, a transaction identifier is allocated by the NEF to identify this AF Session. Then the 5GMSd AF can subsequently invoke the Nnef_AFsessionWithQoS_Update API with this transaction identifier to update the flow description.

Alternatively, the 5GMSd AF in the trusted DN can directly send a request using Npcf_PolicyAuthorization API to provision, update and remove a request to reserve resources for a specific application/flow with specific flow descriptions.

Then the PCF initiates the PDU Session modification procedure to provide the updated PCC rule to the SMF and the SMF updates the PDRs in the UPF for the application/traffic identification and policy handling.

However, when a new TCP connection is opened and the old one is closed, the 5-tuple in the Flow Description needs to be changed. (This may be from the consequence of factors such as load balancing, multiple concurrent requests for different types of resources, use of a shared TCP connection pool, etc.) In such cases, the 5GMSd AF can invoke the NEF/PCF-related APIs with new flow description to update the PDRs installed in UPF to follow the changed transport layer 5-tuples for application/flow identification.

Editor’s Note: Whether a single or multiple modification procedures are needed depends on further check and study.

#### 5.3.4.3	Usage of ToS Traffic Class for Traffic Identification

The following is a simplified call flow when using the ToS Traffic Class for Traffic Identification, meaning, only the Type of Service field is used within a SDF Filter. The Type of Service (ToS) is an 8-bit field within the IP header (both IPv4 and IPv6) that can be used as DiffServ Code Point (DSCP) value [29] and for ECN marking [30]. It is assumed here that the QoS flow should be used (e.g. for Premium QoS) as described in TS 26.512, Annex A.

Figure 5.3.4.3-1: ToS usage within an application traffic detection rule (simplified)

Figure 5.3.4.3-1 depicts a call flow for ToS-based traffic detection. It is assumed here that the 5GMSd AF provides the ToS value for traffic identification in the Policy Activation response message (step 5). Another solution might be that the Media Session Handler allocates a ToS value and then provides the value to the 5GMSd AF.

The call flow works as the following steps:

1:	The Media Session Handler activates a Dynamic Policy and provides the Policy Template Id with the activation request (among other parameters).

The 5GMSd AF triggers the activation of a Dynamic PCC rule:

2:	The 5GMSd AF uses the Policy Authorization Service API and triggers a PCC rule activation. The 5GMSd AF provides the ToS value together with the IP address of the requesting UE and QoS parameters.

3:	As result, the PCF uses the Npcf_SMPolicyControl APIs to provide a new PCC rule to the SMF.

4:	The SMF uses the N4 interface to provide a new Packet Detection Rule (PDR) together with other rules for the UE to the UPF. Once the new rule is installed in the UPF, the UPF starts taking actions on the detected traffic.

5:	If the Dynamic Policy can be activated, the 5GMSd AF provides a value for the ToS field in return.

NOTE 1: The ToS Value is not immediately provided to the Media Session Handler to prevent race conditions.

6:	The Media Player prepares a new TCP connection and sets the ToS value nominated by the 5GMSd AF on the TCP socket using the setsockopt() API or equivalent. As a result, all TCP packets for the flow will be marked by the UE with the ToS value.

7:	The TCP Connection is established, and the traffic is marked with the ToS field. The UPF detects the traffic (by inspecting the IP header) and handles it according to the policy in the PCC Rule.

NOTE 2:	The PCC Rule is scoped by the PDU Session, so the treatment of the ToS field value by the UPF is limited to the requesting UE. The UPF first looks up the relevant PDRs for a PDU session based on the incoming GTP Tunnel Id.

The UPF also needs to detect the downlink traffic matching the uplink traffic. There are different solutions to achieve this:

A:	The 5GMSd AS uses the same ToS field for downlink traffic as used for uplink traffic.

NOTE 3:	This solution may not work for cases where traffic crosses operational domain boundaries, since the ToS header field is often reset by border IP routers.

B:	The UPF captures the 5-tuple carrying a specific ToS field from the TCP SYN Packet that establishes the connection in the uplink direction. As result, the UPF automatically creates a new PDR in the opposite direction derived by inverting the address fields found in the SYN packet.

NOTE 4:	The connection handshake of other transport protocols may be more difficult to detect.

C:	Often, the UEs in a PLMN are shielded from public Internet traffic by means of firewalls that employ Network Address Translation (NAT). In order to set the ToS field within the Trusted DN to an appropriate value, the N6-NAT may set the downlink ToS to the same value as the uplink ToS.

NOTE 5:	This is similar to solution A above.

#### 5.3.4.4	Usage of Packet Flow Descriptions for Traffic Identification

The following are potential and simplified call flows for the realization of the traffic identification.

In the first call flow (Figure 5.3.4.4-1) the provisioning step is described, in which one or more PFDs for a single application are provisioned. The provisioned PFDs for a single application are identified by the Application Identifier.

Figure 5.3.4.4-1: PFD Provisioning using the PFD Management API (simplified)

In the second call flow (Figure 5.3.4.4-2) the update procedure for the PFD to adjust to an actual session is described.

Figure 5.3.4.4-2: PFD usage within an application detection filter (simplified)

#### 5.3.4.3	Usage of ToS Traffic Class for Traffic Identification

The following is a simplified call flow when using the ToS Traffic Class for Traffic Identification, meaning, only the Type of Service field is used within a SDF Filter. The Type of Service (ToS) is an 8-bit field within the IP header (both IPv4 and IPv6) that can be used as DiffServ Code Point (DSCP) value [29] and for ECN marking [30]. It is assumed here that the QoS flow should be used (e.g. for Premium QoS) as described in TS 26.512, Annex A.

Figure 5.3.4.3-1: ToS usage within an application traffic detection rule (simplified)

Figure 5.3.4.3-1 depicts a call flow for ToS-based traffic detection. It is assumed here that the 5GMSd AF provides the ToS value for traffic identification in the Policy Activation response message (step 5). Another solution might be that the Media Session Handler allocates a ToS value and then provides the value to the 5GMSd AF.

The call flow works as the following steps:

1:	The Media Session Handler activates a Dynamic Policy and provides the Policy Template Id with the activation request (among other parameters).

The 5GMSd AF triggers the activation of a Dynamic PCC rule:

2:	The 5GMSd AF uses the Policy Authorization Service API and triggers a PCC rule activation. The 5GMSd AF provides the ToS value together with the IP address of the requesting UE and QoS parameters.

3:	As result, the PCF uses the Npcf_SMPolicyControl APIs to provide a new PCC rule to the SMF.

4:	The SMF uses the N4 interface to provide a new Packet Detection Rule (PDR) together with other rules for the UE to the UPF. Once the new rule is installed in the UPF, the UPF starts taking actions on the detected traffic.

5:	If the Dynamic Policy can be activated, the 5GMSd AF provides a value for the ToS field in return.

NOTE 1: The ToS Value is not immediately provided to the Media Session Handler to prevent race conditions.

6:	The Media Player prepares a new TCP connection and sets the ToS value nominated by the 5GMSd AF on the TCP socket using the setsockopt() API or equivalent. As a result, all TCP packets for the flow will be marked by the UE with the ToS value.

7:	The TCP Connection is established, and the traffic is marked with the ToS field. The UPF detects the traffic (by inspecting the IP header) and handles it according to the policy in the PCC Rule.

NOTE 2:	The PCC Rule is scoped by the PDU Session, so the treatment of the ToS field value by the UPF is limited to the requesting UE. The UPF first looks up the relevant PDRs for a PDU session based on the incoming GTP Tunnel Id.

The UPF also needs to detect the downlink traffic matching the uplink traffic. There are different solutions to achieve this:

A:	The 5GMSd AS uses the same ToS field for downlink traffic as used for uplink traffic.

NOTE 3:	This solution may not work for cases where traffic crosses operational domain boundaries, since the ToS header field is often reset by border IP routers.

B:	The UPF captures the 5-tuple carrying a specific ToS field from the TCP SYN Packet that establishes the connection in the uplink direction. As result, the UPF automatically creates a new PDR in the opposite direction derived by inverting the address fields found in the SYN packet.

NOTE 4:	The connection handshake of other transport protocols may be more difficult to detect.

C:	Often, the UEs in a PLMN are shielded from public Internet traffic by means of firewalls that employ Network Address Translation (NAT). In order to set the ToS field within the Trusted DN to an appropriate value, the N6-NAT may set the downlink ToS to the same value as the uplink ToS.

NOTE 5:	This is similar to solution A above.

#### 5.3.4.4	Usage of 5-tuples for Traffic Identification

Besides the PFD related traffic identification method which identifies the 3-tuple and/or the domain name, the packet detection filters required in the UPF can also be configured in the SMF and provided to the UPF, which can be used to detect a specific 5-tuples streaming within one specific application, e.g. subtitles, video, audio and bullet screen comments. The 5GMS AF is able to provision, update and remove a dynamic PCC rule which contains Service Data Flow description parameters for traffic handling and application/flow detection in the UPF. When using 5-tuples for application traffic detection, the following fields of an IP Packet Filter are used:

-	Source/destination IP address or IPv6 prefix.

-	Source/destination port number.

-	Protocol ID of the protocol above IP/Next header type.

-	Packet Filter direction.

NOTE:	These fields are encoded in the Flow Description field, defined in clause 5.3.8 of TS 29.514 [28].

As shown in figure 5.3.4.4-1 (below), the 5GMSd AF in the external DN can send a request using Nnef_AFsessionWithQos API to provision, update or remove a request to reserve resources for a specific application/flow with specific flow descriptions. After the AF request authorization, NEF interacts with the PCF, providing the flow description together with the QoS reference, the optional other parameters like Alternative Service Requirements, period of time or traffic volume, etc.

Figure 5.3.4.4-1:Flow description usage for traffic flow identification

If the request is authorised, the PCF determines the required QoS parameters based on the information provided by NEF/AF. After the Nnef_AFsessionWithQoS_Create Procedure, a transaction identifier is allocated by the NEF to identify this AF Session. Then the 5GMSd AF can subsequently invoke the Nnef_AFsessionWithQoS_Update API with this transaction identifier to update the flow description.

Alternatively, the 5GMSd AF in the trusted DN can directly send a request using Npcf_PolicyAuthorization API to provision, update and remove a request to reserve resources for a specific application/flow with specific flow descriptions.

Then the PCF initiates the PDU Session modification procedure to provide the updated PCC rule to the SMF and the SMF updates the PDRs in the UPF for the application/traffic identification and policy handling.

However, when a new TCP connection is opened and the old one is closed, the 5-tuple in the Flow Description needs to be changed. (This may be from the consequence of factors such as load balancing, multiple concurrent requests for different types of resources, use of a shared TCP connection pool, etc.) In such cases, the 5GMSd AF can invoke the NEF/PCF-related APIs with new flow description to update the PDRs installed in UPF to follow the changed transport layer 5-tuples for application/flow identification.

Editor’s Note: Whether a single or multiple modification procedures are needed depends on further check and study.

### 5.3.5	Potential open issues

The exact behaviour and information that needs to be provided to and by the 5GMSd AF as well as the MSH need to be specified.

The following open issues have been identified

The Npcf_PolicyAuthorization API as defined in TS 23.502 only supports usage of a flow description or an application identifier. A flow description represents only a 5-tuple. Other information elements of the Service Data Flow Filter are not supported.

The Nnef_ChargeableParty and Nnef_AFsessionWithQOS APIs only support usage of a flow description. Other information elements of the Service Data Flow Filter are not supported.

The Npcf_PolicyAuthorization API Stage 3 as defined in TS 29.514, only supports a flow description and a ToS value. However, it is not possible to define whether the ToS value should be used in uplink traffic detection or downlink traffic detection.

The Nnef_AFsessionWithQOS and Nnef_ChargeableParty APIs Stage 3, as defined in TS 29.522, only supports a flow description. Other information elements of the Service Data Flow Filter are not supported.

### 5.3.6	Candidate Solutions

Editor’s Note: Provide candidate solutions (including call flows) for each of the identified issues.

**** Last Change ****