Source:	Nokia Corporation

Title:	Updates to IVAS MASA C Reference Software

Document for:	Information

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

In [1], the source announced plans to develop a common software toolbox for generating IVAS encoder input signals according to the proposed metadata-assisted spatial audio (MASA) format. The IVAS MASA C Reference Software package was made available in the Busan meeting [2].

In 2020, the IVAS work item was mainly progressed through offline work to define a more effective way forward for the standardization. During various parallel discussions and practical experiments, the source has identified potential improvements to the IVAS MASA C Reference Software package.

This input document provides a corresponding update, as was previously planned and announced [3]. Specifically, the updates aim to improve the flexibility of audio capture for encoder input signal generation, as well as tune the analysis of coherence parameters.

The accompanying zip archive includes the updated software package.

# 2. Description of the updates

The updated MASA C reference is provided with this contribution as an accompanying zip archive. The package contains the following parts, as already known from [2]:

- Complete C code (according to C11 standard) for MASA Analyzer and MASA Renderer

- CMake project files to build for multiple platforms

- Binary files for various data needed by the software

- Instructions

- Matlab tools for helping on various tasks

- Pre-built release x64 versions of the software for Windows (Visual Studio 2019 ver. 16.9.4)

- Version of EVS release (CLDFB, FFT)

The updates to the MASA C reference relate to functional improvements, mainly in terms of more flexible audio capture, and algorithmic improvements. The updates are discussed in more detail in the following.

## 2.1 Audio capture updates

The original version of the MASA C reference [2], or more precisely the MASA Analyzer part of it, utilizes only raw Eigenmike signals. While this was shown to be a very useful method for generating MASA signals, being limited to this option only has two drawbacks. Firstly, the capture system based on Eigenmike EM32 can be considered rather costly, which can limit access and overall interest in experimentation. Secondly, since only either one or two of the raw microphone channel(s) from the Eigenmike are used as the MASA format transport signals, the signal-to-noise ratio can be limiting in some recording and testing scenarios. Thus, there is significant motivation to improve the flexibility of audio capture for the MASA C reference both in terms of affordability and signal quality. This is achieved with two new approaches.

The updated MASA Analyzer accepts the following alternative inputs:

- 32-channel raw Eigenmike signal in PCM format

  - Full range of mono and stereo MASA configurations according to [2] with internal transport signal generation

- 32-channel raw Eigenmike signal + 1 channel for mono transport in PCM format

  - Mono MASA with 1 or 2 analyzed directions

- 32-channel raw Eigenmike signal + 2 channels for stereo transport in PCM format

  - Stereo MASA with 1 or 2 analyzed directions

- FOA signal + 1 channel for mono transport in PCM format

  - Mono MASA with 1 analyzed direction

- FOA signal + 2 channels for stereo transport in PCM format

  - Stereo MASA with 1 analyzed direction

- HOA2 signal + 1 channel for mono transport in PCM format

  - Mono MASA with 1 or 2 analyzed directions

- HOA2 signal + 2 channels for stereo transport in PCM format

  - Stereo MASA with 1 or 2 analyzed directions

In other words, the MASA signal generation can now be based on Eigenmike only, Eigenmike + external microphone(s), or FOA/HOA2 + external microphone(s). For example, two large diaphragm condenser microphones in spaced (e.g., 17 cm) cardioid (pointing outwards) formation can be used to capture the stereo PCM transport stream, while corresponding Eigenmike or FOA/HOA signal is used to analyze and produce the spatial MASA metadata. We note that when external microphone(s) for transport signal capture are used, care should be taken in placing them relatively close to the spatial analysis microphone ensuring that they can capture good representation of the sound field. Furthermore, it is important to ensure that there is no significant difference in time sync between the spatial analysis and transport signals.

As before, MASA Analyzer uses as input raw PCM with signed 16-bit samples with a sampling rate of 48 kHz. Accompanied documentation provides details on how the channels are ordered in each configuration. In short, Ambisonics / Eigenmike channels come first and they are followed by 0, 1, or 2 transport channels. This results in 16-bit, signed, little endian, channel interleaved, PCM files with 5 to 34 channels.

## 2.1.1 Example of Eigenmike + external microphones

Figure 1 illustrates a first example configuration, where the raw Eigenmike signal is captured together with a separate stereo signal using a large-diaphragm stereo condenser pair. This example configuration allows for capturing high transport signal SNR in varied environments.

Figure 1. Eigenmike EM32 and Rode NT2-A microphones used for capturing IVAS MASA stereo signals

To further clarify the operation, example command line for this configuration would be:

masaAnalyzer -stereo -2dir –eigenext 34chn_eigenext.pcm stereo.pcm 2Dir.met

## 2.1.2 Example of Ambisonics + external cardioids

Figure 2 illustrates a second example configuration, where the Eigenmike is replaced by a First-order Ambisonics microphone, namely Rode NT-SF1.

Figure 2. Rode NT-SF1 and Rode NT2-A microphones used for capturing IVAS MASA stereo signals

When utilizing the Ambisonics (FOA/HOA2) and external microphone mode of the updated MASA C reference software, a more affordable microphone setup can be used to capture the signals for spatial analysis.

For the first order Ambisonics and external stereo microphones following example command line can be used:

masaAnalyzer -stereo -1dir –foaext 6hn_foaext.pcm stereo.pcm 1Dir.met

For the second order Ambisonics and external (mono) microphone, the following example command line can be used:

masaAnalyzer -mono -2dir –hoa2ext 10chn_hoa2ext.pcm mono.pcm 2Dir.met

As always, care is to be taken with Ambisonics signals, and the MASA Analyzer expects, e.g., a specific FOA representation. For correct usage, e.g., the A-Format signal originating directly from a microphone needs to be converted to an ACN order, SN3D normalized Ambisonics representation either using the software provided with the Ambisonics microphone or, e.g., other suitable external tools, such as Matlab, with proper coefficients for the microphone [4].

## 2.2 Algorithmic updates

The spatial analysis of the MASA C reference is based on the FOA/HOA2 input or a corresponding transformation from the raw Eigenmike signals. Therefore, in this context, Ambisonics signals are being used. It is well known that Ambisonics signals (created from Eigenmike or other microphone arrays) may be noisy at low frequencies. This is due to small phase differences between the microphones at low frequencies (where the wavelengths are large), and thus large equalization gains being used in the generation of the Ambisonics signals.

As a result, the spread coherence and the surround coherence analysis of this example software may also be noisy at the lowest frequencies. In order to mitigate the effect of the noise, the coherence analysis was modified to use an average value over all frequencies for the lowest frequency band (0-400 Hz). This allows for more reliable coherence analysis, especially since the software now supports arbitrary Ambisonics signals at the input in addition to raw Eigenmike signals.

# 3. Discussion

The functionality of the IVAS MASA C Reference Software [2] is extended by adding support for new audio capture configurations. This allows for both higher-quality audio capture (e.g., higher SNR) and use of more affordable audio capture setups for generating IVAS MASA encoder input signals. No previous functionality is removed, and details of the algorithm and past learnings based on experimentation with [2] thus remain valid. For details of the operation, users are thus advised to refer to [2]. Instructions for using the new audio capture configurations are included in current document and the accompanying documentation.

While certain algorithmic changes are present in the current iteration of the IVAS MASA C Reference Software, the current system remains interoperable with [2]. In other words, MASA input signals generated using software version of [2] can be synthesized with the latest software version and vice versa. Future updates, e.g., relating to decisions on IVAS-4 design constraints can affect interoperability, and therefore also deprecate past and present versions.

# References

[1] 	S4-190695: “Development for common IVAS MASA Reference Software”, Nokia Corporation

[2] 	S4-191167: “Description of the IVAS MASA C Reference Software”, Nokia Corporation

[3]	S4-210130: “Schedule for planned IVAS MASA C Reference Software updates”, Nokia Corporation

[4]	“Higher Order Ambisonics Library”, https://github.com/polarch/Higher-Order-Ambisonics. Retrieved May 12th, 2021.