3GPP TSG-SA4 Meeting #115-e	S4- 211047

# Online, August 19 - 28, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.998 | CR | - | rev | - | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Coordinating operation of Uu and Wirelss Connectivity in 5G WireLess Tethered AR (WLAR) UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Facebook, Qualcomm Incorporated, Samsung Electronics Co. Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GSTAR |  |  |  |  |  | Date: |  |  | 2021-08-18 |
| Category: |  |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Outline the need for coordinating operation of Uu and wireless connectivity for low-latency services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start change 1 ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TR 26.928: “Extended Reality (XR) in 5G”

[4.3.0]	ETSI GS ISG ARF 003 v1.1.1 (2020-03): “Augmented Reality Framework (ARF) AR framework architecture”

[4.3.a]	3GPP TR 26.928: “Extended Reality (XR) in 5G”

[4.3.b]	3GPP TS 22.261: “Service requirements for the 5G system”

[4.3.c]	3GPP TR 22.873: “Study on evolution of the IP Multimedia Subsystem (IMS) multimedia telephony service”

[4.3.d]	3GPP TS 26.114: “IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction”

[4.3.e]	3GPP RP-193241: “New SID on XR Evaluations for NR”

[4.3.f]	ISO/IEC 23090-2:2019: “Information technology — Coded representation of immersive media — Part 2: Omnidirectional media format”

[4.3.g]	ISO/IEC 23090-3:2020 FDIS: “Information technology — Coded representation of immersive media — Part 3: Versatile video coding”

[4.3.h]	ISO/IEC 23090-5:2020 FDIS: “Information technology — Coded representation of immersive media — Part 5: Visual Volumetric Video-based Coding (V3C) and Video-based Point Cloud Compression (V-PCC)”

[4.3.i]	ISO/IEC 23090-8:2020 FDIS: “Information technology — Coded representation of immersive media — Part 8: Network based media processing”

[4.5.a]	Khronos Group, The OpenXR Specification, 1.0, https://www.khronos.org/registry/OpenXR/specs/1.0/html/xrspec.html

[4.5.b]	W3C, WebXR Device API, W3C Working Group Draft, https://www.w3.org/TR/webxr/

[4.5.c]	ISO/IEC 23090-14:2021 DIS: “Information technology — Coded representation of immersive media — Part 14: Scene Description for MPEG-I Media”

[4.5.d]	Khronos Group, The GL Transmission Format (glTF) 2.0 Specification, https://github.com/KhronosGroup/glTF/tree/master/specification/2.0/

[A.1]	Google Draco: https://google.github.io/draco/

[A.2]	T.Ebner, O.Schreer, I. Feldmann, P.Kauff, T.v.Unger, “m42921 HHI Point cloud dataset of boxing trainer”, MPEG 123rd meeting, Ljubljana, Slovenia

[A.3]	Scene understanding, https://docs.microsoft.com/en-us/windows/mixed-reality/scene-understanding

[A.4]	Serhan Gül, Dimitri Podborski, Jangwoo Son, Gurdeep Singh Bhullar, Thomas Buchholz, Thomas Schierl, Cornelius Hellge, “Cloud Rendering-based Volumetric Video Streaming System for Mixed Reality Services”, Proceedings of the 11th ACM Multimedia Systems Conference (MMSys'20), June 2020

[A.5]	Scene lighting: https://docs.microsoft.com/en-us/azure/remote-rendering/overview/features/lights

[A.6]	PBR material: https://docs.microsoft.com/en-us/azure/remote-rendering/overview/features/pbr-materials

[A.7]	Color Material: https://docs.microsoft.com/en-us/azure/remote-rendering/overview/features/color-materials

[A.8]	S. N. B. Gunkel, H. M. Stokking, M. J. Prins, N. van der Stap, F.B.T. Haar, and O.A. Niamut, 2018, June. Virtual Reality Conferencing: Multi-user immersive VR experiences on the web. In Proceedings of the 9th ACM Multimedia Systems Conference (pp. 498-501). ACM.

[A.9]	Dijkstra-Soudarissanane, Sylvie, et al. "Multi-sensor capture and network processing for virtual reality conferencing." Proceedings of the 10th ACM Multimedia Systems Conference. 2019.

[A.10]	VRTogether, a media project funded by the European Commission as part of the H2020 program, https://vrtogether.eu/, November 2020.

[A.11]	MPEG131 Press Release: Point Cloud Compression – WG11 (MPEG) promotes a Video-based Point Cloud Compression Technology to the FDIS stage: https://multimediacommunication.blogspot.com/2020/07/mpeg131-press-release-point-cloud.html

[xx]	Wireless Broadband Alliance, “5G and Wi-Fi RAN Convergence”, April 2021.

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

*** End change 1 ***

*** Start change 2 ***

#### 4.2.2.4	Type 3: 5G WireLess Tethered AR UE

Figure 4.2.2.4-1 provides a functional structure for Type 3: 5G WireLess Tethered AR UE.

Figure 4.2.2.4-1: Functional structure for Type-3: 5G WireLess Tethered AR UE

Main characteristics of Type 3: 5G WireLess Tethered AR UE:

-	5G connectivity is provided through a tethered device which embeds the 5G modem. Wireless tethered connectivity is through WiFi or 5G sidelink. BLE (Bluetooth Low Energy) connectivity may be used for audio.

-	The AR Runtime is local and uses from sensors, audio inputs or video inputs, but may be assisted by functionalities on phone.

-	While media processing (for 2D media) can be done on the AR glasses, heavy AR/MR media processing may be done on the AR/MR tethered device or split.

-	Some devices might have limited support for immersive media decoding and rendering and may need to rely on 5G cloud/edge

-	While such devices are likely to use significantly less processing than Type 1: 5G STAR devices by making use of the processing capabilities of the tethered device, they can still support a lot of local media and AR/MR processing. Such devices are expected to provide 8-10h of battery life while keeping a significantly low weight.

-	The tethered glass itself is not a regular 5G UE, but the combination of the glass and the phone results in a regular 5G UE.

-	Media Access functions are provided that support the delivery of media content components over the 5G system. Examples of the media access functions are 5GMS functions, MTSI functions, web-connectivity or edge-related client functions. Detailed requirements are for study in this report.

For services with latency limitations, such as MTSI or FLUS, it can be necessary to take the status of wireless connectivity into account when configuring the services, such that the link between AR glass and 5G phone is not overly loaded. How to coordinate the operation of Uu and wireless connectivity in such services is FFS [xx].

*** End change 2 ***