3GPP TSG-SA4#115e Meeting 	S4- 211256

18th August – 27th August 2021

Source: 	Tencent

Title: 	Draft CR updates on Multiple grouping of 360 video

Document for	Agreement

Agenda item: 	12.5

# Introduction:

In SA4#114, Multiple grouping of 360 videos (S4-210932) was accepted to be included in the PD. This document proposes moving the text for the draft CR. The below updates are proposed to be added in section Y.6.8.1 of the draft CR.

First Change

Y.6.8.1 General

[content from 1126]

Y.6.8.2 Excluding other participants’ overlays

When an ITT4RT-Tx client in terminal sends a 360-degree video media stream to the MRF, it may include an attribute "a= no_other_overlays", which indicates that the MRF shall not group the 360-degree media stream from that ITT4RT-Tx client with overlay media streams from other ITT4RT clients. In this case, the MRF shall group the 360-degree video media stream and one or more overlays of that ITT4RT-Tx client in a separate <rest-group> in the itt4rt_group attribute when describing them to any ITT4RT-Rx client.

The ABNF syntax for this attribute is the following:

no_other_overlays	= "a= no_other_overlays"

NOTE: If multiple itt4rt_group are created, an ITT4RT-Rx client in terminal would need to re-negotiate the session to switch to media streams from other itt4rt_group. However, doing so may add further burden on the signaling nodes.

End of Changes

# Proposal

The proposal is to move the agreed changes to the draft CR.