3GPP TSG SA WG4#115-e meeting	S4-211190

18th– 27th August 2021                   revision of S4-211031 and merged with S4-211047

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.998 | CR | pseudo | rev |  | Current version: | 0.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5GSTAR] Improved user experience for WLAR UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Facebook, Samsung Electronics Co. Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GSTAR |  |  |  |  |  | Date: |  |  | 11/08/2021 |
| Category: | B |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TR 26.928: “Extended Reality (XR) in 5G”

[4.2.4a]	Wireless Broadband Alliance, “5G and Wi-Fi RAN Convergence”, April 2021.

[4.3.0]	ETSI GS ISG ARF 003 v1.1.1 (2020-03): “Augmented Reality Framework (ARF) AR framework architecture”

[4.3.a]	3GPP TR 26.928: “Extended Reality (XR) in 5G”

[4.3.b]	3GPP TS 22.261: “Service requirements for the 5G system”

[4.3.c]	3GPP TR 22.873: “Study on evolution of the IP Multimedia Subsystem (IMS) multimedia telephony service”

[4.3.d]	3GPP TS 26.114: “IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction”

[4.3.e]	3GPP RP-193241: “New SID on XR Evaluations for NR”

[4.3.f]	ISO/IEC 23090-2:2019: “Information technology — Coded representation of immersive media — Part 2: Omnidirectional media format”

[4.3.g]	ISO/IEC 23090-3:2020 FDIS: “Information technology — Coded representation of immersive media — Part 3: Versatile video coding”

[4.3.h]	ISO/IEC 23090-5:2020 FDIS: “Information technology — Coded representation of immersive media — Part 5: Visual Volumetric Video-based Coding (V3C) and Video-based Point Cloud Compression (V-PCC)”

[4.3.i]	ISO/IEC 23090-8:2020 FDIS: “Information technology — Coded representation of immersive media — Part 8: Network based media processing”

[4.5.a]	Khronos Group, The OpenXR Specification, 1.0, https://www.khronos.org/registry/OpenXR/specs/1.0/html/xrspec.html

[4.5.b]	W3C, WebXR Device API, W3C Working Group Draft, https://www.w3.org/TR/webxr/

[4.5.c]	ISO/IEC 23090-14:2021 DIS: “Information technology — Coded representation of immersive media — Part 14: Scene Description for MPEG-I Media”

[4.5.d]	Khronos Group, The GL Transmission Format (glTF) 2.0 Specification, https://github.com/KhronosGroup/glTF/tree/master/specification/2.0/

[A.1]	Google Draco: https://google.github.io/draco/

[A.2]	T.Ebner, O.Schreer, I. Feldmann, P.Kauff, T.v.Unger, “m42921 HHI Point cloud dataset of boxing trainer”, MPEG 123rd meeting, Ljubljana, Slovenia

[A.3]	Scene understanding, https://docs.microsoft.com/en-us/windows/mixed-reality/scene-understanding

[A.4]	Serhan Gül, Dimitri Podborski, Jangwoo Son, Gurdeep Singh Bhullar, Thomas Buchholz, Thomas Schierl, Cornelius Hellge, “Cloud Rendering-based Volumetric Video Streaming System for Mixed Reality Services”, Proceedings of the 11th ACM Multimedia Systems Conference (MMSys'20), June 2020

[A.5]	Scene lighting: https://docs.microsoft.com/en-us/azure/remote-rendering/overview/features/lights

[A.6]	PBR material: https://docs.microsoft.com/en-us/azure/remote-rendering/overview/features/pbr-materials

[A.7]	Color Material: https://docs.microsoft.com/en-us/azure/remote-rendering/overview/features/color-materials

[A.8]	S. N. B. Gunkel, H. M. Stokking, M. J. Prins, N. van der Stap, F.B.T. Haar, and O.A. Niamut, 2018, June. Virtual Reality Conferencing: Multi-user immersive VR experiences on the web. In Proceedings of the 9th ACM Multimedia Systems Conference (pp. 498-501). ACM.

[A.9]	Dijkstra-Soudarissanane, Sylvie, et al. "Multi-sensor capture and network processing for virtual reality conferencing." Proceedings of the 10th ACM Multimedia Systems Conference. 2019.

[A.10]	VRTogether, a media project funded by the European Commission as part of the H2020 program, https://vrtogether.eu/, November 2020.

[A.11]	MPEG131 Press Release: Point Cloud Compression – WG11 (MPEG) promotes a Video-based Point Cloud Compression Technology to the FDIS stage: https://multimediacommunication.blogspot.com/2020/07/mpeg131-press-release-point-cloud.html

=====  CHANGE  =====

#### 4.2.2.1	Overview

In TR 26.928, different AR and VR device types had been introduced in clause 4.8. This clause provides an update and refinement in particular for AR devices. The focus in this clause mostly on functional components and not on physical implementation of the glass/HMD. Also, in this context the device is viewed as a UE, i.e. which functions are included in the UE.

A summary of the different device types is provided in Table 4.2.2.1-1. The table also covers:

-	how the devices are connected to get access to information,

-	where the 5G Uu modem is expected to be placed,

-	where the AR Runtime (as specified in 4.2.1) is placed,

-	where the Scene Manager (as specified in 4.2.1) is placed,

-	where the AR/MR application is running,

-	where the power supply/battery is placed.

For all glass device types, it is assumed that sensors, cameras and microphones are on the device.

The definition for Split AR/MR in Table 4.2.2.1-1 is as follows:

-	Split: the tethered device (phone/puck) or external entity (cloud/edge) does some pre-processing (e.g, a pre-rendering of the viewport based on sensor and pose information), and the AR/MR device and/or tethered device performs a rendering considering the latest sensor information (e.g. applying pose correction). Different degrees of split exist, between different devices and entities. Similarly, vision engine functionalities and other AR/MR functions (such as AR/MR media reconstruction, encoding and decoding) can be subject to split computation.

Table 4.2.2.1-1: 5G Augmented Reality device types

| Device Type Name | Reference | Tethering | 5G Uu Modem | AR Runtime | Scene Manager | AR/MR Application | Power Supply |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5G Standalone AR UE | 1: STAR | N/A | Glass | Glass | Glass/Split 1) | Glass | Glass |
| 5G EDGe-Dependent AR UE | 2: EDGAR | N/A | Glass | Glass/Split 1) | Split 1) | Split | Glass |
| 5G WireLess Tethered AR UE | 3: WLAR | 802.11ad, 5G sidelink, etc. | Tethered device (phone/puck) | Glass | Split 2) | Tethered device | Glass |
| 5G Wired Tethered AR UE 3) | 4: WTAR | USB-C | Tethered device | Tethered device | Split 2) | Tethered device | Tethered device |
| 1) Cloud/Edge 2) Phone/Puck and/or Cloud/Edge 3) Not considered in this document |  |  |  |  |  |  |  |  |

The Wired Tethered AR UE device type is for reference purposes only and not considered in this document as it is not included as part of the study item objectives.

Generally, the STAR and WLAR device according to Table 4.2.2.1-1 are expected to have similar functionalities from a 5G System perspective.

Based on this, the focus is on three main different device types in the remainder of this document following the rows 1 to 3 in Table 4.2.2.1-1.

=====  CHANGE  =====

#### 4.2.2.4	Type 3: 5G WireLess Tethered AR UE

This clause introduces the 5G WireLess Tethered AR UE. Two sub-types are differentiated:

- Split Rendering WLAR UE. In this case the 5G phone that includes the modem also acts to support rendering of complex scenes and provides the pre-rendered data to the glass

- Relay WLAR UE: In this case, the 5G phone acts as a relay to provide IP connectivity.

Figure 4.2.2.4-1 provides a functional structure for Type 3a: 5G Split Rendering WireLess Tethered AR UE.

Figure 4.2.2.4-1: Functional structure for Type-3a: 5G Split Rendering WireLess Tethered AR UE

Main characteristics of Type 3: 5G WireLess Tethered AR UE:

-	5G connectivity is provided through a tethered device which embeds the 5G modem. Wireless tethered connectivity is through WiFi or 5G sidelink. BLE (Bluetooth Low Energy) connectivity may be used for audio. The motion-to-render-to-photon loop runs from the glass to the phone. While the connectivity is outside of the 5G Uu domain, it is still expected that for proper performance when used for split rendering, a stable and constant delay link can be setup on the tethered connection.

-	The AR Runtime is local and uses from sensors, audio inputs or video inputs, but may be assisted by functionalities on phone.

-	While media processing (for 2D media) can be done on the AR glasses, heavy AR/MR media processing may be done on the AR/MR tethered device or split.

-	Some devices might have limited support for immersive media decoding and rendering and may need to rely on 5G cloud/edge.

-	While such devices are likely to use significantly less processing than Type 1: 5G STAR devices by making use of the processing capabilities of the tethered device, they can still support a lot of local media and AR/MR processing. Such devices are expected to provide 8-10h of battery life while keeping a significantly low weight.

-	The tethered glass itself is not a regular 5G UE, but the combination of the glass and the phone results in a regular 5G UE.

-	Media Access functions are provided that support the delivery of media content components over the 5G system. Examples of the media access functions are 5GMS functions, MTSI functions, web-connectivity or edge-related client functions. Detailed requirements are for study in this report.

Figure 4.2.2.4-2 provides a functional structure for Type 3b: 5G Relay WireLess Tethered AR UE.

Figure 4.2.2.4-2: Functional structure for Type-3b: 5G Relay WireLess Tethered AR UE

Main characteristics of Type 3: 5G Relay WireLess Tethered AR UE:

-	5G connectivity is provided through a tethered device which embeds the 5G modem. Wireless tethered connectivity is through WiFi or 5G sidelink. BLE (Bluetooth Low Energy) connectivity may be used for audio.

-	The 5G Phone acts as a relay to forward IP packets. The 5G Phone runs a Media Session Handler including EDGE functionalities to support QoS control on the 5G System. To support proper end-to-end QoS, the media session handling needs to take into account the constraints of the tethering link to provide sufficient QoS on the 5G System link to provide adequate QoE for the end user. Details on the exact function of the relay, for example of it is on IP layer (layer 3) or on lower layer is for further study.

-	Media Access functions are provided on the glass device to support the delivery of media content components over the 5G and wireless tethered link.

-	The motion-to-render-to-photon loop runs from the glass to the edge and hence includes in total 4 wireless links. It is expected that for proper performance when used for split rendering, a stable and constant delay end to end link needs to be setup.

-	The AR Runtime is local and uses from sensors, audio inputs or video inputs, but may be assisted by functionalities on phone.

-	Media Processing is either done on the glass device or it is split with the network. In particular relevant is that many devices have limited support for immersive media decoding and rendering and may need to rely on 5G cloud/edge

-	While such devices are likely to use significantly less processing than Type 1: 5G STAR devices by making use of the processing capabilities of the tethered device, they can still support a lot of local media and AR/MR processing. Such devices are expected to provide 8-10h of battery life while keeping a significantly low weight.

-	The tethered glass itself is not a regular 5G UE, but the combination of the glass and the phone results in a regular 5G UE.

For services with low latency requirements, such as MTSI or those provided by FLUS, it may be necessary to take the status of wireless connectivity into account when configuring the services, such that the link between AR glass and 5G phone is not overly loaded. How to coordinate the operation of Uu and wireless connectivity in such services is FFS [xx].