3GPP TSG SA WG4#116e	S4-211346

E-meeting, 10th – 19th November 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.502 | CR | pseudo | rev |  | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5MBUSA] Re-use of MBMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBUSA |  |  |  |  |  | Date: |  |  | 03/11/2021 |
| Category: | B |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

## 6.1	Object Delivery Method

This section deals with

b)	Object delivery Method that includes:

-	Download delivery method, File Delivery as defined in TS 26.346, clause 7.

-	DASH/HLS over MBMS as defined in TS 26.346, clause 5.6 and 5.7, including Low-Latency CMAF as defined in 5GMS.

For the object delivery method, it is proposed to differentiate two different cases.

- Non-real-time file delivery including Carouselling

  - Selected properties of this mode include

    - Scheduled delivery

    - File repair

    - Carousel (for example supporting functionalities defined in DSM-CC)

    - Post-delivery reporting

    - File delivery QoS

    - Usage of FEC for file delivery

    - Support of single large file distribution

  - On stage-3 it is expected that we use FLUTE as defined in TS 26.346 with the following questions and comments:

    - Upgrade to the latest version of ALC, FLUTE and LCT? (stage 3 decision, what is the value? do we need a legacy mode?)

    - Profile/remove any non-used functionalities based on MBMS Download Profile in TS 26.346, Annex L.4 (stage 3 decision, legacy?)

- Object Streaming addressing DASH/HLS

  - Selected properties of this mode include

    - Timed delivery

      - Object deadline that is relevant for proper application operation.

    - Concurrent metrics reporting

    - Usage of FEC for object delivery

    - Sequence of multiple objects

    - Possibly multiple flows

    - Limited size

    - Partial objects

  - Excluded at least in Rel-17 are

    - Unicast such as fast startup, service continuity, unicast repair? At this stage no, it is deferred to the application (e.g. as part of 5GMS). This holds for Rel-17, may be revisited later.

  - On stage 3, it is expected that enhancements are needed beyond the existing FLUTE.

    - Resolve and address object timing model (stage-3).

    - Address real-time and low-latency streaming, e.g. ROUTE or FLUTE extensions, but stage-3 discussion?

It is agreed that:

- It is beneficial to create two distinct object delivery methods: object carouselling and object streaming.

- Does not imply that we need two different protocols.

- We should clearly define two different call flows for the different delivery modes. The may be common procedures, but also distinct ones.

Develop at least one call flow for each of the above.

- For object carouselling:

  - Scheduled delivery of one or multiple files.

  - Carousel.

- For object streaming:

  - Regular-latency object streaming.

  - (Low-latency streaming if any substantial differences would be observed and there is sufficient time).

=====  CHANGE  =====

## 6.2	[Packet/Transparent] Delivery Method

Editor’s Note: The name of this delivery method is pending to further discussion, for example also transparent mode was suggested. The discussion was around whether to describe the MBS delivery function or the service that is supported to the outside by the delivery.

This section deals with

c)	A common packet delivery method that includes the relevant delivery aspects of transparent delivery method, group communication delivery method and streaming delivery method as defined in TS 26.346, clause 8B, 8A and 8 respectively.

For the packet delivery method, it is proposed to only support the Transparent Delivery Method as defined in clause 8B, both the proxy and the forward-only mode. This includes RTP-based delivery as a special case.

The following functions are expected to be included:

- Packet sequencing.

- FEC.

- QoS, bit rates.

- Multiple flows?

- Specific protocol support such as RTP/AVP.

The following functions are expected not to be included:

- Metrics – this is considered to be an application function.

- Unicast is not supported.

- Codecs will not be addressed. It is an assumption that an SDP is available with RTP/AVP.

At least the following call flows are expected to created:

- Packet delivery in proxy-mode.

- Packet delivery in forward-only mode.