3GPP TSG SA WG4#116e	S4-211364

E-meeting, 10th – 19th November 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR |  | rev |  | Current version: | 1.3.6 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5G_Video] Metrics and Characterization Updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 04/11/2021 |
| Category: |  |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

### 5.5.8	Reference computation of HDR metrics

Computation of wPSNR(Y), wPSNR(U) and wPSNR(V) metrics as defined in clause 5.5.5 is performed with HDRMetrics tool version 0.23 [67] with with reference config file HDRMetrics_wtPSNR.cfg as attached and

where hdrTable.txt is attached to this report.

Computation of DeltaE100 and PSNRL100 metrics requires conversion from YUV to linear light RGB data which is performed with the use of HDRConvert tool [67] and reference config file HDRConvertYCbCr420ToEXR2020.cfg as attached.

After the conversion for reference and decoded video clips is done, computation of DeltaE100 and PSNRL100 metric as defined in clause 5.5.5 is performed with HDRMetrics tool version 0.23 [67] with reference config file HDRMetrics_deltaE100.cfg as attached.

=====  CHANGE  =====

## 5.7	Characterization

Characterization is the comparison of a codec under test with an anchor based on the framework introduced in this clause. Characterization in this report is based on Bjöntegard-Delta (BD)-rate information according to [44].

Characterization is expected to provide a summary of the expected gains a codec under test would provide, compared to a reference codec. For characterization, the metric results in this Technical Report are used to derive summary numbers. It is important for a codec to understand the performance for individual scenarios, for individual configurations within a scenario, but also for individual reference sequences. At the same time, a summary comparison is beneficial to provide an overview of the overall performance. A summary based on averages of selected sequences as an example can only provide indication of the performance if the reference sequences would be fully representative. However, it is also of interest to understand maximum and minimum gains.

Figure 5.7-1: Characterization Framework

Based on this, a full characterization of a codec for a scenario against a 3GPP codec is expected to provide at least the following metrics

-   The BD-Rate Gain (for a given metric and a given anchor codec) for each reference sequence in the test configuration.

-   The minimum, maximum and average BD-Rate Gain (for a given metric and a given anchor codec) across all reference sequences in the test configuration.

Note that this results typically in the following results for a codec under test. For each scenario and for each configuration, a table is provided to compare against each anchor codec

-    where the row header documents the reference sequence for the test, e.g. S5-R<i> to the specific configuration.

-    where the column header documents the key of the metric

-    that documents in the cell with BD-Rate Gain

-    the above is extended with three summary rows, where the column header documents average, minimum and maximum gain

An example is provided in Table 5.7-1.

Table 5.7-1 BD-Rate Gain example table for an example scenario SX with 4 reference sequences, a given codec under test, a given anchor codec and given test configuration

| Reference sequence | y_psnr | psnr | ms-ssim | vmaf |
| --- | --- | --- | --- | --- |
| SX-R01 |  |  |  |  |
| SX-R02 |  |  |  |  |
| SX-R03 |  |  |  |  |
| SX-R04 |  |  |  |  |
| Average |  |  |  |  |
| Minimum |  |  |  |  |
| maximum |  |  |  |  |

BD-Rate Gain is computed according to the Common Test Condition (CTC) method used in JVET and specified in [44]. According to the IETF draft on "Video Codec Testing and Quality Measurement" [70], clause 4.2, the BD-Rate Gain is defined as a measure of the bitrate reduction offered by a codec or codec feature, while maintaining the same quality as measured by objective metrics.  The rate change is computed as the average percent difference in rate over a range of qualities.  Metric score ranges are not static - they are calculated either from a range of bitrates of the reference codec, or from quantizers of a third, anchor codec. Given an anchor codec and test codec, the BD-Rate Gain values are calculated as follows:

- Rate/quality points are calculated for the anchor and test code

-	At least four points need to be computed.  These points should be the same quantizers when comparing two versions of the same codec.

-	Additional points outside of the range should be discarded.

- The rates are converted into logarithmic scale.

- A piecewise cubic hermite interpolating polynomial is fit to the points for each codec to produce functions of log-rate in terms of quality measure. Interpolation functions require that two quality values are different.  Hence, if provided data does have identical quality values, the higher rate point is adjusted to a slightly higher value that is withing the rounding error of the identical value, for example if twice 23.24 is reported, value 1 is set to 23.240 and the second value is set to 23.241.

- Metric score ranges are chosen by the minimum value of the codec under test and the maximum value of the anchor codec.

- The log-rate is numerically integrated over the metric range for each curve, using at least 1000 samples and trapezoidal integration.

- The resulting integrated log-rates are converted back into linear rate, and then the percent difference is calculated from the anchor to the test codec. This difference is reported as the BD-Rate Gain.

An example is provided in Figure 5.7-2.

Figure 5.7-2: BD-Rate Gain computation example

The JVET Excel files for the CTC include a VBS script bdrate() to compute the BD-Rate performance between a test codec and a reference from four or five rate-distortion points. However, to address different corner cases as well as to ensure applicability to the reported csv metrics files, the above algorithm is converted into a script python to generate the BD-Rate Gain for a codec under test vs. an anchor. For details on the script compare.py available here https://github.com/haudiobe/5GVideo, please refer to Annex F.

For providing the BD-Rate Gain values for a single reference sequence and configuration, please use

Usage: python3 compare.py -s <key anchor> <key test>

Example: python3 compare.py -s S1-T01-264 S1-T01-265

For providing the BD-Rate Gain values for all reference sequences of one configuration, please use

Usage: python3 compare.py -c <key anchor configuration> <key test configuration>

Example: python3 compare.py -c S1-JM-01 S1-HM-01

It is assumed that the directory structure is configured as in the attachments and the compare function is placed in the root directory where all scenarios are provided. In case all information for one configuration is provided, the data is written into a csv file in the directory Characterization of the codec under test with the naming convention

<anchor configuration>.<test configuration>.csv

Example: S1-JM-01.S1-HM-01.csv

NOTE: For consistency, the JVET excel files have been extended in the Random-Access and low delay tabs to contain new columns for the new metrics: VMAF and MS-SSIM, in the SDR case only. The “SA4 extended excel files” for SDR and HDR are attached as S4-template-HDR.xlsx and S4-template-SDR.xlsx, but are not considered to be used for BD-Rate computation.

=====  CHANGE  =====

### 6.2.5	Performance Metrics

All metrics as defined in clause 5.5 are expected to be reported.

For BD-Rate computation and SDR, only the following metrics are expected to be provided:

-	Peak-Signal to Noise Ratio PSNR(Y) of luma component as defined in clause 5.5.3,

-	Average colour component PSNR, PSNR over all colour components PSNR as defined in clause 5.5.3,

-	Structural similarity metric MS-SSIM asdefined in clause 5.5.4.2,

-	Video Multimethod Assessment Fusion (VMAF) VMAF as defined in clause 5.5.3.

For BD-Rate computation and HDR, only the following metrics are expected to be provided:

-	Peak-Signal to Noise Ratio PSNR(Y) of luma component as specified in [44] as specified in clause 5.5.4 and 5.5.8,

-	Average colour component PSNR, PSNR over all colour components PSNR, as specified in clause 5.5.4 and 5.5.8,

-	the weighted Peak-Signal To Noise Ratio metric for the luminance colour components wPSNR(Y), as specified in clause 5.5.4 and 5.5.8,

-	the PSNRL100 metric as specified in clause 5.5.4 and 5.5.8,

-	DE100 metric as specified in clause 5.5.4 and 5.5.8.

=====  CHANGE  =====

### 6.3.5	Performance Metrics

Performance is assessed using BD-Rate computation, with PSNR, SSIM and VMAF metrics as objective quality criterion. Regarding complexity considerations, encoding/decoding runtime is provided.

All metrics as defined in clause 5.5 are expected to be reported.

For BD-Rate computation and SDR, only the following metrics are expected to be provided:

-	Peak-Signal to Noise Ratio PSNR(Y) of luma component as defined in clause 5.5.3,

-	Average colour component PSNR, PSNR over all colour components PSNR as defined in clause 5.5.3,

-	Structural similarity metric MS-SSIM asdefined in clause 5.5.4.2,

-	Video Multimethod Assessment Fusion (VMAF) VMAF as defined in clause 5.5.3.

For BD-Rate computation and HDR, only the following metrics are expected to be provided:

-	Peak-Signal to Noise Ratio PSNR(Y) of luma component as specified in [44] as specified in clause 5.5.4 and 5.5.8,

-	Average colour component PSNR, PSNR over all colour components PSNR, as specified in clause 5.5.4 and 5.5.8,

-	the weighted Peak-Signal To Noise Ratio metric for the luminance colour components wPSNR(Y), as specified in clause 5.5.4 and 5.5.8,

-	the PSNRL100 metric as specified in clause 5.5.4 and 5.5.8,

-	DE100 metric as specified in clause 5.5.4 and 5.5.8.