3GPP TSG-SA4 Meeting #116e	S4-211379

# Electronic meeting, Telco, November 10-19, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.532 | CR |  | rev |  | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ndcaf_DataReportingProvisioning Service API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVEX |  |  |  |  |  | Date: |  |  | 2021-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Absence of text in TS 26.532 on the data collection and reporting provisioning API exposed by the Data Collection AF to the ASP at the R1 reference point. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Renaming of clause 6.2 title to “Provisioning Sessions API” and proposed text for a portion of this clause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Absence of necessary stage 3 text in TS 26.532 on data collection and reporting functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2 and its sub-clauses |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------- START OF  CHANGE -----------

# 6	Ndcaf_DataReportingProvisioning service

## 6.1	General

This clause specifies the API used by the ASP (its Provisioning AF) to provision data collection and reporting in the Data Collection AF.

## 6.2	Provisioning Sessions API

### 6.2.1	Overview

This clause specifies the provisioning API used by an Application Service Provider server to provision a data collection and reporting configuration in a Data Collection AF.

The Provisioning Sessions API is used by the ASP to instantiate and manipulate Provisioning Sessions in the Data Collection AF, as described in clause 4.2.3.2. Having created a Provisioning Session, the ASP can then go on to provision data collection and reporting configuration instances in the context of that Provisioning Session, using the APIs specified in clause 6.2.3 et seq.

### 6.2.2	Resource structure

The Provisioning Sessions API is accessible through the following URL base path:

{apiRoot}/3gpp-r1/v1/provisioning-sessions/

Table 6.2.2.2-1 specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the sub-resource path specified in the second column of the table shall be appended to the above URL base path.

Table 6.2.2.2-1: Operations supported by the Provisioning Sessions API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Provisioning Session |  | POST | Used to create a new Provisioning Session resource. If the operation succeeds, the URL of the created Provisioning Session resource shall be returned in the Location header of the response. |
| Retrieve Provisioning Session | {provisioningSessionId} | GET | Used to retrieve a Provisioning Session resource for inspection. |
| Destroy Provisioning Session |  | DELETE | Used to destroy an existing Provisioning Session resource. |

### 6.2.3	Data model

The data model for the ProvisioningSession resource is specified in Table 6.2.3-1 below. Different properties are present in the resource depending on the type of Provisioning Session indicated in the provisioningSessionType property, and this is specified in the Applicability column.

Table 6.2.3-1: Definition of ProvisioningSession resource

| Property name | Type | Cardinality | Usage | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| provisioningSessionId | ResourceId | 1..1 | C: R R: RO | A unique identifier for this Provisioning Session. | All types. |
| provisioningSession‌Type | Provisioning‌Session‌Type | 1..1 | C: RW R: RO U: – | The type of Provisioning Session. | All types. |
| aspId | AspId | 0..1 | C: W R: RO | The identity of the Application Service Provider responsible for this Provisioning Session, as specified in clause 5.6.2.3 of TS 29.514 [34]. Absence of the aspId property implies that the identity of the ASP associated with the Provisioning Session is well known to the Data Collection AF. | All types. |
| appIds | Array(Applica-tionId) | 0..1 | C: W R: RO | The External Application Identifier associated with the data reported by Data Collection Client to Data Collection AF. Absence of the appIds property implies that the application(s) associated with the Provisioning Session is(are) well known to the Data Collection AF. | All types. |
| eventIds | Array(EventId) | 0..1 | C: W R: RW | Identification of the UE data event type(s) associated with this Provisioning Session, as specified by clause 4.15.1 of TS 23.502 [3]. | All types. |
| serviceExperience ConfigurationIds | Array(ResourceId) | 0..1 | C: W R: RW | A list of Service Experience Configuration identifiers associated with this Provisioning Session. Each instance of this property represents a different collection of service flow level QoE types and/or QoE metrics observed at one or more UEs as specified by clause 6.4.2 of TS 23.288 [4] on Service Experience information. Note that according to [4], the per-service flow QoE types associated with the Service Experience Configuration identifier may correspond to voice MOS or video MOS as specified in ITU-T P.1203.3 [X], as well as a customized MOS for any kind of service including those not related to video or voice). Also, according to [4], the QoE metrics observed at UE(s) may pertain to MTSI [X], MBMS [Y], VR [Z], 5GMS [XX] or ASP-specific QoE metrics. | All types. |

| performanceData ConfigurationIds | Array(ResourceId) | 0..1 | C: W R: RW | A list of Performance Data Configuration identifiers associated with this Provisioning Session. Each instance of this property represents a different collection of performance-related data types associated with a communication session as specified by clauses 6.4.2 of and 6.14.2 of TS 23.288 [4] on Performance Data information. | All types. |
| --- | --- | --- | --- | --- | --- |
| collectiveBehavior ConfigurationIds | Array(ResourceId) | 0..1 | C: W R: RW | A list of UE Collective Behavior Configuration identifiers associated with this Provisioning Session. Each instance of this property represents a different set of mobility-related behaviors for a collective of UEs within a defined area of interest, as specified by clause 6.5.2 of TS 23.288 [4] on UE Collective Behavior information. | All types. |
| uEMobility ConfigurationIds | Array(ResourceId) | 0..1 | C: W R: RW | A list of individual UE Mobility Configuration identifiers associated with this Provisioning Session. Each instance of this property represents the same set of mobility-related parameters for a unique UE within a defined area of interest, as specified by clause 6.7.2.2 of TS 23.288 [4] on UE Mobility information. | All types. |
| uECommunication ConfigurationIds | Array(ResourceId) | 0..1 | C: W R: RW | A list of individual or group-based UE Communication Configuration identifiers associated with this Provisioning Session. Each instance of this property represents a different set of expected UE behavior and/or UE communication parameters for a given Application Identifier as specified by clause 6.7.2.2 of TS 23.288 [4] and clause 4.15.6.3 of TS 23.502 [3]. | All types. |

### 6.2.4	Mediation by NEF

------------- END OF  CHANGE -----------