3GPP TSG-SA4 Meeting #116-e	S4-211483

# Online, 10th – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.443 | CR | 0041 | rev | - | Current version: | 12.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to EVS Floating-Point Source Code |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fraunhofer IIS, Nokia Corporation, VoiceAge Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVS_codec-SA4 |  |  |  |  |  | Date: |  |  | 2021-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Correct various issues as listed below: - Regression in AVQ decoding which was introduced with the previous CR and which could cause glitches in the decoder audio output: - Correct wrong condition in encoder-preprocessing to make sure LP analysis at 16kHz if ACELP@16k core was selected and buffers are updated - Dependency on <float.h> system header, unnecessary usage of double precision operation |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct issues listed above, as described here: - re8_decode_base_index(): Correct AVQ sign handling. This affects the bit-exactness of the testsequences at various operation points. - pre_proc(): Correct erroneous condition to trigger LP analysis at 16kHz if ACELP@16k core was selected and buffers are updated. This doesn’t affect the bit-exactness of the testsequences. - IGF_CalculateEnvelope(): Remove dependency on <float.h> system header for FLT_EPSILON and replace this by EPSILON as used elsewhere; replace log() by logf(), as value is anyway converted from double to float afterwards. This doesn’t affect the bit-exactness of the testsequences. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Severe issues for the deployed codec, incl. reading of unintialized memory or out-of-bound, leading to possibly crashes and glitches in the audio signal. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Electronic attachment to 26.443, specifically: c-code/EVS_dec.exe c-code/EVS_cod.exe c-code/lib_com/disclaimer.c c-code/lib_dec/lead_deindexing.c c-code/lib_enc/igf_enc.c c-code/lib_enc/pre_proc.c |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 26.442 CR0044 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 26.444 CR0041 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start code change 1

diff -rwNBu 26443-g20/c-code/lib_com/disclaimer.c 26443-g30/c-code/lib_com/disclaimer.c

--- 26443-g20/c-code/lib_com/disclaimer.c	2021-08-02 16:56:06.000000000 +0200

+++ 26443-g30/c-code/lib_com/disclaimer.c	2021-11-02 16:21:21.920604000 +0100

@@ -10,8 +10,8 @@

{

fprintf(fPtr, "\n===========================================================================\n");

-    fprintf(fPtr,   " EVS Codec (Floating Point) 3GPP TS26.443 August 12, 2021.\n");

-    fprintf(fPtr,   " Version 12.13.0 / 13.9.0 / 14.5.0 / 15.3.0 / 16.2.0\n");

+    fprintf(fPtr,   " EVS Codec (Floating Point) 3GPP TS26.443 November 04, 2021.\n");

+    fprintf(fPtr,   " Version 12.14.0 / 13.10.0 / 14.6.0 / 15.4.0 / 16.3.0\n");

fprintf(fPtr,   "===========================================================================\n\n\n");

return 0;

diff -rwNBu 26443-g20/c-code/lib_dec/lead_deindexing.c 26443-g30/c-code/lib_dec/lead_deindexing.c

--- 26443-g20/c-code/lib_dec/lead_deindexing.c	2021-08-02 16:56:11.000000000 +0200

+++ 26443-g30/c-code/lib_dec/lead_deindexing.c	2021-11-02 16:21:22.447568000 +0100

@@ -151,7 +151,13 @@

*--------------------------------------------------------------------*/

m1 = k1-1;

m2 = 8;

-        if (k1 == 7)

+        l = 1;

+        for ( i = 0; i < 8; i++ )

+        {

+            l *= x[i];

+        }

+        /* if the signs are constrained and all components are non-zero  */

+        if ( ( k1 == 7 ) && ( l > 0 ) )

{

m2 = 7;

}

diff -rwNBu 26443-g20/c-code/lib_enc/igf_enc.c 26443-g30/c-code/lib_enc/igf_enc.c

--- 26443-g20/c-code/lib_enc/igf_enc.c	2021-08-02 16:56:18.000000000 +0200

+++ 26443-g30/c-code/lib_enc/igf_enc.c	2021-11-02 16:21:22.080585000 +0100

@@ -6,7 +6,6 @@

#include <stdlib.h>

#include <memory.h>

#include <math.h>

-#include <float.h>

#include <assert.h>

#include "options.h"

#include "prot.h"

@@ -82,9 +81,9 @@

width           = swb_offset[sfb + 1] - swb_offset[sfb];

-            sfbEnergyTileR  = FLT_EPSILON;

-            sfbEnergyTileC  = FLT_EPSILON;

-            sfbEnergyC      = FLT_EPSILON;

+            sfbEnergyTileR  = EPSILON;

+            sfbEnergyTileC  = EPSILON;

+            sfbEnergyC      = EPSILON;

if (pPowerSpectrum)

{

@@ -105,12 +104,12 @@

}

else

{

-                sfbEnergyR  = FLT_EPSILON + sum2_f(pMDCTSpectrum + swb_offset[ sfb ], width) / width;

+                sfbEnergyR  = EPSILON + sum2_f(pMDCTSpectrum + swb_offset[ sfb ], width) / width;

gain        = (float)(sfbEnergyR);

}

-            gain = 0.5f + (float) ((2.885390081777927f * log(gain) + 16.f));

+            gain = 0.5f + (float) ((2.885390081777927f * logf(gain) + 16.f));

gain = min(gain, 91.f);  /* 13+15+63, see arithocde encode residual */

gain = max(gain,  0.f);

diff -rwNBu 26443-g20/c-code/lib_enc/pre_proc.c 26443-g30/c-code/lib_enc/pre_proc.c

--- 26443-g20/c-code/lib_enc/pre_proc.c	2021-08-02 16:56:17.000000000 +0200

+++ 26443-g30/c-code/lib_enc/pre_proc.c	2021-11-02 16:21:22.215585000 +0100

@@ -769,7 +769,7 @@

* update buffers

*-----------------------------------------------------------------*/

-    if( ( ((st->tcxonly == 0) || !(st->core_brate != FRAME_NO_DATA || st->core_brate != SID_2k40)) && st->L_frame == L_FRAME16k && st->codec_mode == MODE2 ) ||

+    if( ( ((st->tcxonly == 0) || !(st->core_brate != FRAME_NO_DATA && st->core_brate != SID_2k40)) && st->L_frame == L_FRAME16k && st->codec_mode == MODE2 ) ||

( st->L_frame == L_FRAME16k && st->codec_mode == MODE1 ) )

{

/* update signal buffers */

End code change 1