3GPP TSG- S4 Meeting #116e	S4-211518

# Electronic Meeting, 10th November – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.502 | CR | <CR#> | Rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5MBUSA] Clause 4: MBS User Service functional entity definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMS_EXT |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | The intention of this document is to start introducing MBUSA functional entities, focusing on MBSTF and the reference point between MBSTF and MBSF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# 4	Reference architecture for 5G Multicast–Broadcast User Services

## 4.1	General

This clause defines a reference architecture for 5G Multicast–Broadcast User Services, including the logical functions involved and the logical reference points between them.

## 4.2	System description

Editor’s Note: Explanation of fundamental concepts in the MBS User Services architecture.

### 4.2.1	Network architecture

Editor’s Note: How this specification relates to the SA2 architecture in TS 23.247.

### 4.2.2	User Service architecture

Editor’s Note: Introduction to MBS User Services and how they are manifested in the MBSF and MBSTF.

### 4.2.3	Delivery methods

Editor’s Note: Explanation of what a delivery method is and what delivery methods are for.

### 4.2.4	Service Announcement / Service Discovery

The Service Announcement / Service Discovery provides information, which are needed by the MBS Client for activating the reception of the MBS User Service. Service Announcement information may be delivered via MBS Sessions or via the regular PDU Sessions.

Editor’s Note: The concept of ancillary information need to be described and defined in context of Service Announcement,

## 4.3	Functional entities

Editor’s Note: Reference architecture for MBS User Services, including client functions.

### 4.3.1	General

The MBSF and MBSTF offer service layer functionality for sending data via MBS Sessions. The MBSTF acts as a media anchor and sources the IP multicast traffic. The MBSF offers control plane functionality while the MBSTF offers user plane functionality. A new interface Nmb2 is introduced between the control and user plane functions.

### 4.3.2	MBSF

The functionality of the MBSF is defined in clause 5.3.2.11 of TS 23.247 [5]. It receives provisioning and control commands either directly at reference point Nmb10 or (via the NEF) at reference point Nmb5. If the MBSTF is deployed, it is controlled by the MBSF. The MBSF invokes MBS Session operations on the MB-SMF at reference point Nmb1.

The present document further elaborates the control plane functionality to support MBS User Services as follows:

-	Generating the Service Announcement for the MBS Session.

-	Controlling the Service Announcement version and configure the carousel delivery if the Service Announcement is delivered by the MBSTF (SACH).

[-	Monitoring the status of ancillary information and configure the ancillary information delivery in the same MBS Session with MBS Data if ancillary information is changed and the MBSTF is used.]

Editor’s Note: Usage of QoS is FFS

### 4.3.3	MBSTF

#### 4.3.3.1	General

The functionality of the MBSF is defined in clause 5.3.2.12 of TS 23.247 [5]. It receives User Plane traffic at reference point Nmb8 and sends MBS data traffic via reference point Nmb9.

NOTE:	The MBSTF may not be present in all deployments of the MBS System.

The present document further elaborates the user plane functionality to support MBS User Services as follows:

-	Multicast delivery of input objects for the Object distribution method.

-	Multicast delivery of input packet as packets sequencing for the Packet distribution method.

-	Multiplexing of ancillary information into the MBS Session.

-	Carousel delivery control for Object distribution method.

-	Sending notification events to the MBSF, i.e. cannot receive the input data, session terminated, delivery started.

#### 4.3.3.2	MBSTF subfunctions to support Object distribution

Details about the MBSTF subfunctions for supporting Object Distribution is given in Figure 4.3.3.2-1.

Figure 4.3.3.2-1 MBSTF architecture overview for Object distribution method

The Object ingest subfunction supports:

-	Pull based ingest over Nmb8: The Object Ingest function fetches one or more objects using HTTPS from an AF/AS.

-	Push based ingest over Nmb8: The Object Distribution Ingestion function receives one or more objects using HTTPS from an AF/AS.

The Object segmentation subfunction supports the partitioning of an object into suitable payload units for MBS transmission.

The optional Application Level FEC subfunction supports calculation of FEC redundancy.

The Packetization subfunction creates (based input) transmission packets

The Packet Scheduler subfunction supports:

-	Encapsulation and transmission of Object distribution PDUs into Nmb9 SDUs.

-	Schedules the outgoing data stream according to target bit rate configuration.

The control subfunction offers support for MBSTF service configuration and service notifications.

#### 4.3.3.3	MBSTF subfunctions to support Packet distribution

Details about the MBSTF subfunctions for supporting Packet Distribution is given in Figure 4.3.3.3-1.

Figure 4.3.3.3-1: MBSTF architecture overview for Packet distribution method

The Packet ingest subfunction supports the reception of a packet sequence from authorized sources.

The optional Application Level FEC subfunction supports calculation of FEC redundancy.

The Packetization subfunction creates (based input) transmission packets. Ingested packets may get reformatted suitable for MBS transmission.

The Packet Scheduler subfunction supports

-	Encapsulation and transmission of Packet Distribution PDUs into Nmb9 SDUs.

-	Schedules the outgoing data stream according to target bit rate configuration.

The control subfunction offers support for MBSTF service configuration and service notifications.

### 4.3.4	MBS AS

The MBS AS performs the following functions to support MBS User Services:

-	Providing the byte-range file repair service for the Object distribution method.

### 4.3.5	MBS Client

The MBS Client is part of the UE. The functionality of the UE is defined in clause 5.3.2.8 of TS 23.247 [5].

The MBS Client performs the following functions to support MBS:

-	Reception of IP multicast data using either MBS Multicast or MBS Broadcast.

-	MBS services exposure towards a 5MBS-aware Application.

-	Unicast recovery of the application payload data carried in multicast/broadcast packets that are not successfully received via MBS-4, if unicast repair is configured.

-	Using AL-FEC repair to the received packets if FEC is used,

Editor’s Note: Handling the roaming is FFS.

Editor’s Note: The MBMS Reception Reporting Service is FFS. In principle, the Reception Reporting is used by the Network Operators to analyse the packet loss rate (Block Error Rates - BLER), and the main target is to adjust the FEC redundancy level to leverage the FEC redundancy level and radio frequency usage efficiency.

## 4.4	Reference points and interfaces

Editor’s Note: Description of the reference points.

## 4.4	Domain model

Editor’s Note: The static domain model for services and sessions.

## 4.5	Life-cycle model

Editor’s Note: State charts explaining the dynamics of MBS User Services.

## 4.6	QoS model

Editor’s Note: How MBS User Services make use of the network Quality of Service primitives defined by SA2 is TS 23.247.

## 4.7	Security

Editor’s Node: How MBS User Services makes use of the security primitives studied by SA3 in TR 33.850.

**** Last Change ****