Source:	Tencent Cloud

Title:	[FS_5GSTAR] Updates to 6.5.5. Instantiation #2: DCMTSI-based architecture extension with immersive media processing

Agenda Item:	10.9

Document for:	Discussion and Agreement

- Introduction

This contribution provides two minor improvements to the system diagrams and call flows for AR conversational services. Namely, it

- adds a scene graph generator to STAR conservational services architecture, and

- adds the pose and viewport information of the terminating device in call flow as an input to IMS AGW/MRF since the composed and rendered AR scene should be based on this information.

- Improvements

### 6.5.3	Basic architecture

To describe the functional architecture for AR conversational use-cases such as clause Annex A.4 and identify the content delivery protocols and performance indicators an end-to-end architecture is addressed. The end-to-end workflow for AR conferencing (one direction) is shown in Figure 6.5.3-1.

Figure 6.5.3-1: One direction conversational service for STAR UEs.

Camera(s) are capturing the participant(s) in an AR conferencing scenario. The camera(s) for each participant are connected to a UE (e.g. laptop or mobile phone or AR glasses) via a data network (wired/wireless). Live camera feeds, sensors and audio signals are provided to a UE which processes, encodes, and transmits immersive media content to the 5G system for distribution.  In multi-party AR conversational services, it may exist an immersive media processing function on the cloud/network that receives the uplink streams from various devices and creates a composite scene description defining the arrangement of individual participants in a single virtual conference room.   The composite scene description as well as the encoded media streams are delivered to each receiving participant. A receiving participant’s 5G STAR UE receives, decodes and processes the 3D video and audio streams, and render them using the received scene description and the information receives from its AR Runtime creating an AR scene of the virtual conference room with all other participants.

Note that in the case of two-party AR conferencing, the immersive media processing function on Cloud may not exist and the UE devices directly talk to each other through 5G Networks. In this case, each receiver locally creates the AR scene of the virtual conference room which includes the other participant.

Also note that if the format conversion is desired, the immersive media processing function on the cloud may optionally use media services such as pre-processing of the captured 3D video, format conversion, and any other processing before compression of immersive media content including 3D representation, such as in form of meshes or point clouds, of participants in an AR conferencing scenario.

Finally, while the sender’s functionality shown in Figure 6.5.3-1 is identical in the STAR and EDGAR devices, if an EDGAR device is used to receive the AR conversational services, it uses the split-rendering function on Cloud/Edge. In this case, the 5G STAR UE (receiver) of Figure 6.5.3-1, is replaced with the 5G EDGAR UE (receiver) and the cloud/edge functionality shown in Figure 6.5.3-2.

Figure 6.5.3-2: Replacement of the STAR UE device with EDGAR and cloud/edge prerendering.

### 6.5.4 Various 5G System instantiations

There are different options for mapping to 5G system:

a)	The MTSI architecture (TS 26.114 [15]) supports audio and 2D video conversational services. Extending the MTSI architecture to support AR signalling and immersive media. This includes both MTSI/RTP and MTSI/Data channel (DC) stack options.

b)	Extending the 5GMS architecture (TS 26.501 [26]) to support AR conversational services by combining live uplink and live downlink. 5GMS offers basic functionality such as QoS support, reporting, and in the future also edge, which will be beneficial for all types of applications. The typical/expected QoS parameters (especially delay) need to be clarified.

c)	An architecture based on something different than MTSI / IMS or 5GMS, for example, browser implementations such as WebRTC. WebRTC is widely deployed today for conversational services and is built on a flexible ecosystem on the device side, which is important in this case since conversational AR will require significant device-side changes.

Table 6.5.3-1: Comparison of different architecture options for supporting AR conversational services

| Component | MTSI: RTP and DC (TS 26.114) | 5GMS/HTTP (26.501) | Other architecture (e.g. WebRTC-based) |
| --- | --- | --- | --- |
| Protocol | SIP- and RTP-based or DC-based. SDP signalling and formats for AR are missing and need to be defined. Encoding and decoding at the MTSI client need to be extended beyond ITT4RT with improved support of AR immersive media formats (e.g. meshes, point clouds). | TCP- and HTTP-based streaming, using DASH/HLS and MPEG OMAF/CMAF technology. AR & immersive media content and signalling are assumed to work with HTTP-based streaming in the other use-case mappings. | For example, using non-IMS WebRTC data channel and/or extending WebRTC audio/video for AR media such as immersive media communications. AR signalling aspects to be studied. |
| Connection establishment | Find and connect is solved through SIP and E.164 addressing in IMS. | Find-and-connect for the conversational, UE-to-UE, case is undefined. | WebRTC implementations offer dedicated APIs for connection establishment in various contexts such as social media platforms. Browser applications are widely available. |
| Performance | Technically possible, latency should in principle not be a problem to achieve, building on the existing QoS and policy framework in 5GC. | Low latency and QoS DASH support in 5GMS to be studied. | WebRTC is designed with low latency in mind but has no defined relation to QoS and policy framework in 5GC and use of that need to be studied. |
| Deployments | Cross-operator interconnect aspects are included. Edge processing functions to be studied. | Cross-operator interconnect aspects are currently ignored. Edge processing functions to be studied (e.g. EMSA). | Cross-operator interconnect aspects are currently not applicable since WebRTC is used OTT today, but will become relevant and need study, especially if used with QoS. Edge processing functions to be studied. |
| Legal Intercept (only in scope of SA3, not SA4) | LI framework exist. Possible extensions to cover new AR media formats to be studied in SA3. | Not in scope since it is not a telephony service. | Not in scope if only OTT. |

NOTE:	There is no support of WebRTC media stack in 3GPP today, except for the WebRTC data channel stack in MTSI. WebRTC access to IMS was studied in TR 23.701 [41] and TR 23.706, [42] and OTT WebRTC client access to 3GPP core network through a gateway is specified in TS 24.371 [43].

### 6.5. 4 Generic Call flow

Figure 6.5.5-2 illustrates the call flow for an immersive AR conversational for a receiving EDGAR UE. Only one sender is shown in this diagram without showing its detailed call flow.

Figure 6.2.4.1-1: AR Conversational service call flow for a receiving EDGAR UE

Procedures:

- The EDGAR Application requests to start a session through EDGE.

- The EDGE negotiates with the Scene Composite Generator (SCG) and the sender UE to establish the session.

- The EDGE acknowledge the session establishment to the EDGAR UE.

Media pipeline configuration:

- The EDGE and EDGAR UE Media pipelines are separately configured.

The conversational stage starts here:

- The EDGAR Application requests the start of the session.

AR data are provided to the EDGE in a parallel loop with the virtual conference loop, in stages 6-8:

- The AR Runtime sends the AR data to the Application.

- The Application processes the data and send it to the Media Client.

- The Media Client streams up the AR data to the Edge.

The virtual conference loop:

- The sender UE streams up its encoded media to the Scene Graph Compositor (SGC).

- Parallel to 9, the sender UE streams up its AR data to the SGC.

- Using all streamed media and AR data from various participants, the SCG creates the composted scene.

- The composted scene and media components are delivered to the EDGE.

- The EDGE parses the scene description and media components, partially render the scene, and create a simple scene description as well as the media component.

- The simplified scene is delivered to the Media Client and Scene Manager.

Media session loop:

- The EDGAR AR Runtime captures the media and processes them.

- The AR Runtime sends the media data to the Media Client.

In parallel to 7:

- The Media Client encoded the received media data.

- The Media Client streams up the encoded media.

- The simplified scene description and pre-rendered media components are streamed to the Media Client.

- The Media Client decodes the media streams

- The Scene Manager parses the basic scene description and composes the scene.

- The AR manager after correcting the pose, renders the immersive scene including the registration of AR content into the real world.

### 6.5.5	Instantiation #2: DCMTSI-based architecture extension with immersive media processing

### Change this section to a call flow for  without being SIP specific and IMS specific.

Compared with the instantiation for MTSI-based architecture extension, this instantiation emphasises that the IMS-AGW/MRF may support immersive media processing. It is necessity for 5G EDGAR UEs with poor media capabilities. Figure 6.5.5-1 provides an DCMTSI-based architecture of AR conversational services for EDGAR UE. A 5G EDGAR UE integrated with DCMTSI client in terminal is denoted as an EDGAR-DCMTSI client. An EDGAR-DCMTSI client may request an AR application (i.e., an entry point) via a bootstrap data channel from the data channel server. An EDGAR-DCMTSI client may also generate or retrieve some AR specific data (e.g., pose and viewpoint information) which is transmitted via additional data channels, given that non-media data is handled by using SCTP as specified in IETF RFC 8831 [44]. When an EDGAR-DCMTSI client initiates an AR call with another one, the IMS-AGW/MRF with a support of immersive media processing ( shown in the Cloud/Edge box in Figure 6.5.3-2) may perform pre-rendering with the media stream originated from the parties of this AR session if they receive the corresponding AR-specific data (i.e. the pose and viewpoint information).

EDGAR-DCMTSI clients negotiate the properties such as reliable or unreliable message transmission, in-order or out-of-order message delivery and an optional protocol for data channel using SDP as defined in IETF RFC 8864 [45]. Based on the user plane protocol stack for a basic MTSI client defined in clause 4.2 of TS 26.114 [15] and the clause 6.5 of IETF RFC 8827 [46], all data channels (e.g., both an AR application via bootstrap data channels and AR-specific data via additional data channels) are secured via DTLS.

Figure 6.5.5-1: DCMTSI-based conversational service architecture for EDGAR UE

Furthermore, the IMS-AGW/MRF with a support of immersive media processing are also desirable to 5G STAR UEs due to saving power consumption. Note that the IMS-AGW/MRF with a support of immersive media processing may perform pre-rendering and scene generation based on the request of the STAR UEs carried in these additional data channels. Particularly, the logical function of immersive media processing may be integrated in the MRF or other media functions.

Figure 6.5.5-2 illustrates the procedure diagram for an immersive AR conversational with two party, each using EDGAR UEs including an EDGAR-DCMTSI client in the context of the IMS-AGW/MRF with a support of immersive media processing. The procedure is also applicable to establish an immersive AR call where the two parties of a session are STAR UEs or one is STAR UE and the other is EDGAR UE.

Figure 6.5.5-2: AR-DCMTSI client to AR-DCMTSI client call establishment for EDGAR UE

Figure 6.5.5-2: AR-DCMTSI client to AR-DCMTSI client call establishment for EDGAR UE

Assumptions:

-	AR immersive media may be sent over RTP/UDP/IP and/or SCTP/UDP/IP.

-	AR immersive media may be negotiated and configured using SDP.

-	A data channel application may provide rich user experiences by utilizing both user’s underlying scene and pose of objects representing users in the scene.

- Each  party has its own MRF capable of immersive media processing..

Procedures:

1.	A EDGAR UE initiates a SIP INVITE request, containing the SDP offer with AR media capabilities.

2.	The call propagates to the terminating EDGAR UE.

3.	The terminating EDGAR UE returns an SDP answer in a SIP 183 progress message. The P-CSCF uses the SDP answer to allocate the required resources.

4.	The originating EDGAR UE generate a PRACK which is transited to the terminating side of the call.

5.	The originating EDGAR UE receives an associated 200 OK (PRACK).

6.	The terminating EDGAR UE reserves internal resources to reflect the SDP answer and configures media pipelines.

7.	The originating EDGAR UE sends a SIP UPDATE message with a new SDP offer confirming the selected media parameters.

8.	The 200 OK (UPDATE) response is received for the terminating STAR UE containing the SDP answer.

9.	The terminating EDGAR UE is now alerted and sends a SIP 180 Ringing response.

10.	When the terminating EDGAR UE has answered the call, it sends a 200 OK to the originating EDGAR UE.

11.	The terminating EDGAR UE receives the 200 OK, and sends a SIP ACK message to acknowledge that the call has been established.

12.	The EDGAR UEs retrieve a data channel application for AR through the bootstrap data channel. If the EDGAR UE enables to provide native AR application, this step is not required.

13.	Any additional data channels created and used by the data channel application for AR itself are requested.

14.	The originating EDGAR UE initiates SIP re-INVITE request, containing an updated SDP offer to establish those additional data channels.

15.	The AS/S-CSCF identify an updated SDP offer for additional data channels and modify the "c=" as the IP address of the MRF, and then send this SDP offer to the remote party.

16.	The AS/S-CSCF send this updated SDP offer to the remote party.

17.	The AS/S-CSCF receive an updated SDP answer from the remote party.

18.	The AS/S-CSCF identify this updated SDP answer for additional data channels and modify the "c=" as the IP address of the MRF, and then send this SDP answer to the remote party.

19.	The additional data channels for the data channel application has been established.

20.	The EDGAR UE processes the immersive media to be transmitted.

a.	The AR runtime function captures and processes the immersive media to be sent.

b.	The AR runtime function passes the immersive media data to the AR-DCMTSI client.

c.	The AR-DCMTSI client encodes the immersive media to be transmitted to the IMS-AGW/MRF supporting immersive media processing.

NOTE 1: The capturing process may be done by an external camera. In this case, the processing and encoding processes are done outside EDGAR UE (e.g., AR-DCMTSI client)

21.	The data channel application for AR collects the AR-specific data, and decide to send them to the AR-DCMTSI client if the AR experiences requires assistance from the network side.

22.	The AR-DCMTSI client sends the AR-specific data (e.g., virtual objects info) to the IMS-AGW/MRF via the designated data channel 1 based on the previous SDP negotiation.

23.	The IMS-AGW/MRF composes, renders and encodes the AR immersive media based on the received media stream, the AR-specific data from the originating party.

24.	The data channel application for AR collects the AR-specific data, and decides to send them to the AR-DCMTSI client if the AR experiences requires assistance from the network side.

25.	The AR-DCMTSI client sends the AR-specific data (e.g. pose info and/or viewport info) to the IMS-AGW/MRF via the designated data channel 2 based on the previous SDP negotiation.

26.	The IMS-AGW/MRF decodes and pre-renders media stream based on the received scene graphs and media stream) from the terminating party and the AR-specific data from the originating party. The composed AR immersive media scene may include a simple scene description. Then the IMS-AGW/MRF sends the simplified scene graph and media streams to the originating party.

27.	The EDGAR UE processes the received immersive media.

a.	The AR-DCMTSI client decodes and process the received immersive media.

b.	The AR-DCMTSI client passes the immersive media data to the Scene Manager.

The Scene Manager renders the immersive media, which includes the registration of the AR content into the real world accordingly.