3GPP TSG SA WG4#116e	S4-211565

E-meeting, 10th – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.998 | CR |  | rev |  | Current version: | 1.0.3 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5GSTAR] On Spatial Computing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Qualcomm, Tencent, b<>com, Samsung, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GSTAR |  |  |  |  |  | Date: |  |  | 2021-11-02 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 2	References

[X] 	Younes, Georges, et al. "Keyframe-based monocular SLAM: design, survey, and future directions." Robotics and Autonomous Systems 98 (2017): 67-88. https://arxiv.org/abs/1607.00470

[XX] 	David G. Lowe, "Distinctive Image Features from Scale-Invariant Keypoints". https://www.cs.ubc.ca/~lowe/papers/ijcv04.pdf

[XXX] 	Herber Bay et. al., " SURF: Speeded Up Robust Features”. https://people.ee.ethz.ch/~surf/eccv06.pdf

[XXXX] 	E. Rublee, V. Rabaud, K. Konolige and G. Bradski, "ORB: An efficient alternative to SIFT or SURF," 2011 International Conference on Computer Vision, 2011, pp. 2564-2571, doi: 10.1109/ICCV.2011.6126544.

=====  CHANGE  =====

## 3.1	Definitions

Spatial Computing: AR functions which process sensor data to generate information about the world 3D space surrounding the AR user.

XR Spatial Description: a data structure describing the spatial organisation of the real world using anchors, trackables, camera parameters and visual features.

XR spatial compute server: is an edge or cloud server that provides spatial computing AR functions.

XR Spatial Description server: is a cloud server for storing, updating and retrieving XR Spatial Description

=====  CHANGE  =====

### 4.2.1	Device Functions

AR glasses contain various functions that are used to support a variety of different AR services as highlight by the different use cases in clause 5. AR devices share some common functionalities in order to create AR/XR experiences. Figure 4.2.1-1 provides a basic overview of the relevant functions of an AR device.

The primary defined functions are

-	AR/MR Application: a software application that integrates audio-visual content into the user’s real-world environment.

-	AR Runtime: a set of functions that interface with a platform to perform commonly required operations such as accessing controller/peripheral state, getting current and/or predicted tracking positions, general spatial computing and submitting rendered frames to the display processing unit.

-	Media Access Function: A set of functions that enables access to media and other AR related data that is needed in the scene manager or AR Runtime in order to provide an AR experience. In the context of this report, the Media Access function typically uses 5G system functionalities to access media.

-	Peripheries: The collection of sensors, cameras, displays and other functionalities on the device that provide a physical connection to the environment.

-	Scene Manager: a set of functions that supports the application in arranging the logical and spatial representation of a multisensorial scene based on support from the AR Runtime.

Figure 4.2.1-1: 5G AR Device Functions

The various functions that are essential for enabling AR glass-related services within an AR device functional structure include:

a)	Tracking and sensing (assigned to the AR Runtime)

-	Inside-out tracking for 6DoF user position

-	Eye Tracking

-	Hand Tracking

-	Sensors

b)	Capturing (assigned to the peripheries)

-	Vision camera: capturing (in addition to tracking and sensing) of the user’s surroundings for vision related functions

-	Media camera: capturing of scenes or objects for media data generation where required

NOTE: vision and media camera logical functions may be mapped to the same physical camera, or to separate cameras. Camera devices may also be attached to other device hardware (AR glasses or smartphone),or exist as a separate external device.

-	Microphones: capturing of audio sources including environmental audio sources as well as users’ voice.

c)	AR Runtime functions

-	XR Spatial Compute: AR functions which process sensor data to generate information about the world 3D space surrounding the AR user. It includes functions such as SLAM for spatial mapping (creating a map of the surrounding area) and localization (establishing the position of users and objects within that space), 3D reconstruction and semantic perception.

-	Pose corrector: function for pose correction that helps stabilise AR media when the user. Typically, this is done by asynchronous time warping (ATW) or late stage reprojection (LSR).

-	Semantic perception: process of converting signals captured on the AR glass into semantical concept. Typically uses some sort of Artificial Intelligence (AI) and/or Machine Learning (ML). Examples include object recognition, object classification, etc.

d)	Scene Manager

-	Scene graph handler: a function that supports the management of a scene graph that represents an object-based hierarchy of the geometry of a scene and permits interaction with the scene.

-	Compositor: compositing layers of images at different levels of depth for presentation

-	Immersive media renderer: the generation of one (monoscopic displays) or two (stereoscopic displays) eye buffers from the visual content, typically using GPUs.  Rendering operations may be different depending on the rendering pipeline of the media, and may include 2D or 3D visual/audio rendering, as well as pose correction functionalities. Also includes rendering of other senses such as audio or haptics.

e)	Media Access Function includes

-	Tethering and network interfaces for AR/MR immersive content delivery

-	The AR glasses may be tethered through non-5G connectivity (wired, WiFi)

-	The AR glasses may be tethered through 5G connectivity

-	The AR glasses may be tethered through different flavours for 5G connectivity

-	Content Delivery: Connectivity and protocol framework to deliver the content and provide functionalities such as synchronization, encapsulation, loss and jitter management, bandwidth management, etc.

-	Digital representation and delivery of scene graphs and XR Spatial Descriptions

-	Codecs to compress the media provided in the scene.

-	2D media codecs

-	Immersive media decoders: media decoders to decode compressed immersive media as inputs to the immersive media renderer.  Immersive media decoders include both 2D and 3D visual/audio media decoder functionalities.

-	Immersive media encoders: encoders providing compressed versions of visual/audio immersive media data.

-	Media Session Handler: A service on the device that connects to 5G System Network functions, typically AFs, in order to support the delivery and QoS requirements for the media delivery. This may include prioritization, QoS requests, edge capability discovery, etc.

-	Other media-delivery related functions such as security, encryption, etc.

f)	Physical Rendering (assigned to the peripheries)

-	Display: Optical see-through displays allow the user to see the real world “directly” (through a set of optical elements though). AR displays add virtual content by adding additional light on top of the light coming in from the real-world.

-	Speakers: Speakers that allow to render the audio content to provide an immersive experience. A typical physical implementation are headphones.

g)	AR/MR Application with additional unassigned functions

-	An application that makes use of the AR and MR functionalities on the device and the network to provide an AR user experience.

-	Semantic perception: process of converting signals captured on the AR glass into semantical concept. Typically uses some sort of Artificial Intelligence (AI) and/or Machine Learning (ML). Examples include object recognition, object classification, etc.

=====  CHANGE  =====

#### 4.2.2.2	Type 1: 5G STandalone AR (STAR) UE

Figure 4.2.2.2-1 provides a functional structure for Type 1: 5G STandalone AR (STAR) UE.

Figure 4.2.2.2-1: Functional structure for Type 1: 5G STandalone AR (STAR) UE

Main characteristics of Type 1: 5G STandalone AR (STAR) UE:

-	The STAR UE is a regular 5G UE. 5G connectivity is provided through an embedded 5G modem.

-	The AR Runtime is local and uses input from sensors, audio inputs or video inputs. XR Spatial Compute is primarily local, but may access or share information on the network.

-	The AR Scene Manager is local and provides immersive rendering capabilities. Support of compute on the network may be provided, but scenes may typically be composed on the UE.

-	The AR/MR application is resident on the device.

-	An AR/MR application provider is providing a service and the service may be supported/assisted by network-based AR functions and rendering.

-	Due to the amount of processing required, such devices are likely to require a higher power consumption in comparison to the other device types.

-	As the device includes all UE functionalities, the application resides and pre-dominantly is executed on the device and all essential AR/MR functions are available for typical media processing use cases, the device referred to as STandalone AR (STAR) UE.

-	Media Access Functions are provided that support the delivery of media content components over the 5G system. For details refer to clause 4.2.5.

-	The application may also communicate though the 5G System using a dedicated interface.

#### 4.2.2.3	Type 2: 5G EDGe-Dependent AR (EDGAR) UE

Figure 4.2.2.3-1 provides a functional structure for Type 2: 5G EDGe-Dependent AR (EDGAR) UE.

Figure 4.2.2.3-1: Functional structure for Type 2: 5G EDGe-Dependent AR (EDGAR) UE

Main characteristics of Type 2: 5G EDGe-Dependent AR (EDGAR) UE:

-	The 5G EDGAR UE is a regular 5G UE. 5G connectivity is provided through an embedded 5G modem and 5G System components.

-	The AR Runtime is local and uses data from sensors, audio inputs or video inputs. The AR Runtime, in particular the XR spatial compute, may be assisted by the cloud/edge application for example spatial localization and mapping provided by a spatial computing service.

-	Media processing is local, the device needs to embed all media codecs required for decoding pre-rendered 2D view.

-	A Lightweight Scene Manager is local to the AR/MR device, but the main scene management and composition is done on the cloud/edge. A scene description is generated and exchanged to establish the split work flow.

-	The main AR/MR application resides on the cloud/edge, but a basic application functionality is on the UE to support regular UE functionalities and launching services and applications.

-	Power consumption on such glasses must be low enough to fit the form factors. Heat dissipation is essential.

-	Media Access Functions are provided that support the delivery of media content components over the 5G system, in particular cloud and split rendering supporting functions. Media Access Functions are divided in control on M5 (Media Session Handler and Media AF) and user data on M4 (Media Client and Media Application Server). Detailed requirements are for study in this report.

-	While the EDGAR UE may have additional functionalities, for example those available in a STAR UE, generally for media centric or compute heavy use cases processing needs to be supported by the edge, hence referred to as EDGe-Dependent AR (EDGAR) UE.

-	The application may also communicate though the 5G System using a dedicated interface.

#### 4.2.2.4	Type 3: 5G WireLess Tethered AR UE

This clause introduces the 5G WireLess Tethered AR UE. Two sub-types are differentiated:

- Split Rendering WLAR UE. In this case the 5G phone that includes the modem also acts to support rendering of complex scenes and provides the pre-rendered data to the glass

- Relay WLAR UE: In this case, the 5G phone acts as a relay to provide IP connectivity.

Figure 4.2.2.4-1 provides a functional structure for Type 3a: 5G Split Rendering WireLess Tethered AR UE.

Figure 4.2.2.4-1: Functional structure for Type 3a: 5G Split Rendering WireLess Tethered AR UE

Main characteristics of Type 3a: 5G Split Rendering WireLess Tethered AR UE:

-	5G connectivity is provided through a tethered device which embeds the 5G modem. Wireless tethered connectivity is provided through WiFi or 5G sidelink. BLE (Bluetooth Low Energy) connectivity may be used for audio. The motion-to-render-to-photon loop runs from the glass to the phone. While the connectivity is outside of the 5G Uu domain, it is still expected that for proper performance when used for split rendering, a stable and constant delay link may be setup on the tethered connection.

-	The AR Runtime is local and uses from sensors, audio inputs or video inputs, but may be assisted by functionalities on phone.

-	While media processing (for 2D media) may be done on the AR glasses, energy intensive AR/MR media processing may be done on the AR/MR tethered device or split.

-	Some devices might have limited support for immersive media decoding and rendering and may need to rely on 5G cloud/edge

-	While such devices are likely to use significantly less processing than Type 1: 5G STAR devices by making use of the processing capabilities of the tethered device, they still support a lot of local media and AR/MR processing. Such devices are expected to provide 8-10h of battery life while keeping a significantly low weight.

-	The tethered glass itself is not a regular 5G UE, but the combination of the glass and the phone results in a regular 5G UE.

-	Media Access Functions are provided that support the delivery of media content components over the 5G system. Examples of the Media Access Functions are 5GMS functions, MTSI functions, web-connectivity or edge-related client functions. Detailed requirements are for study in this report.

Figure 4.2.2.4-2 provides a functional structure for Type 3b: 5G Relay WireLess Tethered AR UE.

Figure 4.2.2.4-2: Functional structure for Type 3b: 5G Relay WireLess Tethered AR UE

Main characteristics of Type 3b: 5G Relay WireLess Tethered AR UE:

-	5G connectivity is provided through a tethered device which embeds the 5G modem. Wireless tethered connectivity is through WiFi or 5G sidelink. BLE (Bluetooth Low Energy) connectivity may be used for audio.

-	The 5G Phone acts as a relay to forward IP packets. The 5G Phone runs a Media Session Handler including EDGE functionalities to support QoS control on the 5G System. To support proper end-to-end QoS, the media session handling needs to take into account the constraints of the tethering link to provide sufficient QoS on the 5G System link to provide adequate QoE for the end user. Details on the exact function of the relay, for example of it is on IP layer (layer 3) or on lower layer is for further study.

-	Media Access functions are provided on the glass device to support the delivery of media content components over the 5G and wireless tethered link.

-	The motion-to-render-to-photon loop runs from the glass to the edge and hence includes in total 4 wireless links. It is expected that for proper performance when used for split rendering, a stable and constant delay end to end link needs to be setup.

-	The AR Runtime is local and uses from sensors, audio inputs or video inputs, but may be assisted by functionalities on phone.

-	Media Processing is either done on the glass device or it is split with the network. In particular, relevant is that many devices have limited support for immersive media decoding and rendering and may need to rely on 5G cloud/edge.

-	While such devices are likely to use significantly less processing than Type 1: 5G STAR devices by making use of the processing capabilities of the tethered device, they still support a lot of local media and AR/MR processing. Such devices are expected to provide 8-10h of battery life while keeping a significantly low weight.

-	The tethered glass itself is not a regular 5G UE, but the combination of the glass and the phone results in a regular 5G UE.

-	For services with low latency requirements, such as MTSI or those provided by FLUS, it may be necessary to take the status of wireless connectivity into account when configuring the services, such that the link between AR glass and 5G phone is not overly loaded. How to coordinate the operation of Uu and wireless connectivity in such services is FFS [3].

=====  CHANGE  =====

### 4.2.3	AR Runtime

The AR Runtime is a device-resident software or firmware that implements a set of APIs to provide access to the underlying AR/MR hardware. These APIs are referred to as AR Runtime APIs. An AR Runtime typically provides the following functions:

-	System capability discovery: allows applications to discover capabilities of the AR glasses

-	Session management: manages an AR session and its state

-

-	Input and Haptics: receives information about user’s actions, e.g. through usage of trackpads, and passes that information to the application. On request by the application, it may trigger haptics feedback using the AR glasses and associated hardware.

-	Rendering: synchronizes the display and renders the composited frame onto the AR glasses displays.

-	Spatial Computing: processes sensor data to generate information about the world 3D space surrounding the AR user. Spatial computing includes functions such as

-	Tracking to estimate the movement of the AR device at a high frequency

-	Relocalisation to estimate the pose of the AR device at initialization, when tracking is lost or regularly to correct the drift of the tracking.

-	Mapping, for reconstructing the surrounding space, for example through triangulation of identified points. This reconstruction can be sparse for localization purposes, or dense for visualization.

-	A combination of tracking, mapping and relocalization functions, for example through Simultaneous Localization and Mapping (SLAM) to build a map of the environment and establish the position of users and objects within that environment

-	Semantic perception: to process the captured information into a semantical concept, typically uses some sort of Artificial Intelligence (AI) and/or Machine Learning (ML). Examples include object or user activity segmentation, recognition, and classification.

Spatial computing functions typically include data exchange and requires network architecture. Clause 4.2.5 provides more details on XR Spatial computing.

AR runtimes are usually extensible to add support for a wide range of AR glasses and controllers that are on the market or that might be released in the future. This will allow different vendors to add custom functionality such as gaze tracking, hand control, new reference spaces, etc.

Two key representative and standardized AR runtime APIs are Khronos defined OpenXR [4] and W3C defined WebXR [5]. More details are provided in clause 4.6.4.

=====  CHANGE  =====

### 4.2.5	XR Spatial Computing

XR Spatial Computing summarizes functions which process sensor data to generate information about the world 3D space surrounding the AR user. It includes functions such as SLAM for spatial mapping (creating a map of the surrounding area) and localization (establishing the position of users and objects within that space), 3D reconstruction and semantic perception. This requires accurately localizing the AR device worn by the end-user in relation to a spatial coordinate system of the real-world space. Vision-based localization systems reconstruct a sparse spatial mapping of the real-world space in parallel (e.g. SLAM). Beyond the localization within a world coordinate system based on a sparse spatial map, additionally dense spatial mapping of objects is essential in order to place 3D objects on real surfaces, but also provides the ability to occlude objects behind surfaces, doing physics-based interactions based on surface properties, providing navigation functions or providing a visualization of the surface. Thirdly, for the purpose of understanding and perceiving the scene semantically, machine-learning and/or artificial intelligence may be used to provide context of the observed scene. The output of spatial computing is spatial mapping information that is organized in a data structure called the XR Spatial Description for storing and exchanging the information. Further details on XR Spatial Description formats are provided in clause 4.4.7.

XR Spatial compute processes may be carried out entirely on the AR device. However, it may be beneficial or necessary to use cloud or edge resources to support spatial computing functions. At least two primary scenarios may be differentiated:

- spatial computing is done on the AR device, but an XR Spatial Description server is used for storage and retrieval of XR Spatial Description.

- At least parts of the spatial compute functions are offloaded to a XR Spatial compute server

Both cases are discussed further in the following.

Typically, the device can store a certain amount of XR Spatial Description locally on the device. However, in particular to create a world-experience, the AR device may not be able to store all information related to XR Spatial Description on the device, and hence, such information may continuously be updated by downloading or streaming updated information from an XR Spatial Description server as shown in Figure 4.2.5-1. In addition, the device may use personalized storage on the cloud to offload device-generated XR spatial information components. This may for example include so-called key frames, i.e. frames that are useful to provide accurate spatial mapping with relevant key points.

The architecture in Figure 4.2.5-1 relates to the case where XR Spatial computing is done standalone on the device, and hence we refer to this as STAR architecture in context of XR Spatial computing.

Figure 4.2.5-1 Functional diagram for XR spatial computing with network/cloud support

If the device is limited in processing power, or if complex XR compute functionalities need to be carried out, the XR compute function on the device may be assisted by or even depend on compute resources in the network, for example on the edge or cloud. Figure 4.2.5-2 provide a basic architecture for the case where XR spatial computing is delegated partially or completely to an XR spatial computing edge server.

- The device sends sensor data or pre-processed sensor data (e.g. capture frames or visual features extracted from such frames) to the XR spatial compute server.

- The XR spatial compute server carries out supporting functions to extract relevant information and returns directly XR Spatial Compute-related AR Runtime data (according to the AR Runtime API), e.g. pose information, or pre-computed XR Spatial information that is used by a lightweight XR spatial compute function on the device to create AR Runtime data. Pre-computed XR Spatial information may, for example, be dense map of segmented objects for visualization, labels or id of recognized objects, 2D contours of recognized object to highlight them, or labels of the recognized user activity.

- The XR spatial edge compute server may further fetch the XR Spatial Description from the XR Spatial Description server and perform spatial computing based on device sensor data.

Figure 4.2.5-2 Functional diagram for spatial computing with XR spatial compute edge

The architecture in Figure 4.2.5-2 relates to the case for XR Spatial computing depends on an edge function, and hence we refer to this as EDGAR architecture in context of XR Spatial computing.

=====  CHANGE  =====

### 4.2.6	5G Media Access Function

The Media Access Function supports the AR UE to access and stream media. For this purpose, a Media Access Function as shown in Figure 4.2.5-1 includes:

-	Codecs: are used to compress and decompress the rich media. In several cases, not only a single instance of a codec per media type is needed, but multiple ones.

-	Digital representation and delivery for scene description and XR Spatial Description

-	Content Delivery Protocol: Container format and protocol to deliver media content between the UE and the network according to the requirements of the application. This includes timing, synchronization, reliability, reporting and other features.

-	5G connectivity: a modem and 5G System functionalities that allow the UE to connect to a 5G network and get access to the features and service offered by the 5G System.

-	Media Session Handler: A generic function on the device to setup 5G System capabilities. This may setup edge functionalities, provide QoS support, support reporting, etc.

-	Content protection and decryption: This function handles protection of content from being played on unauthorized devices.

Functions are needed in both uplink and downlink, depending on use cases and scenarios.

Example for Media Access Functions are

- 5GMSd client that includes a Media Session Handler and a Media Player as defined in TS 26.501 and TS 26.512.

- 5GMSu client that includes a Media Session Handler and a Media Streamer as defined in TS 26.501 and TS 26.512.

- A real-time communication client that includes either uplink or downlink, or both to support more latency critical communication services.

- A combination of the above based on the needs of the XR application. An XR scene may have a mix of static, streaming, and real-time media that require the usage of multiple transport channels and protocol stacks.

In all cases, the basic function of Media Session Handler and a delivery client (which includes content delivery protocols and codecs) is expected to be maintained. The Media Session Handler is a generic function to support 5G System integration.

As a subject of this report, the needs to support different types of instantiations is for codecs, delivery protocols, session handling and so  is identified. Not all components are necessarily required for all scenarios and even further, not all functions may be available on all device types.

Figure 4.2.5-1 Media Access function for AR

=====  CHANGE  =====

## 4.3	Basic Processes in an AR Session

## 4.3.1.	Introduction

In this clause we provide a generic workflow for setting up an AR session. The basic workflow is provided in Figure 4.3.1-1. In this case, the following steps happen:

1.	The application contacts the application provider to fetch the entry point for the content. The acquisition of the entry point may be performed in different ways and is considered out of scope. An entry point may for example be a URL to a scene description.

2.	The application initializes the Scene Manager using the acquired entry point.

3.	The Scene Manager retrieves the scene description from the scene provider based on the entry point information. It then establishes an AR scene session with the scene provider.

4.	The Scene Manager parses the entry point and creates the immersive scene.

5.	The Scene Manager requests the creation of a new AR/MR session from the AR Runtime.

6.	The AR Runtime creates a new AR/MR session and performs registration with the local environment.

Then two types of sessions are established:

7-11: AR scene delivery session: A session - for accessing a scene and related media over the network. This basically uses the MAF as well as the scene manager as well as the corresponding network functions. Details are introduced clause 4.3.2.

12-16: XR spatial compute delivery session: A session that uses sensor data to provide an understanding of the physical space surrounding the device to determine the device’s position and orientation and placement of AR objects in reference to the real world and uses XR Spatial Description information from the network to support this process. This uses the XR Spatial Description functions as introduced in clause 4.2.6. Details are introduced in clause 4.3.3.

17: Both sessions run independently, but the results of both sessions (e.g. media organized in a scene graph and pose of the AR device) are inputs of the AR/MR scene manager function. This function handles the common processing of the two asynchronous sessions to create an AR experience.

Figure 4.3.1-1: Basic workflow for AR sessions

Editor’s Note: the above call flow needs careful checking for consistent terminology

## Each of the session typically runs independently, but information may be exchanged on the device or in the network, typically moderated by the application.

## 4.3.2	AR Scene Session

In this clause, we provide basic processes and generic workflow description for setting up AR scene sessions for media is accessed over the network. This generic basic process may be extended to address specific applications and use cases. The call flow as shown in Figure 4.3.2-1 aligns with the STAR/EDGAR architecture and serves as a baseline for defining use-case specific call flows.

Figure 4.3.2-1: Basic workflow for AR Scene session

Editor’s Note: the above call flow needs careful checking for consistent terminology and alignment with the basic call flow in clause 4.3.1.

A description of the steps of the general workflow is provided as follows:

Steps 1-8 are described in Figure 4.3.1-1.

The application starts the scene fetching and rendering loop:

- the MAF may receive updates to the scene description from the scene provider

- the MAF passes the scene update to the Scene Manager.

- the Scene Manager updates the current scene.

- The Scene Manager acquires the latest pose information and the user’s actions

- The Scene Manager shares that information with the AR/MR application on the server

The media rendering loop consists of the following steps. Note that steps 8, 9 and 10 are running as 3 parallel loops:

- For each new object in the scene:

  - the Scene Manager triggers the MAF to fetch the related media.

  - the MAF creates a dedicated media pipeline to process the input.

  - the MAF establishes a transport session for each component of the media object.

- For each transport session:

  - The media pipeline fetches the media data. It could be static, segmented, or real-time media streams.

  - The media pipeline processes the media and makes it available in buffers.

- For each object to be rendered:

  - The Scene Manager gets processed media data from the media pipeline buffers

  - The Scene Manager reconstructs and renders the object

- The Scene Manager passes the rendered frame to the AR/MR Runtime for display on the user’s HMD.

### 4.3.3	XR Spatial Computing Session

In this clause, we provide basic processes and generic workflow description for setting up an XR spatial compute session for spatial description related data to be accessed over the network. This generic basic process may be extended to address specific applications and use cases. The call flow as shown in Figure 4.3.3-1 aligns with the STAR/EDGAR architecture and serves as a baseline for defining use-case specific call flows.

Figure 4.3.3-1 Functional diagram for spatial computing with XR spatial compute edge

Editor’s Note: the above call flow needs careful checking for consistent terminology and alignment with the basic call flow in clause 4.3.1.

A description of the steps of the general workflow is provided as follows:

Step 1-11 are provided in clause 4.3.1.

12.	XR Spatial compute delivery sessions are established.

13.	For a XR Spatial compute downlink delivery session

14. The XR Spatial compute function in the AR Runtime asks the MAF to establish a XR Spatial compute downlink delivery session

15. The MAF communicates with the network to establish the proper resources and QoS

16. The XR Spatial compute function requests access to XR Spatial Description information

17. An XR Spatial Description downlink delivery session is established across the XR Spatial compute server, the media delivery function, the media access function and XR Spatial compute function on the device.

18. XR Spatial Description information is delivered in this downlink delivery session

19. For a XR Spatial compute uplink delivery session

20. The XR Spatial compute function in the AR Runtime asks the MAF to establish a XR Spatial compute uplink delivery session

21. The MAF communicates with the network to establish the proper resources and QoS

22. The MAF established an appropriate uplink delivery pipeline

23. An XR Spatial Description uplink delivery session is established across the XR Spatial compute function on the device, the media access function, the media delivery function and the XR Spatial compute server.

24. Spatial compute information is upstreamed to the XR Spatial compute server.

25. Data is continuously exchanged between the Scene Manager and the AR Runtime

26. The AR Runtime processes the data from AR scene session and XR spatial compute session

=====  CHANGE  =====

### 4.4.7	XR Spatial Description

#### 4.4.7.1	Overview

XR Spatial Description is a data structure (typically organized in a graph) describing the spatial organisation of the real world using:

- Visual features, keyframes and spatial maps as described in more details in clause 4.4.7.3.

- Spatial anchors and trackables as described in more details in clause 4.4.7.4.

- camera parameters as defined in clause 4.4.3.2

XR Spatial Description is derived from or needs to be processed together with camera and sensor information. Typical raw sensor data is summarized in clause 4.4.7.2.

XR Spatial Description describes the real-world including information that can be used for the estimation of position and orientation (pose estimation) of AR devices for the purpose of registration, tracking and positioning, and provides a coordinate reference system in relation to the real world. Generally, it may be used for spatial computing as described in clause 4.2.5.

The XR Spatial Description may be downloaded to the AR device and reside on the device. However, to support mobility and different environments, XR Spatial Description may have to be exchanged over the network and hence a formalized representation of XR Spatial Description may be needed. XR Spatial Description data may have to be downloaded and updated periodically from a XR Spatial Description server.

In addition, the AR function may send XR Spatial Description Updates to a XR Spatial Description Server. Such data may be derived from XR Spatial compute functions, e.g., updated visual spatial features, keyframes attached to camera parameters, or sub-parts of an XR Spatial Description. The server may use these XR Spatial Description Updates to improve the XR Spatial Description for future use of the same user or by other users.

As the data needs to be updated, exchanged as well as stored on the device and the XR Spatial Description server, an efficient and flexible representation of XR Spatial Description is desired. For example, the description needs to be serialized and fragmented to be properly accessed and downloaded over the network.

The size of the XR Spatial Description depends on several parameters, for example, size of the area covered by the XR application, number of supported viewpoints in the area, the amount of keyframes that are provided, etc. The size may for example be from 10MByte for a small room to several hundred MBytes for a building. For a global-scale spatial map, the amount of data would be massively larger. As an example, the Microsoft™ Flight Simulator is around 2 Million GByte (https://www.xboxachievements.com/news/news-34054-microsoft-flight-simulators-world-is-two-million-gigabytes-in-size.html). Regular exchange of data with the network is needed, details on the frequency, the latency requirements, the bitrate requirements typically depend on the application, but more details are for further study.

As an example, the ETSI ISG ARF004 [21] uses the term World Graph for XR Spatial Description. It defines the relative position of Trackables and World Anchors by 3D Transforms. In this case, the world graph is similar to a scene graph including trackables (embedding their features), and anchors representing the real world. This information may be used by the AR Runtime for spatial compute functions including activity and object detection, object recognition, and pose estimation using trackables [21].  At the end of 2021, no non-proprietary XR Spatial Description formats are known.

#### 4.4.7.2	Camera and sensor information

In this clause we provide an overview of different sensors that can provide the input data for spatial compute AR functions. All device captured data requires a common timeline and a common coordinate system in order to be meaningful for XR Spatial compute process. If the data is processed in the networked, such time and spatial synchronization information is expected to be maintained.

AR Glasses typically include multiple cameras (for example one device supports 7 cameras) to build precise motion tracking and gesture recognition. Generally, these camera feeds are processed on the device, but they may be sent across the network to support spatial compute functions. Different cameras exist on a single device, namely

- monochrome image capture cameras,

- RGB image capture cameras,

- infrared capture cameras.

[

Editor’s Note: the following text needs some consolidation and consistency

Light Detection And Ranging (LiDAR) is a method for measuring distances (ranging) by illuminating the target with a light and then measuring the reflection with an optical sensor. There exist different methods of analysing the reflected light to measure the distance. One method is based on knowing the speed of light and clocking the duration between the moment of emission and the moment of reception of the emitted beam of light. These two variables, light speed and time interval, allow to estimate the third one: the distance from the object. This concept leveraging velocity, time interval and distance, is generally known as Time-of-Flight and can be applied to a wide range of domains for waves or particles. In case of the optical domain, the sensor is typically called a ToF camera which constitutes the main source of LiDAR acquisition. Another method analyses the shift in the frequency domain of the reflected light. Based on the Doppler effect concept, this method can resolve both distance and the velocity of the target.

In practice, LiDAR camera can operate in the ultraviolet, visible or infrared spectrum. Since the laser light used is typically collimated, the LiDAR camera needs to scan the scene in order to generate an image with a usable field-of-view. There exists mainly two ways to scan a scene assuming that the camera is static. The first one uses reflecting moving pieces (e.g., mirror) so that the laser light is reflected in various directions in the scene. A second option is to mount the laser on a rotating piece so that the laser light hits the scene all around the camera. Both static and rotating LiDAR have advantages and disadvantages and the application requirements may be more suitable for one or the other type.

In both cases, the output of a LiDAR acquisition is a point cloud which can then be enriched with other sensor data such as RGB data.

Optical 3D sensors may be used to capture and reconstruct three-dimensional depth of objects. Depending on the source of the radiation, optical 3D sensors may be divided in two sub-categories; passive and active systems. Stereoscopic systems, Shape-from-Silhouettes (SfS), and Shape-from-Texture (SfT) are the examples of the passive systems, which do not emit any kind of radiation themselves. The sensors collect images of the scene, eventually from different points of view or with different optical setup. Then the images are analysed in order to compute the 3D depth of points in the scene. On the contrary, the active systems emit some kind of radiation and the interaction between the object and the radiation is captured by a sensor. From the analysis of the captured data, knowing the features of the emitted radiation, the coordinates of the points can be obtained. Time-of-Flight (ToF), phase shift, and active triangulation are the examples of the active systems.

]

Devices may also include microphones. A a typical setup is a two-channel microphone array for audio input. Multichannel microphones or even Higher-Order Ambisonics (HOA) microphone arrays may be supported as well. The resulting signals are two- or multichannel audio signals or HOA signals.

Typical sensor and fusion data are accelerometer, gyroscope, and magnetometer samples. This information for example includes angular velocity from gyroscopes, accelerometer data including the effect or gravity as well statistical data around the measurements. Detailed representations are for further study.

#### 4.4.7.3	Visual features, Keyframes and Spatial Maps

Visual features, keyframes and spatial maps are used for mapping the real world, typically as part of the SLAM process.

Visual features are characteristics of real-world elements that can be searched, recognized and tracked in 2D images captured by the AR device as the user moves in a real environment. These images provide a view of the same real world elements, captured from different positions (as indicated by the camera parameters attached to them) from a single moving camera or multiple cameras. Visual features are mapping 2D points that are recognizable in multiple 2D images.

Keyframes are stored frames that include one or multiple visual features. Visual features may be matched from camera-captured frames by comparing those frames with keyframes available to the AR Runtime in order to support the SLAM process. Keyframes also have attached camera information defined in 4.4.3.2 to triangulate 3D points correctly from multiple cameras.

A spatial map is a digital representation of the real world and of the user’s environment. The geometrical part of the spatial map can be represented by a mesh, or a sparse or dense point cloud. A texture can be assigned to the geometry to provide a textured mesh or textured point cloud. Such a spatial map may be generated from keyframes and their matched visual features using triangulation. This mapping process can be performed either at runtime or offline. A spatial map is then used at runtime to relocalise and thus register the AR device by matching the features extracted from the current captured frames with the keyframes stored as part of the spatial map. The mapping approach is one of well-known keyframe-based SLAM techniques [X].

Editor’s Note: the following text on spatial maps should be improved

A spatial map consists of at least a spatial feature cloud, i.e., 3D points (Vector of 3 float) with their associated descriptors such as SIFT [XX], SURF[XXX], ORB[XXXX]. A spatial feature is a 3D point triangulated from matching features. The descriptors are generally vectors of numbers (e.g vector of 128 floats for SIFT, vector of 64 floats for SURF, vector of 32 integers for ORB). Note that other features such as 3D segments can be also used. To speed-up the 2D-3D matching process, a spatial map typically also includes the following additional information:

- Information required for keyframe retrieval. For example, a keyframe retrieval can use Bag-Of-visual-Words (BoW) model. In this case, the information consists of the vocabulary of the BoW model and corresponding descriptor for each keyframe (vector of occurrence counts of a vocabulary in the keyframe). Depending on the visual descriptor used, the vocabulary size is usually a 10-100 MByte, and this vocabulary can be reused for any spatial map using the same vocabulary.

- The 2D features for each keyframes (e.g. 2D points with their associated descriptors such as SURF, SIFT, ORB represented by a vector of numbers). The number of features extracted per keyframe varies between 200 and 1000.

- The matches between 2D features of keyframes and 3D features of the spatial feature cloud.

Thanks to this additional information, instead of comparing all descriptors of 2D features extracted from the current frame with all spatial feature descriptors (from the spatial feature cloud of the spatial map), resulting in a very high complexity, the vision based localization system can:

- Match the closest keyframe to the current frame by retrieving it with the BoW model

- Match the 2D features between the current frame and the retrieved keyframe

- Match the 2D features between the current frame and spatial feature cloud (knowing matches between 2D features of the keyframes and 3D features of the spatial feature cloud)

Figure 4.4.7.3-1 visualizes the processes for spatial feature mapping using a captured 2D frame in order to estimate the camera pose.

Figure 4.4.7.3-1 Camera pose estimation by spatial feature mapping using a spatial map and a captured 2D frame

#### 4.4.7.4	Spatial Anchors and Trackables

AR objects can be positioned in reference to the real world (e.g., placing a vase on a table) using spatial anchors and trackables.

A spatial anchor provides a fixed position and orientation in the real world based on a common frame of reference that can be used by multiple AR devices. Spatial anchors can also be used independently of other spaces in case global coordinates are available to the device. In this case, the anchors are treated as global anchors as they have global coordinates for which positions can be determined.

However, in many cases an accurate global coordinate system is not available. In this case, spatial anchors refer to trackables for accurate positioning relative to the physical space. Trackables are elements of the real world for which features (visual or non-visual) are available and/or could be extracted. A trackable can for example be a spatial map that defines a full environment composed of floor walls and furniture in the real world consisting of several 3D points with visual features. However, there are other types of trackables, for example:

- A controller with LEDs that can be tracked by an AR headset’s vision sensor. The feature in this case is the constellation of LEDs.

- A fiducial marker that is detected as a black and white pattern by an AR device vision sensor. The feature in this case is the black and white pattern.

- Hands visible through an AR headset’s vision sensor. The feature is a learnt model for hands.

All of the above examples give a position of the trackable in reference to the position of the sensor (generally embedded in the AR headset).

=====  CHANGE  =====

Editor’s Note: Updates need to be done to clause 6

=====  CHANGE  =====

Editor’s Note: Updates need to be done to Conclusions