3GPP TSG SA WG4#117e	S4-220019

E-meeting, 14th – 23rd February 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | draft | rev |  | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5MBUSA] 5GMS via eMBMS - Broadcast on Demand |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBUSA |  |  |  |  |  | Date: |  |  | 07/02/2022 |
| Category: | B |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | See work item |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add 5GMS via eMBMS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Item objectives not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This document assumes that the dCRs in S4-220018 and S4-220020 are agreed. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

### 5.10.6	Procedures for dynamic On-Demand network selection for 5GMS content delivery

#### 5.10.6.1 	General

The scenario is as follows: A content provider offers content via eMBMS, for example using a broadcast network in receive only mode, but also distributes content via a 5G System. The broadcast system statically configured resources. Distribution of a service via eMBMS is for example only used for services, for which there is high demand. Also, based on the demand, the resources and quality of the service distributed through broadcast may be adjusted.

The call flow in Figure 5.10.6-1 extends the call flow defined in clause 5.6.1 to address generic cases for the broadcast on-demand use cases. Specific additional use cases are presented in the remainder of clause 5.10.6.

Figure 5.10.6.1-1: High Level Procedure for DASH content via Broadcast On-Demand

Steps:

1:	The 5GMS Aware Application triggers the Service Provisioning for several or many services and permits broadcast distribution of the media content

2:	Specifically, the 5GMSd AF provisions for MBMS delivery is setup and the BMSC informs the 5GMS AF on ingest resources.

3: The media content is announced to the 5GMSd-aware application and the application request the entry points for the service.

4: Ingest starts from the 5GMSd Application Providers and the 5GMS AS.

5: Consumption Reporting is applied for the session.

6: The media content is selected by the application

7:	The application starts media playback

8:	The MPD/manifest is requested by the Media Player from the 5GMS AS.

9: The Media Player processes the MPD and identifies that resources are available on the 5GMS AS

10: The Media Player continuously together with the application selects the content and different content options.

11: Content is received from the 5GMS AS.

12: The Media Player informs the Media Session Handler on the consumed resources.

13: The MSH sends consumption reports to the 5GMS AF.

14: Demand for broadcast is identified by the 5GMS AF.

15: Service provisioning is updated to add MBMS delivery sessions

16: MBMS delivery sessions are provisioned to add delivery of certain service on MBMS.

17: Ingest is updated such that the 5GMS AS now forwards selected resources to the BMSC.

18:	MBMS delivery starts.

19: The 5GMS AF informs the MSH that MBMS delivery is initiated and provides the service access information.

20:	MBMS delivery reception is initiated.

21: Once the service is ready, the content delivered on MBMS is used by the media player. Consumption reporting continues. Specific cases may used different policies, similar to the hybrid case in clause 5.10.5.

#### 5.10.6.2 	Different Operation modes

Different operation modes may be considered in the above use case:

1)	Every 5GMS media service is mapped to exactly one MBMS User Service. Whether the MBMS User service is announced and delivered, depends on the demand. The delivery session is dynamically adjusted depending on the demand, for example the delivery session is switched off, or the bitrate in the delivery session is changed depending on demand and content requirements.

2)	A set of MBMS User services and delivery sessions are defined in the initial provisioning. 5GMS media services are dynamically mapped to MBMS User services based on demand and content requirements.

3)	Components of the 5GMS User Service, for example languages, are dynamically assigned to MBMS delivery depending on demand.