| 3GPP TS 26.532 V0.2.1 (2022-02) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Data Collection and Reporting; Protocols and Formats; (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Procedures for Data Collection and Reporting	9

4.1	General	9

4.2	Network-side procedures	9

4.2.1	General	9

4.2.2	Data Collection AF registration with NRF	9

4.2.3	Data collection and reporting provisioning	10

4.2.3.1	General	10

4.2.3.2	Provisioning Session procedures	10

4.2.3.2.1	General	10

4.2.3.2.2	Create Provisioning Session	10

4.2.3.2.3	Retrieve Provisioning Session properties	10

4.2.3.2.4	Update Provisioning Session properties	10

4.2.3.2.5	Destroy Provisioning Session	10

4.2.3.3	Data Reporting Provisioning procedures	11

4.2.3.3.1	General	11

4.2.3.3.2	Data Reporting Configuration types	11

4.2.3.3.3	Create Data Reporting Configuration	11

4.2.3.3.4	Retrieve Data Reporting Configuration	11

4.2.3.3.5	Update Data Reporting Configuration	11

4.2.3.3.6	Destroy Data Reporting Configuration	12

4.2.4	Configuration of Indirect Data Collection Client	12

4.2.5	Configuration of Application Server	12

4.2.6	Indirect data reporting	12

4.2.7	Reporting by Application Server	12

4.2.8	Event subscription, management and publication	12

4.3	UE-to-network procedures	13

4.3.1	General	13

4.3.2	Configuration of Direct Data Reporting Client	13

4.3.3	Direct data reporting	13

4.4	UE-internal procedures	13

4.4.1	General	13

5	General Aspects of APIs for Data Collection and Reporting	14

5.1	Overview	14

5.2	HTTP resource URIs and paths	14

5.3	Usage of HTTP	14

5.4	Common API data types	14

5.5	Explanation of API data model notation	14

6	Ndcaf_DataReportingProvisioning service	14

6.1	General	14

6.2	Provisioning Sessions API	14

6.2.1	Overview	14

6.2.2	Resource structure	15

6.2.3	Data model	15

6.2.4	Mediation by NEF	15

6.3	Data Reporting Provisioning API	15

6.3.1	Overview	15

6.3.2	Resource structure	15

6.3.3	Data model	15

6.3.4	Mediation by NEF	15

7	Ndcaf_DataReporting service	15

7.1	General	15

7.2	Data Collection and Reporting Configuration API	15

7.2.1	Overview	15

7.2.2	Resources	15

7.2.2.1	Resource Structure	15

7.2.2.2	Data Reporting Sessions resource collection	16

7.2.2.2.1	Description	16

7.2.2.2.2	Resource definition	16

7.2.2.2.3	Resource Standard Methods	16

7.2.2.2.3.1	Ndcaf_DataReporting_CreateSession operation using POST method	16

7.2.2.3	Data Reporting Session resource	18

7.2.2.3.1	Description	18

7.2.2.3.2	Resource definition	18

7.2.2.3.3	Resource standard methods	18

7.2.2.3.3.1	Ndcaf_DataReporting_RetrieveSession operation using GET method	18

7.2.2.3.3.2	Ndcaf_DataReporting_UpdateSession operation using PUT method	18

7.2.2.3.3.1	Ndcaf_DataReporting_DestroySession operation using DELETE method	20

7.2.3	Data Model	21

7.2.3.1	General	21

7.2.3.2	Structured data types	22

7.2.3.2.1	DataReportingSession resource type	22

7.2.3.2.2	ReportCondition type	22

7.2.3.3	Simple data types and enumerations	22

7.2.3.3.1	DataDomain enumeration	22

7.2.3.3.2	ConditionType enumeration	23

7.2.3.3.3	Event enumeration	23

7.2.4	Error handling	23

7.2.5	Mediation by NEF	23

7.3	Data Reporting API	23

7.3.1	Overview	23

7.3.2	Resources	23

7.3.2.1	Resource structure	23

7.3.2.2	Data Report resource	24

7.3.2.2.1	Description	24

7.3.2.2.2	Resource definition	24

7.3.2.2.3	Resource Standard Methods	24

7.3.2.2.3.1	Ndcaf_DataReporting_Report operation using POST method	24

7.3.3	Data Model	25

7.3.3.1	General	25

7.3.3.2	Structured data types	25

7.3.3.2.1	DataReport type	25

7.3.3.3	Simple data types and enumerations	26

7.3.4	Mediation by NEF	26

8	Client API	26

8.1	General	26

9	Security and Access Control	26

Annex A (normative): Data reporting data models	27

A.1	Introduction	27

A.2	Service Experience reporting	27

A.2.1	ServiceExperienceRecord type	27

A.2.2	PerFlowServiceExperienceInfo type	27

A.3	Location reporting	28

A.3.1	LocationRecord type	28

A.4	Communication reporting	28

A.4.1	CommunicationRecord type	28

A.5	Performance Data reporting	28

A.5.1	PerformanceDataRecord type	28

A.6	Application-specific reporting	29

A.6.0	Introduction	29

A.6.1	ApplicationSpecificRecord type	29

A.7	Trip Plan reporting	29

A.7.0	Introduction	29

A.7.1	TripPlanRecord type	29

Annex B (normative): OpenAPI representation of REST APIs for data collection and reporting	30

A.1	General	30

Annex <C> (informative): <Informative annex for a Technical Specification>	31

C.1	Heading levels in an annex	31

Annex X (informative): Change history	32

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The in the clause 1, as defined in 3GPP TS 23.501 [2], TS 23.502 [3], TS 23.288 [4], TS 29.517 [5], TS 29.510 [6] and TS 26.531 [7] .

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[5]	3GPP TS 29.517: "5G System; Application Function Event Exposure Service; Stage 3".

[6]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[7]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[8]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[9]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[10]	"CORS (Cross-Origin Resource Sharing)" protocol as defined in the ‘Fetch’ standard of WHATWG: https://fetch.spec.whatwg.org/#cors-protocol.

[11]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[12]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[13]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[14]	3GPP TS 26.512: “5G Media Streaming (5GMS); Protocols”.

[15]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[16]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[17]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], TS 23.501 [2], TS 23.502 [3], TS 23.288 [4], TS 29.517 [5], TS 29.510 [6], TS 26.531 [7] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2], TS 23.502 [3], TS 23.288 [4], TS 29.517 [5], TS 29.510 [6], TS 26.531 [7] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AF	Application Function

AS	Application Server

ASP	Application Service Provider

DC-AF	Data Collection AF

DC-Client	Data Collection Client

NEF	Network Exposure Function

NRF	Network Repository Function

NWDAF	Network Data Analytics Function

# 4	Procedures for Data Collection and Reporting

## 4.1	General

This clause specifies the stage 3 procedures for data collection and reporting.

## 4.2	Network-side procedures

### 4.2.1	General

This clause specifies the procedures used between network-side entities for UE data collection and reporting, along with related functionality pertaining to the provisioning, management, and delivery of such data between the Data Collection AF and consumer entities.

### 4.2.2	Data Collection AF registration with NRF

This clause specifies the use of the Nnrf_NFManagement service API as defined in TS 29.510 [7] and invoked by a Data Collection AF instance to register its profile with the NRF in order to enable the discovery of the Data Collection AF by consumer entities.

### 4.2.3	Data collection and reporting provisioning

#### 4.2.3.1	General

An Application Service Provider, via its Provisioing AF, may use the procedures in this clause to supply data collection and reporting provisioning information, as defined in clause 4.2 of TS 26.531 [7], to the Data Collection AF via reference point R1 in the form of Data Reporting Configuration resources. A given Data Reporting Configuration comprises instructions and other information to be followed/used by data collection clients in their collection, processing and reporting of UE data for the associated application service and Event ID(s).

The provisioning process begins with the Provisioning AF using the procedures defined in clause 4.2.3.2 to create a Provisioning Session resource as an umbrella for subsequent Data Reporting Configuration resources.

The process then proceeds with the Provisioning AF using the procedures defined in clause 4.2.3.3 to provide the Data Collection AF with one or more Data Reporting Configuration resources. Each set of provisioning information pertains to one application, identified by its External Application Identifier, and one type of exposed event, uniquely identified in the 5G System by its Event ID, as defined in clause 4.15.1 of TS 23.502 [3].

#### 4.2.3.2	Provisioning Session procedures

##### 4.2.3.2.1	General

Prior to provisioning of data collection and reporting , the Provisioning AF shall create a new Provisioning Session. The following CRUD operations are used to manage Provisioning Session resources. Additional details, including definition of the Provisioning Sessions API, are provided under clause 6.2.

##### 4.2.3.2.2	Create Provisioning Session

This procedure shall be used by the Provisioning AF to create a new Provisioning Session. The HTTP POST method shall be used for this purpose.

Editor’s Note: Describe key attributes of the Provisioning Session resource here, especially the access controls that realise the data exposure restrictions affecting all Data Reporting Configuration children of the Provisioning Session.

Upon successful creation, the Data Collection AF shall respond with a 201 (Created) response message that includes the resource identifier of the newly created Provisioning Session in the body of the reply and the URL of the resource, including its resource identifier, shall be returned as part of the HTTP Location header field.

##### 4.2.3.2.3	Retrieve Provisioning Session properties

This procedure is used by the Provisioning AF to obtain the properties of an existing Provisioning Session from the Data Collection AF. The HTTP GET method shall be used for this purpose.

##### 4.2.3.2.4	Update Provisioning Session properties

The Update operation is not allowed on Provisioning Session resources.

##### 4.2.3.2.5	Destroy Provisioning Session

This procedure is used by the Provisioning AF to destroy a Provisioning Session. The Data Collection AF shall use the HTTP DELETE method for this purpose.

As a side-effect of destroying a Provisioning Session, the Data Collection AF shall release any associated resources, purge any cached data, and delete all UE data reporting configurations associated with this Provisioning Session.

#### 4.2.3.3	Data Reporting Provisioning procedures

##### 4.2.3.3.1	General

Upon the successful creation of a Provisioning Session, the Provisioning AF shall use the procedures defined in this clause to configure UE data collection and reporting functionality specific to an application in the Data Collection AF. This clause defines the basic procedures. Additional details, including definition of the Data Reporting Provisioning Sessions API are provided under clause 6.3.

A given instance of a Data Reporting Configuration resource is identified by the dataReportingConfigurationType and dataReportingConfigurationId properties of the DataReportingConfiguration resource, The properties of that resource, as described in the following sub-clauses, pertain to UE data collection and reporting by different Data Collection Clients to the Data Collection AF.

##### 4.2.3.3.2	Data Reporting Configuration types

The type of a Data Reporting Configuration resource is identified by the dataCollectionClientType property of the ProvisioningSession resource as specified in clause 6.2.3.

NOTE:	The dataCollectionClientType property corresponds to the Data collection client type parameter in table 4.6.2-1 of TS 26.531 [7].

Editor’s Note: Define a common enumeration DataCollectionClientType in clause 5.4.

##### 4.2.3.3.3	Create Data Reporting Configuration

This procedure is used by the Provisioning AF to create a Data Reporting Configuration resource within the scope of a particular Provisioning Session. The HTTP POST method shall be used for this purpose and the request message body may include a DataReportingConfiguration resource, as specified under clause 6.3.

Editor’s Note: Describe key attributes of the Data Reporting Configuration resource here.

Upon success, the Data Collection AF shall respond with a 201 (Created) response message and the resource URL for the newly-created Data Reporting Configuration resource shall be returned in the Location header field.

If the procedure is unsuccessful, the Data Collection AF shall provide a response code as defined in clause 5.3.

This procedure may be performed multiple times to provision different Data Reporting Configurations in the scope of a particular Provisioning Session.

##### 4.2.3.3.4	Retrieve Data Reporting Configuration

This procedure is used by the Provisioning AF to obtain the properties of an existing Data Reporting Configuration resource from the Data Collection AF. The HTTP GET method shall be used for this purpose.

If successful, the Data Collection AF shall respond with a 200 (OK) and the requested DataReportingConfiguration resource shall be returned in the body of the HTTP response message.

If the procedure is unsuccessful, the Data Collection AF shall provide a response code as defined in clause 5.3.

##### 4.2.3.3.5	Update Data Reporting Configuration

The update operation is invoked by the Provisioning AF to initially upload the representation of a Data Reporting Configuration resource created using the procedure in clause 4.2.3.3.3 or, in the case of an existing Data Reporting Configuration resource, to entirely replace or modify certain properties of that resource. All available properties may be updated. The HTTP PATCH or HTTP PUT methods shall be used for the update operation.

If the procedure is successful, the Data Collection AF shall respond with a 200 (OK) reflecting the successful update operation.

If the procedure is unsuccessful, the Data Collection AF shall provide a response code as defined in clause 5.3.

##### 4.2.3.3.6	Destroy Data Reporting Configuration

This operation is used by the Provisioning AF to destroy a Data Reporting Configuration resource and to terminate the related UE data reporting procedure. The HTTP DELETE method shall be used for this purpose.

As a result, the Data Collection AF should release any associated resources, deliver any pending data to subscribed recipients, as appropriate to the Event ID in question, and delete any corresponding configurations.

If the procedure is successful, the Data Collection AF shall respond with a 200 (OK) response message.

If the procedure is unsuccessful, the Data Collection AF shall provide a response code as defined in clause 5.3.

### 4.2.4	Configuration of Indirect Data Collection Client

Indirect reporting operation involves first a UE Application instance sending domain-specific UE data to an Application Service Provider (ASP) server instance across reference point R8. That UE data is passed from to an Indirect Data Reporting Client function operated by the Application Service Provider to be subsequently sent as data reports, possibly in processed form, to a Data Collection AF instance. Operation of the latter procedure is conditioned upon the Indirect Data Collection Client having acquired its data collection and reporting configuration from the Data Collection AF by means of the Ndcaf_DataReporting service (either directly across the reference point R3 or via an equivalent service exposed by the NEF, depending on whether the Indirect Data Collection Client and the Data Collection AF reside in the same or separate trust domains).

The Indirect Data Collection Client shall obtain its configuration by invoking the Data Collection and Reporting Configuration API associated with the Ndcaf_DataReporting service, as described under clause 7.2.

The configuration information is contained in a generic data collection and reporting configuration envelope that shall include at minimum the baseline configuration parameters defined in clause 4.6.3 of TS 26.531 [7]. In particular, the configuration shall specify the domain-specific parameters associated with the specified Event ID(s) to be reported to the Data Collection AF.

### 4.2.5	Configuration of Application Server

An Application Server (AS) instance, as a type of data collection client, acquires its domain-specific data collection and reporting configuration from a Data Collection AF instance by means of the Ndcaf_DataReporting service (either directly across reference point R4 or via an equivalent service exposed by the NEF, depending on whether the AS and the Data Collection AF reside in the same or separate trust domains).

Similar to clause 4.2.4, the AS shall obtain its configuration by invoking the Data Collection and Reporting Configuration API associated with the Ndcaf_DataReporting service, as described under clause 7.2.

The configuration information is contained in a generic data collection and reporting configuration envelope that shall include at minimum the baseline configuration parameters defined in clause 4.6.3 of TS 26.531 [7]. The configuration shall specify the domain-specific parameters associated with the specified Event ID(s) to be reported to the Data Collection AF.

### 4.2.6	Indirect data reporting

### 4.2.7	Reporting by Application Server

### 4.2.8	Event subscription, management and publication

This clause pertains to the use of the Naf_EventExposure service API as defined in TS 29.517 [5] and invoked by the NWDAF or an Application Server Provider AF to subscribe to and receive UE data related event information from a Data Collection AF.

## 4.3	UE-to-network procedures

### 4.3.1	General

This clause specifies the procedures used between the UE and Network Functions in support of provisioning a data collection and reporting configuration in the UE’s Direct Data Collection Client, and subsequent reporting of the collected UE data to the Data Collection AF.

### 4.3.2	Configuration of Direct Data Reporting Client

A Direct Data Reporting Client instance acquires its domain-specific data collection and reporting configuration from a Data Collection AF instance by means of the Ndcaf_DataReporting service across reference point R2.

The Direct Data Reporting Client shall obtain its configuration by invoking the Data Collection and Reporting Configuration API associated with the Ndcaf_DataReporting service, as described under clause 7.2.

The configuration information is contained in a generic data collection and reporting configuration envelope that shall include at minimum the baseline configuration parameters defined in clause 4.6.3 of TS 26.531 [7]. The configuration shall specify the domain-specific parameters associated with the specified Event ID(s) to be reported to the Data Collection AF.

### 4.3.3	Direct data reporting

After acquiring its data collection and configuration from the Data Collection AF, and in accordance with this configuration, the Direct Data Collection Client shall send domain-specific data reports to the Data Collection AF by invoking the Data Reporting API associated with Ndcaf_DataReporting service across reference point R2 as described under clause 7.3. The data reports shall be supplied in a generic data report envelope that includes at minimum the baseline information for data reporting defined in clause 4.6.4 of TS 26.531 [7].

## 4.4	UE-internal procedures

### 4.4.1	General

This clause specifies the procedures used by internal UE entities, namely a UE Application and the associated Direct Data Collection Client, in support of UE data collection by the Direct DC-Client for subsequent reporting to the DC-AF.

# 5	General Aspects of APIs for Data Collection and Reporting

## 5.1	Overview

## 5.2	HTTP resource URIs and paths

## 5.3	Usage of HTTP

## 5.4	Common API data types

## 5.5	Explanation of API data model notation

# 6	Ndcaf_DataReportingProvisioning service

## 6.1	General

This clause specifies the API used to provision data collection and reporting in the Data Collection AF.

## 6.2	Provisioning Sessions API

### 6.2.1	Overview

This clause specifies the provisioning API used by an Application Service Provider server to provision a data collection and reporting configuration in a Data Collection AF.

### 6.2.2	Resource structure

### 6.2.3	Data model

### 6.2.4	Mediation by NEF

## 6.3	Data Reporting Provisioning API

### 6.3.1	Overview

### 6.3.2	Resource structure

### 6.3.3	Data model

### 6.3.4	Mediation by NEF

# 7	Ndcaf_DataReporting service

## 7.1	General

This clause specifies the APIs used by clients of the Data Collection AF to obtain a data collection and reporting configuration from. and then report data to, the Data Collection AF.

## 7.2	Data Collection and Reporting Configuration API

### 7.2.1	Overview

This clause specifies the configuration API used by data collection clients to obtain their data collection and reporting configurations from the Data Collection AF.

### 7.2.2	Resources

#### 7.2.2.1	Resource Structure

Figure 7.2.2.1-1 depicts the URL path model for the Data Reporting Sessions resource collection and the Data Reporting Session resources of the Ndcaf_DataReporting service.

/sessions

{apiRoot}/ndcaf_data-reporting/v1

/{sessionId}

Figure 7.2.2.1-1: URL path model of Data Reporting Session related resources

Table 7.2.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.2.2.1-1: Resources and methods overview

| Service name | Operation name | Resource name | Resource path suffix | HTTP method | Description |
| --- | --- | --- | --- | --- | --- |
| Ndcaf_DataReporting | CreateSession | Data Reporting Sessions | /sessions | POST | Data collection client establishes a UE data reporting session with the Data Collection AF, providing information about what UE data it can report, and is provided with a configuration in response. |
|  | RetrieveSession | Data Reporting Session | /sessions/{sessionId} | GET | Retreives a Data Reporting Session resource from the Data Collection AF. |
|  | UpdateSession |  |  | PUT | Modifies an existing Data Reporting Session resource . |
|  | DestroySession |  |  | DELETE | Destroys a Data Reporting Session resource. |

#### 7.2.2.2	Data Reporting Sessions resource collection

##### 7.2.2.2.1	Description

The Data Reporting Sessions resource collection represents the set of all Data Collection Sessions at a given Data Collection AF (service) instance. The resource collection allows a data collection client to create a new Data Reporting Session resource at, and to receive configuration details for that session from, the Data Collection AF.

##### 7.2.2.2.2	Resource definition

Resource URL: {apiRoot}/ndcaf_data-reporting/v1/sessions

This resource shall support the resource URL variables defined in table 7.2.2.2.2-1.

Table 7.2.2.2.2-1: Resource URL variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Fully-Qualified Doman Name of the Data Collection AF and path prefix. |

##### 7.2.2.2.3	Resource Standard Methods

###### 7.2.2.2.3.1	Ndcaf_DataReporting_CreateSession operation using POST method

This method shall support the URL query parameters specified in table 7.2.2.2.3.1-1.

Table 7.2.2.2.3.1-1: URL query parameters supported by the POST method on this resource

| Parameter | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.2.2.2.3.1-2 and the response data structures and response codes specified in table 7.2.2.2.3.1-4.

Table 7.2.2.2.3.1-2: Data structures supported by the POST request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataReportingSession | M | 1 | Data supplied by the data collection client to enable creation of a new Data Reporting Session at the Data Collection AF. |

Table 7.2.2.2.3.1-3: Headers supported for POST requests on this resource

| HTTP request header | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Authorization | string | M | 1 | For authentication of the data collection client. (NOTE 1) |
| Origin | string | O | 0..1 | Indicates the origin of the requester. (NOTE 2) |
| NOTE 1: If OAuth2.0 authorization is used the value would be “Bearer” followed by a string representing the token, see section 2.1 of RFC 6750 [8]. NOTE 2: The Origin header is always supplied if the data collection client is deployed in a web browser. |  |  |  |  |

Table 7.2.2.2.3.1-4: Data structures supported by the POST response body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataReportingSession | M | 1 | 201 Created | The creation of a Data Reporting Session is confirmed and configuration data for the data collection client for the session is provided by the Data Collection AF. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [9] also apply. |  |  |  |  |

Table 7.2.2.2.3.1-5: Headers supported by the 201 (Created) response code on this resource

| HTTP response header | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | The URL of the newly created resource at the Data Collection AF, according to the structure: {apiRoot}/ndcaf-datareporting/v1/sessions/{sessionId} |
| Access-Control-Allow-Origin | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. |
| Access-Control-Allow-Methods | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Valid values: POST, PUT, DELETE |
| Access-Control-Expose-Headers | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Valid values: Location |

NOTE:	Standard HTTP redirection using a 3xx response code with the Location header as well as Alt-Svc are allowed.

#### 7.2.2.3	Data Reporting Session resource

##### 7.2.2.3.1	Description

The Data Reporting Session resource represents a single session within the collection of Data Reporting Sessions at a given Data Collection AF.

##### 7.2.2.3.2	Resource definition

Resource URL: {apiRoot}/nnwdaf-eventssubscription/v1/sessions/{sessionionId}

This resource shall support the resource URI variables defined in table 7.2.2.3.2-1.

Table 7.2.2.3.2-1: Resource URL variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.2.2.2.2 |
| sessionId | string | Identifies a session to the Ndcaf_DataReporting_Sessions Service |

##### 7.2.2.3.3	Resource standard methods

###### 7.2.2.3.3.1	Ndcaf_DataReporting_RetrieveSession operation using GET method

Editor’s Note: To be added.

###### 7.2.2.3.3.2	Ndcaf_DataReporting_UpdateSession operation using PUT method

This method shall support the URL query parameters specified in table 7.2.2.3.3.2-1.

Table 7.2.2.3.3.2-1: URL query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.2.2.3.3.2-2 and the response data structures and response codes specified in table 7.2.2.3.3.2-4.

Table 7.2.2.3.3.2-2: Data structures supported by the PUT request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataReportingSession | M | 1 | Parameters to replace data collection client provided configuration data for a Data Reporting Session resource. |

Table 7.2.2.3.3.2-3: Headers supported for PUT requests on this resource

| HTTP request header | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Authorization | string | M | 1 | For authentication of the data collection client. NOTE1 |
| Origin | string | O | 0..1 | Indicates the origin of the requester. NOTE2 |
| NOTE 1: If OAuth2.0 authorization is used the value would be “Bearer” followed by a string representing the token, see section 2.1 RFC 6750 [8]. NOTE 2: The Origin header is always supplied if the data collection client is deployed in a Web Browser. |  |  |  |  |

Table 7.2.2.3.3.2-4: Data structures supported by the PUT response body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataReportingSession | M | 1 | 200 OK | The Data Reporting Session resource was modified successfully by configuration data provided by the data collection client. |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during a Data Reporting Session modification. The response shall include a Location header field containing an alternative URL of the resource located in another Data Collection AF (service) instance. Applicable if the feature "ES3XX" (Extended Support of HTTP 307/308 redirection as defined in TS 29.502 [11]) is supported. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during a Data Reporting Session modification. The response shall include a Location header field containing an alternative URL of the resource located in another Data Collection AF (service) instance. Applicable if the feature "ES3XX" is supported. |
| ProblemDetails | O | 0..1 | 404 Not Found | This Data Reporting Session resource does not exist. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [9] also apply. NOTE 2: Failure cases are described in subclause 7.2.4. |  |  |  |  |

Table 7.2.2.3.3.2-5: Headers supported by the 200 response code on this resource

| HTTP response header | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Access-Control-Allow-Origin | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. |
| Access-Control-Allow-Methods | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Valid values: POST, PUT, DELETE. |
| Access-Control-Expose-Headers | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Valid values: Location. |

Table 7.2.2.3.3.2-6: Headers supported by the 307 and 308 response codes on this resource

| HTTP response header | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URL of the resource located in another Data Collection AF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |
| Access-Control-Allow-Origin | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. |
| Access-Control-Allow-Methods | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Valid values: POST, PUT, DELETE |
| Access-Control-Expose-Headers | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Valid values: Location |

###### 7.2.2.3.3.1	Ndcaf_DataReporting_DestroySession operation using DELETE method

This method shall support the URL query parameters specified in table 7.2.2.3.3.1-1.

Table 7.2.2.3.3.1-1: URL query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.2.2.3.3.1-2 and the response data structures and response codes specified in table 7.2.2.3.3.1-4.

Table 7.2.2.3.3.1-2: Data structures supported by the DELETE request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |

Table 7.2.2.3.3.1-3: Headers supported for DELETE requests on this resource

| HTTP request header | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Authorization | string | M | 1 | For authentication of the data collection client. (NOTE 1) |
| Origin | string | O | 0..1 | Indicates the origin of the requester. (NOTE 2) |
| NOTE 1: If OAuth2.0 authorization is used the value would be “Bearer” followed by a string representing the token, see section 2.1 of RFC 6750 [12]. NOTE 2: The Origin header is always supplied if the data collection client is deployed in a web browser. |  |  |  |  |

Table 7.2.2.3.3.1-4: Data structures supported by the DELETE response body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case: The Data Reporting Session resource matching the sessionId was destroyed at the Data Collection AF. |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection during Data Reporting Session destruction. The response shall include a Location header field containing an alternative URL of the resource located in another Data Collection AF (service) instance. Applicable if the feature "ES3XX" as defined in TS 29.502 [11] is supported. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection during Data Reporting Session destruction. The response shall include a Location header field containing an alternative URL of the resource located in another Data Collection AF (service) instance. Applicable if the feature "ES3XX" is supported. |
| ProblemDetails | O | 0..1 | 404 Not Found | The Data Reporting Session resource does not exist. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [9] also apply. NOTE 2: Failure cases are described in subclause 7.2.4. |  |  |  |  |

Table 7.2.2.3.3.1-5: Headers supported by the 204 response code on this resource

| HTTP response header | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Access-Control-Allow-Origin | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. |
| Access-Control-Allow-Methods | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Valid values: POST, PUT, DELETE. |
| Access-Control-Expose-Headers | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Valid values: Location. |

Table 7.2.2.3.3.1-6 Headers supported by the 307 and 308 response codes on this resource

| HTTP response header | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URL of the resource located in another Data Collection AF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |
| Access-Control-Allow-Origin | string | O | 0..1 | Part of CORS [10].Supplied if the request included the Origin header. |
| Access-Control-Allow-Methods | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Valid values: POST, PUT, DELETE. |
| Access-Control-Expose-Headers | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Valid values: Location. |

### 7.2.3	Data Model

#### 7.2.3.1	General

Table 7.2.3.1-1 specifies the data types used by the Ndcaf_DataReporting_CreateSession, Ndcaf_DataReporting_‌Retrieve‌Session, Ndcaf_DataReporting_UpdateSession and Ndcaf_DataReporting_DestroySession operations.

Table 7.2.3.1-1: Data types specific to Ndcaf_DataReporting_CreateSession, Ndcaf_DataReporting_RetrieveSession, Ndcaf_DataReporting_UpdateSession and Ndcaf_DataReporting_DestroySession operations

| Data type | Clause defined | Description |
| --- | --- | --- |
| DataReportingSession | 7.2.3.2.1 | Configuration by the Data Collection AF of the data collection client, specifying the data to be reported. |

Table 7.2.3.1-2 specifies data types re-used from other specifications by the Ndcaf_DataReporting_CreateSessions, Ndcaf_DataReporting_RetrieveSession, Ndcaf_DataReporting_UpdateSession and Ndcaf_DataReporting_‌Destroy‌Session operations, including a reference to their respective specifications.

Table 7.2.3.1-2: Externally defined data types used by  Ndcaf_DataReporting_CreateSession, Ndcaf_DataReporting_RetrieveSession, Ndcaf_DataReporting_UpdateSession and Ndcaf_DataReporting_DestroySession operations

| Data type | Comments | Reference |
| --- | --- | --- |
| ApplicationId | Identifies the reporting application. | 3GPP TS 29.571 [13] |
| DateTime |  |  |
| DurationSec |  |  |
| Double |  |  |
| Float |  |  |
| Int32 |  |  |
| Int64 |  |  |
| Uint16 |  |  |
| Uint32 |  |  |
| Uint64 |  |  |
| Uinteger |  |  |

#### 7.2.3.2	Structured data types

##### 7.2.3.2.1	DataReportingSession resource type

Table 7.2.3.2.1-1: Definition of DataReportingSession resource type

| Property name | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| sessionId | string | 1 | C: RO R: RO U: RO | Unique identifier for this Data Reporting Session assigned by the Data Collection AF. |
| externalApplicationId | ApplicationID | 1 | C: RW R: RO U: RW | The external application identifier, nominated by the data collection client, to which this Data Reporting Session pertains. |
| supportedDomains | array(DataDomain) | 1 | C: RW R: RO U: RW | Set of domains for which the data collection client declares that it is able to report UE data. (See clause 7.2.3.3.1). An empty array indicates that no UE data can currently be reported. |
| reportForDomains | array(DataDomain) | 0..1 | C: RO R: RO U: RO | Subset of supportedDomains above for which the data collection client is requested to report UE data. If the Data Collection AF signals an empty array, no UE data should be reported. |
| reportingCondition | ReportCondition | 0..1 | C: — R: RO U: RO | The condition for reporting, signalled by the Data Collection AF. (See clause 7.2.3.2.2.) |

##### 7.2.3.2.2	ReportCondition type

Table 7.2.3.2.2-1: Definition of ReportCondition type

| Property name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| type | ConditionType | M | 1 | Type of condition, see clause 7.2.3.3.2 |
| intervalLength | DurationSec | C | 0..1 | Only applicable when type is INTERVAL. (NOTE 1) |
| threshold | Double, Float, Int32, Int64, Uint16, Uint32, Uint64, or Uinteger | C | 0..1 | Only applicable when type is THRESHOLD. (NOTE 1) |
| parameter | string | C | 0..1 | Only applicable when type is THRESHOLD. (NOTE 1) |
| reportWhenBelow | boolean | C | 0..1 | Only applicable when type is THRESHOLD. (NOTE 1) |
| event | Event | C | 0..1 | Only applicable when type is EVENT. (NOTE 2) |
| NOTE 1: See clause 7.2.3.3.2 and table 7.2.3.1-2. NOTE 2: See clauses 7.2.3.3.2 and 7.2.3.3.3. |  |  |  |  |

#### 7.2.3.3	Simple data types and enumerations

##### 7.2.3.3.1	DataDomain enumeration

Table 7.2.3.3.1-1: DataDomain enumeration

| Enumeration value | Description | Applicability (refer to Table 7.3.3.2.1-1) |
| --- | --- | --- |
| SERVICE_EXPERIENCE | Service Experience related data. | serviceExperienceRecords |
| LOCATION | Location data. | locationRecords |
| COMMUNICATION | Communication data. | communicationRecords |
| PERFORMANCE | Performance data. | performanceDataRecords |
| APPLICATION_SPECIFIC | Combination of QoE metrics and application service-specific data | applicationSpecificRecords |
| DL_ACCESS_RECORDS | 5GMS downlink access data. | mediaStreamingDownlinkAccessRecords |
| PLANNED_TRIPS | Data related to planned trips. | tripPlanRecords |

##### 7.2.3.3.2	ConditionType enumeration

Table 7.2.3.3.2-1: ConditionType enumeration

| Enumeration value | Description |
| --- | --- |
| INTERVAL | Report at a regular interval. |
| THRESHOLD | Report when a threshold is passed. |
| EVENT | Report on event. |
| OFF | Do not report. |

##### 7.2.3.3.3	Event enumeration

This clause lists the possible events (EVENT in table 7.2.3.3.2-1) that can be used to trigger a report.

Table 7.2.3.3.3-1: Event enumeration

| Enumeration value | Description |
| --- | --- |
| DESTINATION | A new destination has been recorded (refer to clause A.7). |

### 7.2.4	Error handling

Editor’s Note: TBA

### 7.2.5	Mediation by NEF

Editor’s Note: TBA

## 7.3	Data Reporting API

### 7.3.1	Overview

This clause specifies the reporting API used by a data collection client to report UE data that has been collected to the Data Collection AF.

### 7.3.2	Resources

#### 7.3.2.1	Resource structure

/sessions

{apiRoot}/ndcaf_data-reporting/v1

/{sessionId}

/report

Figure 7.2.2.1-1 depicts the URL path model for the Data Report resource pertaining to an established Data Reporting Session of the Ndcaf_DataReporting service.

Figure 7.3.2.1-1: URL path model of Data Report resource

Table 7.3.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.3.2.1-1: Resources and methods overview

| Service name | Operation name | Resource name | Resource path suffix | HTTP method | Description |
| --- | --- | --- | --- | --- | --- |
| Ndcaf_DataReporting | Report | Data Report | /sessions/{sessionId}/report | POST | Data collection client reports data to the Data Collection AF via the established session. |

#### 7.3.2.2	Data Report resource

##### 7.3.2.2.1	Description

The Data Report resource allows a data collection client to report data pertaining to an established Data Reporting Session to the Data Collection AF. The Data Collection AF can provide an updated configuration in the response.

##### 7.3.2.2.2	Resource definition

Resource URL: {apiRoot}/ndcaf_data-reporting/v1/sessions/{sessionId}/report

This resource shall support the resource URL variables defined in table 7.3.2.2.2-1.

Table 7.3.2.2.2-1: Resource URL variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Fully-Qualified Domain Name of the Data Collection AF and path prefix. |
| sessionId | string | Identifier of the Data Reporting Session unique within the scope of the Data Collection AF. |

##### 7.3.2.2.3	Resource Standard Methods

###### 7.3.2.2.3.1	Ndcaf_DataReporting_Report operation using POST method

This method shall support the URI query parameters specified in table 7.3.2.2.3.1-1.

Table 7.3.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Parameter | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.3.2.2.3.1-2 and the response data structures and response codes specified in table 7.3.2.2.3.1-4.

Table 7.3.2.2.3.1-2: Data structures supported by the POST request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataReport | M | 1 | UE data reported by the data collection client. |

Table 7.3.2.2.3.1-3: Headers supported for POST requests on this resource

| HTTP request header | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Authorization | string | M | 1 | For authentication of the data collection client. (NOTE 1) |
| Origin | string | O | 0..1 | Indicates the origin of the requester. (NOTE 2) |
| NOTE 1: If OAuth2.0 authorization is used the value would be “Bearer” followed by a string representing the token, see section 2.1 of RFC 6750 [12]. NOTE 2: The Origin header is always supplied if the data collection client is deployed in a web browser. |  |  |  |  |

Table 7.3.2.2.3.1-4: Data structures supported by the POST response body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataReportingSession | O | 0..1 | 200 OK | The report was accepted by the Data Collection AF. A data collection client configuration (updated or unchanged) may optionally be provided in the response. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [9] also apply. |  |  |  |  |

Table 7.3.2.2.3.1-5: Headers supported by the 200 response code on this resource

| HTTP response header | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Access-Control-Allow-Origin | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. |
| Access-Control-Allow-Methods | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Value: POST |

NOTE:	Standard HTTP redirection (using a 3xx response with a Location response header) as well as Alt-Svc are allowed for this method.

### 7.3.3	Data Model

#### 7.3.3.1	General

Table 7.3.3.1-1 specifies the data types used by the Ndcaf_DataReporting_Report operation.

Table 7.3.3.1-1: Data types specific to Ndcaf_DataReporting_Report operation

| Data type | Clause defined | Description |
| --- | --- | --- |
| DataReport | 7.3.3.2.1 | Reported data by the data collection client to the Data Collection AF. |
| DataReportingSession | 7.2.3.2.1 | Configuration of the data collection client. |

Table 7.3.3.1-2 specifies data types re-used from other specifications by the Ndcaf_DataReporting_Report operation, including a reference to their respective specifications.

Table 7.3.3.1-2: Externally defined data types used by Ndcaf_DataReporting_Report operation

| Data type | Comments | Reference |
| --- | --- | --- |
| ApplicationId | Identifies the reporting application. | 3GPP TS 29.571 [13] |

#### 7.3.3.2	Structured data types

##### 7.3.3.2.1	DataReport type

Table 7.3.3.2.1-1: Definition of DataReport type

| Property name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| externalApplicationId | ApplicationID | 1 | External application identifier. |
| serviceExperienceRecords | array(ServiceExperienceRecord) | 0..1 (NOTE) | See clause A.2. |
| locationRecords | array(LocationRecord) |  | See clause A.3. |
| communicationRecords | array(CommunicationRecord) |  | See clause A.4. |
| performanceDataRecords | array(PerformanceDataRecord) |  | See clause A.5. |
| applicationSpecificRecords | array(ApplicationSpecificRecord) |  | See clause A.6. |
| tripPlanRecords | array(TripPlanRecord) |  | See clause A.7. |
| mediaStreaming‌DownlinkAccessRecords | array(MediaStreaming‌DownlinkAccessRecord) |  | See TS 26.512 [14] clause X. |
| NOTE: At least one of these properties must be present in a DataReport. |  |  |  |

#### 7.3.3.3	Simple data types and enumerations

There are no simple data types and enumerations specified in this release.

### 7.3.4	Mediation by NEF

# 8	Client API

## 8.1	General

This clause specifies the API used by internal UE entities, namely a UE Application and the associated Direct Data Collection Client, in support of UE data collection by the Direct Data Collection Client for subsequent reporting to the Data Collection AF.

# 9	Security and Access Control

This clause specifies the security functionality associated with UE data collection, reporting and exposure.

Editor’s Note: Text in this clause is expected to be based on the TS 26.531 stage 2 description of the Access Profile data model (also referred to as the combination of event processing instructions and event data restrictions).

Annex A (normative):
Data reporting data models

# A.1	Introduction

This annex describes the format of the data reports submitted to the Data Collection AF, as required by TS 23.288 [4].

Table A.1-1 specifies data types re-used from other specifications, including a reference to their respective specifications.

Table A.1-1: Externally defined data types used by Ndcaf_DataReporting_Report operation

| Data type | Comments | Reference |
| --- | --- | --- |
| BitRate |  | 3GPP TS 29.571 [13] |
| PacketDelBudget |  |  |
| PacketLossRate |  |  |
| DateTime |  |  |
| DurationSec |  |  |
| SvcExperience |  | 3GPP TS 29.517 [5] |
| AddrFqdn |  |  |
| TimeWindow |  | 3GPP TS 29.122 [15] |
| Volume |  |  |
| FlowInfo |  |  |
| LocationData5G |  | 3GPP TS 29.572 [16] |
| HorizontalSpeed |  |  |

# A.2	Service Experience reporting

## A.2.1	ServiceExperienceRecord type

Table A.2.1-1: Definition of ServiceExperienceRecord type

| Property name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| timestamp | DateTime | 1 | Time stamp of this record. |
| serviceExperienceInfos | array(PerFlowServiceExperienceInfo) | 1 | See clause A.2.2. |

## A.2.2	PerFlowServiceExperienceInfo type

Table A.2.2-1: Definition of PerFlowServiceExperienceInfo type

| Property name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| serviceExperience | SvcExperience | 1 |  |
| timeInterval | TimeWindow | 1 |  |
| remoteEndpoint | array(AddrFqdn) | 1 | FQDN or IP Address of remote endpoint (e.g., server). |

# A.3	Location reporting

## A.3.1	LocationRecord type

Table A.3.1-1: Definition of type LocationRecord

| Attribute name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| timestamp | DateTime | 1 | Time stamp of this record. |
| location | LocationData5G | 1 |  |

# A.4	Communication reporting

## A.4.1	CommunicationRecord type

Table A.4.1-1: Definition of type CommunicationRecord

| Attribute name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| timestamp | DateTime | 1 | Time stamp of this record. |
| timeInterval | TimeWindow | 1 |  |
| uplinkVolume | Volume | 0..1 | See NOTE. |
| downlinkVolume | Volume | 0..1 | See NOTE. |
| NOTE: at least one of uplinkVolume and downlinkVolume must be present |  |  |  |

# A.5	Performance Data reporting

## A.5.1	PerformanceDataRecord type

Table A.5.1-1: Definition of type PerformanceDataRecord

| Attribute name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| timestamp | DateTime | 1 | Time stamp of this record. |
| timeInterval | TimeWindow | 1 |  |
| location | LocationArea5G | 0..1 | Represents the UE location. |
| remoteEndpoint | AddrFqdn | 0..1 | FQDN or IP Address of remote endpoint (e.g., server) |
| packetDelayBudget | PacketDelayBudget | 0..1 | Indicates average Packet Delay. |
| packetLossRate | PacketLossRate | 0..1 | Indicates average Packet Loss Rate. |
| uplinkThroughput | BitRate | 0..1 | Indicates the average uplink throughput. |
| downlinkThroughput | BitRate | 0..1 | Indicates the average downlink throughput. |

# A.6	Application-specific reporting

## A.6.0	Introduction

Application-specific reporting is intended to allow reporting of any application-specific data.

## A.6.1	ApplicationSpecificRecord type

Different services will have different data that are of interest (depending on the service type). The ApplicationSpecificRecord type is intended to enable services to report data specific for the service or application.

Editor’s Note: To do that the recordIdentifier in form of a unique uri (see Table A.6.1-1) must be provided, and that uri must match a specification of the container used to hold the data.

Table A.6.1-1: Definition of ApplicationSpecificRecord type

| Property name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| timestamp | DateTime | 1 | Time stamp of this record. |
| recordIdentifier | string | 1 | A controlled term in form of a URI that uniquely identifies the type of record that follows. |
| recordContainer | Object container or array container | 1 | Container with the actual application-specific data. |

# A.7	Trip Plan reporting

## A.7.0	Introduction

Trip Plan(s) enable the Data Collection AF to identify collective behavior amongst UEs. See tables 6.5.2-4 and 6.5.2-5 in 3GPP TS 23.288 [4].

## A.7.1	TripPlanRecord type

Table A.7.1-1: Definition of TripPlanRecord type

| Property name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| timestamp | DateTime | 1 | Time stamp of this record. |
| startingPoint | LocationData5G | 1 | The starting point of the planned trip. |
| destination | LocationData5G | 1 | The destination of the planned trip. |
| Route | array(LocationData5G) | 0..1 | The route of the planned trip. |
| averageSpeed | HorizontalSpeed | 0..1 | Estimated average speed of the planned trip. |
| arrivalTime | DateTime | 0..1 | Estimated time of arrival at the destination of the planned trip. |

Annex B (normative):
OpenAPI representation of REST APIs for data collection and reporting

# A.1	General

This annex is based on the OpenAPI 3.0.0 specification [17] and provides corresponding representations of all APIs defined in the present document.

NOTE 1:	An OpenAPIs representation embeds JSON Schema representations of HTTP message bodies.

This Annex shall take precedence when being discrepant to other parts of the present document with respect to the encoding of information elements and methods within the API(s).

NOTE 2:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Annex <C> (informative):
<Informative annex for a Technical Specification>

Informative annexes may appear in both Technical Specifications and Technical Reports. Use style "Heading 8" for use in TSs.

Informative annexes shall not contain requirements for the implementation of the Technical Specification.

# C.1	Heading levels in an annex

Heading levels within an annex are used as in the main document, but for Heading level selection, the "A.", "B.", etc. are ignored. e.g. B.1.2 is formatted using Heading 2 style.

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-08 | Sa4#115-e | S4-211220 |  |  |  | Initial specification skeleton | 0.1.0 |
| 2021-11 | SA4#116-e | S4-211422 S4-211578 S4-211593 |  |  |  | Document reorganization, and miscellaneous modifications and corrections to previous document outline. Added text under clause previously empty clause 4.2.3. Added text to previously empty clauses 4.2.4, 4.2.5, 4.2.6, 4.2.7, 4.3.2 and 4.3.3 | 0.2.0 |
| 2022-02 | SA4#117-e | S4-220xxx other agreed CRs to TS 25.532 |  |  |  | Additional references under clause 2, corrections and added text under clauses 7.2 and 7.3, new Annex A, and demoted existing Annexes A and B by one level. Possibly additional changes reflecting other agreed CRs to TS 26.532 at SA4#117-e | 0.3.0 |