3GPP TSG-SA4 Meeting #117-e	S4-220075

14th – 22nd February 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | 0015 | rev |  | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | dCR on Edge Provisioning for Media Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incoroporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS_EDGE_3 |  |  |  |  |  | Date: |  |  | 8th February 2022 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR introduces extensions to the 5GMS provisioning procedures to add support for provisioning edge resources for media services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new API on the provisioning of edge resources and the corresponding REST resource definitions are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G media sessions will not be able to make use of edge resources. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2, 5.2, 6.4.3.8, 6.4.3.9, 6.4.4.4, 6.4.4.5, 7.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 2	References

## …

[42]	3GPP TS29.558, Enabling Edge Applications; Application Programming Interface (API) specification; Stage 3

2nd Change

## 4.2	APIs relevant to Downlink Media Streaming

Table 4.2-1 summarises the APIs used to provision and use the various downlink media streaming features specified in TS 26.501 [2].

Table 4.2-1: Summary of APIs relevant to downlink media streaming features

| 5GMSd feature | Abstract | Relevant APIs |  |  |
| --- | --- | --- | --- | --- |
|  |  | Interface | API name | Clause |
| Content protocols discovery | Used by the 5GMSd Application Provider to interrogate which content ingest protocols are supported by 5GMSd AS(s). | M1d | Content Protocols Discovery API | 7.5 |
| Content hosting | Content is ingested, hosted and distributed by the 5GMSd AS according to a Content Hosting Configuration associated with a Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Server Certificates Provisioning API | 7.3 |
|  |  |  | Content Preparation Templates Provisioning API | 7.4 |
|  |  |  | Content Hosting Provisioning API | 7.6 |
|  |  | M2d | HTTP-pull based content ingest protocol | 8.2 |
|  |  |  | DASH-IF push based content ingest protocol | 8.3 |
|  |  | M4d | DASH [4] or 3GP [37] | 10 |
|  |  | M5d | Service Access Information API | 11.2 |
| Metrics reporting | The 5GMSd Client uploads metrics reports to the 5GMSd AF according to a provisioned Metrics Reporting Configuration it obtains from the Service Access Information for its Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Metrics Reporting Provisioning API | 7.8 |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  |  | Metrics Reporting API | 11.4 |
| Consumption reporting | The 5GMSd Client provides feedback reports on currently consumed content according to a provisioned Consumption Reporting Configuration it obtains from the Service Access Information for its Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Consumption Reporting Provisioning API | 7.7 |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  |  | Consumption Reporting API | 11.3 |
| Dynamic Policy invocation | The 5GMSd Client activates different traffic treatment policies selected from a set of Policy Templates configured in its Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Policy Templates Provisioning API | 7.9 |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  |  | Dynamic Policies API | 11.5 |
| Network Assistance | The 5GMSd Client requests bit rate recommendations and delivery boosts from the 5GMSd AF. | M5d | Service Access Information API | 11.2 |
|  |  |  | Network Assistance API | 11.6 |
| Edge resources | Edge resources are provisioned for 5GMS media sessions. | M1d | Edge Resource Provisioning API | 7.10 |
|  |  | M5d | Service Access Information API | 11.2 |

3rd Change

## 5.2	APIs relevant to Uplink Media Streaming

Table 5.2-1 summarises the APIs used to provision and use the various uplink media streaming features specified in TS 26.501 [2].

Table 5.2-1: Summary of APIs relevant to uplink media streaming features

| 5GMSu feature | Abstract | Relevant APIs |  |  |
| --- | --- | --- | --- | --- |
|  |  | Interface | API name | Clause |
| Content protocols discovery | Used by the 5GMSu Application Provider to query which content egest protocols are supported by 5GMSu AS(s). | M1u | Content Protocols Discovery API | 7.5 |
| Content preparation | Supports manipulation by the 5GMSu AS of streaming media content uploaded by 5GMSu Client over M4u, prior to egest of the manipulated content over M2u. | M1u | Content Preparation Templates Provisioning API | 7.4 |
| Metrics reporting | The 5GMSu Client uploads metrics reports to the 5GMSu AF according to a provisioned Metrics Reporting Configuration it obtains from the Service Access Information for its Provisioning Session. | M1u | Provisioning Sessions API | 7.2 |
|  |  |  | Metrics Reporting Provisioning API | 7.8 |
|  |  | M5u | Service Access Information API | 11.2 |
|  |  |  | Metrics Reporting API | 11.4 |
| Dynamic Policy invocation | The 5GMSu Client activates different traffic treatment policies selected from a set of Policy Templates configured in its Provisioning Session. | M1u | Provisioning Sessions API | 7.2 |
|  |  |  | Policy Templates Provisioning API | 7.9 |
|  |  | M5u | Service Access Information API | 11.2 |
|  |  |  | Dynamic Policies API | 11.5 |
| Network Assistance | The 5GMSu Client requests bit rate recommendations and delivery boosts from the 5GMSu AF. | M5u | Service Access Information API | 11.2 |
|  |  |  | Network Assistance API | 11.6 |
| Edge resources | Edge resources are provisioned for 5GMS media sessions. | M1u | Edge Resource Provisioning API | 7.10 |
|  |  | M5u | Service Access Information API | 11.2 |

4th Change

#### 6.4.3.8	ActivationTrigger type

The definition for the ActivationTrigger type is specified in table 6.4.3.8-1 below:

Table 6.4.3.7-1: Definition of ActivationTrigger type

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| serviceDataFlowDescriptions | Array(ServiceDataFlowDescription) | 0..1 | Provides a list of service data flow descriptions that are to be used as triggers for invoking edge media processing. |
| ueLocation | Array(LocationArea5G) | 0..1 | provides a list of geographical areas. If the UE is in one of these areas, the session will invoke edge media processing. |
| timeWindow | Array(TimeWindow) | 0..1 | edge media processing will only be triggered when the session is taking place during one of the indicated time windows. |
| appRequest | Boolean | 0..1 | Only trigger edge media processing based on application request, when this value is set to True. |

#### 6.4.3.9	ACRRequirement type

The definition for the ACRRequirement type is specified in table 6.4.3.9-1 below:

Table 6.4.3.7-1: Definition of ACRRequirement type

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| tolerance | ACRTolerance | 1..1 | Indicates whether an operation was successful (TRUE) or not (FALSE). |
| maxInterruptionDuration | integer | 0..1 | the maximum allowed downtime during application context relocation that an application may experience. |
| maxLatencyDifference | integer | 0..1 | the maximum allowed difference between the previously experienced average latency and the expected latency to the new EAS. |
| maxAppContextSize | integer | 0..1 | the maximum application context data size that may need to be transferred to the target EAS as part of an ACR procedure. |

5th Change

#### 6.4.4.4	EdgeManagement enumeration

The data model for the EdgeManagement enumeration is specified in Table 6.4.4.4-1 below:

Table 6.4.4.4-1: Definition of EdgeManagement enumeration

| Enumeration value | Description |
| --- | --- |
| EM_NETWORK_DRIVEN | The AF in coordination with the MSH assign edge resources and direct application traffic to the EAS transparently to the application on the UE. |
| EM_APP_DRIVEN | The application on the UE requests edge resources and manages the connection to the edge. |

#### 6.4.4.5	ACRTolerance enumeration

The data model for the ACRTolerance enumeration is specified in Table 6.4.4.5-1 below:

Table 6.4.4.5-1: Definition of ACRTolerance enumeration

| Enumeration value | Description |
| --- | --- |
| ACRT_UNAWARE | the application is not aware of any context relocation that may happen. ACR procedures may be started without any restrictions. |
| ACRT_TOLERANT | the application may tolerate ACR but requirements for the ACR procedure must be met. There is potentially an application context that may need to be transferred. |
| ACRT_INTOLERANT | the application does not tolerate context transfer. |

6th Change

## 7.10	Edge Resources API

### 7.10.1	General

The Edge Resources API is used by the Application Provider to provision the edge resource usage for media sessions associated with the parent Provisioning Session. The information serves as a template to select or instantiate the proper EAS that will serve the media session on the UE.

### 7.10.2	Resource structure

The Edge Resources API is accessible through the following URL base path:

{apiRoot}/3gpp-m1/v1.1/provisioning-sessions/{provisioningSessionId}/

Table 7.10.2-1 specifies the operations and the corresponding HTTP methods that are supported by the Edge Resources API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path indicated by the second column of the table shall be appended to the resulting URL base path.

Table 7.10.2-1: Operations supported by the Edge Resources API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Configure Edge Resources | edge-resources-configuration | POST | Invoked on an Edge Resources collection to create a new Edge Resources Configuration. If the operation succeeds, the URL of the newly created Edge Resources Configuration resource shall be returned in the Location header of the response. |
| Retrieve Edge Resources Configuration | edge-resources-configuration/‌{edgeResourcesConfigurationId} | GET | Used to retrieve a specific Edge Resources Configuration resource. |
|  |  | PUT, PATCH | Used to modify or replace an existing Edge Resources Configuration resource. |
|  |  | DELETE | Used to destroy an existing Edge Resources Configuration resource. |

### 7.10.3	Data model

The data model for the Edge Resources Configuration resource is specified in table 7.10.3-1:

Table 7.10.3-1: Definition of EdgeResourcesConfiguration resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| edgeResourcesConfigurationId | ResourceId | 1..1 | An identifier for this Metrics Reporting Configuration that is unique within the scope of the enclosing Provisioning Session. |
| edgeManagementMode | EdgeManagment | 1..1 | Indicates if the management of the edge resource session is application-driven or network-driven. |
| activationTrigger | Activation‌Trigger‌ | 0..1 | Condition to activate edge resources for this ProvisioningSession. If the activationTrigger element is not provided, it shall be assumed that all media sessions related to the parent Provisioning Session will use edge resources. |
| profile | EASProfile | 1..1 | The EAS profile used by the 5GMS AF or by the EEC to discover and select one or more EAS instances to serve media streaming sessions. The format of the EASProfile is defined in table 8.1.5.2.3-1 of TS 29.558 [42]. |
| application‌Context‌Relocation‌Requirements | Array(ACR‌Requirement) | 0..1 | Application Context Relocation tolerance and requirements. If not present, the 5GMS AF shall assume that the application is unaware of context transfer and that transfers to new EAS are allowed. |