3GPP TSG-SA4 Meeting #117-e	S4-220080

# Online, 14th – 22nd February 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.998 | CR |  | rev |  | Current version: | 1.1.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to generic call flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_STAR |  |  |  |  |  | Date: |  |  | 8th February 2022 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 3.1	Definitions

…

AR Session: a session that is established between the application and the XR runtime to gain access to the AR features and input/output data.

AR Media Session: A session that is established by the UE to exchange AR/MR content.

2nd Change

## 4.3	Basic Processes in an AR Media Session

### 4.3.1	Introduction

In this clause we provide a generic call flow for setting up and operating an AR Media session. This geneic call flow is applicable to all types of AR applications, such as 6DoF streaming, AR conferencing, and spatial computing. The components used in this call flow can be instantiated differently depending on the target AR application. For example a Media Server may be an MRF in an AR Call application or a mapping server in a spatial computing application.

The basic workflow is provided in Figure 4.3.1-1. In this case, the following steps happen:

1.	The application contacts the application provider to fetch the entry point for the content. The acquisition of the entry point may be performed in different ways and is considered out of scope. An entry point may for example be a URL to a scene description document or an SDP entry declaring a data channel that carries scene description data.

2.	The application initializes the Scene Manager using the acquired entry point.

3.	If not done in step 1, the Scene Manager retrieves the scene description document from the scene provider based on the entry point information.  The scene provide may e.g. be an MRF that composes a scene for an AR conference or it may be a media server that serves a pre-stored scene.

4.	The Scene Manager parses the entry point and creates the immersive scene.

5.	The Scene Manager requests the creation of a new AR/MR session from the AR Runtime.

6.	The AR Runtime creates a new AR/MR session and performs registration with the local environment.

7.	Delivery sessions are established based on requests from the Scene Manager. Downlink delivery sessions are used for fetching media and uplink delivery sessions are used for sharing spatial computing data, sensor data, pose information, …

8.	The AR media session loop consists of three main steps:

- scene update loop: scene updates are fetched and processed by the scene manager.

- Spatial Compute loop: spatial compute data is collected and sent uplink to the media servers (e.g. spatial compute servers). This may include collection and sharing of pose information. The results of the spatial compute operations are fetched.

- Media Rendering loop: media data is fetched, processed, and rendered.

In addition to the AR/MR session, an AR Media Session may trigger the establishment of one or more downlink or uplink delivery sessions as follows: :

- downlink delivery session: A session for accessing scene description, scene description updates, media, and spatial compute data over the network. These delivery sessions are established by the MAF based on requests from the Scene Manager.

- uplink delivery session: A session for sharing sensor data (e.g. pose information or depth data) with the network to perform different tasks such as spatial description functions, 3D reconstruction, etc. XR Spatial Description functions are introduced in clause 4.2.5.

NOTE: 	Both types of sessions run independently and there could be multiple sessions of each type. Some delivery sessions may be last for the whole lifetime of an AR media session, while others may be short lived. The data from each of these sessions is fed into the Scene Manager to create the AR experience.

Figure 4.3.1-1: Basic workflow for AR sessions