3GPPSA4-e (AH) Video SWG 117-e	S4-220128

E-meeting, 14-23 February 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.998 | CR |  | rev | - | Current version: | 1.1.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5GSTAR] pCR on clause 6.6 of TR 26.998 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics, Co. Ltd., Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GSTAR |  |  |  |  |  | Date: |  |  | today |
| Category: | B |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

### 6.6.3	Basic architecture

To describe the functional architecture for shared AR conversational experience use case such as clause Annex A.7 and identify the content delivery protocols and performance indicators, an end-to-end architecture is addressed. The end-to-end architecture for shared AR conferencing (one direction) is shown in Figure 6.6.3-1. To simplify the architecture, only 5G STAR UE is considered in this figure.

Figure 6.6.3-1: Shared AR conversational service for STAR UE

Camera(s) are capturing the participant(s) in an AR conferencing scenario. The camera(s) for each participant are connected to a UE (e.g. laptop or mobile phone or AR glasses) via a data network (wired/wireless). Live camera feeds, sensors, and audio signals are provided to a UE which processes, encodes, and transmits immersive media content to the 5G system for distribution.  In shared AR conversational services, the immersive media processing function on the cloud/network receives the uplink streams from various devices and composes a scene description defining the arrangement of individual participants in a single virtual conference room. The scene description as well as the encoded media streams are delivered to each receiving participant. A receiving participant’s 5G STAR UE receives, decodes, and processes the 3D video and audio streams, and renders them using the received scene description and the information received from its AR Runtime, creating an AR scene of the virtual conference room with all other participants.

Also note that if the format conversion is desired, the immersive media processing function on the cloud may optionally use media services such as pre-processing of the captured 3D video, format conversion, and any other processing before compression of immersive media content including 3D representation, such as in form of meshes or point clouds, of participants in an AR conferencing scenario.

Note: As an example of the composite scene generation, the immersive media processing function may take the input from the participants’ physical constraints, so that the generated scene is consistent with every participants’ environment and can be rendered at each device consistently.

Figure 6.6.3-2 illustrates the architecture for shared AR conversational experience use case when an 5G EDGAR UE (receiver) is used. While the functionalities of the sender and the network/cloud shown in Figure 6.6.3-1 are identical in the STAR and EDGAR devices, an EDGAR device uses a split-rendering function on Cloud/Edge.

Figure 6.6.3-2: Shared AR conversational service for EDGAR UE and cloud/edge pre-rendering

In this case, AR Scene Manager on cloud/edge generates the lightweight scene description and simple format of AR media that match AR glass display capabilities of the individual participant’s 5G EDGAR device. The lightweight scene description and encoded rendered scene are delivered to the UE. The UE receives the simple format of AR media and audio streams, decodes and renders them using the received lightweight scene description and the information received from its AR Runtime, creating an AR scene of the virtual conference room with all other participants.

=====  CHANGE  =====

### 6.6.7	Standardization areas

The list of potential standardization area that has been collected is provided in the following:

-	Immersive media format and simplified media formats with integration into relevant 5G architecture

-	Scene description format, functionality, and profile as an entry point of immersive media

-	Scene description update mechanism

-	Relevant subset of media codecs for different media types and formats

-	CMAF encapsulation of immersive media for 5G media streaming

-	Media payload format to be mapped into RTP streams

-	Capability exchange mechanism and relevant signalling

-	Low-latency uplink streaming of captured AR data

-	Functionalities to support split rendering and network-based media processing allocation with 5G edge/MRF

-	Required QoS and QoE for shared AR/MR conversational experience service