Source: 	InterDigital Inc.

Title: 	Candidate 2D video capabilities for MeCAR

Document for	Discussion and Agreement

Agenda item: 	9.5 - MeCAR (Media Capabilities for Augmented Reality)

# Introduction

The MeCAR work item defines service-independent media capabilities for AR devices. Among the objectives of this work item is “to define media types and formats produced and consumed by the AR device (…), define decoding capabilities, including support for multiple parallel decoders and define encoding capabilities”. One justification is that “media capabilities typically contribute to (..) basic media applications on such AR glasses with simple rendering functionalities”

This contribution proposes to start listing encoding/decoding capabilities which may be of interest for the Edgar-1 MeCAR device, referring to video capabilities which have been referenced in TS 26.511 (“5G Media Streaming, Profiles, Codecs and Formats”).

Video profiles which have been referenced in TS 26.118 (“Virtual Reality (VR) profiles for streaming applications”) refer to a 360° immersive context, which may not be relevant for the optical see-through device type which we are focusing on in a first step. As it is presented in the next chapter, focusing on 2D streaming may be sufficient as a first step.

# Interest of 2D video streaming

In this chapter, we discuss that 2D video streaming is a necessary capability. In addition, it may be a sufficient capability to implement any AR application within the frame of an edge architecture based on split rendering.

## 2D Video streaming as a use case

Several use cases have been described in TR 26.928 annex A and TR 26.998 annex A. In addition to these scenarios which are focusing on AR experiences, and therefore including an AR element, basic use cases that use only 2D videos as visual media need to be also supported with AR glasses.

A basic scenario only using 2D video is precisely the display of 2D video stream as an overlay to the real world. A typical scenario is the “virtual TV set” as depicted in the figure below, where a 2D virtual TV-set is overlayed on the wall. In the figure below, the TV_set as well as as the prehistoric persons on the table are virtual objects which can only be seen through the AR glasses.

Figure 1: Overlay of virtual objects - 2D screen on the wall, 3D persons on the table

## 3D rendering based on 2D video streaming

Heat and battery drain are two of the main concerns in the design of AR glasses. In order to reduce battery drain and device heating, it is proposed to offload from the AR glasses the computationally intensive tasks to an edge device, ie to a device located in close network proximity. This principle results in the Edge architecture which is proposed to be studied in priority in MeCAR, where the rendering function is split between the Edgar device and the Edge device.

Within the scope of this architecture, one possible distribution of functions is that the Edge concentrates most of the AR related functions. In such a distributed architecture, whenever the pose is computed, the final viewport rendering is performed on the Edge which then sends one or several video streams (depending on the solution for carrying one picture per eye, multi-view codec, legacy side by side or top-bottom stereoscopic 3D video,…), corresponding to the viewport to be rendered to the user. The advantage of such a functional distribution is to only require 2D video codecs on the Edgar UE. It is depicted in the figure below.

Figure 2: Possible use of 2D video codecs for 3D rendering in a split context

Corresponding viewport rendering methods have been described in clauses 6.2.4 and 6.2.5 of TS 26.928.

# List of 2D video profiles to be considered for MeCAR capabilities

Based on the importance of video streaming capability and on the video profiles which have been proposed in TS 26.511 for 2D videos, it is proposed to reference the video profiles proposed in TS_26.511 for candidates for MeCAR video encoding and decoding capabilities. These candidates are reproduced here for sake of clarity below both for AVC and HEVC codecs in chapters 3.1 and 3.2.

## AVC

### Decoding

TS 26.511 has defined the following decoding capabilities for H.264 (AVC4) in its clause 4.2.1.1:

-	AVC-HD-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 3.1 [2] bitstreams, for which the maximum VCL Bit Rate is constrained to be 14 Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1,000 and 1,200, respectively.

-	AVC-FullHD-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 4.0 [2] bitstreams.

-	AVC-UHD-Dec: the capability to decode H.264/AVC Progressive High Profile Level 5.1 [2] bitstreams for H.264/AVC with the following additional restrictions and requirements:

-	the maximum VCL Bit Rate is constrained to be 120 Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively.

-	the bitstream does not contain more than 10 slices per picture.

NOTE:	High Profile for H.264/AVC excludes Flexible macro-block order, Arbitrary slice ordering, Redundant slices, Data partition.

### Encoding

The corresponding encoding capabilities (in the sense that their bitstream are decodable by the decoder with the same name) have been defined:

-	AVC-HD-Enc:

-	up to 108,000 macroblocks per second;

-	up to a frame size of 3,600 macroblocks;

-	up to 120 frames per second;

-	the chroma format being 4:2:0; and

-	the bit depth being 8 bits;

-	AVC-FullHD-Enc:

-	up to 245,760 macroblocks per second;

-	up to a frame size of 8,192 macroblocks;

-	up to 240 frames per second;

-	the chroma format being 4:2:0; and

-	the bit depth being 8 bits;

-	AVC-UHD-Enc:

-	up to 983,040 macroblocks per second;

-	up to a frame size of 36,864 macroblocks;

-	up to 480 frames per second;

-	the chroma format being 4:2:0; and

-	the bit depth being 8 bits.

## HEVC

### Decoding

TS26.511 has defined the following decoding capabilities for H.265 (HEVC) in its clause 4.2.2.1:

-  HEVC-HD-Dec: the capability to decode H.265 (HEVC) Main Profile, Main Tier, Level 3.1[3] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

-	HEVC-FullHD-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 4.1[3] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

-	HEVC-UHD-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 5.1[3] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

-	HEVC-8K-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 6.1[3] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1 with the following further limitations:

-	the bitstream does not exceed the maximum luma picture size in samples of 33,554,432,

-	the maximum VCL Bit Rate is constrained to be 80 Mbps with CpbVclFactor and CpbNalFactor being fixed to be 1000 and 1100, respectively.

### Encoding

The corresponding encoding capabilities (in the sense that their bitstream are decodable by the decoder with the same name) have been defined:

-	HEVC-HD-Enc:

-	up to 33,177,600 luma samples per second;

-	up to a luma picture size of 983,040 samples;

-	up to 120 frames per second;

-	the Chroma format being 4:2:0; and

-	the bit depth being 8 bits;

-	HEVC-FullHD-Enc:

-	up to 133,693,440 luma samples per second;

-	up to a luma picture size of 2,228,224 samples;

-	up to 240 frames per second;

-	the Chroma format being 4:2:0; and

-	the bit depth being either 8 or 10 bits;

-	HEVC-UHD-Enc:

-	up to 534,773,760 luma samples per second;

-	up to a luma picture size of 8,912,896 samples;

-	up to 480 frames per second;

-	the Chroma format being 4:2:0; and

-	the bit depth being either 8 or 10 bits.

0

# Proposal

We propose as potential candidates for MeCAR the codec capabilities which are referenced both for AVC and HEVC in clause 4.2 of TS 26.511.

# References

- TS 26.118 “Virtual Reality (VR) profiles for streaming applications”.

- TS 26.511 “5GMS Profiles, Codecs and Formats”.

- TR 26.928 “Extended Reality in 5G”.

- TR 26.998 “Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices”.