Agenda item: 	10.7

Source: 	Qualcomm Inc., Intel, NTT, Samsung

Title: 	Working Assumption for RTC Architecture

Document for	Discussion and Agreement

# Introduction

During the offline discussions on the RTC architecture, there were some agreements that were made on the working assumption. This contribution documents the agreed working assumption.

# RTC Architecture

## 2.1		Collaboration Scenarios

The following collaboration scenarios are assumed as a basis for the architecture:

- 5G support for OTT WebRTC: e.g. to provide QoS support to an OTT WebRTC session.

- MNO-provided trusted WebRTC functions: e.g. a STUN or a TURN server hosted by the MNO.

- MNO-facilitated WebRTC services: e.g. a service offered by the operator, such as a split rendering service.

- Inter-operable WebRTC services: e.g. a cross-MNO communication service, such as a WebRTC-based telephony service.

## 2.2		RTC Functions

The following table identifies key functions and provides a mapping to the collaboration scenarios.

| Functions/CS | Collaboration scenario 1 | Collaboration scenario 2 | Collaboration scenario 3 | Collaboration scenario 4 |
| --- | --- | --- | --- | --- |
| Provisioning server | Optional | Optional | Optional | Optional |
| Configuration server | Optional | Required | Required | Required |
| MSH | Required | Optional | Optional | Optional |
| Network support function | Required | Required | Optional (maybe fulfilled by WebRTC signalling server) | Optional |
| Trusted ICE function | N/A | Required | Optional | Optional |
| Trusted WebRTC signalling server | N/A | N/A | Required | Required |
| Trusted media server | N/A | Optional | Optional | Optional |

## 2.3 	Working Assumption on the Architecture

For the development of the architecture for RTC, we follow the following principles:

- Principle 1: AF is a box communicating with NF in 5GC (according to SA2).

- Principle 2: AF is for C-plane and AS is for U-plane.

Based on that, we make the following assumptions:

- Network support (NS-AF) is an AF.

- WebRTC SS and ICE/TURN use NS-AF to communicate with 5GC, e.g., PCF/NEF/SMF.

- In this sense, WebRTC SS and ICE/TURN are AS.

We propose the following 5G RTC architecture as the baseline architecture for WebRTC-based real-time communication over 5G.

Note: The details of interface in the above figure are TBD

Note: Not all the functionalities are necessary for specific collaboration scenario, as addressed in Table. The figure may need to be updated to express such variants (for example, required/optional functions, location (Trusted/External DN), etc)

In a simplified deployment scenario, the architecture with only the relevant functionalities is depicted in the following figure:

# RTC Interfaces

TBD

# Functions and Entities

Note: The text in this clause are moved from iRTCW permanent document except 2.5.10 (Trusted Application Supporting Web Function), which was proposed as CSF (Conference Supporting Function) in FS_eiRTCW study.

## 2.5.1 General

This subclause defines minimal and essential functions. Extra functions and entities appear in some cases. The definitions of extra functions and entities are for further study.

## Provisioning server

The provisioning server may enable an application provider to perform provisioning of the following functionalities:

- QoS support provisioning for WebRTC sessions

- Charging provisioning for WebRTC sessions

- Collection of consumption and QoE metrics data provisioning related to WebRTC sessions

- Offering ICE functionality provisioning such as STUN and TURN servers

- Offering WebRTC signalling servers provisioning, potentially with interoperability to other signalling servers.

The provisioning server may not be relevant to all collaboration scenarios and some of the 5G support functionality may be offered without application provider provisioning.

NOTE:The integration/collocation of this AF and WebRTC SS is possible. Co-located WebRTC SS is able to act as an AF which is accessible to 5GC, and replace some of this AF’s IFs and APIs with WebRTC SS signalling. E.g., IFs and APIs between this AF and UE will be replaced to avoid concurrent/redundant requests from UE.

## 2.5.3	Configuration server

The configuration server stores WebRTC-related configuration information and makes them accessible to the UE. It stores information and recommendations to operate network-assisted WebRTC sessions over 5G.

The configuration information may consist of static information such as the following:

- Recommendations for media configurations

- Configurations of STUN and TURN server locations

- Configuration about consumption and QoE reporting

- Discovery information for WebRTC signalling and data channel servers and their capabilities in static and/or dynamic way.

NOTE: The integration/collocation of this AF and WebRTC SS is possible. Co-located WebRTC SS is able to act as an AF which is accessible to 5GC, and replace some of this AF’s IFs and APIs with WebRTC SS signalling. E.g., IFs and APIs between this AF and UE will be replaced to avoid concurrent/redundant requests from UE.

## 2.5.42	Media session handler (MSH)

The MSH is an entity running on the UE, which assists with the 5G integration of the WebRTC application. It exchanges, on behalf of the application, information about the WebRTC sessions with the network.

The MSH receives information about a new WebRTC session from the application. It relays the information to the Support Function. It also receives events and other network information about the WebRTC session from the Support Function, which it may relay to the application.

## 2.5.5	Network support function

The support functionality includes the following:

- Network Support Function receives information about a WebRTC session and its state

- Network Support Function requests QoS allocation for a starting or modified session

- Network Support Function receives notification about changes to the QoS allocation for the ongoing WebRTC session

- Network Support Function exchanges information about the WebRTC session with the trusted STUN/TURN/Signalling Server, e.g. to identify a WebRTC session and associate it with a QoS template.

NOTE: The integration/collocation of this AF and WebRTC SS is possible. Co-located WebRTC SS is able to act as an AF which is accessible to 5GC, and replace some of this AF’s IFs and APIs with WebRTC SS signalling. E.g., IFs and APIs between this AF and UE will be replaced to avoid concurrent/redundant requests from UE.

## 2.5.6	Trusted ICE functions

The MNO may offer trusted ICE functions to the WebRTC application to be used during the WebRTC ICE gathering phase. These functions may be STUN and TURN servers that facilitate NAT and Firewall traversal.

The MNO-operated trusted ICE functions may assist with the 5G integration of the WebRTC application. This could be done by triggering network assistance to starting or ongoing WebRTC sessions.

## 2.5.7	Trusted WebRTC signalling server

The trusted WebRTC signalling server is used to setup and manage MNO-operated WebRTC applications. They offer a standardized signalling protocol for the session setup to both parties of the WebRTC session. The WebRTC signalling server will handle the offer/answer exchange and will have access to the SDP in both directions.

The WebRTC signalling server may use that knowledge to offer network assistance and other 5G features to the endpoints of the WebRTC session.

## 2.5.8	Inter-working function

This function provides inter-working functionality to enable MNO-facilitated WebRTC sessions that involve end-points across different MNOs. They may for example provide cross-network signalling functionality to allow WebRTC signalling server that are hosted in different networks to communicate, in order to establish and manage the WebRTC sessions.

## 2.5.9	Trusted media server

A media server may be offered by the MNO to support WebRTC sessions. It may offer a wide range of functionality such as:

- a content server that serves content to the WebRTC application, e.g. through a data channel

- media processing functionality: may be used by the WebRTC application as a relay that performs some media processing function such as transcoding, recording, 3D reconstruction, etc.

- scene composition functionality: the server may compose a 3D scene and distribute it to several point-to-point WebRTC sessions

- MCU functionality: the server may offer multi-party conferencing functionality to merge a number of point-to-point WebRTC sessions

- SFU (Selective Forwarding Unit) functionality: the server may offer the selection, copy, and forwarding functionality of IP steams produced by multiple WebRTC endpoints (i.e., participants).

## 2.5.10 Trusted Application Supporting Web Function

A web server may be offered by the MNO to support applications by providing web service entry point, authorization/authentication, sharing files, or scheduling conferencing sessions.

# 3 Proposal

We propose to agree the baseline architecture and the interface descriptions in clause 2.