3GPPSA4#120-e	S4-220955

E-meeting, 17 – 26 August 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.857 | CR | pseudo | rev | - | Current version: | 0.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5G_MSE] Example 5GMS Media Player |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_MSE |  |  |  |  |  | Date: |  |  | 11/08/2022 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This pCR assumes that S4-220954 is agreed |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

### 4.2.3	Media Player in 5GMS

Downlink 5G Media Streaming specifies the use of segment formats that are based on the Common Media Application Format (CMAF) in ISO/IEC 23000-19 [G]. By using this format, 5G Media Streaming is compatible with a broad set of segment-based streaming protocols including Dynamic Streaming over HTTP (DASH) and HTTP Live Streaming (HLS). For example, ISO/IEC 23009-1 [H] defines a detailed DASH profile for delivering CMAF content within a DASH Media Presentation using a converged format for segmented media content.

Figure 4.2.3-1: Media Playback in 5G Media Downlink Streaming Architecture
(reproduced from 3GPP TS 26.511 [F])

According to TS 26.511 [F], TS 26.512 [E] and Figure 4.2.3-1 above, the Media Player is further decomposed into an Access Client and a Media Playback Platform. Several APIs are identified for the Media Player:

-	M4d (Media Streaming APIs): APIs exposed by a 5GMSd AS to the Media Player to stream media content.

-	M6d (UE Media Session Handling APIs): APIs exposed by a Media Session Handler to the Media Player for client-internal communication and exposed to the 5GMSd-Aware Application enabling it to make use of 5GMS functions.

-	M7d (UE Media Player APIs): APIs exposed by a Media Player to the 5GMSd-Aware Application and Media Session Handler to make use of the Media Player.

-	A set of internal Media Player APIs that deals with providing accessed data to the Media Playback Platform. These closely follow the W3C APIs for HTML-5 based media playback and the Media Source Extensions.

Most relevant in the discussion is the M7d API provided by the Access Client (see clause 13 of TS 26.512 [E]) defining:

1)	Methods to interact with the Access Client of the Media Player,

2)	Notification and Error Events sent to the Media Session Handler and 5GMSd-Aware Application,

3)	Configuration and Settings methods,

4)	Status Information.

The initial API has largely been designed based on the dash.js API documented here: http://cdn.dashjs.org/latest/jsdoc, but they are abstract.

The Media Player internal APIs are defined to follow closely what is defined in the W3C APIs for HTML-5 based media playback and the Media Source Extensions.

For the Media Player, different states are defined, depending on actions received from any of the APIs.