Source:	SA4 Audio SWG Co-Chairs (Qualcomm Incorporated, Orange)

Title:	Draft report from Audio SWG during 3GPP SA4#121

Document for:	Approval

Agenda Item:	12.1

Executive Summary

The Audio SWG meeting (33 delegates) met in 10 time slots. In total 51 documents were handled; in addition, two output documents were left to be presented to closing Plenary. The meeting outcome is summarized below:

- Maintenance

  - Corrections for idle noise and sidetone requirements (electrical interface UE) in TS 26.131 were agreed in S4-221514

  - S4-221416 is a CR to 26.447 with editorial corrections in notations and formulas. Further items for correction were found, the revision in S4-221515 was agreed.

  - S4-221463 is a CR to 26.973 (basic operators) with editorial modifications on using inclusive language which was agreed.

- IVAS_Codec

  - Based on input contributions, several IVAS Permanent Documents were updated in on-line editing sessions.

    - S4-221526 IVAS-4 v.0.7.0 Design Constraints: this is the completed and agreed outcome at SWG with the request to agree as finalized version at the plenary.

    - A further update was produced in editing sessions and agreed at SWG as the next version of working draft for

      - S4-221525 IVAS-3 v.0.3.0 Performance Requirements

      - S4-221524 IVAS-6 v.0.4.0 Selection Deliverables

      - S4-221521 IVAS-7a v.0.7.0 Processing Plan for Selection Phase

      - S4-221528 IVAS-9 v.0.3.0 Usage Scenarios

    - The proposed updates on IVAS-2 Project Plan in S4-221339 v.0.4.0 were discussed in SWG; the editor produced a further update in S4-221527 IVAS-2 v.0.5.0 based on the discussion (this version was not seen in SWG due to lack of time).

- ATIAS

  - Existing test methods for send performance in TS 26.260, comments on FOA audio tests, and proposals on additional metrics were reviewed based on inputs from Dolby, Qualcomm, Nokia. An CR to TS 26.260 adding a reference and fixing editorial issues in S4-221518 was agreed. Further clarifications of TS 26.260 are expected in future meetings.

  - Wind noise testing was discussed based on an input from Xiaomi. An LS to ETSI TC STQ in S4-221516 was agreed

  - A new permanent document (ATIAS-1, v0.1) collecting proposals on test methods and performance requirements was initiated (Editor: Dolby) and agreed in S4-221517

  - The ATIAS time plan was updated in S4-221519 including two telcos post-121.

- FS_Audio_5GSTAR

  - The CR to 26.998 on audio aspects in S4-221529 was edited in the Audio SWG, some minor fixes to new figures were left to be done offline. The CR was left to be presented directly to SA Plenary. The intention is to complete the study item at this meeting with this CR.

- eUET

  - CRs to 26.131/26.132 removing vehicle-mounted handsfree UE in S4-221522 and S4-221523 were agreed

  - Updated test results on SWB performance were presented by HEAD acoustics. The construction of a realistic JBM profile reflecting VoLTE was presented by Orange.

  - The eUET time plan was updated in S4-221520 including one telco post-121.

- New Work Items

  - S4-221409 is a SID proposal on diverse audio capturing for end-user devices. The proposal was extensively discussed and further improved in S4-221530 which was agreed.

- AOB

  - Rapporteurs (editors) for IVAS specifications:

    - The group will collect further offers at future meetings as Rapporteur / Editor for each spec.

  - The following telcos were agreed:

    - Telco on IVAS: 2 December 2022, 14:00 – 17:00 CET, submission deadline: 1 December 2022, 14:00 CET, host: Dolby

    - Telco on ATIAS (12 Dec. 2022 15:00-17:00 CET, submission deadline is 9 Dec. 2022, 14:00 CET, Host: Dolby)

    - Telco on IVAS: 16 December 2022, 14:00 – 17:00 CET, submission deadline: 15 December 2022, 14:00 CET, host: Dolby

    - Telco on IVAS: 13 January 2023, 14:00 – 17:00 CET, submission deadline: 12 January 2023, 14:00 CET, host: Dolby

    - Telco on ATIAS (16 Jan. 2023 15:00-17:00 CET, submission deadline is 13 Jan. 2023, 14:00 CET, Host: Dolby)

    - Telco on eUET (Jan. 30, 16:00-18:00 CET; Submission deadline: Jan. 27, 14:00 CET; Host: HEAD acoustics GmbH)

    - Telco on IVAS: 3 February 2023, 14:00 – 17:00 CET, submission deadline: 2 February 2023, 14:00 CET, host: Dolby

1.  Opening of the Session

The Audio SWG Co-Chairs, Mr. Imre Varga (Qualcomm) and Mr. Stephane Ragot (Orange), opened the Audio SWG meeting on 15 November 2022, 09:00 CET.

The minutes are shared here:

https://etsihq-my.sharepoint.com/:w:/g/personal/andrijana_brekalo_etsi_org/EYCpkkQyHPFBmHX87gYXVGkBZGqUz5HjccibGf-KJv0bDQ?e=4%3AJUHOmf&at=9&CID=FED9879B-D487-4E41-AB77-9B10FE0744E2&wdLOR=c3D238B62-7682-4971-8708-9BA20067BFD1

2.   Approval of Agenda and Registration of Documents

Stéphane displays a draft revision of agenda in S4-221336R1, including Tdoc allocations and an extra Tdoc already discussed in plenary (S4-221440). The agenda in S4-221336R1 was agreed (see the final agenda in Annex A of the present report).

3.   CRs to Features in Release 17 and earlier

S4-221271

Presenter: Jan Reimes

Discussion:

- Stéphane: SA4 secretary commented that the cover page has some issues, we need to allocate a revision to fix the formal aspects, we need to tick one box on affected boxes, any other comments?

- Andre: more editorial correction, on 5.3.4, it cut the ‘connections’ word, clarify why volume control not applicable to the digital electrical interface?

- Jan: if take BT connection, if volume control on phone it does not apply to electrical interface, for USB it’s not clear how it works, we observe devices that behave like in BT, some do attenuate, we assume that volume control has no impact on signal transmitted

- Stéphane: should there be a note?

- Jan: it is described in 26.132, implicitly it is assumed that there is no volume control for digital case, a note is not necessary

Decision: S4-221271 is revised to S4-221514

S4-221514  is agreed without presentation

S4-221416

Presenter: M. Multrus

Discussion:

- Further corrections are suggested. The source will provide a revision in DRAFTS folder

- Later during the sessions, the update was reviewed and agreed

Decision: S4-221416 is revised into S4-221515 which was agreed and will be brought to plenary under A.I.13

S4-221437 is withdrawn

S4-221463

Presenter: T. Toftgard

Discussion:

- --

Decision: S4-221463 is agreed

4. Liaisons with other groups/meetings

No Tdoc  in this A.I.

5. IVAS_Codec (EVS Codec Extension for Immersive Voice and Audio Services)

S4-221297

Presenter: H-yu Su

Discussion:

- The document reflects the outcome of the two SWG calls and will be the basis for further editing during this meeting.

Decision: S4-221297 is agreed

S4-221335

Presenter: S. Bruhn

Discussion:

- Editor is asked to implement it into working draft (1297-rev); revision expected at editing session.

Decision: S4-221335 is noted

S4-221398

Presenter: M. Szczerba

Discussion:

- Comments were made which will be taken into account in editing session.

- Editor is asked to implement it into working draft (1297-rev); revision expected at editing session.

Decision: S4-221398 is noted

S4-221411

Presenter: W. Bin

Discussion:

- The proposal on including audio source angles into Annex C describes rather a solution than a requirement.

Decision: S4-221411 is noted

S4-221417

Presenter: M. Multrus

Discussion:

- L. Laaksonen: extremely important requirement; concern is whether level-1 will be relevant solution (mono only?), what is the main interest

- M. Multrus: some configurations may output mono only which is better than having not even that

- M. Jelinek: defining some requirements of minimum functionality for each complexity level appears a good solution

- M. Multrus: not straightforward, good option

- S. Bruhn: important concept is to define what should be part of each level; Dolby proposal goes a step further; we may merge the two

- S. Döhla: risk with defining what should be in a level is that the incentive is missing to work hard and include as much functionality as possible

- S. Ragot: rendering is included “hiddenly” here, we can point to it more explicitly

- M. Jelinek: would level 1 also decode bitstream

- M. Multrus: yes, this is the point

- M. Jelinek: interesting proposal, potentially challenging, more thinking will be useful

- Conclusion: see below with S4-221442

Decision: S4-221417 is noted

S4-221442

Presenter: S. Bruhn

Discussion:

- S. Ragot: important that the levels represent requirements? Or recommended?

- S. Bruhn: too loose would not be helpful; manufacturers may get information on resources vs functionality

- M. Szczerba: narrow snapshot of options / small part of the big picture

- L. Laaksonen: complexity requirement is a fundamental element; in general supportive to combine the two proposals; rather think about exact complexity numbers, they are good baseline numbers; immersive output from the codec even in level 1 is desirable

- M. Jelinek: completing design constraints would imply describing functionality constraint for each level

- Conclusion: the editor will merge the content with S4-221417 into IVAS-4 which goes into the editing session

Decision: S4-221442 is noted

S4-221331

Presenter: S. Bruhn

Discussion:

- Further editing will help to make IVAS-6 more complete although this is no 1st prio for this meeting (editing if time allows)

Decision: S4-221331 is revised to S4-221524 which is agreed as IVAS-6 v.0.4.0 and will be brought to plenary under A.I.14.2

S4-221339

Presenter: I. Varga

Discussion:

- On-line editing resulted in a draft for further consideration, to be found in DRAFTS folder.

- Due to lack of time for further on-line editing at SWG, the editor was asked to produce a further update based on the discussions and bring it to the plenary.

Decision: S4-221339 is revised to S4-221527 IVAS-2 v.0.5.0 which was not seen in SWG and will be brought to plenary under A.I.14.2

S4-221464

Presenter: T. Toftgard

Discussion:

- --

Decision: S4-221464 is agreed as IVAS-7a v.0.7.0 in S4-221521 (agreed) and will be brought to plenary under A.I.14.2

S4-221399

Presenter: E. Fotopoulou

Discussion:

- F. Plante: bit rate saving?

- M. Multrus: relatively small

- M. Jelinek: not every operating point in selection likely

- S. Ragot: reference is a challenge (like in EVS, SWB was not avl)

- A, Rämö: listener dependent, stereo reference for multichannel is not useful

- Conclusion: will be included in the working draft of IVAS-3 and be brought to editing session

Decision: S4-221399 is noted

S4-221413

Presenter: L. Laaksonen

Discussion:

- A. Schevciw: ACR test, HOA as a reference?

- L. Laaksonen: HOA available for a half of conditions

- A. Rämö: refers to earlier contribution

- S. Bruhn: difference between conv and MASA renderer, suspected difference?

- A. Rämö: less accurate in positional inf

- Conclusion: proposal is agreed, we will remove brackets in MASA perf reqs

Decision: S4-221413 is agreed

S4-221414

Presenter: L. Laaksonen

Discussion:

- S. Ragot: those bitrates which will not be tested, should still be included even without a reference

- Conclusion: will be included in the working draft of IVAS-3 and be brought to an editing session

Decision: S4-221414 is noted

S4-221443

Presenter: S. Bruhn

Discussion:

- S. Ragot: test methods don’t belong IVAS-3 (in notes)

- Conclusion: will be included in the working draft of IVAS-3 and be brought to an editing session

Decision: S4-221443 is noted

S4-221525

Presenter: S. Bruhn

Discussion:

- IVAS-3 editor’s input contains relevant contributions

- On-line editing session in Audio SWG resulted in S4-221525 v.0.3.0

- Conclusion: will be brought to plenary under A.I. 14.2

Decision: S4-221525 is agreed as IVAS-3 v.0.3.0 and will be brought to plenary under A.I.14.2

S4-221352

Presenter: T. Moriya

Discussion:

- M. Jelinek: coding goal is reproduction of input signal so reference signal is given in DCR or MUSHRA; gain amplification tool to prevent gain manipulation; CCR is not appropriate

- T. Moriya: 24 kb/s is most relevant; codec performance is important, clean speech from commercialization point of view

- M. Jelinek: using noise reduction is usual practice but not as part of the codec; goal with DCR is to score down with quality drop

- T. Moriya: customers will see scores

- T. Toftgard: clean speech was used as reference, but noisy tested

- T. Moriya: input is noisy speech, output is coded noisy speech

- T. Moriya: providing data bases is an offer to this project, D is available right now, A-B-C will be made available later

- Conclusion: NTT offering the data bases is a great offer and we thank for this

Decision: S4-221352 is noted

S4-221410

Presenter: W. Bin

Discussion:

- T. Toftgard: bullet 2, 3DOF rotating students, VR/AR device – will propose an edited update

- The update was reviewed at later sessions and agreed to be included into IVAS-9

Decision: S4-221410 is noted
 
S4-221528

IVAS-9 v.0.3.0 (agreed without presentation) and will be brought to plenary under A.I.14.2

6. ATIAS (Terminal Audio quality performance and Test methods for Immersive Audio Services)

S4-221353

Presenter: Wu Ninghang

Discussion:

- Stéphane: wind noise is a real issue, comments/questions?

- Stefan B: interesting contribution, in introduction talk about STI, is it speech transmission index (intelligibility) or?

- Ning: yes, speech transmission index

- Stefan B: we see for spatial capture, especially outdoors for those doing recording, usually there is a big wind problem, when it comes to what we can do in testing, ensure test are really repeatable and reproducible, here question to making sure we are using a wind generator that produces laminar wind flows, no to suffer effects due to turbulence, just get too many random effects during measurement, be cautious. Under figure 1, note could be clarified. Talking about clipping, maybe you have an idea to see if frame is clipped.

- Ning: with wind speed increasing, it makes overload in output

- Stefan B: could be more detailed discussion, obvious to see when signal is clipped, considering all processing that may occur, not certain if can detect such clipping condition. In that case how to make sure, it’s not obvious.

- Tomas: also question on note, note clear on processing , also define distortion, increase wind speed until you reach certain distortion rate?

- Ning: overload is caused by transducer, not processing

- Tomas: you are measuring after processing not directly after microphone, so processing could have some impact

- Ning: make sure test signal is the same for all devices

- Tomas: note should be clarified because processing would have some impact

- Andre: this is important, should do work in this area, agree with Stefan that detecting clipping may be tricky after signal enhancement and coding-decoding, other metric more related to quality could be welcome. Not clear if ETSI 103 640 setup, if look at annex, it’s informative, using turbulent case for practical reasons, still in round robin. Can work in coordination with ETSI. At moment setup is in informative status

- Jan: regarding wind speed target, in ETSI up to 4 to 5 m/s, already quite challenging for some typical wind generators, need to set max, initial proposal for that?

- Ning: We can define the requirements of wind generator, it’s a question about hardware, after defining requirement we can define wind generator

- Jan: the 4.5 m/s proposed in ETSI was like a moderate wind outside, on the road, but the higher you go with wind speed, the higher requirements on test equipment, quite difficult to setup a wind generator, compromise between laminar and turbulent cases, do you have a wind speed to operate this test?

- Ning: not yet, want to find wind generator that fits the requirements, if 3GPP can define requirements

- Andre: in this case should Ning correspond to ETSI because they work

- Stéphane: any volunteer to draft an LS to ETSI

- Jan: I can

- Stéphane: proposal to start a Pdoc on ATIAS, any text to take ?

- Stefan B: we propose to include things in a Pdoc, how this could be done, in any case having a minimum a document where we collect various contributions with the titles, section heading

- Stéphane: any disagreement to start a Pdoc on ATIAS?

- Stefan B: as co-Rapporteur I can do that

Decision: S4-221353 is noted
Parts will be included in the Pdoc on ATIAS

An LS will be drafted to communicate with ETSI STQ

S4-221412

Presenter: Arvi Lintervo

Discussion:

- Stefan B: good to take opportunity later to discuss things in detailed way and come with joint proposal, Nokia contribution is very much appreciated, on documents we presented two meetings ago. If we want to have a deep discussion, could be beyond session. Good points. To comment on harmonic structure in frequency domain, it’s a mistake, you’re right. On test signal with AM/FM modulation, making reference to P.501, there are certained assumed modifications. Something to discuss on a detailed level what might be good to be done. For level measurement and windowing, these are implementation details that could be discussed. These are practical things. Things would have to be done in more precise way. Maybe should discuss how to proceed. May reflect in a Pdoc. Can work on this towards the next meeting. This could be a way.

- Stéphane: for offline exchange, could expect an input in a next meeting (conference call).

Decision: S4-221412 is noted

S4-221447

Presenter: Stefan Bruhn

Discussion:

- Stéphane: this is a resubmission from a Tdoc in the last AH telco?

- Stefan B: essentially yes, the contribution contained a second part, we split

- Stéphane: there are answers submitted by Qualcomm at this meeting, with Orange hat, for the other method Orange will provide clarifications in a later meeting

Decision: S4-221447 is noted

S4-221422

Presenter: Andre Schevciw

Discussion:

- Stefan B: shows that there is something in TS 26.260, that we need to fix somehow, appears to be be mislead, there may be some assumptions in mind, with periphonic array. And do test with onmidirectionnal microphone, you will not be able to measure directional preferences of resynthesized soundfield, reader should not expect that this is possible. If do such measurement, you have the assumpiton that all components played out will be recombined, in the end you measure a frequency response of this combined signal. Thank you for CR. Assume certain things are fixed, maybe would do more work to make sure that spec is a bit clearer on what it performs.

- Andre: can have some note, title could be descriptive enough, CR is just correction

- Stefan B: not insisting to do all things in one CR, might be further updates related to turn

Decision: S4-221422 is noted

S4-221426

Presenter: Andre Schevciw

Discussion:

- Stéphane: no CR number, ATIAS should not be the WI code if you want it in LiQUiMAS (Rel-16), can change cat to F

- Stefan B: if change to cat F, maybe wait and see how discussion continues based on input we might from ORange, maybe no urgency, not sure we pointed on wrong things, but pointed on things that need to be clarified, more than what we see adding the reference, can try to clarify during this meeting

- Andre: if incorrect

Decision: S4-221426 is revised to S4-221518

S4-221449

Presenter: Stefan Bruhn

Discussion:

- Stéphane: sweep is to shorten test time

- Andre: surpising, send frequency response for ambisonic component, for UE testing, send frequency response for UE itself, similar to TS 26.260, but missing composition of different spherical harmonics and then rendering portion, that would complete to define an actual UE send frequency response. Interested that ideal characteristics for each ambisonic component would be flat. You have a truncation to spherical harmonics series. Is it specific to FOA capture to have a flat response?

- Stefan B: not different from TS 26.260, can be seen as complement to our claim that this ideal characteristics would have a flat frequency response. In practical systems, when we do ambisonic capture, even eigenmike, you have some frequency dependent effect, you are not doing measurement at origin, you get spatial aliasing effects, this gives frequency dependent effects, in theory if measurement is done at origin, it would be frequency independent

- Andre: flat is all components, if eigenmike and truncate at different order, if each component is flat does not mean that the combination to listener is flat. What is missing is the rendering portion of it.

- Stefan B: don’t know what kind of answer we would write, not considering rendering, would output B-format component, to avoid some kind of analysis, or frequency domain analysis, to know frequency dependent level. Spherical harmonic functions are assumed to be frequency independent. In combination.

- Stéphane: even DOA is frequency independent

- Stefan B: yes

- Stéphane: proposal is to collect proposal, to be in brackets in the Pdoc?

- Stefan B: up to group, if in square brackets, can use brackets

- Andre: what is the status of Pdoc with brackets?

- Stéphane: would collect proposals and remove brackets when things are agreed

- Andre: requirements on coefficients rather testing a UE

- Stefan B: avoid rediscuss all things at each meeting again, if certain things are technically correct, then could be candidate metrics

Decision: S4-221449 is noted

Proposals will be included in the Pdoc on ATIAS in square brackets (not agreed)

S4-221516

Presenter: Jan Reimes

Discussion:

- No comment

Decision: S4-221516 is agreed

S4-221517

Presenter: Stefan Bruhn

Discussion:

- Stéphane: ok for Xiaomi?

- Wan Bin: yes

- Stéphane: the Pdoc can be v0.1 and you can set the correct A.I.

Decision: S4-221517 is agreed

S4-221518

Presenter: Andre Schevciw

Discussion:

- Stefan B: fine, but could include later further corrections

- Stéphane: this is what is available at this meeting, can foresee more corrections later

Decision: S4-221518 is agreed

S4-221519

Presenter: Jan Reimes

Discussion:

- Stéphane: ok to have two telcos as proposed?

- Answer: yes

Decision: S4-221519 is agreed

7. eUET

S4-221270

Presenter: Jan Reimes

Discussion:

- Stéphane: note could be improved

- Andre: we consider VMHF as preceding

- Jan: can delete

Online editing takes place to simplify the note and remove VMHF

Decision: S4-221270 is revised to S4-221522

S4-221522 is agreed

S4-221269

Presenter: Jan Reimes

Discussion:

- Andre: keep changes on VMHF, realistic room type needs some improvement, but do not remove completely, define some room like ETSI room used for background noise test, there is some requirement on reverb time, wording intended is also a problem. For this time OK with VHMF changes.

- Jan: what about sentence on handset and headset, see above clause 6.1.2

- Stéphane: with Orange hat, keep with VMHF, avoid changing orthogonal aspects

- Andre: agree not big concern on handset and headset, more on handsfree

Online editing takes place (copy of note from CR to 26.131 and reverted changes on environment)

Decision: S4-221269 is revised to S4-221523

S4-221523 is agreed

S4-221419

Presenter: Jan Reimes

Discussion:

- Stéphane: can wonder if all tested phones really have representative SWB capable headset performance

- Jan: used 2014 phones, but not sure

- Stéphane: next steps?

- Jan: intention is to derive from currently available performance or to achieve in future

- Stéphane: did you listen to sound items?

- Jan: not systematically, just to find mistakes.

- Stéphane: can close this document if no more comments

- Jan: if someone is interested in testing as well, we can merge data

Decision: S4-221419 is noted

S4-221419

Presenter: Stéphane Ragot

Discussion:

- Fabrice: clarify numbers in Table 2

- Stéphane: can clarify offline; any opinion on proposal?

- Jan: should this replace or extend JBM test? Profiles are unrealistic, good to know if replace existing profiles or extend the JBM section.

- Stéphane: up to the group to decide

Decision: S4-221445 is noted

The source is invited to provide a draft Annex to TS 26.132

S4-221520

Presenter: S. Ragot

Discussion:

- Stéphane: OK to have one telco as proposed on eUET?

- Answer: yes

Decision: S4-221520 is agreed

8. FS_Audio_5GSTAR

S4-221446

Presenter: S. Ragot

Discussion:

- A. Schevciw: latency figure – refer to 26.918; 50 Hz implies 20 ms already with no headroom; EDGAR 8.9.2 “may” seems not possible always

- S. Ragot: put references, have editing; EDGAR: at least mention that we make a compromise

- T. Toftgard: agrees to motion-sound latency, full rendering control is the main case

- S. Bruhn: Figure on monolithic block, have a note to express this is not a one-directional flow; Figure on integrated decoding, have a note on API for graphics, elements of AR subsystem are specified outside

- S. Ragot: XR runtime definition could be added, don’t change the figure

- S. Döhla: 50 Hz appears coarse; we can keep it like this; Mecar discussions would be included here?

- S. Ragot: goal is to finalize the SID

- M. Jelinek: EDGAR – last part of the sentence could be dropped, keep the principle that we preserve latency and quality

- We expect a revised proposal in DRAFTS folder for tomorrow’s closing session, with everyone interested contributing; definitive goal is to complete this SID which is on a Rel-17 TR

- The revised proposal was reviewed at later sessions and agreed, although the figures in section 8 will need some editorial clean-up which is outstanding (hence no status in SWG)

Decision: S4-221446 is revised into S4-221529 (with no status) and will be brought to plenary under A.I.15.8

9. New Work / New Work Items and Study Items

S4-221409

Presenter: W. Bin

Discussion:

- T. Toftgard: various audio formats, how are they to categories related

- W. Bin: device categories are low-mid-high, they are based on formats, for ex scene based supported or not

- L. Laaksonen: 8 device types + future devices based on audio formats and device levels; study is very ambitious; focus is not clear; use cases identified already or part of the work

- W. Bin: certain devices are high prio, others are not; usage scenario collected in IVAS-9, which one is most relevant/promising

- S. Döhla: what is the standardized solution envisioned in the last objective, why is it needed

- F. Plante: concern on manufacturer side – no need for a reference design (justification section); also in objectives; last objective is of concern (manufacturer proprietary solution)

- A. Schevciw: provided edits to Wang Bin, the last objective is of general concern

- M-L. Champel: this is a study, no normative work: believe these are the items to look at; on the last objective, study it and see whether keep it proprietary or standardize

- H. Ehara: on the last objective, what is needed to reach interoperability

- The revised proposal was reviewed later during the sessions and agreed

Decision: S4-221409 is revised into S4-221530 (agreed) and will be brought to plenary under A.I.16

10. Any Other Business

S4-221263

Presenter: Sunghwan Ko

Discussion:

- Jan: considered different shifts, changing MECRP up to 1cm, in Rel-16, we introduced HaNTE devices, top edge is included or can be regarded as a HaNTE device, you can have a wider variety to position it, not considered here? 10 cm max shift, when you classify on your own, you can put it in a wider range. This is mentioned that the top edge speakers are included

- Sunghwan: yes, we considered, in experiment 2 can move more, but affects max loudness, it will be a problem in the market, all people use max loudness, cannot use MERCP for that problem, also mobile phone has several volume steps, 18 dB was made more than 10 years ago for feature phone, different from mobile phones

- Jan: min RLR requirement are in the spec to have min level of speech as min volume, this has been valid for years, reducing by 6 dB has lower level, 24 or 22 dB than than default level, it’s quite soft, more sene to remove this requirement. Mandating 24 dB is a soft level. At min RLR, covered in HaNTE WI, not big issue, not critical when at max RLR. The spec should be as generic as possible for HaNTE or non-HaNTE. I am thinking it’s not a good idea.

- Andre: Don’t know about suitability of RLR to measure such low levels, it was discussed in HaNTE. RLR was designed as a metric for loudness for typical loudnes. Concern if move handset, there is a situation where you exceed the max RLR, min RLR of –13 dB has been designed for acoustic safety, if can meet min RLR, then move phone and exceeding, it’s a bit of acoustic concern

- Sunghwan: also considered safety issue, measured acoustic shock, no problem, even if using max RLR, that kind of mobile of phones is in market for 3-5 years, we did not change min RLR, if acoustic shock it would be in market 3-5 years. Max RLR is not problem.

- Andre: we don’t do a test where move handset around and look for min RLR, that’s what could be missing. MECRP. Could be a point where loudness is maximised, to make sure there is no point exceeding –13 dB.

- Jan: in Table 2 of the discussion paper, we support not to measure only at one point, there are cases where you pass both, you are allowed to specify one MECRP. For me, it sounds like we want to achieve one multiple points, but in the spec only one point is needed. Intention to meet requirements for more than one point.

- Sunghwan: if measure multiple for max vol and min vol for MECRP, it would be possible if it would be one solution, we discussed with regulation in Korea, this was not accepted.

Decision: S4-221263 is revised to S4-221531

S4-221263

Presenter: Sunghwan Ko

Discussion:

- Sunghwan: discussed with HEAD acoustics, Qualcomm, Xiaomi offline and they are fine with this updated version

- Stéphane: can we agree on this updated CR?

- Answer: yes

Decision: S4-221263 is agreed

Scheduling interim Audio SWG calls

Audio SWG calls were scheduled to progress the work, as follows:

- Telco on IVAS: 2 December 2022, 14:00 – 17:00 CET, submission deadline: 1 December 2022, 14:00 CET, host: Dolby

- Telco on ATIAS (12 Dec. 2022 15:00-17:00 CET, submission deadline is 9 Dec. 2022, 14:00 CET, Host: Dolby)

- Telco on IVAS: 16 December 2022, 14:00 – 17:00 CET, submission deadline: 15 December 2022, 14:00 CET, host: Dolby

- Telco on IVAS: 13 January 2023, 14:00 – 17:00 CET, submission deadline: 12 January 2023, 14:00 CET, host: Dolby

- Telco on ATIAS (16 Jan. 2023 15:00-17:00 CET, submission deadline is 13 Jan. 2023, 14:00 CET, Host: Dolby)

- Telco on eUET (Jan. 30, 16:00-18:00 CET; Submission deadline: Jan. 27, 14:00 CET; Host: HEAD acoustics GmbH)

- Telco on IVAS: 3 February 2023, 14:00 – 17:00 CET, submission deadline: 2 February 2023, 14:00 CET, host: Dolby

Rapporteurs for IVAS specifications

- The chairman reminded that the group agreed to distribute the workload of editing among several contributing companies.

- Next step is to collect volunteers.

| IVAS Specification Number | IVAS Specification Title | Potential Rapporteur |
| --- | --- | --- |
| TS 26.250 | Codec for Immersive Voice and Audio Services - General Overview |  |
| TS 26.251 | Codec for Immersive Voice and Audio Services - ANSI C code (fixed-point) | Markus Multrus /FhG IIS) |
| TS 26.252 | Codec for Immersive Voice and Audio Services - Test Sequences |  |
| TS 26.253 | Codec for Immersive Voice and Audio Services - Detailed Algorithmic Description incl. RTP payload format and SDP parameter definitions |  |
| TS 26.254 | Codec for Immersive Voice and Audio Services - Rendering |  |
| TS 26.255 | Codec for Immersive Voice and Audio Services - Error Concealment of Lost Packets |  |
| TS 26.256 | Codec for Immersive Voice and Audio Services - Jitter Buffer Management |  |
| TR 26.997 | IVAS Codec Performance Characterization |  |
| TS 26.258 | Codec for Immersive Voice and Audio Services - ANSI C code (floating point) | Markus Multrus /FhG IIS) |

We received the offers indicated in the table. Further offers will be collected at future meetings.

11. Close of the Sessions

The Audio SWG Co-chairs thanked the participants for their contributions.

The meeting was closed on 17 November 2022, at 12:30 CET.

Annex A (Agenda, same as S4-221336R3)

Source:	Audio SWG Co-Chairs[1]

Title:	Draft Audio SWG Agenda

Agenda Item:	7

1. Introduction

This document provides the agenda items and allocation of documents for the Audio SWG sessions.

2. Agenda Items and Allocation of Documents

| 7 | Audio SWG | 1336app |
| --- | --- | --- |
| 7.1 | Opening of the session |  |
| 7.2 | Registration of documents |  |
| 7.3 | CRs to Features in Release 17 and earlier | 1271r->1514awp (HEAD ac./Orange, HiNT corr.) A.I. 13 1416r->1515a (FhG, corrections to 26.447) A.I. 13 1437w 1463a (Ericsson, inclusive language) A.I. 13 |
| 7.4 | Liaisons with other groups/meetings |  |
| 7.5 | IVAS_Codec (EVS Codec Extension for Immersive Voice and Audio Services) | 1297a (IVAS-4 v.0.6.1) 1335n (Dolby/FhG, DC) 1398n (Philips, reverb DC) 1411n (Xiaomi, object-based audio) 1417n (FhG, complexity/memory DC) 1442n (Dolby, complexity/memory DC) 1526a (IVAS-4 v.0.7.0) A.I. 14.2 1399n (FhG, PR multichannel) 1413a (Nokia, PR MASA) 1414n (Nokia, PR multichannel) 1443n (Dolby, SBA PR) 1525a (IVAS-3 v.0.3.0) A.I. 14.2 1331rà1524a (IVAS-6 v.0.4.0) A.I. 14.2 1339rà1527 (IVAS-2 v.0.5.0) A.I. 14.2 1352n (NTT, listening tests for noisy) 1464rà1521a (IVAS-7a v.0.6.1à v.0.7.0) A.I. 14.2 1410n (Xiaomi, usage scenario) 1528a (IVAS-9 v.0.3.0) A.I. 14.2 1438w |
| 7.6 | ATIAS (Terminal Audio quality performance and Test methods for Immersive Audio Services) | 1353n (Xiaomi, wind noise test) 1412n (Nokia, FOA audio tests) 1447n (Dolby, review TS 26.260) 1422n (Qualcomm, send side perf.) 1426r->1518a (Qualcomm, CR 26.260 clause 4) A.I. 14.1 1449n (Dolby, additional metrics) 1516a (LS to ETSI STQ) A.I. 14.1 1517a (Pdoc on ATIAS) A.I. 14.1 1519a (Timeplan v0.6) A.I. 14.1 Telco (12 Dec. 2022 15:00-17:00 CET, submission deadline is 9 Dec. 2022, 14:00 CET, Host: Dolby) Telco (16 Jan. 2023 15:00-17:00 CET, submission deadline is 13 Jan. 2023, 14:00 CET, Host: Dolby) |
| 7.7 | eUET (Enhancements to UE Testing) | 1269r->1523a (HEAD acoustics, VMHF UE) A.I. 14.6 1270r->1522a (HEAD acoustics, VMHF UE) A.I. 14.6 1419n (HEAD acoustics, SWB results) 1445n (Orange, JBM profiles) 1520a (Timeplan) A.I. 14.6 Telco (Jan. 30, 165:00-187:00 CET; Submission deadline: Jan. 27, 14:00 CET; Host: HEAD acoustics GmbH) |
| 7.8 | FS_Audio_5GSTAR (Feasibility Study on Audio Aspects for 5G Glasses-type AR/MR Devices) | 1446rà1529 (Orange/Ericsson, CR 26.998) A.I. 15.8 |
| 7.9 | New Work / New Work Items and Study Items | 1409r->1530a (Xiaomi/Huawei/Bytedance/ZTE/BUPT, draft SID on audio capture) A.I. 16 |
| 7.10 | Any Other Business | 1263r->1531a (Samsung, RLR at min. vol.) A.I. 14.11 |
| 7.11 | Close of the session |  |

n – noted

a – agreed

p – parked

pp – postponed

r – revised

rp – replied

m – missing

[1] Imre Varga, Email: ivarga@qti.qualcomm.com; Stephane Ragot, Email: stephane.ragot@orange.com

Annex B

Participants (list provided by the SA4 Secretary)

| Firstname Lastname | Represented Company |
| --- | --- |
| Stéphane Ragot | Orange |
| Imre Varga | Qualcomm |
| Dejun Zhang | Bytedance (remote) |
| Haiqing Du | BUPT (remote) |
| Jan Holub | Mesaqin (remote) |
| Dong Wang | OPPO (remote) |
| Minjie Xie | OPPO (remote) |
| Tore Stegenborg-Andersen | FORCE Technology (remote) |
| Hu Chenhao | Xiaomi (remote) |
| Takehiro Moriya | NTT |
| Marek Szczerba | Philips International B.V. |
| Lasse Laaksonen | Nokia Corporation |
| Tomas Toftgård | Ericsson LM |
| Markus Multrus | Fraunhofer IIS |
| Eleni Fotopoulou | Fraunhofer IIS |
| Fabrice Plante | Apple |
| Stefan Döhla | Fraunhofer IIS |
| Frans de Bont | Philips International B.V. |
| Jan Reimes | HEAD acoustics GmbH |
| Stefan Bruhn | Dolby Sweden AB |
| Hiroyuki Ehara | Panasonic Holdings Corporation |
| Sunghwan Ko | Samsung |
| Hakju Ryan Lee | Samsung |
| Jaeyeon Song | Samsung |
| Milan Jelinek | VoiceAge |
| Wang Bin | Xiaomi |
| Ninghang Wu | Xiaomi |
| Mary-Luc Champel | Xiaomi |
| Arvi Lintervo | Nokia |
| Anssi Ramo | Nokia |
| Andre Schevciw | Qualcomm |
| Brian Lee | Dolby |
| Antero Tossavainen | Huawei (remote) |

Annex C

Document status

C.1 Agreed documents (to be presented to SA4 plenary)

| Tdoc | Title | Source(s) | Agenda Item(s) | Status |
| --- | --- | --- | --- | --- |
| S4-221463 | Inclusive language review – Update to fixed-point basic operators | Ericsson LM | 7.3 | Agreed |
| S4-221514 | CR 26131-0087 rev1 Corrections for idle noise and sidetone requirements (electrical interface UE) | HEAD acoustics GmbH, Orange | 13 | Agreed (without presentation) |
| S4-221515 | CR26447-0018 rev1 Corrections to the EVS algorithmic description | Fraunhofer IIS | 13 | Agreed |
| S4-221516 | LS on Wind Noise Generation for ATIAS (To: ETSI TC STQ) | 3GPP SA4 | 14.1 | Agreed |
| S4-221517 | ATIAS-1: Permanent Document on ATIAS, v0.1 | Editor (Dolby) | 14.1 | Agreed |
| S4-221518 | CR 26260-0004 Corrections to TS 26.260 Clause 4 (Rel-18) | Qualcomm Incorporated | 14.1 | Agreed (Revision of S4-221426) |
| S4-221519 | Draft time plan for ATIAS, v0.6 | ATIAS Co-Rapporteurs (Orange, Dolby) | 14.1 | Agreed |
| S4-221520 | Draft time plan for eUET, v0.2 | eUET Co-Rapporteurs (Orange, HEAD acoustics) | 14.6 | Agreed |
| S4-221521 | Rev of 1464 (IVAS-7a v.0.7.0) | Editor | 14.2 | Agreed |
| S4-221522 | CR26131-0086 rev1 Performance requirements of vehicle-mounted hands-free UE (Rel-18) | HEAD acoustics GmbH | 14.6 | Agreed (Revision of S4-221270) |
| S4-221523 | CR 26132-0107 rev1 Vehicle-mounted hands-free UE (Rel-18) | HEAD acoustics GmbH | 14.6 | Agreed (Revision of S4-221269) |
| S4-221524 | Rev of 1331 (IVAS-6 v.0.4.0) | Editor | 14.2 | Agreed |
| S4-221525 | IVAS-3 v.0.3.0 | Editor | 14.2 | Agreed |
| S4-221526 | IVAS-4 v.0.7.0 | Editor | 14.2 | Agreed |
| S4-221528 | IVAS-9 v.0.3.0 | Editor | 14.2 | agreed |
| S4-221530 | Rev of 1409 (SID) | Beijing Xiaomi Mobile Software Co., Ltd, Huawei Technologies Co Ltd, Beijing Bytedance Technology Co., Ltd, ZTE Corporation, BUPT, Qualcomm Incorporation, Nokia Corporation | 16 | agreed |
| S4-221531 | CR 26131-0085 rev1 Changing RLR requirement at minimum volume in handset mode (Rel-18) | Samsung Electronics, T-Mobile US | 14.11 | Agreed (Revision of S4-221263) |

C.2 Agreed documents (not to be presented to SA4 plenary)

| Tdoc | Title | Source(s) | Agenda Item(s) | Status |
| --- | --- | --- | --- | --- |
| S4-221297 | IVAS-4 Design Constraints v0.6.1 | HUAWEI TECH. GmbH | 7.5 | Agreed |
| S4-221336 | Draft Audio SWG Agenda | QUALCOMM JAPAN LLC. | 7 | Approved |
| S4-221413 | On IVAS performance requirements for MASA format inputs | Nokia Corporation | 7.5 | Agreed |

C.3 Other status than agreed documents (not to be presented to SA4 plenary)

| Tdoc | Title | Source(s) | Agenda Item(s) | Status |
| --- | --- | --- | --- | --- |
| S4-221263 | CR on changing RLR requirement at minimum volume in handset mode | Samsung R&D Institute India | 7.10 | Revised |
| S4-221269 | CR on vehicle-mounted hands-free UE | HEAD acoustics GmbH | 7.7 | Revised |
| S4-221270 | CR on performance requirements of vehicle-mounted hands-free UE | HEAD acoustics GmbH | 7.7 | Revised |
| S4-221271 | Corrections for idle noise and sidetone requirements (electrical interface UE) | HEAD acoustics GmbH, Orange | 7.3 | Revised |
| S4-221331 | IVAS-6: Selection Deliverables, Suggested version 0.3.1 | Dolby Sweden AB | 7.5 | Revised |
| S4-221335 | Further consolidation of IVAS Design Constraints (IVAS-4) | Dolby Sweden AB, Fraunhofer IIS | 7.5 | Noted |
| S4-221339 | Draft IVAS-2 v.0.4.0 update | QUALCOMM JAPAN LLC. | 7.5 | Revised |
| S4-221352 | Thoughts on subjective listening tests for noisy speech | NTT | 7.5 | Noted |
| S4-221353 | Proposal of wind noise test in ATIAS | Beijing Xiaomi Mobile Software | 7.6 | Noted |
| S4-221398 | IVAS-4 updates on Reverb Generator | Philips International B.V., Qualcomm Incorporated | 7.5 | Noted |
| S4-221399 | Proposed Multichannel Performance Requirements | Fraunhofer IIS | 7.5 | Noted |
| S4-221409 | Draft SID: Feasibility Study on Diverse audio Capturing system for End-user Devices | Xiaomi,Huawei,Bytedance Technology,ZTE Corporation,BUPT | 7.9 | Revised |
| S4-221410 | Update Immersive and focused remote class participation usage scenario in IVAS Usage Scenarios | Xiaomi Technology | 7.5 | Noted |
| S4-221411 | Object-based audio source orientation rendering solution | Xiaomi Technology | 7.5 | Noted |
| S4-221412 | Clarifications for ATIAS FOA audio tests | Nokia Corporation | 7.6 | Noted |
| S4-221414 | On IVAS performance requirements for multi-channel inputs | Nokia Corporation | 7.5 | Noted |
| S4-221416 | Corrections to the EVS algorithmic description | Fraunhofer IIS | 7.3 | Revised |
| S4-221417 | On IVAS codec complexity and memory design constraints | Fraunhofer IIS | 7.5 | Noted |
| S4-221419 | SWB Measurement results for eUET | HEAD acoustics GmbH | 7.7 | Noted |
| S4-221422 | Clarifications addressing send side audio performance assessment for Immersive Audio Systems | Qualcomm Incorporated | 7.6 | Noted |
| S4-221426 | Corrections to TS 26.260 Clause 4 | Qualcomm Incorporated | 7.6 | Revised |
| S4-221437 | Inclusive language review – Update to fixed-point basic operators | Ericsson LM | 7.3 | Withdrawn |
| S4-221438 | Proposed revision of IVAS-7a | Ericsson LM | 7.5 | Withdrawn |
| S4-221442 | On IVAS codec complexity and memory design constraints | Dolby Sweden AB | 7.5 | Noted |
| S4-221443 | Proposal for IVAS Performance Requirements for Scene-based Audio | Dolby Sweden AB | 7.5 | Noted |
| S4-221445 | On profiles for JBM behaviour evaluation | Orange | 7.7 | Noted |
| S4-221446 | Corrections and additions on audio aspects | Orange, Ericsson LM | 7.8 | Revised |
| S4-221447 | On send side audio performance assessment for Immersive Audio Systems – review of TS 26.260 | Dolby Sweden AB | 7.6 | Noted |
| S4-221449 | On send side audio performance assessment for Immersive Audio Systems – additional metrics | Dolby Sweden AB | 7.6 | Noted |
| S4-221464 | Proposed revision of IVAS-7a | Ericsson LM | 7.5 | Revised |

C.4 Other status than agreed documents (to be presented to SA4 plenary)

| Tdoc | Title | Source(s) | Agenda Item(s) | Status |
| --- | --- | --- | --- | --- |
| S4-221527 | Rev of 1339 (IVAS-2 v.0.5.0) | Editor | 14.2 | Transferred to plenary |
| S4-221529 | Rev of 1446 (CR 26.998) | Orange, Ericsson LM | 15.8 | Transferred to plenary |

Imre Varga, Email: ivarga@qti.qualcomm.com; Stephane Ragot, Email: stephane.ragot@orange.com