3GPP TSG-SA4 Meeting #122	S4-230112

# Athens, Greece, 20th Feb 2023 - 24th Feb 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.511 | CR | 0008 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Upgrade of audio profiles for 5G-Advanced |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby Laboratories Inc., Fraunhofer IIS, VoiceAge Corporation, Samsung Electronics Co., Ltd, Philips International B.V., ETRI, Qualcomm incorporated, Panasonic Holdings Corporation, Nokia Corporation, Orange, Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS_Audio_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To guarantee interoperability 3GPP specifications should conditionally mandate the support of at least one Audio codec operation points when the relevant profiles are supported: i.e. eAAC+ and the newly defined xHE-AAC stereo operation point if streaming default and Television (TV) profiles are supported and 3GPP MPEG-H Audio if 360 Virtual Reality (VR) profiles are supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add conditionally mandatory support for eAAC+, 3GPP MPEG-H Audio and the newly defined xHE-AAC stereo Audio operation point to guarantee interoperability when the appropriate profiles are supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3; 5.2.7.4; 5.2.8.3; 5.3.3; 5.3.5.4; 5.4.2; 5.4.4.3; 5.5.2.1; 5.6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: re-submission as basis for discussion to fullfill objective 2 of the Work Item Description: 2) to review and, when appropriate, possibly modify requirements (i.e. may, should or shall support) in TS 26.511 applicable to 5GMS Clients for the support of Audio operation Points. Corrected some cross-references |  |  |  |  |  |  |  |  |

====== 1st CHANGE ======

### 5.2.3	Audio decoding

If the 5GMSd Client supports the reception of audio:

- 	xHE-AAC stereo decoding capability shall be supported as defined in 3GPP TS 26.117 [4] clause 5.2.

NOTE:	xHE-AAC® is a registered trademark of Fraunhofer in Germany and other countries and is used with Fraunhofer’s permission.

-	eAAC+ decoding capability shall be supported as defined in 3GPP TS 26.117 [4] clause 5.2.

-	AMR-WB+ decoding capability may be supported as defined in 3GPP TS 26.117 [4] clause 5.2.

====== 2nd CHANGE ======

#### 5.2.7.4	Audio media profiles

If the 5GMSd Client supports the reception of audio, then the following shall be supported:

-	the xHE-AAC stereo playback requirements as defined in 3GPP TS 26.117 [4] clause 7.8.4.

If the 5GMSd Client supports the reception of audio, then the following shall be supported:

-	the eAAC+ stereo playback requirements as defined in 3GPP TS 26.117 [4] clause 7.5.2.4.

If the 5GMSd Client supports the reception of audio, then the following may be supported:

-	the AMR-WB+ playback requirements as defined in 3GPP TS 26.117 [4] clause 7.6.2.4.

====== 3rd CHANGE ======

#### 5.2.8.3	Audio media profiles

For AMR:

-	the <profiles> parameter is defined in TS 26.117, clause 7.2.2.4 as audio/mp4 profiles='camr'

-	the <codecss> parameter is defined in TS 26.117, clause 7.2.2.4 as 'samr'

For AMR-WB:

-	the <profiles> parameter is defined in TS 26.117, clause 7.3.2.4 as audio/mp4 profiles='camw'

-	the <codecs> parameter is defined in TS 26.117, clause 7.3.2.4 as 'samw'

For EVS:

-	the <profiles> parameter is defined in TS 26.117, clause 7.4.2.4 as audio/mp4 profiles='cevs'

-	the <codecs> parameter is defined in TS 26.117, clause 7.4.2.4 as 'sevs'

For xHE-AAC stereo:

-	the <profiles> parameter is defined in TS 26.117, clause 7.8 as audio/mp4 profiles='casu'

-	the <codecs> parameter is defined in TS 26.117, clause 7.8 as 'mp4a'

For eAAC+ stereo:

-	the <profiles> parameter is defined in TS 26.117, clause 7.6.2.4 as audio/mp4 profiles='ceac'

-	the <codecs> parameter is defined in TS 26.117, clause 7.6.2.4 as 'mp4a'

For AMR-WB+:

-	the <profiles> parameter is defined in TS:26.117, clause 7.7.2.4 as audio/mp4 profiles='camp'

-	the <codecs> parameter is defined in TS:26.117, clause 7.7.2.4 as 'sawp'

====== 4th CHANGE ======

### 5.3.3	Audio encoding

If the 5GMS UE supports the transmission of audio:

-	the sender requirements for the eAAC+ stereo Operation Point as defined in 3GPP TS 26.117 [4] clause 6.3.2.3 shall be supported.

- 	the sender requirements for the xHE-AAC stereo Operation Point as defined in 3GPP TS 26.117 [4] clause 6.3.4.3 shall be supported.

====== 5th CHANGE ======

#### 5.3.5.4	Audio media profile

If the 5GMSu Client supports the transmission of audio, then the following shall be supported:

-	the eAAC+ stereo content generation requirements as defined in 3GPP TS 26.117 [4] clause 7.6.2.5.

If the 5GMSu Client supports the transmission of audio, then the following shall be supported:

-	the xHE-AAC stereo content generation requirements as defined in 3GPP TS 26.117 [4] clause 7.8.5.

====== 6th CHANGE ======

### 5.4.2	Audio decoding

If the 5GMSd Client supports the Television (TV) profile, it shall support eAAC+ stereo Operation point as defined in 3GPP TS 26. 117 [4], clause 6.3.3.

If the 5GMSd Client supports the Television (TV) profile, it shall support xHE-AAC stereo Operation point as defined in 3GPP TS 26. 117 [4], clause 6.3.4.

====== 6th CHANGE ======

#### 5.4.4.3	Audio media profiles

If the 5GMSd Client supports the Television (TV) profile, then the following shall be supported:

-	the eAAC+ stereo playback requirements as defined in 3GPP TS 26.117 [4] clause 7.5.2.4.

If the 5GMSd Client supports the Television (TV) profile, then the following shall be supported:

-	the xHE-AAC stereo playback requirements as defined in 3GPP TS 26.117 [4] clause 7.8.4.

====== 7th CHANGE ======

#### 5.5.2.1	Operation Points

If the 5GMS UE supports 3D/VR audio, it shall include a receiver that complies with:

-	the 3GPP MPEG-H Audio Operation Point Receiver requirements as specified in TS 26.118 [13], clause 6.1.4.

====== 8th CHANGE ======

#### 5.6.2.1	Operation Points

If the 5GMS UE supports 3D/VR audio, it shall include an encoder that can encode a bitstream that can be decoded by a receiver that complies with:

-	the 3GPP MPEG-H Audio Operation Point Receiver requirements as specified in TS 26.118 [13], clause 6.1.4.

====== END OF CHANGES ======