Agenda item: 	9.5

Source: 	Qualcomm Inc.

Title: 	JSON format for Timed Metadata

Document for	Discussion and Agreement

# Introduction

Split rendering requires the transmission of different types of data from the glasses to the remote renderer. This information consists mainly of pose predictions and user actions. Other information may include: hand tracking, eye gaze interaction, keyboard tracking, etc.

Our goal is to align with the OpenXR APIs, so we propose the formats for the core set of metadata as specified by the OpenXR APIs.

# Transport Channel

The timed metadata for split rendering sessions shall be transported over a WebRTC datachannel to the split rendering server.

The PPID of the SCTP user message shall be set to 51 (WebRTC String) to indicate UTF-8 formatted messages. The messages shall be JSON formatted and shall follow the following syntax and semantics.

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| version | number | 1..1 | The version of the datachannel sub-protocol “3gpp-sr”. This version of the specification shall be set to 1. |
| type | string | 1..1 | A unique identifier of the type of data that this message contains. The supported types are defined in clause 3. |
| timestamp | string | 1..1 | The NTP time of the generation of this message. |
| content | Object | 1..1 | A JSON object that carries the content of the message. The exact format is defined by the type of the message |

During the negotiation of the WebRTC data channel, the endpoints shall use the following subprotocol identifier during the data channel setup negotiation: “3gpp-sr”.

# Formats for Core Metadata

# General

In the “3gpp-sr” data channel sub-protocol, the message content depends on the type of the message. Message types shall be unique identifiers in the URN format. This clause defines a set of message types and their formats. The messages are derived from the OpenXR API to ensure smooth operation with AR devices that support OpenXR. In case other XR APIs are used, mapping the message payload to the appropriate XR API structures shall be performed by the split rendering client.

# Pose Prediction Format

The split rendering client on the XR device periodically transmits a set of pose predictions to the split rendering server. The type of the message shall be set to “urn:3gpp:split-rendering:v1:pose”.

Each predicted pose shall contain the associated predicted display time and an identifier of the XR space that was used for that pose.

Depending on the view configuration of the XR session, there could be different pose information for each view.

The payload of the message shall be as follows:

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| poseInfo | Object | 1..n | An array of pose information objects, each corresponding to a target display time and XR space. |
| displayTime | number | 1..1 | The time for which the current view poses are predicted. |
| xrSpace | number | 0..1 | An identifier for the XR space in which the view poses are expressed. The set of XR spaces are agreed on between the split rendering client and the split rendering server at the setup of the split rendering session. |
| viewPoses | Object | 0..n | An array that provides a list of the poses associated with every view. The number of views is determined during the split rendering session setup between the split rendering client and server, depending on the view configuration of the XR session. |
| pose | Object | 1..1 | An object that carries the pose information for a particular view. |
| orientation | Object | 1..1 | Represents the orientation of the view pose as a quaternion based on the reference XR space. |
| x | number | 1..1 | Provides the x coordinate of the quaternion. |
| y | number | 1..1 | Provides the y coordinate of the quaternion. |
| z | number | 1..1 | Provides the z coordinate of the quaternion. |
| w | number | 1..1 | Provides the w coordinate of the quaternion. |
| position | Object | 1..1 | Represents the location in 3D space of the pose based on the reference XR space. |
| x | number | 1..1 | Provides the x coordinate of the position vector. |
| y | number | 1..1 | Provides the y coordinate of the position vector. |
| z | number | 1..1 | Provides the z coordinate of the position vector. |
| fov | Object | 1..1 | Indicates the four sides of the field of view used for the projection of the corresponding XR view. |
| angleLeft | number | 1..1 | The angle of the left side of the field of view. For a symmetric field of view this value is negative. |
| angleRight | number | 1..1 | The angle of the right side of the field of view. |
| angleUp | number | 1..1 | The angle of the top part of the field of view. |
| angleDown | number | 1..1 | The angle of the bottom part of the field of view. For a symmetric field of view this value is negative. |

# Action Format

Actions are grouped into action sets, which may be activated and deactivated during the lifetime of an XR session. The action sets and actions are negotiated at the start of the split rendering session.

The split rendering client reports any changes to action state as soon as it occurs by sending a message of the type “urn:3gpp:split-rendering:v1:action”.

The content of the action message type shall follow the following format:

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| actionSets | Object | 1..n | An array of active action sets, for which there is at least an action that has a state change. |
| actions | Object | 1..n | An array of objects that conveys information about the actions of the parent action set. |
| identifier | string | 1..1 | A unique identifier of the action that was agreed upon during split rendering session setup. |
| subactionPath | string | 1..1 | The sub-action path for which the state has changed. It abstracts a binding between an action and the hardware input associated to it by the XR runtime. |
| state | object | 1..1 | The state of the action that had a change in state. |
| lastChangeTime | number | 1..1 | The timestamp of the last change to the state of this action. |
| currentStateBool | Bool | 0..1 | The current Boolean state of the action |
| currentStateNum | number | 0..1 | The current numerical state of the action. |
| currentStateVec2 | Array | 0..1 | An array of numerical state values for the action. |

# JSON Schema

The JSON schema for the message and the defined message types is provided in the following table:

{ "$schema" : "http://json-schema.org/draft-07/schema", "title" : "Split Rendering Metadata Format", "type" : "object", "description": "Defines the split rendering timed metadata message format.", "properties" : { "version": { "type": "number", "default": 1 }, "type": { "oneOf": [ {"type": "string",}, {"enum": ["sr-pose", "sr-action"]} ] }, "timestamp": { "type": "string" }, "content": { "oneOf": [ {"$ref": "#/$message/poseInfo"}, {"$ref": "#/$message/actionSet"}, {"type": "object"} ] }, "$message": { "poseInfo": { "type": "object", "properties": { "displayTime": { "type": "number" }, "xrSpace": { "type": "number" }, "viewPoses": { "type": "array", "items": { "$ref": "#/$components/viewPose" } } } }, "actionSets": { "type": "object", "properties": { "type": "array", "items": { "$ref": "#/$components/action" } } } }, "$components": { "ViewPose": { "type": "object", "properties": { "pose": { "type": "#/$components/pose" }, "fov": { "type": "#/$components/fov" } } }, "pose": { "type": "object", "properties": { "orientation": { "type": "#/$components/position" }, "position": { "type": "#/$components/orientation" } } }, "position": { "type": "object", "properties": { "x": { "type": "number" }, "y": { "type": "number" }, "z": { "type": "number" } } }, "orientation": { "type": "object", "properties": { "x": { "type": "number" }, "y": { "type": "number" }, "z": { "type": "number" }, "w": { "type": "number" } } }, "fov": { "type": "object", "properties": { "angleLeft": { "type": "number" }, "angleRight": { "type": "number" }, "angleUp": { "type": "number" }, "angleDown": { "type": "number" } } }, "action": { "type": "object", "properties": { "identifier": { "type": "string" }, "subactionPath": { "type": "string" }, "state": { "$ref": "#/$components/state" } } }, "state": { "type": "object", "properties": { "lastChange": "number", "oneOf": [ { "currentStateBool": { "type": "boolean" } }, { "currentStateNum": { "type": "number" } }, { "currentStateVec2": { "type": "object", "properties": { "v1": {"type": "number"}, "v2": {"type": "number"} } } } ] } } } }, "required": [ "version", "type", "timestamp" ] }

# Proposal

We propose to agree the content of section 2 and 3 into TS26.565.