Agenda item: 	9.11

Source: 	InterDigital Communications

Title: 	V3C Adaptive Streaming Demo on AR Glasses

Document for:	Discussion

# Introduction

While traditional multimedia applications such as videos are still popular, there has been a substantial interest towards new media, such as VR and immersive 3D graphics. Advances in technologies for capturing and rendering 3D points have realized novel applications in the areas of tele-presence, augmented reality, and large-scale dynamic 3D maps. The 3D Graphics working group ISO/IEC JTC1/SC29/WG7 of MPEG has developed two visual volumetric media standards: a video-based compression standard for dynamic point clouds and other volumetric videos [1], and a geometry-based compression standard for static point clouds [2]. In addition, the MPEG Systems working group ISO/IEC JTC1/SC29/WG3 has developed standards to support efficient and interoperable storage and transport of compressed visual volumetric media content with 6 degrees of freedom [3] [4].

This contribution provides an overview of an end-to-end implementation of a visual volumetric video-based coding (V3C) streaming system running on Nreal Light AR glasses that is based on the MPEG standards mentioned above along with the overall architecture of the end-to-end system and the streaming client architecture.

# Description

The V3C adaptive streaming system includes a V3C encoder, ISOBMFF MP4 encapsulator, a DASH server, a DASH client supporting streaming of V3C media with GPU accelerated real-time HEVC video decoding, reconstruction, and viewport-based rendering of the point cloud object to NReal Light AR glasses as well as conventional displays. Figure 1 shows the architecture of the V3C adaptive video streaming demo.

The captured raw point cloud content is encoded using a V3C encoder and the resulting coded V3C bitstreams is encapsulated into an ISOBMFF container file using an MP4 file multiplexer. The MP4 file multiplexer encapsulates the coded V3C content into multiple tracks based on ISO/IEC 23090-10 specification, where the atlas data of the V3C bitstream is stored into an atlas track and the video-coded data for the geometry, occupancy, and one or more attribute components are stored into separate video tracks in the file. A DASH segmenter, splits the MP4 file into multiple MP4 media segments and generates a media presentation descriptor (MPD) file. The generated MPD along with the atlas media segments and video components media segments are stored on a DASH server.

Figure 1. V3C adaptive video streaming system architecture

The stored MP4 media segments are delivered to the DASH client using HTTP requests. Figure 2 shows the V3C adaptive video streaming client architecture. The MPEG-DASH client downloads the MPD file from the server and parses it to identify the various V3C components. The V3C de-multiplexer extracts the V3C content from the downloaded DASH segments and provides the extracted V3C units to a V3C decoder as an input. Using the decoded atlas data and decoded video component data, 3D reconstruction is performed by doing inverse projection of the 2D frame representation to a 3D representation of the volumetric video. The reconstructed volumetric video frame is then rendered on to the Nreal Light glasses in real-time. GPU capabilities of the device are leveraged to perform real-time 3D reconstruction and rendering of the point cloud content.

Figure 2. V3C adaptive video streaming client architecture

# Proposal

The contribution is provided for information to 3GPP SA4 to demonstrate adaptive streaming and playback of V3C content on AR glasses.

# References

- ISO/IEC 23090-5 Information Technology — Coded Representation of Immersive media — Part 5: Visual Volumetric Video-Based Coding (V3C) and Video-Based point Cloud Compression (V-PCC).

- ISO/IEC 23090-9 Information Technology — Coded Representation of Immersive media — Part 9: Geometry-based point Cloud Compression (G-PCC).

- ISO/IEC 23090-10 Information Technology — Coded Representation of Immersive media — Part 10: Carriage of Visual Volumetric Video-Based Coding Data.

- ISO/IEC 23090-18 Information Technology — Coded Representation of Immersive media — Part 18: Carriage of Geometry-based Point Cloud Compression Data.

- ISO/IEC 14496-12, Information technology — Coding of audio-visual objects — Part 12: ISO base media file format